From Wikipedia, the free encyclopedia

Jump to: navigation, search

"Computer networks" redirects here. For the periodical, see Computer Networks (journal).

A computer network, often simply referred to as a network, is a group of computers and devices interconnected by communications channels that facilitate communications among users and allows users to share resources. Networks may be classified according to a wide variety of characteristics.

- Contents [hide] - 1 Introduction - 1.1 Purpose - 2 Network classification - 2.1 Connection method - 2.2 Wired technologies - 2.3 Wireless technologies - 2.4 Scale - 2.5 Functional relationship (network architecture) - 2.6 Network topology - 3 Types of networks based on physical scope - 3.1 Local area network - 3.1.1 Personal area network - 3.1.2 Home area network - 3.2 Wide area network - 3.2.1 Campus network - 3.3 Metropolitan area network - 3.3.1 Enterprise private network - 3.3.2 Virtual private network - 3.3.3 Internetwork - 3.4 Backbone network - 3.4.1 Global area network - 3.4.2 Internet - 3.4.3 Intranets and extranets - 3.4.4 Overlay network - 4 Basic hardware components - 4.1 Network interface cards - 4.2 Repeaters - 4.3 Hubs - 4.4 Bridges - 4.5 Switches - 4.6 Routers - 5 See also - 6 References - 7 Further reading

- [edit] Introduction

A computer network allows sharing of resources and information among interconnected devices. In the 1960s, the Advanced Research Projects Agency (ARPA) started funding the design of the Advanced Research Projects Agency Network (ARPANET) for the United States Department of Defense. It was the first computer network in the world.[1] Development of the network began in 1969, based on designs developed during the 1960s.

- [edit] Purpose

Computer networks can be used for several purposes:

- Facilitating communications. Using a network, people can communicate efficiently and easily via email, instant messaging, chat rooms, telephone, video telephone calls, and video conferencing.

- Sharing hardware. In a networked environment, each computer on a network may access and use hardware resources on the network, such as printing a document on a shared network printer.

- Sharing files, data, and information. In a network environment, authorized user may access data and information stored on other computers on the network. The capability of providing access to data and information on shared storage devices is an important feature of many networks.

- Sharing software. Users connected to a network may run application programs on remote computers.

- Information preservation.

- Security.

- Speed up.

- [edit] Network classification

The following list presents categories used for classifying networks.

- [edit] Connection method

Computer networks can be classified according to the hardware and software technology that is used to interconnect the individual devices in the network, such as optical fiber, Ethernet, wireless LAN, HomePNA, power line communication or G.hn.

Ethernet uses physical wiring to connect devices. Frequently deployed devices include hubs, switches, bridges, or routers. Wireless LAN technology is designed to connect devices without wiring. These devices use radio waves or infrared signals as a transmission medium. ITU-T G.hn technology uses existing home wiring (coaxial cable, phone lines and power lines) to create a high-speed (up to 1 Gigabit/s) local area network.

- [edit] Wired technologies

- Twisted pair wire is the most widely used medium for telecommunication. Twisted-pair wires are ordinary telephone wires which consist of two insulated copper wires twisted into pairs and are used for both voice and data transmission. The use of two wires twisted together helps to reduce crosstalk and electromagnetic induction. The transmission speed ranges from 2 million bits per second to 100 million bits per second.

- Coaxial cable is widely used for cable television systems, office buildings, and other worksites for local area networks. The cables consist of copper or aluminum wire wrapped with insulating layer typically of a flexible material with a high dielectric constant, all of which are surrounded by a conductive layer. The layers of insulation help minimize interference and distortion. Transmission speed range from 200 million to more than 500 million bits per second.

- Optical fiber cable consists of one or more filaments of glass fiber wrapped in protective layers. It transmits light which can travel over extended distances. Fiber-optic cables are not affected by electromagnetic radiation. Transmission speed may reach trillions of bits per second. The transmission speed of fiber optics is hundreds of times faster than for coaxial cables and thousands of times faster than a twisted-pair wire.[citation needed]

- [edit] Wireless technologies

- Terrestrial microwave – Terrestrial microwaves use Earth-based transmitter and receiver. The equipment look similar to satellite dishes. Terrestrial microwaves use low-gigahertz range, which limits all communications to line-of-sight. Path between relay stations spaced approx, 30 miles apart. Microwave antennas are usually placed on top of buildings, towers, hills, and mountain peaks.

- Communications satellites – The satellites use microwave radio as their telecommunications medium which are not deflected by the Earth's atmosphere. The satellites are stationed in space, typically 22,000 miles (for geosynchronous satellites) above the equator. These Earth-orbiting systems are capable of receiving and relaying voice, data, and TV signals.

- Cellular and PCS systems – Use several radio communications technologies. The systems are divided to different geographic areas. Each area has a low-power transmitter or radio relay antenna device to relay calls from one area to the next area.

- Wireless LANs – Wireless local area network use a high-frequency radio technology similar to digital cellular and a low-frequency radio technology. Wireless LANs use spread spectrum technology to enable communication between multiple devices in a limited area. An example of open-standards wireless radio-wave technology is IEEE.

- Infrared communication , which can transmit signals between devices within small distances not more than 10 meters peer to peer or ( face to face ) without any body in the line of transmitting.

- [edit] Scale

Networks are often classified as local area network (LAN), wide area network (WAN), metropolitan area network (MAN), personal area network (PAN), virtual private network (VPN), campus area network (CAN), storage area network (SAN), and others, depending on their scale, scope and purpose, e.g., controller area network (CAN) usage, trust level, and access right often differ between these types of networks. LANs tend to be designed for internal use by an organization's internal systems and employees in individual physical locations, such as a building, while WANs may connect physically separate parts of an organization and may include connections to third parties.

- [edit] Functional relationship (network architecture)

Computer networks may be classified according to the functional relationships which exist among the elements of the network, e.g., active networking, client–server and peer-to-peer (workgroup) architecture.

- [edit] Network topology

Main article: Network topology

Computer networks may be classified according to the network topology upon which the network is based, such as bus network, star network, ring network, mesh network. Network topology is the coordination by which devices in the network are arranged in their logical relations to one another, independent of physical arrangement. Even if networked computers are physically placed in a linear arrangement and are connected to a hub, the network has a star topology, rather than a bus topology. In this regard the visual and operational characteristics of a network are distinct. Networks may be classified based on the method of data used to convey the data, these include digital and analog networks.

- [edit] Types of networks based on physical scope

Common types of computer networks may be identified by their scale.

- [edit] Local area network

A local area network (LAN) is a network that connects computers and devices in a limited geographical area such as home, school, computer laboratory, office building, or closely positioned group of buildings. Each computer or device on the network is a node. Current wired LANs are most likely to be based on Ethernet technology, although new standards like ITU-T G.hn also provide a way to create a wired LAN using existing home wires (coaxial cables, phone lines and power lines).[2]

Typical library network, in a branching tree topology and controlled access to resources

All interconnected devices must understand the network layer (layer 3), because they are handling multiple subnets (the different colors). Those inside the library, which have only 10/100 Mbit/s Ethernet connections to the user device and a Gigabit Ethernet connection to the central router, could be called "layer 3 switches" because they only have Ethernet interfaces and must understand IP. It would be more correct to call them access routers, where the router at the top is a distribution router that connects to the Internet and academic networks' customer access routers.

The defining characteristics of LANs, in contrast to WANs (Wide Area Networks), include their higher data transfer rates, smaller geographic range, and no need for leased telecommunication lines. Current Ethernet or other IEEE 802.3 LAN technologies operate at speeds up to 10 Gbit/s. This is the data transfer rate. IEEE has projects investigating the standardization of 40 and 100 Gbit/s.[3]

- [edit] Personal area network

A personal area network (PAN) is a computer network used for communication among computer and different information technological devices close to one person. Some examples of devices that are used in a PAN are personal computers, printers, fax machines, telephones, PDAs, scanners, and even video game consoles. A PAN may include wired and wireless devices. The reach of a PAN typically extends to 10 meters.[4] A wired PAN is usually constructed with USB and Firewire connections while technologies such as Bluetooth and infrared communication typically form a wireless PAN.

- [edit] Home area network

A home area network (HAN) is a residential LAN which is used for communication between digital devices typically deployed in the home, usually a small number of personal computers and accessories, such as printers and mobile computing devices. An important function is the sharing of Internet access, often a broadband service through a CATV or Digital Subscriber Line (DSL) provider. It can also be referred as an office area network (OAN).

- [edit] Wide area network

A wide area network (WAN) is a computer network that covers a large geographic area such as a city, country, or spans even intercontinental distances, using a communications channel that combines many types of media such as telephone lines, cables, and air waves. A WAN often uses transmission facilities provided by common carriers, such as telephone companies. WAN technologies generally function at the lower three layers of the OSI reference model: the physical layer, the data link layer, and the network layer.

- [edit] Campus network

A campus network is a computer network made up of an interconnection of local area networks (LAN's) within a limited geographical area. The networking equipments (switches, routers) and transmission media (optical fiber, copper plant, Cat5 cabling etc.) are almost entirely owned (by the campus tenant / owner: an enterprise, university, government etc.).

In the case of a university campus-based campus network, the network is likely to link a variety of campus buildings including; academic departments, the university library and student residence halls.

- [edit] Metropolitan area network

A Metropolitan area network is a large computer network that usually spans a city or a large campus.

Sample EPN made of Frame relay WAN connections and dialup remote access.

Sample VPN used to interconnect 3 offices and remote users

- [edit] Enterprise private network

An enterprise private network is a network build by an enterprise to interconnect various company sites, e.g., production sites, head offices, remote offices, shops, in order to share computer resources.

- [edit] Virtual private network

A virtual private network (VPN) is a computer network in which some of the links between nodes are carried by open connections or virtual circuits in some larger network (e.g., the Internet) instead of by physical wires. The data link layer protocols of the virtual network are said to be tunneled through the larger network when this is the case. One common application is secure communications through the public Internet, but a VPN need not have explicit security features, such as authentication or content encryption. VPNs, for example, can be used to separate the traffic of different user communities over an underlying network with strong security features.

VPN may have best-effort performance, or may have a defined service level agreement (SLA) between the VPN customer and the VPN service provider. Generally, a VPN has a topology more complex than point-to-point.

- [edit] Internetwork

An internetwork is the connection of two or more private computer networks via a common routing technology (OSI Layer 3) using routers. The Internet is an aggregation of many internetworks, hence its name was shortened to Internet.

- [edit] Backbone network

- [edit] Global area network

A global area network (GAN) is a network used for supporting mobile communications across an arbitrary number of wireless LANs, satellite coverage areas, etc. The key challenge in mobile communications is handing off the user communications from one local coverage area to the next. In IEEE Project 802, this involves a succession of terrestrial wireless LANs.[5]

- [edit] Internet

The Internet is a global system of interconnected governmental, academic, corporate, public, and private computer networks. It is based on the networking technologies of the Internet Protocol Suite. It is the successor of the Advanced Research Projects Agency Network (ARPANET) developed by DARPA of the United States Department of Defense. The Internet is also the communications backbone underlying the World Wide Web (WWW).

Participants in the Internet use a diverse array of methods of several hundred documented, and often standardized, protocols compatible with the Internet Protocol Suite and an addressing system (IP addresses) administered by the Internet Assigned Numbers Authority and address registries. Service providers and large enterprises exchange information about the reachability of their address spaces through the Border Gateway Protocol (BGP), forming a redundant worldwide mesh of transmission paths.

- [edit] Intranets and extranets

Intranets and extranets are parts or extensions of a computer network, usually a local area network.

An intranet is a set of networks, using the Internet Protocol and IP-based tools such as web browsers and file transfer applications, that is under the control of a single administrative entity. That administrative entity closes the intranet to all but specific, authorized users. Most commonly, an intranet is the internal network of an organization. A large intranet will typically have at least one web server to provide users with organizational information.

An extranet is a network that is limited in scope to a single organization or entity and also has limited connections to the networks of one or more other usually, but not necessarily, trusted organizations or entities—a company's customers may be given access to some part of its intranet—while at the same time the customers may not be considered trusted from a security standpoint. Technically, an extranet may also be categorized as a CAN, MAN, WAN, or other type of network, although an extranet cannot consist of a single LAN; it must have at least one connection with an external network.

- [edit] Overlay network

An overlay network is a virtual computer network that is built on top of another network. Nodes in the overlay are connected by virtual or logical links, each of which corresponds to a path, perhaps through many physical links, in the underlying network.

A sample overlay network: IP over SONET over Optical

For example, many peer-to-peer networks are overlay networks because they are organized as nodes of a virtual system of links run on top of the Internet. The Internet was initially built as an overlay on the telephone network .[6]

Overlay networks have been around since the invention of networking when computer systems were connected over telephone lines using modem, before any data network existed.

Nowadays the Internet is the basis for many overlaid networks that can be constructed to permit routing of messages to destinations not specified by an IP address. For example, distributed hash tables can be used to route messages to a node having a specific logical address, whose IP address is not known in advance.

Overlay networks have also been proposed as a way to improve Internet routing, such as through quality of service guarantees to achieve higher-quality streaming media. Previous proposals such as IntServ, DiffServ, and IP Multicast have not seen wide acceptance largely because they require modification of all routers in the network.[citation needed] On the other hand, an overlay network can be incrementally deployed on end-hosts running the overlay protocol software, without cooperation from Internet service providers. The overlay has no control over how packets are routed in the underlying network between two overlay nodes, but it can control, for example, the sequence of overlay nodes a message traverses before reaching its destination.

For example, Akamai Technologies manages an overlay network that provides reliable, efficient content delivery (a kind of multicast). Academic research includes End System Multicast and Overcast for multicast; RON (Resilient Overlay Network) for resilient routing; and OverQoS for quality of service guarantees, among others.

- [edit] Basic hardware components

All networks are made up of basic hardware building blocks to interconnect network nodes, such as Network Interface Cards (NICs), Bridges, Hubs, Switches, and Routers. In addition, some method of connecting these building blocks is required, usually in the form of galvanic cable (most commonly Category 5 cable). Less common are microwave links (as in IEEE 802.12) or optical cable ("optical fiber").

- [edit] Network interface cards

A network card, network adapter, or NIC (network interface card) is a piece of computer hardware designed to allow computers to communicate over a computer network. It provides physical access to a networking medium and often provides a low-level addressing system through the use of MAC addresses.

- [edit] Repeaters

A repeater is an electronic device that receives a signal, cleans it of unnecessary noise, regenerates it, and retransmits it at a higher power level, or to the other side of an obstruction, so that the signal can cover longer distances without degradation. In most twisted pair Ethernet configurations, repeaters are required for cable that runs longer than 100 meters. Repeaters work on the Physical Layer of the OSI model.

- [edit] Hubs

A network hub contains multiple ports. When a packet arrives at one port, it is copied unmodified to all ports of the hub for transmission. The destination address in the frame is not changed to a broadcast address.[7] It works on the Physical Layer of the OSI model..

- [edit] Bridges

A network bridge connects multiple network segments at the data link layer (layer 2) of the OSI model. Bridges broadcast to all ports except the port on which the broadcast was received. However, bridges do not promiscuously copy traffic to all ports, as hubs do, but learn which MAC addresses are reachable through specific ports. Once the bridge associates a port and an address, it will send traffic for that address to that port only.

Bridges learn the association of ports and addresses by examining the source address of frames that it sees on various ports. Once a frame arrives through a port, its source address is stored and the bridge assumes that MAC address is associated with that port. The first time that a previously unknown destination address is seen, the bridge will forward the frame to all ports other than the one on which the frame arrived.

Bridges come in three basic types:

- Local bridges: Directly connect local area networks (LANs)

- Remote bridges: Can be used to create a wide area network (WAN) link between LANs. Remote bridges, where the connecting link is slower than the end networks, largely have been replaced with routers.

- Wireless bridges: Can be used to join LANs or connect remote stations to LANs.

- [edit] Switches

A network switch is a device that forwards and filters OSI layer 2 datagrams (chunk of data communication) between ports (connected cables) based on the MAC addresses in the packets.[8] A switch is distinct from a hub in that it only forwards the frames to the ports involved in the communication rather than all ports connected. A switch breaks the collision domain but represents itself as a broadcast domain. Switches make forwarding decisions of frames on the basis of MAC addresses. A switch normally has numerous ports, facilitating a star topology for devices, and cascading additional switches.[9] Some switches are capable of routing based on Layer 3 addressing or additional logical levels; these are called multi-layer switches. The term switch is used loosely in marketing to encompass devices including routers and bridges, as well as devices that may distribute traffic on load or by application content (e.g., a Web URL identifier).

- [edit] Routers

A router is an internetworking device that forwards packets between networks by processing information found in the datagram or packet (Internet protocol information from Layer 3 of the OSI Model). In many situations, this information is processed in conjunction with the routing table (also known as forwarding table). Routers use routing tables to determine what interface to forward packets (this can include the "null" also known as the "black hole" interface because data can go into it, however, no further processing is done for said data).

- [edit] See also

|  | Wikimedia Commons has media related to: Computer network |
| --- | --- |

|  | Wikiversity has learning materials about Computer network |
| --- | --- |

- Access point

- Active networking

- Bluetooth Networks

- Client–server model

- Cloud-based networking

- Computer network diagram

- Computer networking

- Computer networking device

- Decentralized computing

- Expander graph

- History of the Internet

- Home network

- Institute of Electrical and Electronic Engineers

- International Organization for Standardization

- Internet

- International Telecommunications Union - Telecommunication Standards Sector

- Nanoscale network

- Network monitoring

- Network tomography

- Network topology

- Node (networking)

- Protocols

- Scale-free network

- Wireless network

- Network complexity

- Cloud computing

From Wikipedia, the free encyclopedia

Jump to: navigation, search

Not to be confused with Wireless Application Protocol.

See also: Wireless router

|  | This article does not cite any references or sources. Please help improve this article by adding citations to reliable sources. Unsourced material may be challenged and removed. (March 2010) (Find sources: Wireless access point – news, books, scholar) |
| --- | --- |

Industrial Wireless Access Point

In computer networking, a wireless access point (WAP) is a device that allows wired communication devices to connect to a wireless network using Wi-Fi, Bluetooth or related standards. The WAP usually connects to a router, and can relay data between the wireless devices (such as computers or printers) and wired devices on the network.

Industrial grade WAPs are rugged, with a metal cover and a DIN rail mount. During operations they can tolerate a wider temperature range, high humidity and exposure to water, dust, and oil. Wireless security includes: WPA-PSK, WPA2, IEEE 802.1x/RADIUS, WDS, WEP, TKIP, and CCMP (AES) encryption. Unlike home consumer models, industrial wireless access points can also be used as a bridge, router, or a client.

- Contents [hide] - 1 Introduction - 2 Common WAP Applications - 3 Wireless Access Point vs. Ad Hoc Network - 4 Limitations - 5 Security - 6 See also - 7 References - 8 External links

- [edit] Introduction

Linksys WAP54G 802.11g Wireless Access Point

embedded RouterBoard 112 with U.FL-RSMA pigtail and R52 mini PCI Wi-Fi card widely used by wireless Internet service providers (WISPs) across the world

Prior to wireless networks, setting up a computer network in a business, home, or school often required running many cables through walls and ceilings in order to deliver network access to all of the network-enabled devices in the building. With the advent of the Wireless Access Point, network users are now able to add devices that access the network with few or no cables. Today's WAPs are built to support a standard for sending and receiving data using radio frequencies rather than cabling. Those standards, and the frequencies they use are defined by the IEEE. Most WAPs use IEEE 802.11 standards.

- [edit] Common WAP Applications

A typical corporate use involves attaching several WAPs to a wired network and then providing wireless access to the office LAN. The wireless access points are managed by a WLAN Controller which handles automatic adjustments to RF power, channels, authentication, and security. Further, controllers can be combined to form a wireless mobility group to allow inter-controller roaming. The controllers can be part of a mobility domain to allow clients access throughout large or regional office locations. This saves the clients time and administrators overhead because it can automatically re-associate or re-authenticate.

A Hot Spot is a common public application of WAPs, where wireless clients can connect to the Internet without regard for the particular networks to which they have attached for the moment. The concept has become common in large cities, where a combination of coffeehouses, libraries, as well as privately owned open access points, allow clients to stay more or less continuously connected to the Internet, while moving around. A collection of connected Hot Spots can be referred to as a lily-pad network.

The majority of WAPs are used in Home wireless networks.[citation needed] Home networks generally have only one WAP to connect all the computers in a home. Most are wireless routers, meaning converged devices that include the WAP, a router, and, often, an ethernet switch. Many also include a broadband modem. In places where most homes have their own WAP within range of the neighbors' WAP, it's possible for technically savvy people to turn off their encryption and set up a wireless community network, creating an intra-city communication network without the need of wired networks.

A WAP may also act as the network's arbitrator, negotiating when each nearby client device can transmit. However, the vast majority of currently installed IEEE 802.11 networks do not implement this, using a distributed pseudo-random algorithm called CSMA/CA instead.

- [edit] Wireless Access Point vs. Ad Hoc Network

Some people confuse Wireless Access Points with Wireless Ad Hoc networks. An Ad Hoc network uses a connection between two or more devices without using a wireless access point: the devices communicate directly when in range. An Ad Hoc network is used in situations such as a quick data exchange or a multiplayer LAN game because setup is easy and does not require an access point. Due to its peer-to-peer layout, Ad Hoc connections are similar to Bluetooth ones and are generally not recommended for a permanent installation.

Internet access via Ad Hoc networks, using features like Windows' Internet Connection Sharing, may work well with a small number of devices that are close to each other, but Ad Hoc networks don't scale well. Internet traffic will converge to the nodes with direct internet connection, potentially congesting these nodes. For internet-enabled nodes, Access Points have a clear advantage, with the possibility of having multiple access points connected by a wired LAN.

- [edit] Limitations

One IEEE 802.11 WAP can typically communicate with 30 client systems located within a radius of 100 m.[citation needed] However, the actual range of communication can vary significantly, depending on such variables as indoor or outdoor placement, height above ground, nearby obstructions, other electronic devices that might actively interfere with the signal by broadcasting on the same frequency, type of antenna, the current weather, operating radio frequency, and the power output of devices. Network designers can extend the range of WAPs through the use of repeaters and reflectors, which can bounce or amplify radio signals that ordinarily would go un-received. In experimental conditions, wireless networking has operated over distances of several kilometers.[citation needed]

Most jurisdictions have only a limited number of frequencies legally available for use by wireless networks. Usually, adjacent WAPs will use different frequencies (Channels) to communicate with their clients in order to avoid interference between the two nearby systems. Wireless devices can "listen" for data traffic on other frequencies, and can rapidly switch from one frequency to another to achieve better reception. However, the limited number of frequencies becomes problematic in crowded downtown areas with tall buildings using multiple WAPs. In such an environment, signal overlap becomes an issue causing interference, which results in signal droppage and data errors.

Wireless networking lags behind wired networking in terms of increasing bandwidth and throughput. While (as of 2010) typical wireless devices for the consumer market can reach speeds of 300 Mbit/s (megabits per second) (IEEE 802.11n) or 54 Mbit/s (IEEE 802.11g), wired hardware of similar cost reaches 1000 Mbit/s (Gigabit Ethernet). One impediment to increasing the speed of wireless communications comes from Wi-Fi's use of a shared communications medium, so a WAP is only able to use somewhat less than half the actual over-the-air rate for data throughput. Thus a typical 54 MBit/s wireless connection actually carries TCP/IP data at 20 to 25 Mbit/s. Users of legacy wired networks expect faster speeds, and people using wireless connections keenly want to see the wireless networks catch up.

By 2008 draft 802.11n based access points and client devices have already taken a fair share of the market place but with inherent problems integrating products from different vendors.

- [edit] Security

Main article: Wireless LAN Security

Wireless access has special security considerations. Many wired networks base the security on physical access control, trusting all the users on the local network, but if wireless access points are connected to the network, anyone on the street or in the neighboring office could connect.

The most common solution is wireless traffic encryption. Modern access points come with built-in encryption. The first generation encryption scheme WEP proved easy to crack; the second and third generation schemes, WPA and WPA2, are considered secure if a strong enough password or passphrase is used.

Some WAPs support hotspot style authentication using RADIUS and other authentication servers.

- [edit] See also

- Hotspots - access points or wireless networks open to the public

- Wireless LAN - networks consisting of zero or more access points plus devices

- Wireless bridge - between networks sharing same Service Set Identifier (SSID) and radio channel

- Wi-Fi Array System of multiple APs

- LWAPP - Lightweight Access Point Protocol used to manage a large set of WAPs

- WarXing - searching for open networks

- Femtocell - a local-area base station using cellular network standards such as UMTS, rather than Wi-Fi

- WiMAX - wide-area wireless standard that has a few elements in common with Wi-Fi

From Wikipedia, the free encyclopedia

Jump to: navigation, search

Active networking is a communication pattern that allows packets flowing through a telecommunications network to dynamically modify the operation of the network.

- Contents [hide] - 1 How it works - 2 What does it offer? - 3 Fundamental Challenges - 4 Nanoscale Active Networks - 5 See also - 6 Sources - 7 Notes - 8 External links

- [edit] How it works

Active network architecture is composed of execution environments (similar to a unix shell that can execute active packets), a node operating system capable of supporting one or more execution environments. It also consists of active hardware, capable of routing or switching as well as executing code within active packets. This differs from the traditional network architecture which seeks robustness and stability by attempting to remove complexity and the ability to change its fundamental operation from underlying network components. Network processors are one means of implementing active networking concepts. Active networks have also been implemented as overlay networks.

- [edit] What does it offer?

Active networking allows the possibility of highly tailored and rapid "real-time" changes to the underlying network operation. This enables such ideas as sending code along with packets of information allowing the data to change its form (code) to match the channel characteristics. The smallest program that can generate a sequence of data can be found in the definition of Kolmogorov Complexity. The use of real-time genetic algorithms within the network to compose network services is also enabled by active networking.

- [edit] Fundamental Challenges

Active network research addresses the nature of how best to incorporate extremely dynamic capability within networks[1].

In order to do this, active network research must address the problem of optimally allocating computation versus communication within communication networks[2]. A similar problem related to the compression of code as a measure of complexity is addressed via algorithmic information theory.

- [edit] Nanoscale Active Networks

As the limit in reduction of transistor size is reached with current technology, active networking concepts are being explored as a more efficient means accomplishing computation and communication[3][4]. More on this can be found in nanoscale networking.

- [edit] See also

- Nanoscale networking

- Programmable networking

- Network processing

- [edit] Sources

- Towards an Active Network Architecture (1996), David L. Tennenhouse, et al., Computer Communication Review

- Active Networks and Active Network Management: A Proactive Management Framework by Stephen F. Bush and Amit Kulkarni, Kluwer Academic/Plenum Publishers, New York, Boston, Dordrecht, London, Moscow, 2001, 196 pp. Hardbound, ISBN 0-306-46560-4.

- Programmable Networks for IP Service Deployment" by Galis, A., Denazis, S., Brou, C., Klein, C.- Artech House Books, London, June 20;, 450 pp., ISBN 1-58053-745-6

This article is about the electronic protocol. For the medieval King of Denmark, see Harald I of Denmark.

Bluetooth logo

Bluetooth is an open wireless technology standard for exchanging data over short distances (using short wavelength radio transmissions) from fixed and mobile devices, creating personal area networks (PANs) with high levels of security. Created by telecoms vendor Ericsson in 1994,[1] it was originally conceived as a wireless alternative to RS-232 data cables. It can connect several devices, overcoming problems of synchronization. Today Bluetooth is managed by the Bluetooth Special Interest Group.

- Contents [hide] - 1 Name and logo - 2 Implementation - 2.1 Communication and connection - 3 Uses - 3.1 Bluetooth profiles - 3.2 List of applications - 3.3 Bluetooth vs. Wi-Fi IEEE 802.11 in networking - 3.4 Bluetooth devices - 4 Computer requirements - 4.1 Operating system support - 5 Mobile phone requirements - 6 Specifications and features - 6.1 Bluetooth v1.0 and v1.0B - 6.2 Bluetooth v1.1 - 6.3 Bluetooth v1.2 - 6.4 Bluetooth v2.0 + EDR - 6.5 Bluetooth v2.1 + EDR - 6.6 Bluetooth v3.0 + HS - 6.7 Bluetooth v4.0 - 6.7.1 Bluetooth low energy - 6.8 Future - 6.8.1 UWB for AMP - 7 Technical information - 7.1 Bluetooth protocol stack - 7.1.1 LMP (Link Management Protocol) - 7.1.2 L2CAP (Logical Link Control & Adaptation Protocol) - 7.1.3 SDP (Service Discovery Protocol) - 7.1.4 HCI (Host/Controller Interface) - 7.1.5 RFCOMM (Serial Port Emulation) - 7.1.6 BNEP (Bluetooth Network Encapsulation Protocol) - 7.1.7 AVCTP (Audio/Video Control Transport Protocol) - 7.1.8 AVDTP (Audio/Video Distribution Transport Protocol) - 7.1.9 Telephony control protocol - 7.1.10 Adopted protocols - 7.2 Baseband Error Correction - 7.3 Setting up connections - 7.4 Pairing - 7.4.1 Motivation - 7.4.2 Implementation - 7.4.3 Pairing mechanisms - 7.4.4 Security Concerns - 7.5 Air interface - 8 Security - 8.1 Overview - 8.2 Bluejacking - 8.3 History of security concerns - 8.3.1 Early - 8.3.2 2005 - 8.3.3 2006 - 8.3.4 2007 - 9 Health concerns - 10 Bluetooth Innovation World Cup marketing initiative - 11 See also - 12 References - 13 External links

- [edit] Name and logo

The word Bluetooth is an anglicised version of Danish/Swedish Blåtand, the epithet of the tenth-century king Harald I of Denmark and parts of Norway who united dissonant Danish tribes into a single kingdom. The implication is that Bluetooth does the same with communications protocols, uniting them into one universal standard.[2][3][4]

The Bluetooth logo is a bind rune merging the Younger Futhark runes  (Hagall) (ᚼ) and  (Bjarkan) (ᛒ), Harald's initials.

- [edit] Implementation

Bluetooth uses a radio technology called frequency-hopping spread spectrum, which chops up the data being sent and transmits chunks of it on up to 79 bands (1 MHz each) in the range 2402-2480 MHz. This range is in the globally unlicensed Industrial, Scientific and Medical (ISM) 2.4 GHz short-range radio frequency band.

Originally Gaussian frequency-shift keying (GFSK) modulation was the only modulation scheme available; subsequently, since the introduction of Bluetooth 2.0+EDR, π/4-DQPSK and 8DPSK modulation may also be used between compatible devices. Devices functioning with GFSK are said to be operating in basic rate (BR) mode where a gross data rate of 1 Mbit/s is possible. The term enhanced data rate (EDR) is used to describe π/4-DPSK and 8DPSK schemes, each giving 2 and 3 Mbit/s respectively. The combination of these (BR and EDR) modes in Bluetooth radio technology is classified as a "BR/EDR radio".

Bluetooth is a packet-based protocol with a master-slave structure. One master may communicate with up to 7 slaves in a piconet; all devices share the master's clock. Packet exchange is based on the basic clock, defined by the master, which ticks at 312.5 µs intervals. Two clock ticks make up a slot of 625 µs; two slots make up a slot pair of 1250 µs. In the simple case of single-slot packets the master transmits in even slots and receives in odd slots; the slave, conversely, receives in even slots and transmits in odd slots. Packets may be 1, 3 or 5 slots long but in all cases the master transmit will begin in even slots and the slave transmit in odd slots.

Bluetooth provides a secure way to connect and exchange information between devices such as faxes, mobile phones, telephones, laptops, personal computers, printers, Global Positioning System (GPS) receivers, digital cameras, and video game consoles.

The Bluetooth specifications are developed and licensed by the Bluetooth Special Interest Group (SIG). The Bluetooth SIG consists of more than 13,000 companies in the areas of telecommunication, computing, networking, and consumer electronics.[5]

To be marketed as a Bluetooth device, it must be qualified to standards defined by the SIG.

- [edit] Communication and connection

A master Bluetooth device can communicate with up to seven devices in a Wireless User Group. This network group of up to eight devices is called a piconet. The devices can switch roles, by agreement, and the slave can become the master at any time.

At any given time, data can be transferred between the master and one other device.

The master switches rapidly from one device to another in a round-robin fashion. Simultaneous transmission from the master to multiple other devices is possible via broadcast mode, but this capability is infrequently used in practice.

The Bluetooth Core Specification provides for the connection of two or more piconets to form a scatternet, in which certain devices serve as bridges, simultaneously playing the master role in one piconet and the slave role in another.

Many USB Bluetooth adapters or "dongles" are available, some of which also include an IrDA adapter. Older (pre-2003) Bluetooth dongles, however, have limited capabilities, offering only the Bluetooth Enumerator and a less-powerful Bluetooth Radio incarnation. Such devices can link computers with Bluetooth with a distance of 100 meters, but they do not offer much in the way of services that modern adapters do.

- [edit] Uses

Bluetooth is a standard communications protocol primarily designed for low power consumption, with a short range (power-class-dependent: 100 m, 10 m and 1 m, but ranges vary in practice; see table below) based on low-cost transceiver microchips in each device.[6] Because the devices use a radio (broadcast) communications system, they do not have to be in line of sight of each other.[5]

| Class | Maximum Permitted Power |  | Range (approximate) |
| --- | --- | --- | --- |
|  | mW | dBm |  |
| Class 1 | 100 | 20 | ~100 meters |
| Class 2 | 2.5 | 4 | ~10 meters |
| Class 3 | 1 | 0 | ~1 meters |

In most cases the effective range of class 2 devices is extended if they connect to a class 1 transceiver, compared to a pure class 2 network. This is accomplished by the higher sensitivity and transmission power of Class 1 devices.[7]

| Version | Data Rate |
| --- | --- |
| Version 1.2 | 1 Mbit/s |
| Version 2.0 + EDR | 3 Mbit/s |
| Version 3.0 + HS | 24 Mbit/s |

While the Bluetooth Core Specification does mandate minimums for range, the range of the technology is application specific and is not limited. Manufacturers may tune their implementations to the range needed to support individual use cases.

- [edit] Bluetooth profiles

Main article: Bluetooth profile

To use Bluetooth wireless technology, a device must be able to interpret certain Bluetooth profiles, which are definitions of possible applications and specify general behaviors that Bluetooth enabled devices use to communicate with other Bluetooth devices. There are a wide range of Bluetooth profiles that describe many different types of applications or use cases for devices.[8]

- [edit] List of applications

A typical Bluetooth mobile phone headset.

- Wireless control of and communication between a mobile phone and a handsfree headset. This was one of the earliest applications to become popular.

- Wireless networking between PCs in a confined space and where little bandwidth is required.

- Wireless communication with PC input and output devices, the most common being the mouse, keyboard and printer.

- Transfer of files, contact details, calendar appointments, and reminders between devices with OBEX.

- Replacement of traditional wired serial communications in test equipment, GPS receivers, medical equipment, bar code scanners, and traffic control devices.

- For controls where infrared was traditionally used.

- For low bandwidth applications where higher USB bandwidth is not required and cable-free connection desired.

- Sending small advertisements from Bluetooth-enabled advertising hoardings to other, discoverable, Bluetooth devices.[9]

- Wireless bridge between two Industrial Ethernet (e.g., PROFINET) networks.

- Three seventh-generation game consoles, Nintendo's Wii[10] and Sony's PlayStation 3 and PSP Go, use Bluetooth for their respective wireless controllers.

- Dial-up internet access on personal computers or PDAs using a data-capable mobile phone as a wireless modem like Novatel mifi.

- Short range transmission of health sensor data from medical devices to mobile phone, set-top box or dedicated telehealth devices.[11]

- Allowing a DECT phone to ring and answer calls on behalf of a nearby cell phone

- Real-time location systems (RTLS), are used to track and identify the location of objects in real-time using “Nodes” or “tags” attached to, or embedded in the objects tracked, and “Readers” that receive and process the wireless signals from these tags to determine their locations[12]

- [edit] Bluetooth vs. Wi-Fi IEEE 802.11 in networking

Bluetooth and Wi-Fi have many applications: setting up networks, printing, or transferring files.

Wi-Fi is intended for resident equipment and its applications. The category of applications is outlined as WLAN, the wireless local area networks. Wi-Fi is intended as a replacement for cabling for general local area network access in work areas.

Bluetooth is intended for non-resident equipment and its applications. The category of applications is outlined as the wireless personal area network (WPAN). Bluetooth is a replacement for cabling in a variety of personally carried applications in any ambience and can also support fixed location applications such as smart energy functionality in the home (thermostats, etc.).

Wi-Fi is wireless version of a traditional Ethernet network, and requires configuration to set up shared resources, transmit files, and to set up audio links (for example, headsets and hands-free devices). Wi-Fi uses the same radio frequencies as Bluetooth, but with higher power, resulting in a faster connection and better range from the base station. The nearest equivalents in Bluetooth are the DUN profile, which allows devices to act as modem interfaces, and the PAN profile, which allows for ad-hoc networking.[citation needed]

- [edit] Bluetooth devices

A Bluetooth USB dongle with a 100 m range. The MacBook Pro, shown, also has a built in Bluetooth adaptor.

Bluetooth exists in many products, such as telephones, the Wii, PlayStation 3, PSP Go, Lego Mindstorms NXT, iPod Touch and in some high definition watches,[13] modems and headsets. The technology is useful when transferring information between two or more devices that are near each other in low-bandwidth situations. Bluetooth is commonly used to transfer sound data with telephones (i.e., with a Bluetooth headset) or byte data with hand-held computers (transferring files).

Bluetooth protocols simplify the discovery and setup of services between devices.[citation needed] Bluetooth devices can advertise all of the services they provide.[citation needed] This makes using services easier because more of the security, network address and permission configuration can be automated than with many other network types.[citation needed]

- [edit] Computer requirements

A typical Bluetooth USB dongle.

An internal notebook Bluetooth card (14×36×4 mm).

A personal computer that does not have embedded Bluetooth can be used with a Bluetooth adapter or "dongle" that will enable the PC to communicate with other Bluetooth devices (such as mobile phones, mice and keyboards). While some desktop computers and most recent laptops come with a built-in Bluetooth radio, others will require an external one in the form of a dongle.

Unlike its predecessor, IrDA, which requires a separate adapter for each device, Bluetooth allows multiple devices to communicate with a computer over a single adapter.

- [edit] Operating system support

For more details on this topic, see Bluetooth stack.

Apple has supported Bluetooth since Mac OS X v10.2 which was released in 2002.[14]

For Microsoft platforms, Windows XP Service Pack 2 and SP3 releases have native support for Bluetooth 1.1, 2.0 and 2.0+EDR.[15] Previous versions required users to install their Bluetooth adapter's own drivers, which were not directly supported by Microsoft.[16] Microsoft's own Bluetooth dongles (packaged with their Bluetooth computer devices) have no external drivers and thus require at least Windows XP Service Pack 2. Windows Vista RTM/SP1 with the Feature Pack for Wireless or Windows Vista SP2 support Bluetooth 2.1+EDR.[15] Windows 7 supports Bluetooth 2.1+EDR and Extended Inquiry Response (EIR).[15]

The Windows XP and Windows Vista/Windows 7 Bluetooth stacks support the following Bluetooth profiles natively: PAN, SPP, DUN, HID, HCRP. The Windows XP stack can be replaced by a third party stack which may support more profiles or newer versions of Bluetooth. The Windows Vista/Windows 7 Bluetooth stack supports vendor-supplied additional profiles without requiring the Microsoft stack to be replaced.[15]

Linux has two popular Bluetooth stacks, BlueZ and Affix. The BlueZ stack is included with most Linux kernels and was originally developed by Qualcomm.[17] The Affix stack was developed by Nokia. FreeBSD features Bluetooth support since its 5.0 release. NetBSD features Bluetooth support since its 4.0 release. Its Bluetooth stack has been ported to OpenBSD as well.

- [edit] Mobile phone requirements

A Bluetooth-enabled mobile phone is able to pair with many devices. To ensure the broadest support of feature functionality together with legacy device support, the Open Mobile Terminal Platform (OMTP) forum has published a recommendations paper, entitled "Bluetooth Local Connectivity".[18]

- [edit] Specifications and features

The Bluetooth specification was developed in 1994 by Jaap Haartsen and Sven Mattisson, who were working for Ericsson in Lund, Sweden.[19] The specification is based on frequency-hopping spread spectrum technology.

The specifications were formalized by the Bluetooth Special Interest Group (SIG). The SIG was formally announced on May 20, 1998. Today it has a membership of over 13,000 companies worldwide. It was established by Ericsson, IBM, Intel, Toshiba, Motorola and Nokia, and later joined by many other companies.

- [edit] Bluetooth v1.0 and v1.0B

Versions 1.0 and 1.0B had many problems, and manufacturers had difficulty making their products interoperable. Versions 1.0 and 1.0B also included mandatory Bluetooth hardware device address (BD_ADDR) transmission in the Connecting process (rendering anonymity impossible at the protocol level), which was a major setback for certain services planned for use in Bluetooth environments.

- [edit] Bluetooth v1.1

- Ratified as IEEE Standard 802.15.1-2002[20]

- Many errors found in the 1.0B specifications were fixed.

- Added support for non-encrypted channels.

- Received Signal Strength Indicator (RSSI).

- [edit] Bluetooth v1.2

This version is backward compatible with 1.1 and the major enhancements include the following:

- Faster Connection and Discovery

- Adaptive frequency-hopping spread spectrum (AFH), which improves resistance to radio frequency interference by avoiding the use of crowded frequencies in the hopping sequence.

- Higher transmission speeds in practice, up to 721 kbit/s,[21] than in 1.1.

- Extended Synchronous Connections (eSCO), which improve voice quality of audio links by allowing retransmissions of corrupted packets, and may optionally increase audio latency to provide better support for concurrent data transfer.

- Host Controller Interface (HCI) support for three-wire UART.

- Ratified as IEEE Standard 802.15.1-2005[22]

- Introduced Flow Control and Retransmission Modes for L2CAP.

- [edit] Bluetooth v2.0 + EDR

This version of the Bluetooth Core Specification was released in 2004 and is backward compatible with the previous version 1.2. The main difference is the introduction of an Enhanced Data Rate (EDR) for faster data transfer. The nominal rate of EDR is about 3 megabits per second, although the practical data transfer rate is 2.1 megabits per second.[21] EDR uses a combination of GFSK and Phase Shift Keying modulation (PSK) with two variants, π/4-DQPSK and 8DPSK.[23] EDR can provide a lower power consumption through a reduced duty cycle.

The specification is published as "Bluetooth v2.0 + EDR" which implies that EDR is an optional feature. Aside from EDR, there are other minor improvements to the 2.0 specification, and products may claim compliance to "Bluetooth v2.0" without supporting the higher data rate. At least one commercial device states "Bluetooth v2.0 without EDR" on its data sheet.[24]

- [edit] Bluetooth v2.1 + EDR

Bluetooth Core Specification Version 2.1 + EDR is fully backward compatible with 1.2, and was adopted by the Bluetooth SIG on July 26, 2007.[23]

The headline feature of 2.1 is secure simple pairing (SSP): this improves the pairing experience for Bluetooth devices, while increasing the use and strength of security. See the section on Pairing below for more details.[25]

2.1 allows various other improvements, including "Extended inquiry response" (EIR), which provides more information during the inquiry procedure to allow better filtering of devices before connection; sniff subrating, which reduces the power consumption in low-power mode

- [edit] Bluetooth v3.0 + HS

Version 3.0 + HS of the Bluetooth Core Specification[23] was adopted by the Bluetooth SIG on April 21, 2009. It supports theoretical data transfer speeds of up to 24 Mbit/s, though not over the Bluetooth link itself. Instead, the Bluetooth link is used for negotiation and establishment, and the high data rate traffic is carried over a colocated 802.11 link. Its main new feature is AMP (Alternate MAC/PHY), the addition of 802.11 as a high speed transport. Two technologies had been anticipated for AMP: 802.11 and UWB, but UWB is missing from the specification.[26]

Alternate MAC/PHY

Enables the use of alternative MAC and PHYs for transporting Bluetooth profile data. The Bluetooth radio is still used for device discovery, initial connection and profile configuration, however when large quantities of data need to be sent, the high speed alternate MAC PHY 802.11 (typically associated with Wi-Fi) will be used to transport the data. This means that the proven low power connection models of Bluetooth are used when the system is idle, and the low power per bit radios are used when large quantities of data need to be sent.

Unicast connectionless data

Permits service data to be sent without establishing an explicit L2CAP channel. It is intended for use by applications that require low latency between user action and reconnection/transmission of data. This is only appropriate for small amounts of data.

Enhanced Power Control

Updates the power control feature to remove the open loop power control, and also to clarify ambiguities in power control introduced by the new modulation schemes added for EDR. Enhanced power control removes the ambiguities by specifying the behaviour that is expected. The feature also adds closed loop power control, meaning RSSI filtering can start as the response is received. Additionally, a "go straight to maximum power" request has been introduced, this is expected to deal with the headset link loss issue typically observed when a user puts their phone into a pocket on the opposite side to the headset.

- [edit] Bluetooth v4.0

On June 12, 2007, Nokia and Bluetooth SIG had announced that Wibree will be a part of the Bluetooth specification, as an ultra-low power Bluetooth technology.[27]

On December 17, 2009, the Bluetooth SIG adopted Bluetooth low energy technology as the hallmark feature of the version 4.0.[28] The provisional names Wibree and Bluetooth ULP (Ultra Low Power) are abandoned.

On April 21, 2010, the Bluetooth SIG completed the Bluetooth Core Specification version 4.0, which includes Classic Bluetooth, Bluetooth high speed and Bluetooth low energy protocols. Bluetooth high speed is based on Wi-Fi, and Classic Bluetooth consists of legacy Bluetooth protocols.

- [edit] Bluetooth low energy

Main article: Bluetooth low energy

Bluetooth low energy is an enhancement to the Bluetooth standard that was introduced in Bluetooth v4.0. It allows two types of implementation, dual-mode and single-mode. In a dual-mode implementation, Bluetooth low energy functionality is integrated into an existing Classic Bluetooth controller. The resulting architecture shares much of Classic Bluetooth’s existing radio and functionality resulting in a minimal cost increase compared to Classic Bluetooth. Additionally, manufacturers can use current Classic Bluetooth (Bluetooth v2.1 + EDR or Bluetooth v3.0 + HS) chips with the new low energy stack, enhancing the development of Classic Bluetooth enabled devices with new capabilities.

Single-mode chips, which will enable highly integrated and compact devices, will feature a lightweight Link Layer providing ultra-low power idle mode operation, simple device discovery, and reliable point-to-multipoint data transfer with advanced power-save and secure encrypted connections at the lowest possible cost. The Link Layer in these controllers will enable Internet connected sensors to schedule Bluetooth low energy traffic between Bluetooth transmissions.

Expected use cases for Bluetooth low energy technology include sports and fitness, security and proximity and smart energy. Bluetooth low energy technology is designed for devices to have a battery life of up to one year such as those powered by coin-cell batteries. These types of devices include watches that will use Bluetooth low energy technology to display Caller ID information and sports sensors that will be used to monitor the wearer's heart rate during exercise. The Medical Devices Working Group of the Bluetooth SIG is also creating a medical devices profile and associated protocols to enable Bluetooth applications for this vertical market.

- [edit] Future

Broadcast channel

Enables Bluetooth information points. This will drive the adoption of Bluetooth into mobile phones, and enable advertising models based on users pulling information from the information points, and not based on the object push model that is used in a limited way today.

Topology management

Enables the automatic configuration of the piconet topologies especially in scatternet situations that are becoming more common today. This should all be invisible to users of the technology, while also making the technology "just work."

QoS improvements

Enable audio and video data to be transmitted at a higher quality, especially when best effort traffic is being transmitted in the same piconet.

- [edit] UWB for AMP

Main article: ultra-wideband

The high speed (AMP) feature of Bluetooth v3.0 is based on 802.11, but the AMP mechanism was designed to be usable with other radios as well. It was originally intended for UWB, but the WiMedia Alliance, the body responsible for the flavor of UWB intended for Bluetooth, announced in March 2009 that it was disbanding.

On March 16, 2009, the WiMedia Alliance announced it was entering into technology transfer agreements for the WiMedia Ultra-wideband (UWB) specifications. WiMedia has transferred all current and future specifications, including work on future high speed and power optimized implementations, to the Bluetooth Special Interest Group (SIG), Wireless USB Promoter Group and the USB Implementers Forum. After the successful completion of the technology transfer, marketing and related administrative items, the WiMedia Alliance will cease operations.[29][30][31][32][33][34]

In October 2009 the Bluetooth Special Interest Group suspended development of UWB as part of the alternative MAC/PHY, Bluetooth v3.0 + HS solution. A small, but significant, number of former WiMedia members had not and would not sign up to the necessary agreements for the IP transfer. The Bluetooth SIG is now in the process of evaluating other options for its longer term roadmap.[35]

- [edit] Technical information

- [edit] Bluetooth protocol stack

Main articles: Bluetooth stack and Bluetooth protocols

"Bluetooth is defined as a layer protocol architecture consisting of core protocols, cable replacement protocols, telephony control protocols, and adopted protocols."[36] Mandatory protocols for all Bluetooth stacks are: LMP, L2CAP and SDP. Additionally, these protocols are almost universally supported: HCI and RFCOMM.

- [edit] LMP (Link Management Protocol)

Used for control of the radio link between two devices. Implemented on the controller.

- [edit] L2CAP (Logical Link Control & Adaptation Protocol)

Used to multiplex multiple logical connections between two devices using different higher level protocols. Provides segmentation and reassembly of on-air packets.

In Basic mode, L2CAP provides packets with a payload configurable up to 64kB, with 672 bytes as the default MTU, and 48 bytes as the minimum mandatory supported MTU.

In Retransmission & Flow Control modes, L2CAP can be configured for reliable or isochronous data per channel by performing retransmissions and CRC checks.

Bluetooth Core Specification Addendum 1 adds two additional L2CAP modes to the core specification. These modes effectively deprecate original Retransmission and Flow Control modes:

- Enhanced Retransmission Mode (ERTM): This mode is an improved version of the original retransmission mode. This mode provides a reliable L2CAP channel.

- Streaming Mode (SM): This is a very simple mode, with no retransmission or flow control. This mode provides an unreliable L2CAP channel.

Reliability in any of these modes is optionally and/or additionally guaranteed by the lower layer Bluetooth BDR/EDR air interface by configuring the number of retransmissions and flush timeout (time after which the radio will flush packets). In-order sequencing is guaranteed by the lower layer.

Only L2CAP channels configured in ERTM or SM may be operated over AMP logical links.

- [edit] SDP (Service Discovery Protocol)

Service Discovery Protocol (SDP) allows a device to discover services supported by other devices, and their associated parameters. For example, when connecting a mobile phone to a Bluetooth headset, SDP will be used for determining which Bluetooth profiles are supported by the headset (Headset Profile, Hands Free Profile, Advanced Audio Distribution Profile (A2DP) etc.) and the protocol multiplexer settings needed to connect to each of them. Each service is identified by a Universally Unique Identifier (UUID), with official services (Bluetooth profiles) assigned a short form UUID (16 bits rather than the full 128)

- [edit] HCI (Host/Controller Interface)

Standardised communication between the host stack (e.g., a PC or mobile phone OS) and the controller (the Bluetooth IC). This standard allows the host stack or controller IC to be swapped with minimal adaptation.

There are several HCI transport layer standards, each using a different hardware interface to transfer the same command, event and data packets. The most commonly used are USB (in PCs) and UART (in mobile phones and PDAs).

In Bluetooth devices with simple functionality (e.g., headsets) the host stack and controller can be implemented on the same microprocessor. In this case the HCI is optional, although often implemented as an internal software interface.

- [edit] RFCOMM (Serial Port Emulation)

Radio frequency communications (RFCOMM) is a cable replacement protocol used to create a virtual serial data stream. RFCOMM provides for binary data transport and emulates EIA-232 (formerly RS-232) control signals over the Bluetooth baseband layer.

RFCOMM provides a simple reliable data stream to the user, similar to TCP. It is used directly by many telephony related profiles as a carrier for AT commands, as well as being a transport layer for OBEX over Bluetooth.

Many Bluetooth applications use RFCOMM because of its widespread support and publicly available API on most operating systems. Additionally, applications that used a serial port to communicate can be quickly ported to use RFCOMM.

- [edit] BNEP (Bluetooth Network Encapsulation Protocol)

BNEP is used for transferring another protocol stack's data via an L2CAP channel. It's main purpose is the transmission of IP packets in the Personal Area Networking Profile. BNEP performs a similar function to SNAP in Wireless LAN.

- [edit] AVCTP (Audio/Video Control Transport Protocol)

Used by the remote control profile to transfer AV/C commands over an L2CAP channel. The music control buttons on a stereo headset use this protocol to control the music player.

- [edit] AVDTP (Audio/Video Distribution Transport Protocol)

Used by the advanced audio distribution profile (A2DP) to stream music to stereo headsets over an L2CAP channel. Intended to be used by video distribution profile.

- [edit] Telephony control protocol

Telephony control protocol-binary (TCS BIN) is the bit-oriented protocol that defines the call control signaling for the establishment of voice and data calls between Bluetooth devices. Additionally, "TCS BIN defines mobility management procedures for handling groups of Bluetooth TCS devices."

TCS-BIN is only used by the cordless telephony profile, which failed to attract implementers. As such it is only of historical interest.

- [edit] Adopted protocols

Adopted protocols are defined by other standards-making organizations and incorporated into Bluetooth’s protocol stack, allowing Bluetooth to create protocols only when necessary. The adopted protocols include:

Point-to-Point Protocol (PPP)

Internet standard protocol for transporting IP datagrams over a point-to-point link.

TCP/IP/UDP

Foundation Protocols for TCP/IP protocol suite

Object Exchange Protocol (OBEX)

Session-layer protocol for the exchange of objects, providing a model for object and operation representation

Wireless Application Environment/Wireless Application Protocol (WAE/WAP)

WAE specifies an application framework for wireless devices and WAP is an open standard to provide mobile users access to telephony and information services.[36]

- [edit] Baseband Error Correction

Three types of error correction are implemented in Bluetooth systems,

- 1/3 rate forward error correction (FEC)

- 2/3 rate FEC

- Automatic repeat-request (ARQ)

- [edit] Setting up connections

Any Bluetooth device in discoverable mode will transmit the following information on demand:

- Device name

- Device class

- List of services

- Technical information (for example: device features, manufacturer, Bluetooth specification used, clock offset)

Any device may perform an inquiry to find other devices to connect to, and any device can be configured to respond to such inquiries. However, if the device trying to connect knows the address of the device, it always responds to direct connection requests and transmits the information shown in the list above if requested. Use of a device's services may require pairing or acceptance by its owner, but the connection itself can be initiated by any device and held until it goes out of range. Some devices can be connected to only one device at a time, and connecting to them prevents them from connecting to other devices and appearing in inquiries until they disconnect from the other device.

Every device has a unique 48-bit address. However, these addresses are generally not shown in inquiries. Instead, friendly Bluetooth names are used, which can be set by the user. This name appears when another user scans for devices and in lists of paired devices.

Most phones have the Bluetooth name set to the manufacturer and model of the phone by default. Most phones and laptops show only the Bluetooth names and special programs are required to get additional information about remote devices. This can be confusing as, for example, there could be several phones in range named T610 (see Bluejacking).

- [edit] Pairing

- [edit] Motivation

Many of the services offered over Bluetooth can expose private data or allow the connecting party to control the Bluetooth device. For security reasons it is therefore necessary to control which devices are allowed to connect to a given Bluetooth device. At the same time, it is useful for Bluetooth devices to automatically establish a connection without user intervention as soon as they are in range.

To resolve this conflict, Bluetooth uses a process called pairing, which is generally manually started by a device user—making that device's Bluetooth link visible to other devices. Two devices need to be paired to communicate with each other; the pairing process is typically triggered automatically the first time a device receives a connection request from a device with which it is not yet paired. Once a pairing has been established it is remembered by the devices, which can then connect to each without user intervention. When desired, the pairing relationship can later be removed by the user.

- [edit] Implementation

During the pairing process, the two devices involved establish a relationship by creating a shared secret known as a link key. If a link key is stored by both devices they are said to be paired or bonded. A device that wants to communicate only with a bonded device can cryptographically authenticate the identity of the other device, and so be sure that it is the same device it previously paired with. Once a link key has been generated, an authenticated ACL link between the devices may be encrypted so that the data that they exchange over the airwaves is protected against eavesdropping.

Link keys can be deleted at any time by either device. If done by either device this will implicitly remove the bonding between the devices; so it is possible for one of the devices to have a link key stored but not be aware that it is no longer bonded to the device associated with the given link key.

Bluetooth services generally require either encryption or authentication, and as such require pairing before they allow a remote device to use the given service. Some services, such as the Object Push Profile, elect not to explicitly require authentication or encryption so that pairing does not interfere with the user experience associated with the service use-cases.

- [edit] Pairing mechanisms

Pairing mechanisms have changed significantly with the introduction of Secure Simple Pairing in Bluetooth v2.1. The following summarizes the pairing mechanisms:

- Legacy pairing: This is the only method available in Bluetooth v2.0 and before. Each device must enter a PIN code; pairing is only successful if both devices enter the same PIN code. Any 16-byte UTF-8 string may be used as a PIN code, however not all devices may be capable of entering all possible PIN codes.

  - Limited input devices: The obvious example of this class of device is a Bluetooth Hands-free headset, which generally have few inputs. These devices usually have a fixed PIN, for example "0000" or "1234", that are hard-coded into the device.

  - Numeric input devices: Mobile phones are classic examples of these devices. They allow a user to enter a numeric value up to 16 digits in length.

  - Alpha-numeric input devices: PCs and smartphones are examples of these devices. They allow a user to enter full UTF-8 text as a PIN code. If pairing with a less capable device the user needs to be aware of the input limitations on the other device, there is no mechanism available for a capable device to determine how it should limit the available input a user may use.

- Secure Simple Pairing (SSP): This is required by Bluetooth v2.1. A Bluetooth v2.1 device may only use legacy pairing to interoperate with a v2.0 or earlier device. Secure Simple Pairing uses a form of public key cryptography, and has the following modes of operation:

  - Just works: As implied by the name, this method just works. No user interaction is required; however, a device may prompt the user to confirm the pairing process. This method is typically used by headsets with very limited IO capabilities, and is more secure than the fixed PIN mechanism which is typically used for legacy pairing by this set of limited devices. This method provides no man in the middle (MITM) protection.

  - Numeric comparison: If both devices have a display and at least one can accept a binary Yes/No user input, they may use Numeric Comparison. This method displays a 6-digit numeric code on each device. The user should compare the numbers to ensure they are identical. If the comparison succeeds, the user(s) should confirm pairing on the device(s) that can accept an input. This method provides MITM protection, assuming the user confirms on both devices and actually performs the comparison properly.

  - Passkey Entry: This method may be used between a device with a display and a device with numeric keypad entry (such as a keyboard), or two devices with numeric keypad entry. In the first case, the display is used to show a 6-digit numeric code to the user, who then enters the code on the keypad. In the second case, the user of each device enters the same 6-digit number. Both cases provide MITM protection.

  - Out of band (OOB): This method uses an external means of communication (such as NFC) to exchange some information used in the pairing process. Pairing is completed using the Bluetooth radio, but requires information from the OOB mechanism. This provides only the level of MITM protection that is present in the OOB mechanism.

SSP is considered simple for the following reasons:

- In most cases, it does not require a user to generate a passkey.

- For use-cases not requiring MITM protection, user interaction has been eliminated.

- For numeric comparison, MITM protection can be achieved with a simple equality comparison by the user.

- Using OOB with NFC will enable pairing when devices simply get close, rather than requiring a lengthy discovery process.

- [edit] Security Concerns

Prior to Bluetooth v2.1, encryption is not required and can be turned off at any time. Moreover, the encryption key is only good for approximately 23.5 hours; using a single encryption key longer than this time allows simple XOR attacks to retrieve the encryption key.

- Turning off encryption is required for several normal operations, so it is problematic to detect if encryption is disabled for a valid reason or for a security attack.

- Bluetooth v2.1 addresses this in the following ways:

  - Encryption is required for all non-SDP (Service Discovery Protocol) connections

  - A new Encryption Pause and Resume feature is used for all normal operations requiring encryption to be disabled. This enables easy identification of normal operation from security attacks.

  - The encryption key is required to be refreshed before it expires.

Link keys may be stored on the device file system, not on the Bluetooth chip itself. Many Bluetooth chip manufacturers allow link keys to be stored on the device; however, if the device is removable this means that the link key will move with the device.

- [edit] Air interface

The protocol operates in the license-free ISM band at 2.402-2.480 GHz.[37] To avoid interfering with other protocols that use the 2.45 GHz band, the Bluetooth protocol divides the band into 79 channels (each 1 MHz wide) and changes channels up to 1600 times per second. Implementations with versions 1.1 and 1.2 reach speeds of 723.1 kbit/s. Version 2.0 implementations feature Bluetooth Enhanced Data Rate (EDR) and reach 2.1 Mbit/s. Technically, version 2.0 devices have a higher power consumption, but the three times faster rate reduces the transmission times, effectively reducing power consumption to half that of 1.x devices (assuming equal traffic load).

- [edit] Security

- [edit] Overview

Bluetooth implements confidentiality, authentication and key derivation with custom algorithms based on the SAFER+ block cipher. Bluetooth key generation is generally based on a Bluetooth PIN, which must be entered into both devices. This procedure might be modified if one of the devices has a fixed PIN (e.g., for headsets or similar devices with a restricted user interface). During pairing, an initialization key or master key is generated, using the E22 algorithm.[38] The E0 stream cipher is used for encrypting packets, granting confidentiality and is based on a shared cryptographic secret, namely a previously generated link key or master key. Those keys, used for subsequent encryption of data sent via the air interface, rely on the Bluetooth PIN, which has been entered into one or both devices.

An overview of Bluetooth vulnerabilities exploits was published in 2007 by Andreas Becker.[39]

In September 2008, the National Institute of Standards and Technology (NIST) published a Guide to Bluetooth Security that will serve as reference to organization on the security capabilities of Bluetooth and steps for securing Bluetooth technologies effectively. While Bluetooth has its benefits, it is susceptible to denial of service attacks, eavesdropping, man-in-the-middle attacks, message modification, and resource misappropriation. Users/organizations must evaluate their acceptable level of risk and incorporate security into the lifecycle of Bluetooth devices. To help mitigate risks, included in the NIST document are security checklists with guidelines and recommendations for creating and maintaining secure Bluetooth piconets, headsets, and smart card readers.[40]

Bluetooth v2.1 - finalized in 2007 with consumer devices first appearing in 2009 - makes significant changes to Bluetooth's security, including pairing. See the #Pairing mechanisms section for more about these changes.

- [edit] Bluejacking

Main article: Bluejacking

Bluejacking is the sending of either a picture or a message from one user to an unsuspecting user through Bluetooth wireless technology. Common applications include short messages (e.g., "You’ve just been bluejacked!"). [41] Bluejacking does not involve the removal or alteration of any data from the device. Bluejacking can also involve taking control of a mobile wirelessly and phoning a premium rate line, owned by the bluejacker.

- [edit] History of security concerns

- [edit] Early

In 2001, Jakobsson and Wetzel from Bell Laboratories discovered flaws in the Bluetooth pairing protocol and also pointed to vulnerabilities in the encryption scheme.[42] In 2003, Ben and Adam Laurie from A.L. Digital Ltd. discovered that serious flaws in some poor implementations of Bluetooth security may lead to disclosure of personal data.[43] In a subsequent experiment, Martin Herfurt from the trifinite.group was able to do a field-trial at the CeBIT fairgrounds, showing the importance of the problem to the world. A new attack called BlueBug was used for this experiment.[44] In 2004 the first purported virus using Bluetooth to spread itself among mobile phones appeared on the Symbian OS.[45] The virus was first described by Kaspersky Lab and requires users to confirm the installation of unknown software before it can propagate. The virus was written as a proof-of-concept by a group of virus writers known as "29A" and sent to anti-virus groups. Thus, it should be regarded as a potential (but not real) security threat to Bluetooth technology or Symbian OS since the virus has never spread outside of this system. In August 2004, a world-record-setting experiment (see also Bluetooth sniping) showed that the range of Class 2 Bluetooth radios could be extended to 1.78 km (1.08 mile) with directional antennas and signal amplifiers.[46] This poses a potential security threat because it enables attackers to access vulnerable Bluetooth devices from a distance beyond expectation. The attacker must also be able to receive information from the victim to set up a connection. No attack can be made against a Bluetooth device unless the attacker knows its Bluetooth address and which channels to transmit on.

- [edit] 2005

In January 2005, a mobile malware worm known as Lasco.A began targeting mobile phones using Symbian OS (Series 60 platform) using Bluetooth enabled devices to replicate itself and spread to other devices. The worm is self-installing and begins once the mobile user approves the transfer of the file (velasco.sis ) from another device. Once installed, the worm begins looking for other Bluetooth enabled devices to infect. Additionally, the worm infects other .SIS files on the device, allowing replication to another device through use of removable media (Secure Digital, Compact Flash, etc.). The worm can render the mobile device unstable.[47]

In April 2005, Cambridge University security researchers published results of their actual implementation of passive attacks against the PIN-based pairing between commercial Bluetooth devices, confirming the attacks to be practicably fast and the Bluetooth symmetric key establishment method to be vulnerable. To rectify this vulnerability, they carried out an implementation which showed that stronger, asymmetric key establishment is feasible for certain classes of devices, such as mobile phones.[48]

In June 2005, Yaniv Shaked and Avishai Wool published a paper describing both passive and active methods for obtaining the PIN for a Bluetooth link. The passive attack allows a suitably equipped attacker to eavesdrop on communications and spoof, if the attacker was present at the time of initial pairing. The active method makes use of a specially constructed message that must be inserted at a specific point in the protocol, to make the master and slave repeat the pairing process. After that, the first method can be used to crack the PIN. This attack's major weakness is that it requires the user of the devices under attack to re-enter the PIN during the attack when the device prompts them to. Also, this active attack probably requires custom hardware, since most commercially available Bluetooth devices are not capable of the timing necessary.[49]

In August 2005, police in Cambridgeshire, England, issued warnings about thieves using Bluetooth enabled phones to track other devices left in cars. Police are advising users to ensure that any mobile networking connections are de-activated if laptops and other devices are left in this way.[50]

- [edit] 2006

In April 2006, researchers from Secure Network and F-Secure published a report that warns of the large number of devices left in a visible state, and issued statistics on the spread of various Bluetooth services and the ease of spread of an eventual Bluetooth worm.[51]

- [edit] 2007

In October 2007, at the Luxemburgish Hack.lu Security Conference, Kevin Finistere and Thierry Zoller demonstrated and released a remote root shell via Bluetooth on Mac OS X v10.3.9 and v10.4. They also demonstrated the first Bluetooth PIN and Linkkeys cracker, which is based on the research of Wool and Shaked.

- [edit] Health concerns

Main article: Wireless electronic devices and health

Bluetooth uses the microwave radio frequency spectrum in the 2.402 GHz to 2.480 GHz range.[37] Maximum power output from a Bluetooth radio is 100 mW, 2.5 mW, and 1 mW for Class 1, Class 2, and Class 3 devices respectively, which puts Class 1 at roughly the same level as mobile phones, and the other two classes much lower.[52] Accordingly, Class 2 and Class 3 Bluetooth devices are considered less of a potential hazard than mobile phones, and Class 1 may be comparable to that of mobile phones : the maximum for a Class 1 is 100 mW for Bluetooth but 250 mW for UMTS W-CDMA, 1 W for GSM1800/1900 and 2 W for GSM850/900 for instance.

- [edit] Bluetooth Innovation World Cup marketing initiative

The Bluetooth Innovation World Cup is an international competition encouraging the development of new innovations and ideas for applications leveraging the Bluetooth low energy wireless technology in sports, fitness and health care products.The Bluetooth Innovation World Cup is a marketing initiative of the Bluetooth Special Interest Group (SIG).

The aim of the competition is to stimulate new markets, creating new fields of applications and establishing Bluetooth low energy technology as the wireless data transfer standard for low energy applications is ordinary business in the competition of wireless standards. The initiative will go on for three years, having started 1 June 2009.[53]

Bluetooth Innovation World Cup 2009

The first international Bluetooth Innovation World Cup 2009 drew more than 250 international entries illustrating the abundance of opportunities for product development with the new Bluetooth low energy wireless technology.

The Bluetooth Innovation World Cup 2009 was sponsored by Nokia, Freescale Semiconductor, Texas Instruments, Nordic Semiconductor, STMicroelectronics and Brunel.

Bluetooth Innovator of the Year 2009

On February 8, 2010, the Bluetooth SIG has awarded Edward Sazonov, Physical Activity Innovations LLC, the title of Bluetooth Innovator of the Year for 2009. Sazonov received this recognition at the official award ceremony held in-line with the Wearable Technologies Show at ispo 2010, the world’s largest trade show for sporting goods. The award includes a cash prize of €5,000 and a Bluetooth Qualification Program voucher (QDID) valued at up to US$ 10,000. Sazonov’s winning idea, The Fit Companion, is a small, unobtrusive sensor that when clipped-on to a user’s clothing or integrated in to a shoe, provides feedback about their physical activity. The data, transmitted via Bluetooth low energy technology, can help individuals to lose weight and achieve optimal physical activity. Intended for use in both training and daily activities like walking or performing chores, this simple, measuring device may offer a solution for reducing obesity.

Bluetooth Innovation World Cup 2010

The Bluetooth Special Interest Group (SIG) announced the start of the second Bluetooth Innovation World Cup (IWC) on 1 June 2010. The 2010 Bluetooth Innovation World Cup has a focus on applications for the sports & fitness, health care and home information and control markets. The competition will close for registrations on September 15, 2010.

- [edit] See also

- Bluesniping

- BlueSoleil - Proprietary driver

- Continua Health Alliance

- DASH7

- Java APIs for Bluetooth

- Jellingspot Data Server

- Near Field Communication

- Tethering

- ZigBee - low power lightweight wireless protocol in the ISM band.

- [edit] References

- ^ "Bluetooth traveler". www.hoovers.com. http://www.hoovers.com/business-information/--pageid__13751--/global-hoov-index.xhtml. Retrieved 9 April 2010.

- ^ Monson, Heidi (1999-12-14). "Bluetooth Technology and Implications". SysOpt.com. http://www.sysopt.com/features/network/article.php/3532506. Retrieved 2009-02-17.

- ^ "About the Bluetooth SIG". Bluetooth SIG. http://www.bluetooth.com/Bluetooth/SIG/. Retrieved 2008-02-01.

- ^ Kardach, Jim (2008-05-03). "How Bluetooth got its name". http://www.eetimes.eu/scandinavia/206902019?cid=RSSfeed_eetimesEU_scandinavia. Retrieved 2009-02-24.

- ^ a b Newton, Harold. (2007). Newton’s telecom dictionary. New York: Flatiron Publishing.

- ^ "How Bluetooth Technology Works". Bluetooth SIG. Archived from the original on 2008-01-17. http://web.archive.org/web/20080117000828/http://bluetooth.com/Bluetooth/Technology/Works/. Retrieved 2008-02-01.

- ^ "Class 1 Bluetooth Dongle Test". Amperordirect.com. http://www.amperordirect.com/pc/r-electronic-resource/z-reference-bluetooth-class1-myth.html. Retrieved 2010-09-04.

- ^ "Profiles Overview". Bluetooth.com. http://www.bluetooth.com/English/Technology/Works/Pages/Profiles_Overview.aspx. Retrieved 2010-09-04.

- ^ "Hypertag.com". Hypertag.com. http://www.hypertag.com/company1/what-is-proximity-or-bluetooth-marketing/. Retrieved 2010-09-04. [dead link]

- ^ "Wii Controller". Bluetooth SIG. Archived from the original on February 20, 2008. http://web.archive.org/web/20080220080315/http://bluetooth.com/Bluetooth/Products/Products/Product_Details.htm?ProductID=2951. Retrieved 2008-02-01.

- ^ "Telemedicine.jp". Telemedicine.jp. http://www.telemedicine.jp/. Retrieved 2010-09-04.

- ^ "Real Time Location Systems". clarinox. http://www.clarinox.com/docs/whitepapers/RealTime_main.pdf. Retrieved 2010-8-4.

- ^ "Watch". Bluetooth.com. http://www.bluetooth.com/English/Products/Pages/Watch.aspx. Retrieved 2010-09-04.

- ^ Apple (2002-07-17). "Apple Introduces "Jaguar," the Next Major Release of Mac OS X". Press release. http://www.apple.com/pr/library/2002/jul/17jaguar.html. Retrieved 2008-02-04.

- ^ a b c d "Bluetooth Wireless Technology FAQ - 2010". http://download.microsoft.com/download/9/c/5/9c5b2167-8017-4bae-9fde-d599bac8184a/Bth_FAQ.docx. Retrieved 2010-09-04.

- ^ "Network Protection Technologie". Changes to Functionality in Microsoft Windows XP Service Pack 2. Microsoft Technet. http://www.microsoft.com/technet/prodtechnol/winxppro/maintain/sp2netwk.mspx. Retrieved 2008-02-01.

- ^ "Official Linux Bluetooth protocol stack". BlueZ. http://www.bluez.org/. Retrieved 2010-09-04.

- ^ OMTP.org

- ^ "The Bluetooth Blues". Information Age. 2001-05-24. Archived from the original on 2007-12-22. http://web.archive.org/web/20071222231740/http://www.information-age.com/article/2001/may/the_bluetooth_blues. Retrieved 2008-02-01.

- ^ "IEEE Std 802.15.1-2002 – IEEE Standard for Information technology – Telecommunications and information exchange between systems – Local and metropolitan area networks – Specific requirements Part 15.1: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for Wireless Personal Area Networks (WPANs)". Ieeexplore.ieee.org. doi:10.1109/IEEESTD.2002.93621. http://ieeexplore.ieee.org/servlet/opac?punumber=7932. Retrieved 2010-09-04.

- ^ a b Guy Kewney (2004-11-16). "High speed Bluetooth comes a step closer: enhanced data rate approved". Newswireless.net. http://www.newswireless.net/index.cfm/article/629. Retrieved 2008-02-04.

- ^ "IEEE Std 802.15.1-2005 – IEEE Standard for Information technology – Telecommunications and information exchange between systems – Local and metropolitan area networks – Specific requirements Part 15.1: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for Wireless Personal Area Networks (WPANs)". Ieeexplore.ieee.org. doi:10.1109/IEEESTD.2005.96290. http://ieeexplore.ieee.org/servlet/opac?punumber=9980. Retrieved 2010-09-04.

- ^ a b c "Specification Documents". Bluetooth SIG. http://www.bluetooth.com/Bluetooth/Technology/Building/Specifications/. Retrieved 2008-02-04.

- ^ "HTC TyTN Specification" (PDF). HTC. http://www.europe.htc.com/z/pdf/products/1766_TyTN_LFLT_OUT.PDF. Retrieved 2008-02-04.

- ^ (PDF) Simple Pairing Whitepaper. Version V10r00. Bluetooth SIG. 2006-08-03. Archived from the original on October 18, 2006. http://web.archive.org/web/20061018032605/http://www.bluetooth.com/NR/rdonlyres/0A0B3F36-D15F-4470-85A6-F2CCFA26F70F/0/SimplePairing_WP_V10r00.pdf. Retrieved 2007-02-01.

- ^ David Meyer (2009-04-22). "Bluetooth 3.0 released without ultrawideband". zdnet.co.uk. http://news.zdnet.co.uk/communications/0,1000000085,39643174,00.htm. Retrieved 2009-04-22.

- ^ Nokia (2007-06-12). "Wibree forum merges with Bluetooth SIG" (PDF). Press release. http://www.wibree.com/press/Wibree_pressrelease_final_1206.pdf. Retrieved 2008-02-04.

- ^ "Bluetooth.com". Bluetooth.com. http://www.bluetooth.com/Bluetooth/Press/SIG/SIG_INTRODUCES_BLUETOOTH_LOW_ENERGY_WIRELESS_TECHNOLOGY_THE_NEXT_GENERATION_OF_BLUETOOTH_WIRELESS_TE.htm. Retrieved 2010-09-04.

- ^ "Wimedia.org". Wimedia.org. 2010-01-04. http://www.wimedia.org/. Retrieved 2010-09-04.

- ^ "Wimedia.org". Wimedia.org. http://www.wimedia.org/imwp/download.asp?ContentID=15508. Retrieved 2010-09-04.

- ^ "Wimedia.org". http://www.wimedia.org/imwp/download.asp?ContentID=15506. Retrieved 2010-09-04.

- ^ Bluetooth.com[dead link]

- ^ "USB.org". USB.org. 2009-03-16. http://www.usb.org/press/WiMedia_Tech_Transfer/. Retrieved 2010-09-04.

- ^ "Incisor.tv". Incisor.tv. 2009-03-16. http://www.incisor.tv/2009/03/what-to-make-of-bluetooth-sig-wimedia.html. Retrieved 2010-09-04.

- ^ Bluetooth group drops ultrawideband, eyes 60 GHz, Report: Ultrawideband dies by 2013, Incisor Magazine November 2009

- ^ a b Stallings, William. (2005). Wireless communications & networks.'=' Upper Saddle River, NJ: Pearson Prentice Hall.

- ^ a b D. Chomienne, M. Eftimakis (2010-10-20). "Bluetooth Tutorial" (PDF). http://www.newlogic.com/products/Bluetooth-Tutorial-2001.pdf. Retrieved 2009-12-11.

- ^ Juha T. Vainio (2000-05-25). "Bluetooth Security". Helsinki University of Technology. http://www.iki.fi/jiitv/bluesec.pdf. Retrieved 2009-01-01.

- ^ Andreas Becker (2007-08-16) (PDF). Bluetooth Security & Hacks. Ruhr-Universität Bochum. http://gsyc.es/~anto/ubicuos2/bluetooth_security_and_hacks.pdf. Retrieved 2007-10-10.

- ^ Scarfone, K., and Padgette, J. (September 2008) (PDF). Guide to Bluetooth Security. National Institute of Standards and Technology. http://csrc.nist.gov/publications/nistpubs/800-121/SP800-121.pdf. Retrieved 2008-10-03.

- ^ "What is bluejacking?". Helsinki University of Technology. http://www.bluejackq.com/what-is-bluejacking.shtml. Retrieved 2008-05-01.

- ^ "Security Weaknesses in Bluetooth". RSA Security Conf. – Cryptographer’s Track. http://citeseerx.ist.psu.edu/viewdoc/summary?doi=10.1.1.23.7357. Retrieved 2009-03-01.

- ^ "Bluetooth". The Bunker. Archived from the original on January 26, 2007. http://web.archive.org/web/20070126012417/http://www.thebunker.net/resources/bluetooth. Retrieved 2007-02-01.

- ^ "BlueBug". Trifinite.org. http://trifinite.org/trifinite_stuff_bluebug.html. Retrieved 2007-02-01.

- ^ John Oates (2004-06-15). "Virus attacks mobiles via Bluetooth". The Register. http://www.theregister.co.uk/2004/06/15/symbian_virus/. Retrieved 2007-02-01.

- ^ "Long Distance Snarf". Trifinite.org. http://trifinite.org/trifinite_stuff_lds.html. Retrieved 2007-02-01.

- ^ "F-Secure Malware Information Pages: Lasco.A". F-Secure.com. http://www.f-secure.com/v-descs/lasco_a.shtml. Retrieved 2008-05-05.

- ^ Ford-Long Wong, Frank Stajano, Jolyon Clulow (2005-04) (PDF). Repairing the Bluetooth pairing protocol. University of Cambridge Computer Laboratory. Archived from the original on 2007-06-16. http://web.archive.org/web/20070616082657/http://www.cl.cam.ac.uk/~fw242/publications/2005-WongStaClu-bluetooth.pdf. Retrieved 2007-02-01.

- ^ Yaniv Shaked, Avishai Wool (2005-05-02). Cracking the Bluetooth PIN. School of Electrical Engineering Systems, Tel Aviv University. http://www.eng.tau.ac.il/~yash/shaked-wool-mobisys05/. Retrieved 2007-02-01.

- ^ "Phone pirates in seek and steal mission". Cambridge Evening News. Archived from the original on 2007-07-17. http://web.archive.org/web/20070717035938/http://www.cambridge-news.co.uk/news/region_wide/2005/08/17/06967453-8002-45f8-b520-66b9bed6f29f.lpf. Retrieved 2008-02-04.

- ^ (PDF) Going Around with Bluetooth in Full Safety. F-Secure. 2006-05. http://www.securenetwork.it/bluebag_brochure.pdf. Retrieved 2008-02-04.

- ^ M. Hietanen, T. Alanko (2005-10). "Occupational Exposure Related to Radiofrequency Fields from Wireless Communication Systems" (PDF). XXVIIIth General Assembly of URSI - Proceedings. Union Radio-Scientifique Internationale. Archived from the original on October 6, 2006. http://web.archive.org/web/20061006124651/http://www.ursi.org/Proceedings/ProcGA05/pdf/K03.7(01682).pdf. Retrieved 2007-04-19.

- ^ "Bluetooth Innovation World Cup". Bluetooth.com. http://www.bluetooth.com/Bluetooth/Press/Bluetooth_World_Innovation_Cup.htm. Retrieved 2010-09-04.

- [edit] External links

- Official website

- Use cases for mobile phones

| [hide]Internet access |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Network type | Wired |  |  |  |  | Wireless |  |  |
|  | Optical | Coaxial cable | Twisted pair | Phone line | Power line | Unlicensed terrestrial bands | Licensed terrestrial bands | Satellite |
| LAN | Ethernet | G.hn | Ethernet | HomePNA · G.hn | G.hn · HomePlug Powerline Alliance | Wi-Fi · Bluetooth · DECT · Wireless USB |  |  |
| WAN | PON · Ethernet | DOCSIS | Ethernet | Dial-up · ISDN · DSL | BPL | Muni Wi-Fi | GPRS · iBurst · WiBro/WiMAX · UMTS-TDD, HSPA · EVDO · LTE | Satellite |

From Wikipedia, the free encyclopedia

Jump to: navigation, search

|  | This article needs additional citations for verification. Please help improve this article by adding reliable references. Unsourced material may be challenged and removed. (August 2010) |
| --- | --- |

The client–server model of computing is a distributed application structure that partitions tasks or workloads between the providers of a resource or service, called servers, and service requesters, called clients.[1] Often clients and servers communicate over a computer network on separate hardware, but both client and server may reside in the same system. A server machine is a host that is running one or more server programs which share their resources with clients. A client does not share any of its resources, but requests a server's content or service function. Clients therefore initiate communication sessions with servers which await incoming requests.

- Contents [hide] - 1 Description - 2 Comparison to peer-to-peer architecture - 3 Advantages - 4 Disadvantages - 5 See also - 6 References

- [edit] Description

The client–server characteristic describes the relationship of cooperating programs in an application. The server component provides a function or service to one or many clients, which initiate requests for such services.

Functions such as email exchange, web access and database access, are built on the client–server model. Users accessing banking services from their computer use a web browser client to send a request to a web server at a bank. That program may in turn forward the request to its own database client program that sends a request to a database server at another bank computer to retrieve the account information. The balance is returned to the bank database client, which in turn serves it back to the web browser client displaying the results to the user. The client–server model has become one of the central ideas of network computing. Many business applications being written today use the client–server model. So do the Internet's main application protocols, such as HTTP, SMTP, Telnet, and DNS.

The interaction between client and server is often described using sequence diagrams. Sequence diagrams are standardized in the Unified Modeling Language.

Specific types of clients include web browsers, email clients, and online chat clients.

Specific types of servers include web servers, ftp servers, application servers, database servers, name servers, mail servers, file servers, print servers, and terminal servers. Most web services are also types of servers.

- [edit] Comparison to peer-to-peer architecture

|  | This section requires expansion. |
| --- | --- |

In peer-to-peer architectures, each host or instance of the program can simultaneously act as both a client and a server, and each has equivalent responsibilities and status.

Both client–server and peer-to-peer architectures are in wide usage today. Details may be found in Comparison of Centralized (Client-Server) and Decentralized (Peer-to-Peer) Networking.

- [edit] Advantages

- In most cases, a client–server architecture enables the roles and responsibilities of a computing system to be distributed among several independent computers that are known to each other only through a network. This creates an additional advantage to this architecture: greater ease of maintenance. For example, it is possible to replace, repair, upgrade, or even relocate a server while its clients remain both unaware and unaffected by that change.

- All data is stored on the servers, which generally have far greater security controls than most clients.[citation needed] Servers can better control access and resources, to guarantee that only those clients with the appropriate permissions may access and change data.

- Since data storage is centralized, updates to that data are far easier to administer in comparison to a P2P paradigm. In the latter, data updates may need to be distributed and applied to each peer in the network, which is both time-consuming and error-prone,[citation needed] as there can be thousands or even millions of peers.

- Many mature client–server technologies are already available which were designed to ensure security, friendliness of the user interface, and ease of use.[citation needed]

- It functions with multiple different clients of different capabilities.

- [edit] Disadvantages

- As the number of simultaneous client requests to a given server increases, the server can become overloaded.[citation needed] Contrast that to a P2P network, where its aggregated bandwidth actually increases as nodes are added, since the P2P network's overall bandwidth can be roughly computed as the sum of the bandwidths of every node in that network.

- The client–server paradigm lacks the robustness of a good P2P network.[citation needed] Under client–server, should a critical server fail, clients’ requests cannot be fulfilled. In P2P networks, resources are usually distributed among many nodes. Even if one or more nodes depart and abandon a downloading file, for example, the remaining nodes should still have the data needed to complete the download.

- [edit] See also

- Hybrid client

- Inter-server

- Mainframe computer

- Mobile software

- Name server

- Observer pattern

- Push technology

- Pull technology

- Servent

- Standalone server

- Thin client

- Multitier architecture

- Client (computing)

- Server (computing)

From Wikipedia, the free encyclopedia

Jump to: navigation, search

|  | This article is written like an advertisement. Please help rewrite this article from a neutral point of view. For blatant advertising that would require a fundamental rewrite to become encyclopedic, use {{db-spam}} to mark for speedy deletion. (September 2010) |
| --- | --- |

|  | This article may require cleanup to meet Wikipedia's quality standards. Please improve this article if you can. The talk page may contain suggestions. (September 2010) |
| --- | --- |

Cloud-based networking solutions are beginning to come to market that promise to deliver the same cost, efficiency, and scalability benefits to the networking sphere as software as a service (SaaS) has to enterprise applications. Multi-tenant architecture based SaaS applications like SalesforceTM changed the status quo in the CRM space. Cloud-based networking promises to bring about the same revolution in the networking space.

- Contents [hide] - 1 Definition - 2 Benefits - 3 Players - 4 News articles on cloud-based networking

- [edit] Definition

Cloud-based networking is a new way to roll out distributed enterprise networks that delivers enterprise-class network capabilities around the globe, via a highly resilient, multi-tenant application that requires no capital investment in networking equipment. Unlike traditional hardware-based legacy solutions, cloud-based networking is extremely simple, enabling enterprises to deploy remote locations in minutes and operate their distributed networks via a cloud-based application, while providing unprecedented levels of centralized control and network visibility. These services are also subscription based.

- [edit] Benefits

It offers all the benefits that cloud computing promises to offer. Cloud-based networking products tend to offer all the standard networking features that one would expect from any networking product but delivered in a multi-tenant cloud based model that is simple to use for the customers and IT administrators, offers them granular control and visibility not possible with legacy networking solutions at no capital investment.

- [edit] Players

The marketplace for cloud-based networking promises to get crowded pretty soon. Networking heavyweights like Cisco and Juniper are still coming to terms with the benefits cloud computing brings about to the networking world. Silicon Valley based startups Pareto and Meraki have pioneered cloud-based networking solutions for their customers.

A number of networking vendors such as Cisco Systems, Juniper and Arista Networks position their Ethernet switching products for deployment in Cloud computing data center environments. These devices are not managed with cloud-based services.

- [edit] News articles on cloud-based networking

1. http://www.prnewswire.com/news-releases/pareto-networks-sets-new-standard-for-cloud-based-enterprise-networking-97382414.html

2. http://meraki.com/press-releases/2010/05/26/meraki-expands-international-presence-by-partnering-with-uk-based-cloud-distribution/

3. http://www.aristanetworks.com/en/blogs/?p=311

From Wikipedia, the free encyclopedia

Jump to: navigation, search

A sample network diagram

A computer network diagram is a schematic depicting the nodes and connections amongst nodes in a computer network or, more generally, any telecommunications network.

- Contents [hide] - 1 Symbolization - 1.1 Cisco Symbolization - 2 Topology - 2.1 Gallery - 3 See also - 4 References - 5 External links

- [edit] Symbolization

Readily identifiable icons are used to depict common network appliances e.g. Router, and the style of lines between them indicate the type of connection. Clouds are used to represent networks external to the one pictured for the purposes of depicting connections between internal and external devices, without indicating the specifics of the outside network. For example, in the hypothetical local area network pictured to the right, three personal computers and a server are connected to a switch; the server is further connected to a printer and a gateway router, which is connected via a WAN link to the Internet.

Depending on whether the diagram is intended for formal or informal use, certain details may be lacking and must be determined from context. For example, the sample diagram does not indicate the physical type of connection between the PCs and the switch, but since a modern LAN is depicted, Ethernet may be assumed. If the same style of line was used in a WAN (wide area network) diagram, however, it may indicate a different physical connection.

At different scales,or sizes, diagrams may represent various levels of network granularity. At the LAN level, individual nodes may represent individual physical devices, such as hubs or file servers, while at the WAN level, individual nodes may represent entire cities. In addition, when the scope of a diagram crosses the common LAN/MAN/WAN boundaries, representative hypothetical devices may be depicted instead of showing all actually existing nodes. For example, if a network appliance is intended to be connected through the Internet to many end-user mobile devices, only a single such device may be depicted for the purposes of showing the general relationship between the appliance and any such device.

- [edit] Cisco Symbolization

Cisco uses its own brand of networking symbols. Since Cisco has a large Internet presence and designs a broad variety of network devices, its list of symbols ("Network Topology Icons") is exhaustive. As of November 28, 2006 this list can be found at http://www.cisco.com/web/about/ac50/ac47/2.html.

- [edit] Topology

Main article: Network topology

The physical network topology can be directly represented in a network diagram, as it is simply the physical graph (mathematics) represented by the diagrams, with network nodes as vertices and connections as undirected or direct edges (depending on the type of connection). The logical network topology can be inferred from the network diagram if details of the network protocols in use are also given.

- [edit] Gallery

| Virtual Private Network | Enterprise private network | Overlay network collapsed | Overlay network broken-up |
| --- | --- | --- | --- |
| xDSL to Internet connectivity | Triple play illustration | Internet Distribution and Core | Internet, Access to core |

- [edit] See also

|  | Computer networking portal |
| --- | --- |

- Computer network

- Diagram

- Network diagram

- Network model

- [edit] References

|  | This article does not cite any references or sources. Please help improve this article by adding citations to reliable sources. Unsourced material may be challenged and removed. (October 2006) |
| --- | --- |

- [edit] External links

|  | Wikimedia Commons has media related to: Computer network diagrams |
| --- | --- |

- Dia, a free tool for building network diagrams

- 10-Strike Network Diagram, a commercial program for creating and editing network diagrams under Windows

Retrieved from "http://en.wikipedia.org/wiki/Computer_network_diagram"

From Wikipedia, the free encyclopedia

Jump to: navigation, search

This article is about computer networking, the discipline of engineering computer networks. For the article on computer networks, see Computer network.

"Datacom" redirects here. For other uses, see Datacom (disambiguation).

Network cards such as this one can transmit and receive data at high rates over various types of network cables. This card is a 'Combo' card which supports three cabling standards.

Computer networking or Data communications (Datacom) is the engineering discipline concerned with the communication between computer systems or devices. A computer network is any set of computers or devices connected to each other with the ability to exchange data.[1] Computer networking is sometimes considered a sub-discipline of telecommunications, computer science, information technology and/or computer engineering since it relies heavily upon the theoretical and practical application of these scientific and engineering disciplines. The three types of networks are: the Internet, the intranet, and the extranet. Examples of different network methods are:

- Local area network (LAN), which is usually a small network constrained to a small geographic area. An example of a LAN would be a computer network within a building.

- Metropolitan area network (MAN), which is used for medium size area. examples for a city or a state.

- Wide area network (WAN) that is usually a larger network that covers a large geographic area.

- Wireless LANs and WANs (WLAN & WWAN) are the wireless equivalent of the LAN and WAN.

All networks are interconnected to allow communication with a variety of different kinds of media, including twisted-pair copper wire cable, coaxial cable, optical fiber, power lines and various wireless technologies.[2] The devices can be separated by a few meters (e.g. via Bluetooth) or nearly unlimited distances (e.g. via the interconnections of the Internet[3]). Networking, routers, routing protocols, and networking over the public Internet have their specifications defined in documents called RFCs.[4]

- Contents [hide] - 1 Views of networks - 2 History of computer networks - 3 Networking methods - 3.1 Local area network (LAN) - 3.2 Wide area network (WAN) - 3.3 Wireless networks (WLAN, WWAN) - 4 Network topology - 5 See also - 6 References - 7 External links

- [edit] Views of networks

Users and network administrators typically have different views of their networks. Users can share printers and some servers from a workgroup, which usually means they are in the same geographic location and are on the same LAN. A [community-of-interest network community of interest]has less of a connection of being in a local area, and should be thought of as a set of arbitrarily located users who share a set of servers , and possibly also communicate via peer-to-peer technologies.

Network administrators can see networks from both physical and logical perspectives. The physical perspective involves geographic locations, physical cabling, and the network elements (e.g., routers, bridges and application layer gateways that interconnect the physical media. Logical networks, called, in the TCP/IP architecture, subnets, map onto one or more physical media. For example, a common practice in a campus of buildings is to make a set of LAN cables in each building appear to be a common subnet, using virtual LAN (VLAN) technology.

Both users and administrators will be aware, to varying extents, of the trust and scope characteristics of a network. Again using TCP/IP architectural terminology, an intranet is a community of interest under private administration usually by an enterprise, and is only accessible by authorized users (e.g. employees).[5] Intranets do not have to be connected to the Internet, but generally have a limited connection. An extranet is an extension of an intranet that allows secure communications to users outside of the intranet (e.g. business partners, customers).[5]

Unofficially, the Internet is the set of users, enterprises, and content providers that are interconnected by Internet Service Providers (ISP). From an engineering viewpoint, the Internet is the set of subnets, and aggregates of subnets, which share the registered IP address space and exchange information about the reachability of those IP addresses using the Border Gateway Protocol. Typically, the human-readable names of servers are translated to IP addresses, transparently to users, via the directory function of the Domain Name System (DNS).

Over the Internet, there can be business-to-business (B2B), business-to-consumer (B2C) and consumer-to-consumer (C2C) communications. Especially when money or sensitive information is exchanged, the communications are apt to be secured by some form of communications security mechanism. Intranets and extranets can be securely superimposed onto the Internet, without any access by general Internet users, using secure Virtual Private Network (VPN) technology.

When used for gaming one computer will need to be the server while the others play through it.

- [edit] History of computer networks

Before the advent of computer networks that were based upon some type of telecommunications system, communication between calculation machines and early computers was performed by human users by carrying instructions between them. Many of the social behaviors seen in today's Internet were demonstrably present in the nineteenth century and arguably in even earlier networks using visual signals.

In September 1940 George Stibitz used a teletype machine to send instructions for a problem set from his Model at Dartmouth College in New Hampshire to his Complex Number Calculator in New York and received results back by the same means. Linking output systems like teletypes to computers was an interest at the Advanced Research Projects Agency (ARPA) when, in 1962, J.C.R. Licklider was hired and developed a working group he called the "Intergalactic Network"Bold text, a precursor to the ARPANet.

In 1964, researchers at Dartmouth developed the Dartmouth Time Sharing System for distributed users of large computer systems. The same year, at MIT, a research group supported by General Electric and Bell Labs used a computer DEC's to route and manage telephone connections.

Throughout the 1960s Leonard Kleinrock, Paul Baran and Donald Davies independently conceptualized and developed network systems which used datagrams or packets that could be used in a network between computer systems.

1965 Thomas Merrill and Lawrence G. Roberts created the first wide area network (WAN).

The first widely used PSTN switch that used true computer control was the Western Electric introduced in 1965.

In 1969 the University of California at Los Angeles, SRI (in Stanford), University of California at Santa Barbara, and the University of Utah were connected as the beginning of the ARPANET network using 50 kbit/s circuits. Commercial services using X.25 were deployed in 1972, and later used as an underlying infrastructure for expanding TCP/IP networks.

Computer networks, and the technologies needed to connect and communicate through and between them, continue to drive computer hardware, software, and peripherals industries. This expansion is mirrored by growth in the numbers and types of users of networks from the researcher to the home user.

Today, computer networks are the core of modern communication. All modern aspects of the Public Switched Telephone Network (PSTN) are computer-controlled, and telephony increasingly runs over the Internet Protocol, although not necessarily the public Internet. The scope of communication has increased significantly in the past decade, and this boom in communications would not have been possible without the progressively advancing computer network.

- [edit] Networking methods

One way to categorize computer networks is by their geographic scope, although many real-world networks interconnect Local Area Networks (LAN) via Wide Area Networks (WAN) and wireless wide area networks (WWAN). These three (broad) types are:

- [edit] Local area network (LAN)

A local area network is a network that spans a relatively small space and provides services to a small number of people.

A peer-to-peer or client-server method of networking may be used. A peer-to-peer network is where each client shares their resources with other workstations in the network. Examples of peer-to-peer networks are: Small office networks where resource use is minimal and a home network. A client-server network is where every client is connected to the server and each other. Client-server networks use servers in different capacities. These can be classified into two types:

1. Single-service servers

2. Print servers

The server performs one task such as file server, while other servers can not only perform in the capacity of file servers and print servers, but also can conduct calculations and use them to provide information to clients (Web/Intranet Server). Computers may be connected in many different ways, including Ethernet cables, Wireless networks, or other types of wires such as power lines or phone lines.

The ITU-T G.hn standard is an example of a technology that provides high-speed (up to 1 Gbit/s) local area networking over existing home wiring (power lines, phone lines and coaxial cables).

- [edit] Wide area network (WAN)

A wide area network is a network where a wide variety of resources are deployed across a large domestic area or internationally. An example of this is a multinational business that uses a WAN to interconnect their offices in different countries. The largest and best example of a WAN is the Internet, which is a network composed of many smaller networks. The Internet is considered the largest network in the world.[6]. The PSTN (Public Switched Telephone Network) also is an extremely large network that is converging to use Internet technologies, although not necessarily through the public Internet.

A Wide Area Network involves communication through the use of a wide range of different technologies. These technologies include Point-to-Point WANs such as Point-to-Point Protocol (PPP) and High-Level Data Link Control (HDLC), Frame Relay, ATM (Asynchronous Transfer Mode) and Sonet (Synchronous Optical Network). The difference between the WAN technologies is based on the switching capabilities they perform and the speed at which sending and receiving bits of information (data) occur.

- [edit] Wireless networks (WLAN, WWAN)

A wireless network is basically the same as a LAN or a WAN but there are no wires between hosts and servers. The data is transferred over sets of radio transceivers. These types of networks are beneficial when it is too costly or inconvenient to run the necessary cables. For more information, see Wireless LAN and Wireless wide area network. The media access protocols for LANs come from the IEEE.

The most common IEEE 802.11 WLANs cover, depending on antennas, ranges from hundreds of meters to a few kilometers. For larger areas, either communications satellites of various types, cellular radio, or wireless local loop (IEEE 802.16) all have advantages and disadvantages. Depending on the type of mobility needed, the relevant standards may come from the IETF or the ITU.

- [edit] Network topology

The network topology defines the way in which computers, printers, and other devices are connected, physically and logically. A network topology describes the layout of the wire and devices as well as the paths used by data transmissions.

Network topology has two types:

- Physical

- Logical

Commonly used topologies include:

- Bus

- Star

- Tree (hierarchical)

- Linear

- Ring

- Mesh

  - partially connected

  - fully connected (sometimes known as fully redundant)

The network topologies mentioned above are only a general representation of the kinds of topologies used in computer network and are considered basic topologies

- [edit] See also

|  | Book:Computer Networking |
| --- | --- |
| Books are collections of articles that can be downloaded or ordered in print. |  |

- Data transmission

- Digital communications

- Communication network

- Network architecture

- Data interventions

- [edit] References

- ^ http://www.atis.org/tg2k/_computer_network.html Computer network definition

- ^ http://www.bellevuelinux.org/network.html Computer networks defined.

- ^ Interplanetary Internet, 2000 Third Annual International Symposium on Advanced Radio Technologies, A. Hooke,September 2000

- ^ The Internet Standards Process -- Revision 3, RFC 2026, rushawn o wright, October 1996.

- ^ a b RFC 2547

- ^ http://www.pcmag.com/encyclopedia_term/0,2542,t=internet&j=54184,00.asp "internet" defined

- Andrew S. Tanenbaum, Computer Networks (ISBN 0-13-349945-6).

- Important publications in computer networks

- Vinton G. Cerf "Software: Global Infrastructure for the 21st Century"

- Meyers, Mike, "Mike Meyers' Certifcation Passport: Network+" ISBN 0072253487"

- Odom, Wendall, "CCNA Certification Guide"

- Network Communication Architecture and Protocols: OSI Network Architecture 7 Layers Model.

- [edit] External links

- Easy Network Concepts (Linux kernel specific)

- Computer Networks and Protocol (Research document, 2006)

- Computer Networking Glossary

- Networking at the Open Directory Project

Retrieved from "http://en.wikipedia.org/wiki/Computer_networking"

From Wikipedia, the free encyclopedia

Jump to: navigation, search

Computer networking devices are units that mediate data in a computer network. Computer networking devices are also called network equipment, Intermediate Systems (IS) or InterWorking Unit (IWU). Units which are the last receiver or generate data are called hosts or data terminal equipment.

- [edit] List of computer networking devices

Common basic networking devices:

- Gateway: device sitting at a network node for interfacing with another network that uses different protocols. Works on OSI layers 4 to 7.

- Router: a specialized network device that determines the next network point to which to forward a data packet toward its destination. Unlike a gateway, it cannot interface different protocols. Works on OSI layer 3.

- Bridge: a device that connects multiple network segments along the data link layer. Works on OSI layer 2.

- Switch: a device that allocates traffic from one network segment to certain lines (intended destination(s)) which connect the segment to another network segment. So unlike a hub a switch splits the network traffic and sends it to different destinations rather than to all systems on the network. Works on OSI layer 2.

- Hub: connects multiple Ethernet segments together making them act as a single segment. When using a hub, every attached all the objects, compared to switches, which provide a dedicated connection between individual nodes. Works on OSI layer 1.

- Repeater: device to amplify or regenerate digital signals received while setting them from one part of a network into another. Works on OSI layer 1.

Some hybrid network devices:

- Multilayer Switch: a switch which, in addition to switching on OSI layer 2, provides functionality at higher protocol layers.

- Protocol Converter: a hardware device that converts between two different types of transmissions, such as asynchronous and synchronous transmissions.

- Bridge Router(Brouter): Combine router and bridge functionality and are therefore working on OSI layers 2 and 3.

- Digital media receiver: Connects a computer network to a home theatre

Hardware or software components that typically sit on the connection point of different networks, e.g. between an internal network and an external network:

- Proxy: computer network service which allows clients to make indirect network connections to other network services

- Firewall: a piece of hardware or software put on the network to prevent some communications forbidden by the network policy

- Network Address Translator: network service provide as hardware or software that converts internal to external network addresses and vice versa

Other hardware for establishing networks or dial-up connections:

- Multiplexer: device that combines several electrical signals into a single signal

- Network Card: a piece of computer hardware to allow the attached computer to communicate by network

- Modem: device that modulates an analog "carrier" signal (such as sound), to encode digital information, and that also demodulates such a carrier signal to decode the transmitted information, as a computer communicating with another computer over the telephone network

- ISDN terminal adapter (TA): a specialized gateway for ISDN

- Line Driver: a device to increase transmission distance by amplifying the signal. Base-band networks only.

- Network Device Connectivity

From Wikipedia, the free encyclopedia

Jump to: navigation, search

|  | This article may require cleanup to meet Wikipedia's quality standards. Please improve this article if you can. The talk page may contain suggestions. (March 2008) |
| --- | --- |

Decentralized computing is a trend in modern day business environments. This is the opposite of centralized computing, which was prevalent during the early days of computers. Decentralized computing is the allocation of resources, both hardware and software, to each individual workstation, or office location. In contrast, centralized computing exists when the majority of functions are carried out, or obtained from a remote centralized location.

A decentralized computer system has many benefits over a conventional centralized network. Desktop computers have advanced so rapidly, that their potential performance far exceeds the requirements of most business applications. This results in most desktop computers remaining idle (in relation to their full potential). A decentralized system can utilize the potential of these systems to maximize efficiency. However, it is debatable whether these networks increase overall effectiveness.

All computers have to be updated individually with new software, unlike a centralised computer system. Decentralised systems still enable file sharing and all computers can share peripherals such as printers and scanners as well as modems, allowing all the computers in the network to connect to the internet.

A collection of decentralized computers systems are components of a larger computer network, held together by local stations of equal importance and capability. These systems are capable of running independently of each other.

- [edit] Peer-to-Peer

Based on a “grid model” a peer-to-peer system, or P2P system, is a collection of applications run on several local computers, which connect remotely to each other to complete a function or a task. There is no main operating system to which satellite systems are subordinate. This approach to software development (and distribution) affords developers great savings, as they don’t have to create a central control point. An example application is LAN messaging which allows users to communicate without a central server.

- [edit] File sharing applications

One of the most interesting debates over decentralized computing involves Napster, a music file sharing application, which granted users access to an enormous database of files. Record companies brought legal action against Napster, blaming the system for lost record sales. Napster was found in violation of copyright laws by distributing pirated software, and was shut down.

After the fall of Napster, there was a demand for a file sharing system that would be less vulnerable to litigation. Gnutella, a decentralized system, was developed. This system allowed files to be queried and shared between users, without a central directory, and therefore shielded the network from law.

- [edit] References

- Crowcroft, Jon. Moreton, Tim. Pratt, Ian. Twigg.(2003). “Peer-to-Peer Systems and the Grid” Retrieved Jun. 24, 2006.

- Reid, Alex. (1995). “IT Strategy Review, Distributed Computing – Rough Draft” Retrieved Jun. 24, 2006.

Retrieved from "http://en.wikipedia.org/wiki/Decentralized_computing"

Categories: Centralized computing

Hidden categories: Wikipedia articles needing cleanup from March 2008 | All articles needing cleanup

From Wikipedia, the free encyclopedia

Jump to: navigation, search

In combinatorics, an expander graph is a sparse graph that has strong connectivity properties, quantified using vertex, edge or spectral expansion as described below. Expander constructions have spawned research in pure and applied mathematics, with several applications to complexity theory, design of robust computer networks, and the theory of error-correcting codes.[1]

- Contents [hide] - 1 Definitions - 1.1 Edge expansion - 1.2 Vertex expansion - 1.3 Spectral expansion - 1.4 Expander families - 2 Relationships between different expansion properties - 2.1 Cheeger Inequalities - 3 Examples of expanders - 4 Applications and useful properties - 4.1 Expander mixing lemma - 4.2 Expander walk sampling - 5 See also - 6 Notes - 7 References - 8 External links

- [edit] Definitions

There are several different ways to measure the expansion properties of a finite, undirected multigraph G.

- [edit] Edge expansion

The edge expansion (also isoperimetric number or Cheeger constant) h(G) of a graph G is defined as

where the minimum is over all nonempty sets S of at most n / 2 vertices and is the edge boundary of S, i.e., the set of edges with exactly one endpoint in S.[2] It is closely related to the conductance.[citation needed]

- [edit] Vertex expansion

The vertex isoperimetric number hout(G) (also vertex expansion or magnification) of a graph G is defined as

where is the outer boundary of S, i.e., the set of vertices in with at least one neighbor in S.[3] In a variant of this definition (called unique neighbor expansion) is replaced by the set of vertices in V with exactly one neighbor in S.[citation needed]

The vertex isoperimetric number hin(G) of a graph G is defined as

where is the inner boundary of S, i.e., the set of vertices in S with at least one neighbor in .[4]

- [edit] Spectral expansion

When G is regular, a linear algebraic definition of expansion is possible based on the eigenvalues of the adjacency matrix A = A(G) of G (where Aii is the number of loops at the ith vertex).[5] Because A is symmetric, the spectral theorem implies that A has n real-valued eigenvalues . Because G is regular, λ1 = d where d is the degree of vertices of G. In some contexts, the spectral gap of G is defined to be d − λ2. In other contexts, the spectral gap refers to d − λ, where

λ = max{ | λ2 | , | λn | }.

The normalized version of this definition, where we consider the matrix A / d and get eigenvalues between −1 and 1, is also widely used and more convenient in the statement of some results.

Frequently one will refer to the second-largest eigenvalue of a graph G, which refers to the max of and . Depending on the context it may be either the normalized version (taking value between 0 and 1) or the un-normalized version (taking value between 0 and d).

For not necessarily regular graphs, the spectrum of a graph can be defined similarly using its Laplacian matrix.

- [edit] Expander families

A family of d-regular graphs is an edge expander family if there is a constant c > 0 such that for each . Typically we want d to be constant, though it is sometimes also interesting to consider d = log | G | or even d = | G | O(1). Similarly, is a vertex expander family if there is a constant c > 1 such that for each , and is a spectral expander family if some positive constant is a lower bound for the spectral gap of each .

These definitions can be extended to the case of directed graphs.[citation needed] A directed graph can also be interpreted as a balanced bipartite graph (with all edges going from one copy of V to another copy). The definition of bipartite expanders can further be generalized to the case of unbalanced bipartite graphs.[citation needed]

- [edit] Relationships between different expansion properties

The expansion parameters defined above are related to each other. In particular, for any d-regular graph G,

Consequently, for constant degree graphs, vertex and edge expansion are qualitatively the same.

- [edit] Cheeger Inequalities

When G is d-regular, there is a relationship between h(G) and the spectral gap d − λ2 of G. An inequality "due to Cheeger and Buser in the continuous case and to Tanner, Alon, and Milman in the discrete case"[6] states that

[7]

This inequality is closely related to the Cheeger bound for Markov chains and can be seen as a discrete version of Cheeger's Inequality in Riemannian geometry.

Similar connections between vertex isoperimetric numbers and the spectral gap have also been studied:[8]

Asymptotically speaking, the quantities , hout, and are all upper bounded by the spectral gap O(d − λ2).

- [edit] Examples of expanders

A random d-regular graph has good expansion, with high probability. Ramanujan graphs are a family of d-regular expander graphs with d being constant and with explicit constructions, that have essentially the largest possible spectral gap. Algebraic constructions based on Cayley graphs are known for various variants of expander graphs. Most recently, combinatorial constructions of expanders have also been discovered.

- [edit] Applications and useful properties

The original motivation for expanders is to build economical robust networks (phone or computer): an expander with bounded valence is precisely an asymptotic robust graph with number of edges growing linearly with size (number of vertices), for all subsets.

Expander graphs have found extensive applications in computer science, in designing algorithms, error correcting codes, extractors, pseudorandom generators, sorting networks (Ajtai, Komlós & Szemerédi (1983)) and robust computer networks. They have also been used in proofs of many important results in computational complexity theory, such as SL=L (Reingold (2008)) and the PCP theorem (Dinur (2007)). In cryptography too, expander graphs are used to construct hash functions.

The following are some properties of expander graphs that have proven useful in many areas.

- [edit] Expander mixing lemma

Main article: Expander mixing lemma

The expander mixing lemma states that, for any two subsets of the vertices , the number of edges between S and T is approximately what you would expect in a random d-regular graph, i.e. .

- [edit] Expander walk sampling

Main article: Expander walk sampling

The Chernoff bound states that, when sampling many independent samples from a random variables in the range [ − 1,1], with high probability the average of our samples is close to the expectation of the random variable. The expander walk sampling lemma, due to Ajtai, Komlós & Szemerédi (1987) and Gillman (1998), states that this also holds true when sampling from a walk on an expander graph. This is particularly useful in the theory of derandomization, since sampling according to an expander walk uses much fewer random bits than sampling independently.

- [edit] See also

- Algebraic connectivity

- Zig-zag product

- [edit] Notes

- ^ Hoory, Linial & Widgerson (2006)

- ^ Definition 2.1 in Hoory, Linial & Widgerson (2006)

- ^ Bobkov, Houdré & Tetali (2000)

- ^ Bobkov, Houdré & Tetali (2000)

- ^ cf. Section 2.3 in Hoory, Linial & Widgerson (2006)

- ^ See [1]

- ^ Theorem 2.4 in Hoory, Linial & Widgerson (2006)

- ^ Theorem 1 in Bobkov, Houdré & Tetali (2000)

- [edit] References

Textbooks and surveys

- Chung, Fan R. K. (1997), Spectral Graph Theory, CBMS Regional Conference Series in Mathematics, 92, American Mathematical Society, ISBN 0-8218-0315-8

- Davidoff, Guiliana; Sarnak, Peter; Valette, Alain (2003), Elementary number theory, group theory and Ramanjuan graphs, LMS student texts, 55, Cambridge University Press, ISBN 0-521-53143-8

- Hoory, Shlomo; Linial, Nathan; Widgerson, Avi (2006), "Expander graphs and their applications", Bulletin (New series) of the American Mathematical Society 43 (4): 439–561, doi:10.1090/S0273-0979-06-01126-8, http://www.cs.huji.ac.il/~nati/PAPERS/expander_survey.pdf

Research articles

- Ajtai, M.; Komlós, J.; Szemerédi, E. (1983), "An O(n log n) sorting network", Proceedings of the 15th Annual ACM Symposium on Theory of Computing, pp. 1–9, doi:10.1145/800061.808726

- Ajtai, M.; Komlós, J.; Szemerédi, E. (1987), "Deterministic simulation in LOGSPACE", Proceedings of the 19th Annual ACM Symposium on Theory of Computing, pp. 132–140, doi:10.1145/28395.28410

- Bobkov, S.; Houdré, C.; Tetali, P. (2000), "λ∞, vertex isoperimetry and concentration", Combinatorica 20 (2): {153--172, doi:10.1007/s004930070018 }.

- Dinur, Irit (2007), "The PCP theorem by gap amplification", Journal of the ACM 54 (3): 12, doi:10.1145/1236457.1236459 .

- Gillman, D. (1998), "A Chernoff Bound for Random Walks on Expander Graphs", SIAM Journal on Computing (Society for Industrial and Applied Mathematics) 27 (4,): 1203–1220, doi:10.1137/S0097539794268765

- Reingold, Omer (2008), "Undirected connectivity in log-space", Journal of the ACM 55 (4): Article 17, 24 pages, doi:10.1145/1391289.1391291

- [edit] External links

- Brief introduction in Notices of the American Mathematical Society

- Introductory paper by Michael Nielsen

- Lecture notes from a course on expanders (by Nati Linial and Avi Wigderson)

- Lecture notes from yet another course on expanders (by Prahladh Harsha)

- Definition and application of spectral gap

Retrieved from "http://en.wikipedia.org/wiki/Expander_graph"

From Wikipedia, the free encyclopedia

Jump to: navigation, search

Commemorative plaque listing some of the early Internet pioneers

The concept of data communication - transmitting data between two different places, connected via some kind of electromagnetic medium, such as radio or an electrical wire - actually predates the introduction of the first computers. Such communication systems were typically limited to point to point communication between two end devices. Telegraph systems and telex machines can be considered early precursors of this kind of communication. The earlier computers used the technology available at the time to allow communication between the central processing unit and remote terminals. As the technology evolved new systems were devised to allow communication over longer distances (for terminals) or with higher speed (for interconnection of local devices) that were necessary for the mainframe computer model. Using these technologies it was possible to exchange data (such as files) between remote computers. However, the point to point communication model was limited, as it did not allow for direct communication between any two arbitrary systems; a physical link was necessary. The technology was also deemed as inherently unsafe for strategic and military use, because there were no alternative paths for the communication in case of an enemy attack.

As a response, several research programs started to explore and articulate principles of communications between physically separate systems, leading to the development of the packet switching model of digital networking. These research efforts included those of the laboratories of Vinton G. Cerf at Stanford University, Donald Davies (NPL), Paul Baran (RAND Corporation), and Leonard Kleinrock at MIT and at UCLA. The research led to the development of several packet-switched networking solutions in the late 1960s and 1970s, including ARPANET, Telenet, and the X.25 protocols. Additionally, public access and hobbyist networking systems grew in popularity, including unix-to-unix copy (UUCP) and FidoNet. They were however still disjointed separate networks, served only by limited gateways between networks. This led to the application of packet switching to develop a protocol for internetworking, where multiple different networks could be joined together into a super-framework of networks. By defining a simple common network system, the Internet Protocol Suite, the concept of the network could be separated from its physical implementation. This spread of internetworking began to form into the idea of a global network that would be called the Internet, based on standardized protocols officially implemented in 1982. Adoption and interconnection occurred quickly across the advanced telecommunication networks of the western world, and then began to penetrate into the rest of the world as it became the de-facto international standard for the global network. However, the disparity of growth between advanced nations and the third-world countries led to a digital divide that is still a concern today.

Following commercialization and introduction of privately run Internet service providers in the 1980s, and the Internet's expansion for popular use in the 1990s, the Internet has had a drastic impact on culture and commerce. This includes the rise of near instant communication by electronic mail (e-mail), text based discussion forums, and the World Wide Web. Investor speculation in new markets provided by these innovations would also lead to the inflation and subsequent collapse of the Dot-com bubble. But despite this, the Internet continues to grow, driven by commerce, greater amounts of online information and knowledge and social networking known as Web 2.0.

History of computing

Hardware before 1960

Hardware 1960s to present

Hardware in Soviet Bloc countries

Artificial intelligence

Computer science

Operating systems

Programming languages

Software engineering

Graphical user interface

Internet

Personal computers

Laptops

Video games

World Wide Web

Timeline of computing - 2400 BC–1949 - 1950–1979 - 1980–1989 - 1990–1999 - 2000–2009 - More timelines...

More...

- Contents [hide] - 1 Three terminals and an ARPA - 2 Packet switching - 3 Networks that led to the Internet - 3.1 ARPANET - 3.2 X.25 and public access - 3.3 UUCP - 3.4 NPL - 4 Merging the networks and creating the Internet - 4.1 TCP/IP - 4.2 ARPANET to several federal wide area networks: MILNET, NSI, and NSFNet - 4.3 Transition towards the Internet - 5 TCP/IP becomes worldwide - 5.1 CERN, the European Internet, the link to the Pacific and beyond - 5.2 Digital divide - 5.2.1 Africa - 5.2.2 Asia and Oceania - 5.2.3 Latin America - 6 Opening the network to commerce - 6.1 Internet Engineering Task Force - 6.2 NIC, InterNIC, IANA and ICANN - 6.3 Globalization and 21st century - 7 Use and culture - 7.1 E-mail and Usenet - 7.2 From gopher to the WWW - 7.3 Search engines - 7.4 Dot-com bubble - 7.5 Online population forecast - 7.6 Mobile phones and the Internet - 8 Historiography - 9 See also - 10 References - 11 Further reading - 12 External links

- [edit] Three terminals and an ARPA

Main articles: RAND and ARPANET

In the 1950s and early 1960s, before the widespread inter-networking that led to the Internet, most communication networks were limited in that they only allowed communications between the stations on the network. Some networks had gateways or bridges between them, but these bridges were often limited or built specifically for a single use. One prevalent computer networking method was based on the central mainframe method, simply allowing its terminals to be connected via long leased lines. This method was used in the 1950s by Project RAND to support researchers such as Herbert Simon, at Carnegie Mellon University in Pittsburgh, Pennsylvania, when collaborating across the continent with researchers in Sullivan, Illinois, on automated theorem proving and artificial intelligence.

A fundamental pioneer in the call for a global network, J.C.R. Licklider, articulated the ideas in his January 1960 paper, Man-Computer Symbiosis.

"A network of such [computers], connected to one another by wide-band communication lines [which provided] the functions of present-day libraries together with anticipated advances in information storage and retrieval and [other] symbiotic functions."

—J.C.R. Licklider, [1]

In August, 1962, Licklider and Welden Clark published the paper "On-Line Man Computer Communication", one of the first descriptions of a networked future.

In October, 1962, Licklider was hired by Jack Ruina as Director of the newly established IPTO within DARPA, with a mandate to interconnect the United States Department of Defense's main computers at Cheyenne Mountain, the Pentagon, and SAC HQ. There he formed an informal group within DARPA to further computer research. He began by writing memos describing a distributed network to the IPTO staff, whom he called "Members and Affiliates of the Intergalactic Computer Network". As part of the information processing office's role, three network terminals had been installed: one for System Development Corporation in Santa Monica, one for Project Genie at the University of California, Berkeley and one for the Compatible Time-Sharing System project at the Massachusetts Institute of Technology (MIT). Licklider's identified need for inter-networking would be made obvious by the apparent waste of resources this caused.

"For each of these three terminals, I had three different sets of user commands. So if I was talking online with someone at S.D.C. and I wanted to talk to someone I knew at Berkeley or M.I.T. about this, I had to get up from the S.D.C. terminal, go over and log into the other terminal and get in touch with them. [...]

I said, it's obvious what to do (But I don't want to do it): If you have these three terminals, there ought to be one terminal that goes anywhere you want to go where you have interactive computing. That idea is the ARPAnet."

—Robert W. Taylor, co-writer with Licklider of "The Computer as a Communications Device", in an interview with the New York Times, [2]

Although he left the IPTO in 1964, five years before the ARPANET went live, it was his vision of universal networking that provided the impetus that led his successors such as Lawrence Roberts and Robert Taylor to further the ARPANET development. Licklider later returned to lead the IPTO in 1973 for two years.[3]

- [edit] Packet switching

Main article: Packet switching

At the tip of the problem lay the issue of connecting separate physical networks to form one logical network. During the 1960s, Paul Baran (RAND Corporation), produced a study of surviveable networks for the US military. Information transmitted across Baran's network would be divided into what he called 'message-blocks'. Independently, Donald Davies (National Physical Laboratory, UK), proposed and developed a similar network based on what he called packet-switching, the term that would ultimately be adopted. Leonard Kleinrock (MIT) developed mathematical theory behind this technology. Packet-switching provides better bandwidth utilization and response times than the traditional circuit-switching technology used for telephony, particularly on resource-limited interconnection links.[4]

Packet switching is a rapid store-and-forward networking design that divides messages up into arbitrary packets, with routing decisions made per-packet. Early networks used message switched systems that required rigid routing structures prone to single point of failure. This led Tommy Krash and Paul Baran's US Military funded research to focus on using message-blocks to include network redundancy,[5] which in turn led to the widespread urban legend that the Internet was designed to resist nuclear attack.[6][7]

- [edit] Networks that led to the Internet

- [edit] ARPANET

Main article: ARPANET

Len Kleinrock and the first Interface Message Processor.[8]

Promoted to the head of the information processing office at DARPA, Robert Taylor intended to realize Licklider's ideas of an interconnected networking system. Bringing in Larry Roberts from MIT, he initiated a project to build such a network. The first ARPANET link was established between the University of California, Los Angeles and the Stanford Research Institute on 22:30 hours on October 29, 1969. By December 5, 1969, a 4-node network was connected by adding the University of Utah and the University of California, Santa Barbara. Building on ideas developed in ALOHAnet, the ARPANET grew rapidly. By 1981, the number of hosts had grown to 213, with a new host being added approximately every twenty days.[9][10]

ARPANET became the technical core of what would become the Internet, and a primary tool in developing the technologies used. ARPANET development was centered around the Request for Comments (RFC) process, still used today for proposing and distributing Internet Protocols and Systems. RFC 1, entitled "Host Software", was written by Steve Crocker from the University of California, Los Angeles, and published on April 7, 1969. These early years were documented in the 1972 film Computer Networks: The Heralds of Resource Sharing.

International collaborations on ARPANET were sparse. For various political reasons, European developers were concerned with developing the X.25 networks. Notable exceptions were the Norwegian Seismic Array (NORSAR) in 1972, followed in 1973 by Sweden with satellite links to the Tanum Earth Station and Peter Kirstein's research group in the UK, initially at the Institute of Computer Science, London University and later at University College London.[11]

- [edit] X.25 and public access

Main articles: X.25, Bulletin board system, and FidoNet

Based on ARPA's research, packet switching network standards were developed by the International Telecommunication Union (ITU) in the form of X.25 and related standards. While using packet switching, X.25 is built on the concept of virtual circuits emulating traditional telephone connections. In 1974, X.25 formed the basis for the SERCnet network between British academic and research sites, which later became JANET. The initial ITU Standard on X.25 was approved in March 1976.[12]

The British Post Office, Western Union International and Tymnet collaborated to create the first international packet switched network, referred to as the International Packet Switched Service (IPSS), in 1978. This network grew from Europe and the US to cover Canada, Hong Kong and Australia by 1981. By the 1990s it provided a worldwide networking infrastructure.[13]

Unlike ARPANET, X.25 was commonly available for business use. Telenet offered its Telemail electronic mail service, which was also targeted to enterprise use rather than the general email system of the ARPANET.

The first public dial-in networks used asynchronous TTY terminal protocols to reach a concentrator operated in the public network. Some networks, such as CompuServe, used X.25 to multiplex the terminal sessions into their packet-switched backbones, while others, such as Tymnet, used proprietary protocols. In 1979, CompuServe became the first service to offer electronic mail capabilities and technical support to personal computer users. The company broke new ground again in 1980 as the first to offer real-time chat with its CB Simulator. Other major dial-in networks were America Online (AOL) and Prodigy that also provided communications, content, and entertainment features. Many bulletin board system (BBS) networks also provided on-line access, such as FidoNet which was popular amongst hobbyist computer users, many of them hackers and amateur radio operators.[citation needed]

- [edit] UUCP

Main articles: UUCP and Usenet

In 1979, two students at Duke University, Tom Truscott and Jim Ellis, came up with the idea of using simple Bourne shell scripts to transfer news and messages on a serial line with nearby University of North Carolina at Chapel Hill. Following public release of the software, the mesh of UUCP hosts forwarding on the Usenet news rapidly expanded. UUCPnet, as it would later be named, also created gateways and links between FidoNet and dial-up BBS hosts. UUCP networks spread quickly due to the lower costs involved, ability to use existing leased lines, X.25 links or even ARPANET connections, and the lack of strict use policies (commercial organizations who might provide bug fixes) compared to later networks like CSnet and Bitnet. All connects were local. By 1981 the number of UUCP hosts had grown to 550, nearly doubling to 940 in 1984. - Sublink Network, operating since 1987 and officially founded in Italy in 1989, based its interconnectivity upon UUCP to redistribute mail and news groups messages throughout its Italian nodes (about 100 at the time) owned both by private individuals and small companies. Sublink Network represented possibly one of the first examples of the internet technology becoming progress through popular diffusion.[14]

- [edit] NPL

Main articles: National Physical Laboratory, UK and DCN

In 1965, Donald Davies of the National Physical Laboratory (United Kingdom) proposed a national data network based on packet-switching. The proposal was not taken up nationally but by 1970 he had designed and built a packet-switched network to meet the needs of the multidisciplinary laboratory and prove the technology under operational conditions.[15] By 1976 12 computers and 75 terminal devices were attached and more were added until the network was replaced in 1986.

- [edit] Merging the networks and creating the Internet

- [edit] TCP/IP

Main article: Internet Protocol Suite

Map of the TCP/IP test network in February 1982

With so many different network methods, something was needed to unify them. Robert E. Kahn of DARPA and ARPANET recruited Vinton Cerf of Stanford University to work with him on the problem. By 1973, they had soon worked out a fundamental reformulation, where the differences between network protocols were hidden by using a common internetwork protocol, and instead of the network being responsible for reliability, as in the ARPANET, the hosts became responsible. Cerf credits Hubert Zimmerman, Gerard LeLann and Louis Pouzin (designer of the CYCLADES network) with important work on this design.[16]

The specification of the resulting protocol, RFC 675 - Specification of Internet Transmission Control Program, by Vinton Cerf, Yogen Dalal and Carl Sunshine, Network Working Group, December, 1974, contains the first attested use of the term internet, as a shorthand for internetworking; later RFCs repeat this use, so the word started out as an adjective rather than the noun it is today.

With the role of the network reduced to the bare minimum, it became possible to join almost any networks together, no matter what their characteristics were, thereby solving Kahn's initial problem. DARPA agreed to fund development of prototype software, and after several years of work, the first somewhat crude demonstration of a gateway between the Packet Radio network in the SF Bay area and the ARPANET was conducted. On November 22, 1977[17] a three network demonstration was conducted including the ARPANET, the Packet Radio Network and the Atlantic Packet Satellite network—all sponsored by DARPA. Stemming from the first specifications of TCP in 1974, TCP/IP emerged in mid-late 1978 in nearly final form. By 1981, the associated standards were published as RFCs 791, 792 and 793 and adopted for use. DARPA sponsored or encouraged the development of TCP/IP implementations for many operating systems and then scheduled a migration of all hosts on all of its packet networks to TCP/IP. On January 1, 1983, TCP/IP protocols became the only approved protocol on the ARPANET, replacing the earlier NCP protocol.[18]

- [edit] ARPANET to several federal wide area networks: MILNET, NSI, and NSFNet

Main articles: ARPANET and NSFNet

After the ARPANET had been up and running for several years, ARPA looked for another agency to hand off the network to; ARPA's primary mission was funding cutting edge research and development, not running a communications utility. Eventually, in July 1975, the network had been turned over to the Defense Communications Agency, also part of the Department of Defense. In 1983, the U.S. military portion of the ARPANET was broken off as a separate network, the MILNET. MILNET subsequently became the unclassified but military-only NIPRNET, in parallel with the SECRET-level SIPRNET and JWICS for TOP SECRET and above. NIPRNET does have controlled security gateways to the public Internet.

The networks based on the ARPANET were government funded and therefore restricted to noncommercial uses such as research; unrelated commercial use was strictly forbidden. This initially restricted connections to military sites and universities. During the 1980s, the connections expanded to more educational institutions, and even to a growing number of companies such as Digital Equipment Corporation and Hewlett-Packard, which were participating in research projects or providing services to those who were.

BBN Technologies TCP/IP internet map early 1986

Several other branches of the U.S. government, the National Aeronautics and Space Agency (NASA), the National Science Foundation (NSF), and the Department of Energy (DOE) became heavily involved in Internet research and started development of a successor to ARPANET. In the mid 1980s, all three of these branches developed the first Wide Area Networks based on TCP/IP. NASA developed the NASA Science Network, NSF developed CSNET and DOE evolved the Energy Sciences Network or ESNet.

In 1984 NSF developed CSNET exclusively based on TCP/IP. CSNET connected with ARPANET using TCP/IP, and ran TCP/IP over X.25, but it also supported departments without sophisticated network connections, using automated dial-up mail exchange. This grew into the NSFNet backbone, established in 1986, and intended to connect and provide access to a number of supercomputing centers established by the NSF.[19]

- [edit] Transition towards the Internet

The term "internet" was adopted in the first RFC published on the TCP protocol (RFC 675:[20] Internet Transmission Control Program, December 1974) as an abbreviation of the term internetworking and the two terms were used interchangeably. In general, an internet was any network using TCP/IP. It was around the time when ARPANET was interlinked with NSFNet in the late 1980s, that the term was used as the name of the network, Internet,[21] being a large and global TCP/IP network.

As interest in wide spread networking grew and new applications for it were developed, the Internet's technologies spread throughout the rest of the world. The network-agnostic approach in TCP/IP meant that it was easy to use any existing network infrastructure, such as the IPSS X.25 network, to carry Internet traffic. In 1984, University College London replaced its transatlantic satellite links with TCP/IP over IPSS.[22]

Many sites unable to link directly to the Internet started to create simple gateways to allow transfer of e-mail, at that time the most important application. Sites which only had intermittent connections used UUCP or FidoNet and relied on the gateways between these networks and the Internet. Some gateway services went beyond simple e-mail peering, such as allowing access to FTP sites via UUCP or e-mail.

Finally, the Internet's remaining centralized routing aspects were removed. The EGP routing protocol was replaced by a new protocol, the Border Gateway Protocol (BGP), in order to allow the removal of the NSFNet Internet backbone network. In 1994, Classless Inter-Domain Routing was introduced to support better conservation of address space which allowed use of route aggregation to decrease the size of routing tables.[23] The picture on the right hand side shows a system made with the help of the high-tech company BBN.

- [edit] TCP/IP becomes worldwide

- [edit] CERN, the European Internet, the link to the Pacific and beyond

Between 1984 and 1988 CERN began installation and operation of TCP/IP to interconnect its major internal computer systems, workstations, PCs and an accelerator control system. CERN continued to operate a limited self-developed system CERNET internally and several incompatible (typically proprietary) network protocols externally. There was considerable resistance in Europe towards more widespread use of TCP/IP and the CERN TCP/IP intranets remained isolated from the Internet until 1989.

In 1988 Daniel Karrenberg, from Centrum Wiskunde & Informatica (CWI) in Amsterdam, visited Ben Segal, CERN's TCP/IP Coordinator, looking for advice about the transition of the European side of the UUCP Usenet network (much of which ran over X.25 links) over to TCP/IP. In 1987, Ben Segal had met with Len Bosack from the then still small company Cisco about purchasing some TCP/IP routers for CERN, and was able to give Karrenberg advice and forward him on to Cisco for the appropriate hardware. This expanded the European portion of the Internet across the existing UUCP networks, and in 1989 CERN opened its first external TCP/IP connections.[24] This coincided with the creation of Réseaux IP Européens (RIPE), initially a group of IP network administrators who met regularly to carry out co-ordination work together. Later, in 1992, RIPE was formally registered as a cooperative in Amsterdam.

At the same time as the rise of internetworking in Europe, ad hoc networking to ARPA and in-between Australian universities formed, based on various technologies such as X.25 and UUCPNet. These were limited in their connection to the global networks, due to the cost of making individual international UUCP dial-up or X.25 connections. In 1989, Australian universities joined the push towards using IP protocols to unify their networking infrastructures. AARNet was formed in 1989 by the Australian Vice-Chancellors' Committee and provided a dedicated IP based network for Australia.

The Internet began to penetrate Asia in the late 1980s. Japan, which had built the UUCP-based network JUNET in 1984, connected to NSFNet in 1989. It hosted the annual meeting of the Internet Society, INET'92, in Kobe. Singapore developed TECHNET in 1990, and Thailand gained a global Internet connection between Chulalongkorn University and UUNET in 1992.[25]

- [edit] Digital divide

Main article: Digital divide

While developed countries with technological infrastructures were joining the Internet, developing countries began to experience a digital divide separating them from the Internet. On an essentially continental basis, they are building organizations for Internet resource administration and sharing operational experience, as more and more transmission facilities go into place.

- [edit] Africa

At the beginning of the 1990s, African countries relied upon X.25 IPSS and 2400 baud modem UUCP links for international and internetwork computer communications.

In August, 1995, InfoMail Uganda, Ltd., a privately held firm in Kampala now known as InfoCom <http://www.imul.com>, and NSN Network Services of Avon, Colorado, sold in 1997 and now known as Clear Channel Satellite, established Africa's first native TCP/IP high-speed satellite Internet services. The data connection was originally carried by a C-Band RSCC Russian satellite which connected InfoMail's Kampala offices directly to NSN's MAE-West point of presence using a private network from NSN's leased ground station in New Jersey. InfoCom's first satellite connection was just 64kbps, serving a Sun host computer and twelve US Robotics dial-up modems.

In 1996 a USAID funded project, the Leland initiative, started work on developing full Internet connectivity for the continent. Guinea, Mozambique, Madagascar and Rwanda gained satellite earth stations in 1997, followed by Côte d'Ivoire and Benin in 1998.

Africa is building an Internet infrastructure. AfriNIC, headquartered in Mauritius, manages IP address allocation for the continent. As do the other Internet regions, there is an operational forum, the Internet Community of Operational Networking Specialists.[26]

There are a wide range of programs both to provide high-performance transmission plant, and the western and southern coasts have undersea optical cable. High-speed cables join North Africa and the Horn of Africa to intercontinental cable systems. Undersea cable development is slower for East Africa; the original joint effort between New Partnership for Africa's Development (NEPAD) and the East Africa Submarine System (Eassy) has broken off and may become two efforts.[27]

- [edit] Asia and Oceania

The Asia Pacific Network Information Centre (APNIC), headquartered in Australia, manages IP address allocation for the continent. APNIC sponsors an operational forum, the Asia-Pacific Regional Internet Conference on Operational Technologies (APRICOT).[28]

In 1991, the People's Republic of China saw its first TCP/IP college network, Tsinghua University's TUNET. The PRC went on to make its first global Internet connection in 1994, between the Beijing Electro-Spectrometer Collaboration and Stanford University's Linear Accelerator Center. However, China went on to implement its own digital divide by implementing a country-wide content filter.[29]

- [edit] Latin America

As with the other regions, the Latin American and Caribbean Internet Addresses Registry (LACNIC) manages the IP address space and other resources for its area. LACNIC, headquartered in Uruguay, operates DNS root, reverse DNS, and other key services.

- [edit] Opening the network to commerce

The interest in commercial use of the Internet became a hotly debated topic. Although commercial use was forbidden, the exact definition of commercial use could be unclear and subjective. UUCPNet and the X.25 IPSS had no such restrictions, which would eventually see the official barring of UUCPNet use of ARPANET and NSFNet connections. Some UUCP links still remained connecting to these networks however, as administrators cast a blind eye to their operation.

During the late 1980s, the first Internet service provider (ISP) companies were formed. Companies like PSINet, UUNET, Netcom, and Portal Software were formed to provide service to the regional research networks and provide alternate network access, UUCP-based email and Usenet News to the public. The first commercial dialup ISP in the United States was The World, opened in 1989.[30]

In 1992, Congress allowed commercial activity on NSFNet with the Scientific and Advanced-Technology Act, 42 U.S.C. § 1862(g), permitting NSFNet to interconnect with commercial networks.[31] This caused controversy amongst university users, who were outraged at the idea of noneducational use of their networks.[citation needed] Eventually, it was the commercial Internet service providers who brought prices low enough that junior colleges and other schools could afford to participate in the new arenas of education and research.[citation needed]

By 1990, ARPANET had been overtaken and replaced by newer networking technologies and the project came to a close. In 1994, the NSFNet, now renamed ANSNET (Advanced Networks and Services) and allowing non-profit corporations access, lost its standing as the backbone of the Internet. Both government institutions and competing commercial providers created their own backbones and interconnections. Regional network access points (NAPs) became the primary interconnections between the many networks. The final commercial restrictions ended in May 1995 when the National Science Foundation ended its sponsorship of the Internet backbone.[32]

- [edit] Internet Engineering Task Force

Main article: Internet Engineering Task Force

Requests for Comments (RFCs) started as memoranda addressing the various protocols that facilitate the functioning of the Internet and were previously edited by the late Dr. Postel as part of his IANA functions.[33]

The IETF started in January 1985 as a quarterly meeting of U.S. government funded researchers. Representatives from non-government vendors were invited starting with the fourth IETF meeting in October of that year.[citation needed] In 1992, the Internet Society, a professional membership society, was formed and the IETF was transferred to operation under it as an independent international standards body.[citation needed]

- [edit] NIC, InterNIC, IANA and ICANN

Main articles: InterNIC, Internet Assigned Numbers Authority, and ICANN

The first central authority to coordinate the operation of the network was the Network Information Centre (NIC) at Stanford Research Institute (SRI) in Menlo Park, California. In 1972, management of these issues was given to the newly created Internet Assigned Numbers Authority (IANA). In addition to his role as the RFC Editor, Jon Postel worked as the manager of IANA until his death in 1998.

As the early ARPANET grew, hosts were referred to by names, and a HOSTS.TXT file would be distributed from SRI International to each host on the network. As the network grew, this became cumbersome. A technical solution came in the form of the Domain Name System, created by Paul Mockapetris. The Defense Data Network—Network Information Center (DDN-NIC) at SRI handled all registration services, including the top-level domains (TLDs) of .mil, .gov, .edu, .org, .net, .com and .us, root nameserver administration and Internet number assignments under a United States Department of Defense contract.[34] In 1991, the Defense Information Systems Agency (DISA) awarded the administration and maintenance of DDN-NIC (managed by SRI up until this point) to Government Systems, Inc., who subcontracted it to the small private-sector Network Solutions, Inc.[35][36]

Since at this point in history most of the growth on the Internet was coming from non-military sources, it was decided that the Department of Defense would no longer fund registration services outside of the .mil TLD. In 1993 the U.S. National Science Foundation, after a competitive bidding process in 1992, created the InterNIC to manage the allocations of addresses and management of the address databases, and awarded the contract to three organizations. Registration Services would be provided by Network Solutions; Directory and Database Services would be provided by AT&T; and Information Services would be provided by General Atomics.[37]

In 1998 both IANA and InterNIC were reorganized under the control of ICANN, a California non-profit corporation contracted by the United States Department of Commerce to manage a number of Internet-related tasks. The role of operating the DNS system was privatized and opened up to competition, while the central management of name allocations would be awarded on a contract tender basis.

- [edit] Globalization and 21st century

Main article: Internet governance

Since the 1990s, the Internet's governance and organization has been of global importance to commerce. The organizations which hold control of certain technical aspects of the Internet are both the successors of the old ARPANET oversight and the current decision-makers in the day-to-day technical aspects of the network. While formally recognized as the administrators of the network, their roles and their decisions are subject to international scrutiny and objections which limit them. These objections have led to the ICANN removing themselves from relationships with first the University of Southern California in 2000,[38] and finally in September 2009, gaining autonomy from the US government by the ending of its longstanding agreements, although some contractual obligations with the Department of Commerce continue until at least 2011.[39][40][41] The history of the Internet will now be played out in many ways as a consequence of the ICANN organization.

In the role of forming standard associated with the Internet, the IETF continues to serve as the ad-hoc standards group. They continue to issue Request for Comments numbered sequentially from RFC 1 under the ARPANET project, for example, and the IETF precursor was the GADS Task Force which was a group of US government-funded researchers in the 1980s. Many of the group's recent developments have been of global necessity, such as the i18n working groups who develop things like internationalized domain names. The Internet Society has helped to fund the IETF, providing limited oversight.

- [edit] Use and culture

- [edit] E-mail and Usenet

Main articles: e-mail, Simple Mail Transfer Protocol, and Usenet

E-mail is often called the killer application of the Internet. However, it actually predates the Internet and was a crucial tool in creating it. E-mail started in 1965 as a way for multiple users of a time-sharing mainframe computer to communicate. Although the history is unclear, among the first systems to have such a facility were SDC's Q32 and MIT's CTSS.[42]

The ARPANET computer network made a large contribution to the evolution of e-mail. There is one report[43] indicating experimental inter-system e-mail transfers on it shortly after ARPANET's creation. In 1971 Ray Tomlinson created what was to become the standard Internet e-mail address format, using the @ sign to separate user names from host names.[44]

A number of protocols were developed to deliver e-mail among groups of time-sharing computers over alternative transmission systems, such as UUCP and IBM's VNET e-mail system. E-mail could be passed this way between a number of networks, including ARPANET, BITNET and NSFNet, as well as to hosts connected directly to other sites via UUCP. See the history of SMTP protocol.

In addition, UUCP allowed the publication of text files that could be read by many others. The News software developed by Steve Daniel and Tom Truscott in 1979 was used to distribute news and bulletin board-like messages. This quickly grew into discussion groups, known as newsgroups, on a wide range of topics. On ARPANET and NSFNet similar discussion groups would form via mailing lists, discussing both technical issues and more culturally focused topics (such as science fiction, discussed on the sflovers mailing list).

During the early years of the Internet, e-mail and similar mechanisms were also fundamental to allow people to access resources that were not available due to the absence of online connectivity. UUCP was often used to distribute files using the 'alt.binary' groups. Also, FTP e-mail gateways allowed people that lived outside the US and Europe to download files using ftp commands written inside e-email messages. The file was encoded, broken in pieces and sent by e-mail; the receiver had to reassemble and decode it later, and it was the only way for people living overseas to download items such as the earlier Linux versions using the slow dial-up connections available at the time. After the popularization of the Web and the HTTP protocol such tools were slowly abandoned.

- [edit] From gopher to the WWW

Main articles: History of the World Wide Web and World Wide Web

As the Internet grew through the 1980s and early 1990s, many people realized the increasing need to be able to find and organize files and information. Projects such as Gopher, WAIS, and the FTP Archive list attempted to create ways to organize distributed data. Unfortunately, these projects fell short in being able to accommodate all the existing data types and in being able to grow without bottlenecks.[citation needed]

One of the most promising user interface paradigms during this period was hypertext. The technology had been inspired by Vannevar Bush's "Memex"[45] and developed through Ted Nelson's research on Project Xanadu and Douglas Engelbart's research on NLS.[46] Many small self-contained hypertext systems had been created before, such as Apple Computer's HyperCard. Gopher became the first commonly-used hypertext interface to the Internet. While Gopher menu items were examples of hypertext, they were not commonly perceived in that way.

This NeXT Computer was used by Sir Tim Berners-Lee at CERN and became the world's first Web server.

In 1989, while working at CERN, Tim Berners-Lee invented a network-based implementation of the hypertext concept. By releasing his invention to public use, he ensured the technology would become widespread.[47] For his work in developing the World Wide Web, Berners-Lee received the Millennium technology prize in 2004. One early popular web browser, modeled after HyperCard, was ViolaWWW.

A potential turning point for the World Wide Web began with the introduction[48] of the Mosaic web browser[49] in 1993, a graphical browser developed by a team at the National Center for Supercomputing Applications at the University of Illinois at Urbana-Champaign (NCSA-UIUC), led by Marc Andreessen. Funding for Mosaic came from the High-Performance Computing and Communications Initiative, a funding program initiated by the High Performance Computing and Communication Act of 1991 also known as the Gore Bill.[50] Indeed, Mosaic's graphical interface soon became more popular than Gopher, which at the time was primarily text-based, and the WWW became the preferred interface for accessing the Internet. (Gore's reference to his role in "creating the Internet", however, was ridiculed in his presidential election campaign. See the full article Al Gore and information technology).

Mosaic was eventually superseded in 1994 by Andreessen's Netscape Navigator, which replaced Mosaic as the world's most popular browser. While it held this title for some time, eventually competition from Internet Explorer and a variety of other browsers almost completely displaced it. Another important event held on January 11, 1994, was The Superhighway Summit at UCLA's Royce Hall. This was the "first public conference bringing together all of the major industry, government and academic leaders in the field [and] also began the national dialogue about the Information Superhighway and its implications."[51]

24 Hours in Cyberspace, "the largest one-day online event" (February 8, 1996) up to that date, took place on the then-active website, cyber24.com.[52][53] It was headed by photographer Rick Smolan.[54] A photographic exhibition was unveiled at the Smithsonian Institution's National Museum of American History on January 23, 1997, featuring 70 photos from the project.[55]

- [edit] Search engines

Main article: Search engine (computing)

Even before the World Wide Web, there were search engines that attempted to organize the Internet. The first of these was the Archie search engine from McGill University in 1990, followed in 1991 by WAIS and Gopher. All three of those systems predated the invention of the World Wide Web but all continued to index the Web and the rest of the Internet for several years after the Web appeared. There are still Gopher servers as of 2006, although there are a great many more web servers.

As the Web grew, search engines and Web directories were created to track pages on the Web and allow people to find things. The first full-text Web search engine was WebCrawler in 1994. Before WebCrawler, only Web page titles were searched. Another early search engine, Lycos, was created in 1993 as a university project, and was the first to achieve commercial success. During the late 1990s, both Web directories and Web search engines were popular—Yahoo! (founded 1994) and Altavista (founded 1995) were the respective industry leaders. By August 2001, the directory model had begun to give way to search engines, tracking the rise of Google (founded 1998), which had developed new approaches to relevancy ranking. Directory features, while still commonly available, became after-thoughts to search engines.

Database size, which had been a significant marketing feature through the early 2000s, was similarly displaced by emphasis on relevancy ranking, the methods by which search engines attempt to sort the best results first. Relevancy ranking first became a major issue circa 1996, when it became apparent that it was impractical to review full lists of results. Consequently, algorithms for relevancy ranking have continuously improved. Google's PageRank method for ordering the results has received the most press, but all major search engines continually refine their ranking methodologies with a view toward improving the ordering of results. As of 2006, search engine rankings are more important than ever, so much so that an industry has developed ("search engine optimizers", or "SEO") to help web-developers improve their search ranking, and an entire body of case law has developed around matters that affect search engine rankings, such as use of trademarks in metatags.

The sale of search rankings by some search engines has also created controversy among librarians and consumer advocates.[56] As of June 3, 2009, Microsoft launched its own search engine. Bing [1] became immediately popular with the masses searching the internet. It has multiple sites belonging to separate countries e.g. the United States version is different to the Australian version. In the US, Bing ranked 17th among all websites out of over 450,000 websites, up from 5120 the week before the official launch when the website was merely a placeholder. Within the Search Engines category, Bing ranked 4th out of the search engines tracked by Hitwise and Bing Image Search ranked 15th for the week ending June 6, 2009.

- [edit] Dot-com bubble

Main article: Dot-com bubble

Suddenly the low price of reaching millions worldwide, and the possibility of selling to or hearing from those people at the same moment when they were reached, promised to overturn established business dogma in advertising, mail-order sales, customer relationship management, and many more areas. The web was a new killer app—it could bring together unrelated buyers and sellers in seamless and low-cost ways. Visionaries around the world developed new business models, and ran to their nearest venture capitalist. While some of the new entrepreneurs had experience in business in economics, the majority were simply people with ideas, and didn't manage the capital influx prudently. Additionally, many dot-com business plans were predicated on the assumption that by using the Internet, they would bypass the distribution channels of existing businesses and therefore not have to compete with them; when the established businesses with strong existing brands developed their own Internet presence, these hopes were shattered, and the newcomers were left attempting to break into markets dominated by larger, more established businesses. Many did not have the ability to do so.

The dot-com bubble burst on March 10, 2000, when the technology heavy NASDAQ Composite index peaked at 5,048.62[57] (intra-day peak 5,132.52), more than double its value just a year before. By 2001, the bubble's deflation was running full speed. A majority of the dot-coms had ceased trading, after having burnt through their venture capital and IPO capital, often without ever making a profit.

- [edit] Online population forecast

A study conducted by JupiterResearch anticipates that a 38 percent increase in the number of people with online access will mean that, by 2011, 22 percent of the Earth's population will surf the Internet regularly. The report says 1.1 billion people have regular Web access. For the study, JupiterResearch defined online users as people who regularly access the Internet from dedicated Internet-access devices, which exclude cellular telephones.[58]

- [edit] Mobile phones and the Internet

See also: Mobile Web

The first mobile phone with Internet connectivity was the Nokia 9000 Communicator, launched in Finland in 1996. The viability of Internet services access on mobile phones was limited until prices came down from that model and network providers started to develop systems and services conveniently accessible on phones. NTT DoCoMo in Japan launched the first mobile Internet service, i-Mode, in 1999 and this is considered the birth of the mobile phone Internet services. In 2001 the mobile phone email system by Research in Motion for their Blackberry product was launched in America. To make efficient use of the small screen and tiny keypad and one-handed operation typical of mobile phones, a specific document and networking model was created for mobile devices, the Wireless Application Protocol (WAP). Most mobile device Internet services operate using WAP. The growth of mobile phone services was initially a primarily Asian phenomenon with Japan, South Korea and Taiwan all soon finding the majority of their Internet users accessing resources by phone rather than by PC.[citation needed] Developing countries followed, with India, South Africa, Kenya, Philippines and Pakistan all reporting that the majority of their domestic users accessed the Internet from a mobile phone rather than a PC. The European and North American use of the Internet was influenced by a large installed base of personal computers, and the growth of mobile phone Internet access was more gradual, but had reached national penetration levels of 20–30% in most Western countries.[citation needed] The cross-over occurred in 2008, when more Internet access devices were mobile phones than personal computers. In many parts of the developing world, the ratio is as much as 10 mobile phone users to one PC user.[59]

- [edit] Historiography

Some concerns have been raised over the historiography of the Internet's development. Specifically that it is hard to find documentation of much of the Internet's development, for several reasons, including a lack of centralized documentation for much of the early developments that led to the Internet.

"The Arpanet period is somewhat well documented because the corporation in charge - BBN - left a physical record. Moving into the NSFNET era, it became an extraordinarily decentralized process. The record exists in people's basements, in closets. [...] So much of what happened was done verbally and on the basis of individual trust."

—Doug Gale (2007), [60]

- [edit] See also

|  | Book:Internet |
| --- | --- |
| Books are collections of articles that can be downloaded or ordered in print. |  |

- Minitel

- Timeline of popular Internet services

- History of hypertext

- History of Internet in Colombia

- [edit] References

- ^ J. C. R. Licklider (1960). Man-Computer Symbiosis.

- ^ "An Internet Pioneer Ponders the Next Revolution". An Internet Pioneer Ponders the Next Revolution. December 20, 1999. http://partners.nytimes.com/library/tech/99/12/biztech/articles/122099outlook-bobb.html?Partner=Snap. Retrieved November 25, 2005.

- ^ Licklider and the Universal Network

- ^ Ruthfield, Scott, The Internet's History and Development From Wartime Tool to the Fish-Cam, Crossroads 2.1, September 1995.

- ^ "About Rand". Paul Baran and the Origins of the Internet. http://www.rand.org/about/history/baran.html. Retrieved January 14, 2006.

- ^ "How does the Internet Work?". http://tldp.org/HOWTO/Unix-and-Internet-Fundamentals-HOWTO/internet.html. Retrieved June 15, 2009.

- ^ Johna Till Johnson. "Net was born of economic necessity, not fear". http://www.networkworld.com/columnists/2004/0607johnson.html. Retrieved June 15, 2009. URL is sufficient attribution

- ^ Leonard Kleinrock (2005). Leonard Kleinrock's Personal History/Biography: the Birth of the Internet. http://www.lk.cs.ucla.edu/personal_history.html. Retrieved 2009-05-28. "The history of the Internet," http://www.lk.cs.ucla.edu/personal_history.html

- ^ Hafner, Katie (1998). Where Wizards Stay Up Late: The Origins Of The Internet. Simon & Schuster. ISBN 0-68-483267-4.

- ^ Ronda Hauben (2001). From the ARPANET to the Internet. http://www.columbia.edu/~rh120/other/tcpdigest_paper.txt. Retrieved 2009-05-28.

- ^ "NORSAR and the Internet". NORSAR. http://www.norsar.no/pc-5-30-NORSAR-and-the-Internet.aspx. Retrieved 2009-06-05.

- ^ tsbedh. "History of X.25, CCITT Plenary Assemblies and Book Colors". Itu.int. http://www.itu.int/ITU-T/studygroups/com17/history.html. Retrieved 2009-06-05.

- ^ "Events in British Telecomms History". Events in British TelecommsHistory. Archived from the original on 2003-04-05. http://web.archive.org/web/20030405153523/http://www.sigtel.com/tel_hist_brief.html. Retrieved November 25, 2005.

- ^ UUCP Internals Frequently Asked Questions

- ^ Celebrating 40 years of the net

- ^ Barry M. Leiner, Vinton G. Cerf, David D. Clark, Robert E. Kahn, Leonard Kleinrock, Daniel C. Lynch, Jon Postel, Larry G. Roberts, Stephen Wolff (2003). A Brief History of Internet. http://www.isoc.org/internet/history/brief.shtml. Retrieved 2009-05-28.

- ^ "Computer History Museum and Web History Center Celebrate 30th Anniversary of Internet Milestone". http://www.computerhistory.org/about/press_relations/releases/20071101/. Retrieved November 22, 2007.

- ^ Jon Postel, NCP/TCP Transition Plan, RFC 801

- ^ David Roessner, Barry Bozeman, Irwin Feller, Christopher Hill, Nils Newman (1997). The Role of NSF's Support of Engineering in Enabling Technological Innovation. http://www.sri.com/policy/csted/reports/techin/inter2.html. Retrieved 2009-05-28.

- ^ "RFC 675 - SPECIFICATION OF INTERNET TRANSMISSION CONTROL PROGRAM". Tools.ietf.org. http://tools.ietf.org/html/rfc675. Retrieved 2009-05-28.

- ^ Tanenbaum, Andrew S. (1996). Computer Networks. Prentice Hall. ISBN 0-13-394248-1.

- ^ Hauben, Ronda (2004). "The Internet: On its International Origins and Collaborative Vision". Amateur Computerist 12 (2). http://www.ais.org/~jrh/acn/ACn12-2.a03.txt. Retrieved 2009-05-29.

- ^ "RFC 1871 - CIDR and Classful Routing". Tools.ietf.org. http://tools.ietf.org/html/rfc1871. Retrieved 2009-05-28.

- ^ Ben Segal (1995). A Short History of Internet Protocols at CERN. http://www.cern.ch/ben/TCPHIST.html.

- ^ "Internet History in Asia". 16th APAN Meetings/Advanced Network Conference in Busan. http://www.apan.net/meetings/busan03/cs-history.htm. Retrieved December 25, 2005.

- ^ "ICONS webpage". Icons.afrinic.net. http://icons.afrinic.net/. Retrieved 2009-05-28.

- ^ Nepad, Eassy partnership ends in divorce,(South African) Financial Times FMTech, 2007

- ^ "APRICOT webpage". Apricot.net. 2009-05-04. http://www.apricot.net/. Retrieved 2009-05-28.

- ^ "A brief history of the Internet in China". China celebrates 10 years of being connected to the Internet. http://www.pcworld.idg.com.au/index.php/id;854351844;pp;2;fp;2;fpid;1. Retrieved December 25, 2005.

- ^ "The World internet provider". http://www.std.com/. Retrieved 2009-05-28.

- ^ OGC-00-33R Department of Commerce: Relationship with the Internet Corporation for Assigned Names and Numbers. Government Accountability Office. 7 July 2000. p. 6. http://www.gao.gov/new.items/og00033r.pdf.

- ^ "A Brief History of the Internet". http://www.walthowe.com/navnet/history.html.

- ^ OGC-00-33R Department of Commerce: Relationship with the Internet Corporation for Assigned Names and Numbers. Government Accountability Office. 7 July 2000. p. 5. http://www.gao.gov/new.items/og00033r.pdf.

- ^ "DDN NIC". IAB Recommended Policy on Distributing Internet Identifier Assignment. http://www.rfc-editor.org/rfc/rfc1174.txt. Retrieved December 26, 2005.

- ^ "GSI-Network Solutions". TRANSITION OF NIC SERVICES. http://www.rfc-editor.org/rfc/rfc1261.txt. Retrieved December 26, 2005.

- ^ "Thomas v. NSI, Civ. No. 97-2412 (TFH), Sec. I.A. (DCDC April 6, 1998)". Lw.bna.com. http://lw.bna.com/lw/19980428/972412.htm. Retrieved 2009-05-28.

- ^ "NIS Manager Award Announced". NSF NETWORK INFORMATION SERVICES AWARDS. http://www.ripe.net/ripe/maillists/archives/lir-wg/1992/msg00028.html. Retrieved December 25, 2005.

- ^ USC/ICANN Transition Agreement

- ^ ICANN cuts cord to US government, gets broader oversight: ICANN, which oversees the Internet's domain name system, is a private nonprofit that reports to the US Department of Commerce. Under a new agreement, that relationship will change, and ICANN's accountability goes global Nate Anderson, 2009-09-30

- ^ U.S. Eases Grip Over Web Body: Move Addresses Criticisms as Internet Usage Becomes More Global Christopher Rhoads, 2009-10-02

- ^ The U.S. Abandons the Internet: Multilateral governance of the domain name system risks censorship and repression Jeremy Rabkin and Jeffrey Eisenach, 2009-10-02

- ^ "The Risks Digest". Great moments in e-mail history. http://catless.ncl.ac.uk/Risks/20.25.html#subj3. Retrieved April 27, 2006.

- ^ "The History of Electronic Mail". The History of Electronic Mail. http://www.multicians.org/thvv/mail-history.html. Retrieved December 23, 2005.

- ^ "The First Network Email". The First Network Email. http://openmap.bbn.com/~tomlinso/ray/firstemailframe.html. Retrieved December 23, 2005.

- ^ Vannevar Bush (1945). As We May Think. http://www.theatlantic.com/doc/194507/bush. Retrieved 2009-05-28.

- ^ Douglas Engelbart (1962). Augmenting Human Intellect: A Conceptual Framework. http://www.bootstrap.org/augdocs/friedewald030402/augmentinghumanintellect/ahi62index.html.

- ^ "The Early World Wide Web at SLAC". The Early World Wide Web at SLAC: Documentation of the Early Web at SLAC. http://www.slac.stanford.edu/history/earlyweb/history.shtml. Retrieved November 25, 2005.

- ^ "Mosaic Web Browser History - NCSA, Marc Andreessen, Eric Bina". Livinginternet.com. http://www.livinginternet.com/w/wi_mosaic.htm. Retrieved 2009-05-28.

- ^ "NCSA Mosaic - September 10, 1993 Demo". Totic.org. http://www.totic.org/nscp/demodoc/demo.html. Retrieved 2009-05-28.

- ^ "Vice President Al Gore's ENIAC Anniversary Speech". Cs.washington.edu. 1996-02-14. http://www.cs.washington.edu/homes/lazowska/faculty.lecture/innovation/gore.html. Retrieved 2009-05-28.

- ^ "UCLA Center for Communication Policy". Digitalcenter.org. http://www.digitalcenter.org/webreport94/apph.htm. Retrieved 2009-05-28.

- ^ Mirror of Official site map[dead link]

- ^ Mirror of Official Site[dead link]

- ^ ""24 Hours in Cyberspace" (and more)". Baychi.org. http://www.baychi.org/calendar/19970909/. Retrieved 2009-05-28.

- ^ "The human face of cyberspace, painted in random images". Archive.southcoasttoday.com. http://archive.southcoasttoday.com/daily/02-97/02-22-97/b02li072.htm. Retrieved 2009-05-28.

- ^ Stross, Randall, Planet Google: One Company's Audacious Plan to Organize Everything We Know, New York : Free Press, September 2008. ISBN 978-1-4165-4691-7

- ^ Nasdaq peak of 5,048.62

- ^ "Brazil, Russia, India and China to Lead Internet Growth Through 2011". Clickz.com. http://clickz.com/showPage.html?page=3626274. Retrieved 2009-05-28.

- ^ Friedhelm Hillebrand, ed. (2002): "GSM and UMTS, The Creation of Global Mobile Communications", John Wiley & Sons, ISBN 0-470-84322-5

- ^ "An Internet Pioneer Ponders the Next Revolution". Illuminating the net's Dark Ages. August 23, 2007. http://news.bbc.co.uk/1/hi/technology/6959034.stm. Retrieved February 26, 2008.

- [edit] Further reading

- Abbate, Janet. Inventing the Internet. Cambridge: MIT Press, 1999.

- Bemer, Bob, "A History of Source Concepts for the Internet/Web"

- Campbell-Kelly, Martin; Aspray, William. Computer: A History of the Information Machine. New York: BasicBooks, 1996.

- Clark, David D., "The Design Philosophy of the DARPA Internet Protocols", Computer Communications Review 18:4, August 1988, pp. 106–114

- Graham, Ian S. The HTML Sourcebook: The Complete Guide to HTML. New York: John Wiley and Sons, 1995.

- Krol, Ed. Hitchhiker's Guide to the Internet, 1987.

- Krol, Ed. Whole Internet User's Guide and Catalog. O'Reilly & Associates, 1992.

- Scientific American Special Issue on Communications, Computers, and Networks, September, 1991.

- [edit] External links

- Thomas Greene, Larry James Landweber, George Strawn (2003). A Brief History of NSF and the Internet. http://www.nsf.gov/od/lpa/news/03/fsnsf_internet.htm. Retrieved 2009-05-28.

- "Hobbes' Internet Timeline v10". http://www.zakon.org/robert/internet/timeline/. Retrieved July 23, 2010.

- "Internet History: People". Internet History People. http://www.unc.edu/depts/jomc/academics/dri/pioneers2d.html. Retrieved July 3, 2006.

- "Internet History Timeline". Internet History Timeline. http://www.computerhistory.org/exhibits/internet_history/. Retrieved November 25, 2005.

- "Internet History". Internet History. Archived from the original on October 31, 2005. http://web.archive.org/web/20051031200142/http://www.mkaz.com/ebeab/history/. Retrieved November 25, 2005.

- "History of the Internet". History of the Internet. http://www.nic.funet.fi/index/FUNET/history/internet/en/etusivu-en.html. Retrieved June 11, 2008.

- "Internet Society". http://www.isoc.org/internet/history/. Retrieved December 1, 2007.

- "Living Internet". http://www.livinginternet.com/i/ii.htm. Retrieved January 1, 2009.  Internet History with input from many of the people who helped invent the Internet

- "Overhearing the Internet" —by Robert Wright, The New Republic, 1993

- Cybertelecom :: Internet History Focusing on government, legal, and policy history of the Internet

- History of the Internet is an animated documentary explaining the inventions from time-sharing to filesharing, from Arpanet to Internet.

- Internet History

- The History of the Internet According to Itself: symbolism of Online Internet Histories Available at the Turn of the Century

- How It All Started, W3C

- A Little History of the World Wide Web, W3C

- The World Wide Web: Past, Present and Future, Tim Berners-Lee

A home network or home area network (HAN) is a residential local area network. It is used for communication between digital devices typically deployed in the home, usually a small number of personal computers and accessories, such as printers and mobile computing devices. An important function is the sharing of Internet access, often a broadband service through a cable tv or Digital Subscriber Line (DSL) provider. Additionally, a home server may be added for increased functionality.

One of the most common ways of creating a home network is by using wireless technology Wireless LAN (802.11 network as certified by the IEEE) radio signal. An alternate to wireless is wired. This article deals primarily with wireless. Most products that are wireless capable operate at 2.4 GHz under 802.11b and 802.11g or 5 GHz under 802.11a. Also, there are home networking devices that operate within both radioband signals and are under the standard of 802.11n. A Wi-Fi network can be used to connect electronic devices to each other, to the Internet, and to wired networks which use Ethernet technology. [1]. The WiFi Alliance has tested all products on each band and each product is certified for interoperability with previous generations and coexistence with other appliances on the same or different radio wave frequencies.

As an alternative to WIFI, the existing home wiring (coax in North America, phone wires in multi dwelling units (MDU) and powerline in Europe and USA) can be used to access the internet and allow devices to transfer information. With the installation of a home networking device, the network can be accessed by simply plugging the Computer into a wall socket. The ITU-T G.hn and IEEE Powerline standard, which provide high-speed (up to 1 Gbit/s) local area networking over existing home wiring, are examples of home networking technology designed specifically for IPTV delivery. Recently, the IEEE passed proposal P1901 which grounded a standard within the Market for wireline products produced and sold by companies that are part of the HomePlug Alliance.[2] The IEEE is continuously working to push for P1901 to be completely recognized worldwide as the sole standard for all future products that are produced for Home Networking.

- Contents [hide] - 1 Network devices - 1.1 Infrastructure Devices - 1.2 Client Devices - 2 Network media - 3 Home Coverage - 3.1 Challenges - 3.1.1 Wireless Signal Loss - 3.1.2 Wired Background "Noise" - 3.1.3 ”Leaky” WiFi - 4 See also - 5 References - 6 External links

- [edit] Network devices

An example of a simple home network

A home network may consist of the following components:

- [edit] Infrastructure Devices

- A broadband modem for connection to the internet (either a DSL modem using the phone line, or cable modem using the cable internet connection).

- A residential gateway (sometimes called a router) connected between the broadband modem and the rest of the network. This enables multiple devices to connect to the internet simultaneously. Residential gateways, hubs/switches, DSL modems, and wireless access points are often combined.

- A wireless access point, usually implemented as a feature rather than a separate box, for connecting wireless devices

- [edit] Client Devices

- A PC, or multiple PCs including laptops, Netbooks and Tablet PC’s

- Entertainment peripherals - an increasing number of devices can be connected to the home network, including DVRs like TiVo, digital audio players, games machines, stereo system, and IP set-top box as well as TVs themselves.

- Internet Phones (VoIP)

- Smart Phones connected via Wifi.

- A network bridge connects two networks together, often giving a wired device, e.g. Xbox, access to a wireless network.

- A network hub/switch - a central networking hub containing a number of Ethernet ports for connecting multiple networked devices

- A network attached storage (NAS) device can be used for storage on the network.

- A print server can be used to share printers among computers on the network.

Older devices may not have the appropriate connector to the network. USB and PCI network controllers can be installed in some devices to allow them to connect to networks.

Network devices may also be configured from a computer. For example, broadband modems are often configured through a web client on a networked PC. As networking technology evolves, more electronic devices and home appliances are becoming Internet ready and accessible through the home network. Set-top boxes from cable TV providers already have USB and Ethernet ports "for future use".

- [edit] Network media

Ethernet cables are the standard medium for networks. However, homes are often more difficult to wire than office environments, and other technologies are being developed which don't require new wires.

Home networking may use

- Ethernet Category 5 cable, Category 6 cable - for speeds of 10 Mbit/s, 100 Mbit/s, or 1 Gbit/s.

- Wi-Fi Wireless LAN connections - for speeds up to 450 Mbit/s, dependent on signal strength and wireless standard.

- Coaxial cables (TV antennas) - for speeds of 270 Mbit/s (see Multimedia over Coax Alliance or 320 Mbit/s see HomePNA)

- Electrical wiring - for speeds of 14 Mbit/s to 200 Mbit/s (see Power line communication)

- Phone wiring - for speeds of 160 Mbit/s (see HomePNA)

- Fiber optics - although rare, new homes are beginning to include fiber optics for future use. Optical networks generally use Ethernet.

- All home wiring (coax, powerline and phone wires) - future standard for speeds up to 1 Gbit/s being developed by the ITU-T (see G.hn)

Ethernet and Wireless are the most common standards. As the demand for home networks has increased, the other alliances have formed to produce standards for networking alternatives.

- [edit] Home Coverage

- [edit] Challenges

- [edit] Wireless Signal Loss

- The Wireless signal strength may not be powerful enough to cover the entire house or may not be able to get through to all floors of multiple floor residences.

- [edit] Wired Background "Noise"

- One of the largest challenges posed for those that wanted to utilize the home electrical system for networking is how to combat other electrical “noise” that would be around due to the use of a power outlet to transfer information. Whenever any appliance is turned on or turned off it creates noise that could possibly dissrupt data transfer through the wiring. IEEE products past the HomePlug 1.0 stage have combated this problem and no longer interfere with, or receive interference from, other devices plugged into a power outlet.[3]

- [edit] ”Leaky” WiFi

- As can be construed, WiFi often extends beyond the boundaries of a home and can create coverage and holes where it is least wanted and it can also allow a way for people to compromise a system and retrieve personal data. The most forward and most widespread way to combat this is the investment of an authentication, encryption, or VPN that requires a password to access the WiFi. [4]

- [edit] See also

- Home server

- WikiBooks:Transferring Data between Standard Dial-Up Modems

- [edit] References

- ^ “Discover and Learn,” WiFi Alliance, http://www.wi-fi.org/discover_and_learn.php (accessed June 30, 2010).

- ^ Faure, Jean-Philippe. “IEEE P1901 Draft Standard for Broadband over Power Line Networks: Medium Access Control and Physical Layer Specifications,” IEEE Standards Association, http://grouper.ieee.org/groups/1901/ (accessed June 22, 2010).

- ^ “Frequently Asked Questions,” HomePlug Powerline Alliance, http://www.homeplug.org/about/faqs/ (accessed June 22, 2010).

- ^ Wangerien, Brian. "The Challenges of Wi-Fi." Communications News. Encyclopedia Britanica. Web. <http://www.britannica.com/bps/additionalcontent/18/21597846/The-challenges-of-WiFi>.

- [edit] External links

- Home Network Help Site

Retrieved from "http://en.wikipedia.org/wiki/Home_network"

From Wikipedia, the free encyclopedia

(Redirected from Institute of Electrical and Electronic Engineers)

Jump to: navigation, search

Not to be confused with the Institution of Electrical Engineers (IEE).

| IEEE |  |
| --- | --- |
| Type | Professional Organization |
| Founded | January 1, 1963 |
| Origins | Merger of the American Institute of Electrical Engineers and the Institute of Radio Engineers |
| Key people | Mr. Pedro A. Ray, Current President |
| Area served | Worldwide |
| Focus | Electrical, Electronics, and Information Technology [1] |
| Method | Industry standards, Conferences, Publications |
| Revenue | US$330 million |
| Members | 395,000+ |
| Website | www.ieee.org |

The Institute of Electrical and Electronics Engineers or IEEE (read I-Triple-E) is an international non-profit, professional organization for the advancement of technology related to electricity. It has the most members of any technical professional organization in the world, with more than 395,000 members in around 150 countries.

- Contents [hide] - 1 History - 2 Publications - 3 Educational opportunities - 4 Standards and the IEEE Standards Development Process - 5 Membership and member grades - 6 IEEE-Level Awards - 6.1 Major Medals - 6.2 Technical Field Awards - 6.3 Recognitions - 6.4 Prize Papers - 6.5 Scholarships - 7 IEEE Technical Councils - 8 IEEE Societies - 9 Committees - 10 Organizational Units - 11 Sponsored activities - 12 Student Branches - 13 Criticism - 14 See also - 15 References - 16 External links

- [edit] History

The IEEE corporate office is on the 17th floor of 3 Park Avenue in New York City

The IEEE is incorporated in the State of New York, United States. It was formed in 1963 by the merger of the Institute of Radio Engineers (IRE, founded 1912) and the American Institute of Electrical Engineers (AIEE, founded 1884).

The major interests of the AIEE were wire communications (telegraph and telephony) and light and power systems. The IRE concerned mostly radio engineering, and was formed from two smaller organizations, the Society of Wireless and Telegraph Engineers and the Wireless Institute. With the rise of electronics in the 1930s, electronics engineers usually became members of the IRE, but the applications of electron tube technology became so extensive that the technical boundaries differentiating the IRE and the AIEE became difficult to distinguish. After World War II, the two organizations became increasingly competitive, and in 1961, the leadership of both the IRE and the AIEE resolved to consolidate the two organizations. The two organizations formally merged as the IEEE on January 1, 1963.

Notable Presidents of IEEE and its founding organizations include Elihu Thomson (AIEE, 1889-1890), Alexander Graham Bell (AIEE, 1891-1892), Charles Proteus Steinmetz (AIEE, 1901-1902), Lee De Forest (IRE, 1930), Frederick E. Terman (IRE, 1941), William R. Hewlett (IRE, 1954), Ernst Weber (IRE, 1959; IEEE, 1963), and Ivan Getting (IEEE, 1978).

IEEE's Constitution defines the purposes of the organization as "scientific and educational, directed toward the advancement of the theory and practice of Electrical, Electronics, Communications and Computer engineering, as well as computer science, the allied branches of engineering and the related arts and sciences." In pursuing these goals, the IEEE serves as a major publisher of scientific journals and a conference organizer. It is also a leading developer of industrial standard (having developed over 900 active industry standards) in a broad range of disciplines, including electric power and energy, biomedical technology and healthcare, information technology, information assurance, telecommunications, consumer electronics, transportation, aerospace, and nanotechnology. IEEE develops and participates in educational activities such as accreditation of electrical engineering programs in institutes of higher learning. The IEEE logo is a diamond-shaped design which illustrates the right hand grip rule embedded in Benjamin Franklin's kite. It is properly presented as a master brand with the letter "IEEE" to the right. IEEE also sponsors or cosponsors more than 1000 international technical conferences each year.

IEEE has a dual complementary regional and technical structure - with organizational units based on geography (e.g., for example the IEEE Philadelphia Section, IEEE South Africa Section [2]) and technical focus (e.g., the IEEE Computer Society). It manages a separate organizational unit (IEEE-USA) which recommends policies and implements programs specifically intended to benefit the members, the profession and the public in the United States.

The IEEE consists of 38 societies, organized around specialized technical fields, with more than 300 local organizations that hold regular meetings.

The IEEE Standards Association is in charge of the standardization activities of the IEEE.

- [edit] Publications

Main article: List of IEEE publications

IEEE produces 30% of the world's literature in the electrical and electronics engineering and computer science fields, publishing well over 100 peer-reviewed journals.[1]

The content in these journals as well as the content from several hundred annual conferences are available in the IEEE's online digital library.[2]

IEEE publications have received a Green[3] rating the from SHERPA/RoMEO guide[4] for affirming "authors and/or their companies shall have the right to post their IEEE-copyrighted material on their own servers without permission" (IEEE Publication Policy 8.1.9.D[5]). This effectively allows authors, at their choice, to make their article openly available. Roughly 1/3 of the IEEE authors take this route.

Like most other standards bodies, IEEE distributes its standards documents for a fee.

- [edit] Educational opportunities

The IEEE provides learning opportunities within the engineering sciences, research, and technology. The goal of the IEEE education programs is to ensure the growth of skill and knowledge in the electricity-related technical professions and to foster individual commitment to continuing education among IEEE members, the engineering and scientific communities, and the general public.

IEEE offers educational opportunities such as IEEE Expert Now Online Education Courses, [6] the Education Partners Program,[7] Standards in Education[8] and Continuing Education Units (CEUs).[9]

IEEE Expert Now is a collection of online educational courses designed for self-paced learning. Education Partners, exclusive for IEEE members, offers on-line degree programs, certifications and courses at a 10% discount. The Standards in Education website explains what standards are and the importance of developing and using them. The site includes tutorial modules and case illustrations to introduce the history of standards, the basic terminology, their applications and impact on products, as well as news related to standards, book reviews and links to other sites that contain information on standards. Currently, twenty-nine states require Professional Development Hours (PDH) to maintain P.E. licensure, encouraging engineers to seek Continuing Education Units (CEUs) for their participation in continuing education programs. CEUs readily translate into Professional Development Hours (PDHs) (1 CEU is equivalent to 10 PDHs). Countries outside the United States, such as South Africa, also require continuing professional development (CPD) credits. In South Africa, the IEEE South Africa Section is a voluntary association of the Engineering Council of South Africa (ECSA), a signatory of Washington Accord. To maintain registration with ECSA, professional engineers (Pr Eng) must pursue CPD, and now the IEEE, via its local reflection in South Africa, can enable CPD provision. In 2010, it is anticipated that IEEE Expert Now courses will feature in the CPD listing for the IEEE membership in South Africa to derive benefit.

IEEE also sponsors a website[10] designed to help young people understand better what engineering means, and how an engineering career can be made part of their future. Students (ages 8–18), parents, and teachers can explore the site to prepare for an engineering career, ask experts engineering-related questions, play interactive games, explore curriculum links, and review lesson plans. This website also allows students to search for accredited engineering degree programs in Canada and the United States; visitors are able to search by state/province/territory, country, degree field, tuition ranges, room and board ranges, size of student body, and location (rural, suburban, or urban).

- [edit] Standards and the IEEE Standards Development Process

Main article: IEEE Standards Association

IEEE is one of the leading standards-making organizations in the world. IEEE performs its standards making and maintaining functions through the IEEE Standards Association (IEEE-SA). IEEE standards affect a wide range of industries including: power and energy, biomedical and healthcare, Information Technology (IT), telecommunications, transportation, nanotechnology, information assurance, and many more. In 2005, IEEE had close to 900 active standards, with 500 standards under development. One of the more notable IEEE standards is the IEEE 802 LAN/MAN group of standards which includes the IEEE 802.3 Ethernet standard and the IEEE 802.11 Wireless Networking standard.

- [edit] Membership and member grades

Most IEEE members are electrical engineers, computer engineers, and computer scientists, but the organization's wide scope of interests has attracted engineers in other disciplines (e.g., mechanical and civil) as well as biologists, physicists, and mathematicians. There are several membership grades. In order to qualify for membership, an individual must meet certain academic or experience requirements. Associate membership is available to those who don't meet such requirements. Student memberships are available with a reduced membership fee. Students and Associates have all the privileges of members, except the right to vote and hold certain offices. Upon meeting certain additional requirements, a member can apply automatically for senior membership. Finally, a member who has made a significant contribution to any of the IEEE fields of interest may be elevated to the grade IEEE Fellow, a distinctive honor. Some societies allow "affiliate" members, i.e. members of the society that are not members of IEEE itself.

- [edit] IEEE-Level Awards

Through its awards program, the IEEE advances the interests of its members by recognizing their contributions in advancing the fields of interest to IEEE to the benefit of society. By this means, the image and prestige of the organization, its members, and the profession are all enhanced. The IEEE Awards Program honors members and other professionals for their service to the society.

For nearly a century, the IEEE Awards Program has paid tribute to technical professionals whose exceptional achievements and outstanding contributions have made a lasting impact on technology, society and the engineering profession.

- [edit] Major Medals

- IEEE Medal of Honor

- IEEE Edison Medal

- IEEE Founders Medal

- IEEE James H. Mulligan, Jr. Education Medal

- IEEE Alexander Graham Bell Medal

- IEEE Simon Ramo Medal

- IEEE Medal for Engineering Excellence

- IEEE Medal for Environmental and Safety Technologies

- IEEE Medal in Power Engineering

- IEEE Richard W. Hamming Medal

- IEEE Heinrich Hertz Medal

- IEEE John von Neumann Medal

- IEEE Jack S. Kilby Signal Processing Medal

- IEEE Dennis J. Picard Medal for Radar Technologies and Applications

- IEEE Robert N. Noyce Medal

- IEEE Medal for Innovations in Healthcare Technology

- [edit] Technical Field Awards

- IEEE Cledo Brunetti Award

- IEEE Components, Packaging, and Manufacturing Technologies Award

- IEEE Control Systems Award

- IEEE Electromagnetics Award

- IEEE James L. Flanagan Speech and Audio Processing Award

- IEEE Andrew S. Grove Award

- IEEE Herman Halperin Electric Transmission and Distribution Award

- IEEE Masaru Ibuka Consumer Electronics Award

- IEEE Internet Award

- IEEE Reynold B. Johnson Data Storage Device Technology Award

- IEEE Reynold B. Johnson Information Storage Systems Award

- IEEE Richard Harold Kaufmann Award

- IEEE Joseph F. Keithley Award in Instrumentation and Measurement

- IEEE Gustav Robert Kirchhoff Award

- IEEE Leon K. Kirchmayer Graduate Teaching Award

- IEEE Koji Kobayashi Computers and Communications Award

- IEEE William E. Newell Power Electronics Award

- IEEE Daniel E. Noble Award

- IEEE Donald O. Pederson Award in Solid-State Circuits

- IEEE Frederik Philips Award

- IEEE Photonics Award

- IEEE Emanuel R. Piore Award

- IEEE Judith A. Resnik Award

- IEEE Robotics and Automation Award

- IEEE Frank Rosenblatt Award

- IEEE David Sarnoff Award

- IEEE Charles Proteus Steinmetz Award

- IEEE Marie Sklodowska-Curie Award

- IEEE Eric E. Sumner Award

- IEEE Undergraduate Teaching Award

- IEEE Nikola Tesla Award

- IEEE Kiyo Tomiyasu Award

- [edit] Recognitions

- IEEE Haraden Pratt Award

- IEEE Richard M. Emberson Award

- IEEE Corporate Innovation Recognition

- IEEE Ernst Weber Engineering Leadership Recognition

- IEEE Honorary Membership

- [edit] Prize Papers

- IEEE Donald G. Fink Award

- IEEE W.R.G. Baker Award

- [edit] Scholarships

- IEEE Life Members Graduate Study Fellowship in Electrical Engineering was established by the IEEE in 2000. The fellowship is awarded annually to a first year, full time graduate student obtaining their masters for work in the area of electrical engineering, at an engineering school/program of recognized standing worldwide.[11]

- IEEE Charles LeGeyt Fortescue Graduate Scholarship was established by the IRE in 1939 to commemorate Charles Legeyt Fortescue's contributions to electrical engineering. The scholarship is awarded for one year of full-time graduate work obtaining their masters in electrical engineering an ANE engineering school of recognized standing in the United States.[12]

- [edit] IEEE Technical Councils

- IEEE Biometrics Council

- IEEE Council on Electronic Design Automation

- IEEE Council on Superconductivity

- IEEE Nanotechnology Council

- IEEE Sensors Council

- IEEE Systems Council

- IEEE Technology Management Council

- [edit] IEEE Societies

In 2007 there were the following societies [13]:

- IEEE Aerospace and Electronic Systems Society

- IEEE Antennas & Propagation Society

- IEEE Broadcast Technology Society

- IEEE Circuits & Systems Society

- IEEE Communications Society

- IEEE Components, Packaging & Manufacturing Technology Society

- IEEE Computational Intelligence Society

- IEEE Computer Society

- IEEE Consumer Electronics Society

- IEEE Control Systems Society

- IEEE Dielectrics & Electrical Insulation Society

- IEEE Education Society

- IEEE Electromagnetic Compatibility Society

- IEEE Electron Devices Society

- IEEE Engineering in Medicine and Biology Society

- IEEE Geoscience & Remote Sensing Society

- IEEE Industrial Electronics Society

- IEEE Industry Applications Society

- IEEE Information Theory Society

- IEEE Instrumentation & Measurement Society

- IEEE Intelligent Transportation Systems Society

- IEEE Magnetics Society

- IEEE Microwave Theory and Techniques Society

- IEEE Nuclear & Plasma Sciences Society

- IEEE Oceanic Engineering Society

- IEEE Photonics Society

- IEEE Power Electronics Society

- IEEE Power & Energy Society

- IEEE Product Safety Engineering Society

- IEEE Professional Communication Society

- IEEE Reliability Society

- IEEE Robotics and Automation Society

- IEEE Signal Processing Society

- IEEE Society on Social Implications of Technology

- IEEE Solid-State Circuits Society

- IEEE Systems, Man & Cybernetics Society

- IEEE Ultrasonics, Ferroelectrics & Frequency Control Society

- IEEE Vehicular Technology Society

- [edit] Committees

- The IEEE Committee on Earth Observation (ICEO)

- IEEE Technical Committee on RFID (CRFID)

- [edit] Organizational Units

- Technical Activities Board (TAB)

- [edit] Sponsored activities

The IEEE sponsors or is associated with a number of technical conferences each year.

- [edit] Student Branches

- IEEE CUSB

- IEEE AlexSB

- IEEE FUMSB

- [edit] Criticism

|  | This article needs additional citations for verification. Please help improve this article by adding reliable references. Unsourced material may be challenged and removed. (December 2009) |
| --- | --- |

A number of free software proponents, such as Richard Stallman[14] and Daniel J. Bernstein[15], have criticized IEEE's copyright policy. As with most other scientific and technical publishers, when publishing with the IEEE the author is asked to transfer his copyright[16] to the Institute, which then sells the paper in journals and online. Typically, scientific and technical journal publishers do not pay authors or the reviewers.[citation needed] However, some people have managed to retain their copyright on papers published by the IEEE and incite other scientists to do the same, criticizing the usual policy as "exploitation".[17]

This has prompted the appearance of so-called "Open access" scientific publishers such as the Public Library of Science, which employ different schemes such as charging the author or resorting to advertising, as opposed to charging the reader. However, publishing in IEEE journals is almost mandatory[citation needed] for those operating in the scientific communities of IEEE covered fields: impact factors of IEEE publications are among the highest.[18]

- [edit] See also

- IEEE Computer Society Certified Software Development Professional (CSDP) Program

- ITU

- [edit] References

- ^ About IEEE

- ^ IEEE's online digital library

- ^ Sherpa Romeo color code

- ^ Sherpa Romeo site

- ^ IEEE Publication Policy 8.1.9.D

- ^ IEEE - IEEE Expert Now

- ^ IEEE - IEEE Education Partners Program

- ^ IEEE - The IEEE Standards Education pages have moved

- ^ IEEE - IEEE Continuing Education Units

- ^ Welcome to TryEngineering.org

- ^ IEEE Life Member Graduate Study Fellowship. Retrieved on 2010-01-23.

- ^ Charles LeGeyt Fortescue Graduate Scholarship. Retrieved on 2010-01-23.

- ^ IEEE - IEEE Society Memberships

- ^ "Richard Stallman's Personal Home Page, section "Long-term action items"". Richard Stallman. http://www.stallman.org.

- ^ "Don't Publish with IEEE!". D.J. Bernstein. http://cr.yp.to/writing/ieee.html. Retrieved 2007-07-10.

- ^ "IEEE copyright transfer form". IEEE. http://www.ieee.org/portal/cms_docs_iportals/iportals/publications/rights/IEEECopyrightForm.pdf.

- ^ "The IEEE's copyright policy sucks!". http://www.ieeesucks.com/.

- ^ "Journal Citation Reports". http://admin-apps.isiknowledge.com/JCR/JCR?PointOfEntry=Home. Retrieved 2008-04-22.

- [edit] External links

- IEEE 125th Anniversary Website

- IEEE Global Website

- IEEE One Voice video

- IEEE Graduates of the Last Decade (GOLD)

- IEEE Organization

- IEEE Egypt Section

- IEEE Cairo University Student Branch

- IEEE Long Island Section

- IEEE South Africa Section

- IEEE Standard Association

- IEEE Student

- IEEE-USA Entrepreneurs Village, established to support those people who create entrepreneurial businesses based on technology.

- IEEE Global History Network, a wiki-based site containing a wealth of information about the history of IEEE, its members, their professions, and their technologies.

- IEEE Xplore — over a million online documents

- www.tryengineering.org - resource for students age 8-18, and their parents, teachers, and guidance counselors.

- IEEE.tv — an award-winning Internet-based television network.

From Wikipedia, the free encyclopedia

Jump to: navigation, search

"ISO" redirects here. For other uses, see ISO (disambiguation).

| International Organization for Standardization Organisation internationale de normalization Международная организация по стандартизации [1] |  |
| --- | --- |
| Logo of ISO in American English |  |
| List of members |  |
| Formation | February 23 1947 |
| Type | NGO |
| Purpose/focus | International standardization |
| Headquarters | Geneva, Switzerland |
| Membership | 165 members |
| Official languages | English, French and Russian |
| Website | www.iso.org |

The International Organization for Standardization (French: Organisation internationale de normalisation, Russian: Международная организация по стандартизации, tr. Mezhdunarodnaya organizaciya po standartizacii),[1] widely known as ISO, is an international-standard-setting body composed of representatives from various national standards organizations. Founded on February 23 1947, the organization promulgates worldwide proprietary industrial and commercial standards. It has its headquarters in Geneva, Switzerland.[2] While ISO defines itself as a non-governmental organization, its ability to set standards that often become law, either through treaties or national standards, makes it more powerful than most non-governmental organizations.[citation needed] In practice, ISO acts as a consortium with strong links to governments.[citation needed]

- Contents [hide] - 1 Name and abbreviation - 2 History - 3 International Standards and other publications - 4 Standardization process - 5 ISO document copyright - 6 Members - 7 Products named after ISO - 8 ISO/IEC Joint Technical Committee 1 - 9 IWA document - 10 Criticism - 11 See also - 12 References - 13 External links

- [edit] Name and abbreviation

The organization's logos in two of its official languages, English and French, include the word ISO, and it is usually referred to by this short-form name. ISO is not an acronym or initialism for the organization's full name in either official language. Rather, the organization adopted ISO based on the Greek word isos (ἴσος), meaning equal. Recognizing that the organization’s initials would be different in different languages, the organization's founders chose ISO as the universal short form of its name. This, in itself, reflects the aim of the organization: to equalize and standardize across cultures.[3][4]

- [edit] History

Standards are important in international trade because incongruent standards can be barriers to trade, giving some organizations advantages in certain areas of the world. Standards provide clear identifiable references that are recognized internationally and encourage fair competition in free-market economies. Standards facilitate trade through enhanced product quality and reliability, greater interoperability and compatibility, greater ease of maintenance and reduced costs. ISO covers a wide variety of standards with the exception of electrical and electronic engineering standards covered by the International Electrotechnical Commission (IEC), telecommunication standards covered by the International Telegraph Union (ITU) and information technology covered by JTC 1 (a joint committee between ISO and IEC).[5]

The organization which today is known as ISO began in 1926 as the International Federation of the National Standardizing Associations (ISA). This organization focused heavily on mechanical engineering. It was disbanded in 1942 during the second World War but was re-organized under the current name, ISO, in 1946.

ISO is a voluntary organization whose members are recognized standard authorities, each one representing one country. The bulk of the work of ISO is done by the 2700 technical committees, subcommittees and working groups. Each committee and subcommittee is headed by a Secretariat from one of the member organizations. The American National Standards Institute (ANSI) is the United States representative to ISO. The ANSI ASC Z-1/ASQ Standards Group coordinates the United States representation in the ISO Technical Committees 176 and 207 which are concerned with the ISO 9000 and ISO 14000 standards respectively.

- [edit] International Standards and other publications

See also: List of ISO standards

ISO's main products are the International Standards. ISO also publishes Technical Reports, Technical Specifications, Publicly Available Specifications, Technical Corrigenda, and Guides.[6][7]

International Standards are identified in the format ISO[/IEC][/ASTM] [IS] nnnnn[:yyyy] Title, where nnnnn is the number of the standard, yyyy is the year published, and Title describes the subject. IEC for International Electrotechnical Commission is included if the standard results from the work of ISO/IEC JTC1 (the ISO/IEC Joint Technical Committee). ASTM (American Society for Testing and Materials) is used for standards developed in cooperation with ASTM International. The date and IS are not used for an incomplete or unpublished standard, and may under some circumstances be left off the title of a published work.

Technical Reports are issued when "a technical committee or subcommittee has collected data of a different kind from that which is normally published as an International Standard".[6] such as references and explanations. The naming conventions for these are the same as for standards, except TR prepended instead of IS in the report's name. Examples:

- ISO/IEC TR 17799:2000 Code of Practice for Information Security Management

- ISO/TR 19033:2000 Technical product documentation — Metadata for construction documentation

Technical Specifications can be produced when "the subject in question is still under development or where for any other reason there is the future but not immediate possibility of an agreement to publish an International Standard". Publicly Available Specifications may be "an intermediate specification, published prior to the development of a full International Standard, or, in IEC may be a 'dual logo' publication published in collaboration with an external organization".[6] Both are named by convention similar to Technical Reports, for example:

- ISO/TS 16952-1:2006 Technical product documentation — Reference designation system — Part 1: General application rules

- ISO/PAS 11154:2006 Road vehicles — Roof load carriers

ISO sometimes issues a Technical Corrigendum. These are amendments to existing standards because of minor technical flaws, usability improvements, or to extend applicability in a limited way. Generally, these are issued with the expectation that the affected standard will be updated or withdrawn at its next scheduled review.[6]

ISO Guides are meta-standards covering "matters related to international standardization".[6] They are named in the format "ISO[/IEC] Guide N:yyyy: Title", for example:

- ISO/IEC Guide 2:2004 Standardization and related activities — General vocabulary

- ISO/IEC Guide 65:1996 General requirements for bodies operating product certification

- [edit] Standardization process

A standard published by ISO/IEC is the last stage of a long process that commonly starts with the proposal of new work within a committee. Here are some abbreviations used for marking a standard with its status:[8][9][10][11][12][13][14]

- PWI - Preliminary Work Item

- NP or NWIP - New Proposal / New Work Item Proposal (e.g., ISO/IEC NP 23007)

- AWI - Approved new Work Item (e.g., ISO/IEC AWI 15444-14)

- WD - Working Draft (e.g., ISO/IEC WD 27032)

- CD - Committee Draft (e.g., ISO/IEC CD 23000-5)

- FCD - Final Committee Draft (e.g., ISO/IEC FCD 23000-12)

- DIS - Draft International Standard (e.g., ISO/IEC DIS 14297)

- FDIS - Final Draft International Standard (e.g., ISO/IEC FDIS 27003)

- PRF - Proof of a new International Standard (e.g., ISO/IEC PRF 18018)

- IS - International Standard (e.g., ISO/IEC 13818-1:2007)

Abbreviations used for amendments :[8][9][10][11][12][13][14][15]

- NP Amd - New Proposal Amendment (e.g., ISO/IEC 15444-2:2004/NP Amd 3)

- AWI Amd - Approved new Work Item Amendment (e.g., ISO/IEC 14492:2001/AWI Amd 4)

- WD Amd - Working Draft Amendment (e.g., ISO 11092:1993/WD Amd 1)

- CD Amd / PDAmd - Committee Draft Amendment / Proposed Draft Amendment (e.g., ISO/IEC 13818-1:2007/CD Amd 6)

- FPDAmd / DAM (DAmd) - Final Proposed Draft Amendment / Draft Amendment (e.g., ISO/IEC 14496-14:2003/FPDAmd 1)

- FDAM (FDAmd) - Final Draft Amendment (e.g., ISO/IEC 13818-1:2007/FDAmd 4)

- PRF Amd - (e.g., ISO 12639:2004/PRF Amd 1)

- Amd - Amendment (e.g., ISO/IEC 13818-1:2007/Amd 1:2007

Other abbreviations :[12][13][15][16]

- TR - Technical Report (e.g., ISO/IEC TR 19791:2006)

- DTR - Draft Technical Report (e.g., ISO/IEC DTR 19791)

- TS - Technical Specification (e.g., ISO/TS 16949:2009)

- DTS - Draft Technical Specification (e.g., ISO/DTS 11602-1)

- PAS - Publicly Available Specification

- TTA - Technology Trends Assessment (e.g., ISO/TTA 1:1994)

- IWA - International Workshop Agreement (e.g., IWA 1:2005)

- Cor - Technical Corrigendum (e.g., ISO/IEC 13818-1:2007/Cor 1:2008)

- Guide - a guidance to technical committees for the preparation of standards

International Standards are developed by ISO technical committees (TC) and subcommittees (SC) by a process with six steps:[10][17]

- Stage 1: Proposal stage

- Stage 2: Preparatory stage

- Stage 3: Committee stage

- Stage 4: Enquiry stage

- Stage 5: Approval stage

- Stage 6: Publication stage

The TC/SC may set up working groups (WG) of experts for the preparation of a Working Drafts. Subcommittees may have several working groups, which can have several Sub Groups (SG).[18]

| Stages in the development process of an ISO standard[9][10][11][14][15][17] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Stage code | Stage | Associated document name | Abbreviations | Description |
| 00 | Preliminary stage | Preliminary work item | PWI |  |
| 10 | Proposal stage | New work item proposal | NP or NWIP, NP Amd/TR/TS/IWA |  |
| 20 | Preparatory stage | Working draft(s) | AWI, AWI Amd/TR/TS, WD, WD Amd/TR/TS |  |
| 30 | Committee stage | Committee draft(s) | CD, CD Amd/Cor/TR/TS, PDAmd (PDAM), PDTR, PDTS |  |
| 40 | Enquiry stage | Enquiry draft | DIS, FCD, FPDAmd, DAmd (DAM), FPDISP, DTR, DTS | (CDV in IEC) |
| 50 | Approval stage | final draft International Standard | FDIS, FDAmd (FDAM), PRF, PRF Amd/TTA/TR/TS/Suppl, FDTR |  |
| 60 | Publication stage | International Standard | ISO TR, TS, IWA, Amd, Cor |  |
| 90 | Review stage |  | ISO TR, TS, IWA, Amd, Cor |  |
| 95 | Withdrawal stage |  |  |  |

It is possible to omit certain stages, if there is a document with a certain degree of maturity at the start of a standardization project - for example a standard developed by another organization. ISO/IEC Directives allow also the so-called "Fast-track procedure". In this procedure a document is submitted directly for approval as a draft International Standard (DIS) to the ISO member bodies or as a final draft International Standard (FDIS) if the document was developed by an international standardizing body recognized by the ISO Council.[10]

The first step - a proposal of work (New Proposal) is approved at the relevant subcommittee or technical committee (e.g., SC29 and JTC1 respectively in the case of Moving Picture Experts Group - ISO/IEC JTC1/SC29/WG11). A working group (WG) of experts is set up by the TC/SC for the preparation of a Working Draft. When the scope of a new work is sufficiently clarified, some of the working groups (e.g., MPEG) usually make open request for proposals - known as "Call for proposals". The first document that is produced for example for audio and video coding standards is called a Verification Model (VM) (previously also called a Simulation and Test Model). When a sufficient confidence in the stability of the standard under development is reached, a Working Draft (WD) is produced. This is in the form of a standard but is kept internal to working group for revision. When a Working Draft is sufficiently solid and the working group is satisfied that it has developed the best technical solution to the problem being addressed, it becomes Committee Draft (CD). If it is required, it is then sent to the P-members of the TC/SC (National Bodies) for ballot.

The CD becomes Final Committee Draft (FCD) if the number of positive votes is above the quorum. Successive committee drafts may be considered until consensus is reached on the technical content. When it is reached, the text is finalized for submission as a draft International Standard (DIS). The text is then submitted to National Bodies for voting and comment within a period of five months. It is approved for submission as a final draft International Standard (FDIS) if a two-thirds majority of the P-members of the TC/SC are in favour and not more than one-quarter of the total number of votes cast are negative. ISO will then hold a ballot with National Bodies where no technical changes are allowed (yes/no ballot), within a period of two months. It is approved as an International Standard (IS) if a two-thirds majority of the P-members of the TC/SC is in favour and not more than one-quarter of the total number of votes cast are negative. After approval, only minor editorial changes are introduced into the final text. The final text is sent to the ISO Central Secretariat which publishes it as the International Standard.[8][10]

- [edit] ISO document copyright

ISO documents are copyrighted and ISO charges for copies of most. ISO does not, however, charge for most draft copies of documents in electronic format. Although useful, care must be taken using these drafts as there is the possibility of substantial change before it becomes finalized as a standard. Some standards by ISO and its official U.S. representative (and the International Electrotechnical Commission's via the U.S. National Committee) are made freely available.[19][20]

- [edit] Members

A map of standards bodies who are ISO members
Key:
     members      correspondent members      subscriber members      other places with an ISO 3166-1 code who aren't members of ISO

ISO has 163 national members,[21] out of the 203 total countries in the world.

ISO has three membership categories:

- Member bodies are national bodies that are considered to be the most representative standards body in each country. These are the only members of ISO that have voting rights.

- Correspondent members are countries that do not have their own standards organization. These members are informed about ISO's work, but do not participate in standards promulgation.

- Subscriber members are countries with small economies. They pay reduced membership fees, but can follow the development of standards.

Participating members are called "P" members as opposed to observing members which are called "O" members.

- [edit] Products named after ISO

The fact that many of the ISO-created standards are ubiquitous has led, on occasion, to common use of "ISO" to describe the actual product that conforms to a standard. Some examples of this are:

- CD images end in the file extension "ISO" to signify that they are using the ISO 9660 standard filesystem as opposed to another file system - hence CD images are commonly referred to as "ISOs". Virtually all computers with CD-ROM drives can read CDs that use this standard. Some DVD-ROMs also use ISO 9660 filesystems.

- Photographic film's sensitivity to light, its "film speed", is described by ISO 5800:1987. Hence, the film's speed is often referred to as its "ISO number."

- [edit] ISO/IEC Joint Technical Committee 1

Main article: ISO/IEC JTC1

To deal with the consequences of substantial overlap in areas of standardization and work related to information technology, ISO and IEC formed a Joint Technical Committee known as the ISO/IEC JTC1. It was the first such joint committee. The second joint committee was created in 2009 - Joint Project Committee - Energy efficiency and renewable energy sources - Common terminology (ISO/IEC/JTC 2).[22]

- [edit] IWA document

Like ISO/TS, International Workshop Agreement (IWA) is another armory of ISO for providing rapid response to requirements for standardization in areas where the technical structures and expertise are not currently in place[citation needed]. The utility harmonizes technical urgency industrial wide..

- [edit] Criticism

With the exception of a small number of isolated standards,[23] ISO standards are normally not available free of charge, but for a purchase fee,[24] which has been seen by some as too expensive for small Open source projects.[25]

The ISO/IEC JTC1 fast-track procedures ("Fast-track" as used by OOXML and "PAS" as used by OpenDocument) have garnered criticism in relation to the standardization of Office Open XML (ISO/IEC 29500). Martin Bryan, outgoing Convenor of ISO/IEC JTC1/SC34 WG1, is quoted as saying:

I would recommend my successor that it is perhaps time to pass WG1’s outstanding standards over to OASIS, where they can get approval in less than a year and then do a PAS submission to ISO, which will get a lot more attention and be approved much faster than standards currently can be within WG1.

The disparity of rules for PAS, Fast-Track and ISO committee generated standards is fast making ISO a laughing stock in IT circles. The days of open standards development are fast disappearing. Instead we are getting 'standardization by corporation'.[26]

Computer security entrepreneur and Ubuntu investor, Mark Shuttleworth, commented on the Standardization of Office Open XML process by saying

I think it de-values the confidence people have in the standards setting process,

and Shuttleworth alleged that ISO did not carry out its responsibility. He also noted that Microsoft had intensely lobbied many countries that traditionally had not participated in ISO and stacked technical committees with Microsoft employees, solution providers and resellers sympathetic to Office Open XML.

When you have a process built on trust and when that trust is abused, ISO should halt the process ... ISO is an engineering old boys club and these things are boring so you have to have a lot of passion … then suddenly you have an investment of a lot of money and lobbying and you get artificial results. The process is not set up to deal with intensive corporate lobbying and so you end up with something being a standard that is not clear.[27]

- [edit] See also

| - American National Standards Institute (ANSI) - Deutsches Institut für Normung, German Institute for Standardization (DIN) - British Standards Institution (BSI) - Countries in International Organization for Standardization - Canadian Standards Association - European Committee for Standardization (CEN) - Commonwealth of Independent States (CIS) set of standards (GOST) - International Classification for Standards - International Electrotechnical Commission (IEC) and ISO/IEC standards - International healthcare accreditation - International Telecommunication Union (ITU) - ISO A4 - ISO country code - List of ISO standards | - ISO divisions - ISO/TC 37 - ISO/TC 68 - TC 46/SC 9 - ISO/TC 211 - ISO/TC 215 - ISO/TC 223 - Standardization - Standards Australia - Standards organization - Terminology planning policy - The International Customer Service Institute (TICSI) - CARICOM Regional Organization for Standards and Quality (CROSQ) - AP Stylebook (Associated Press Style) - Interface 2010 (Interface Marketing Supplier Integration Institute) |
| --- | --- |

- [edit] References

- ^ a b The 3 official full names of ISO can be found at the beginning of the foreword sections of the PDF document: ISO/IEC Guide 2:2004 Standardization and related activities — General vocabulary

- ^ "Discover ISO – Meet ISO". ISO. © 2007. http://www.iso.org/iso/about/discover-iso_meet-iso.htm. Retrieved 7 September 2007.

- ^ "ISO's name". ISO. 2007. http://www.iso.org/iso/en/networking/pr/isoname/isoname.html. Retrieved 7 September 2007.

- ^ "Discover ISO – ISO's name". ISO. 2007. http://www.iso.org/iso/about/discover-iso_meet-iso/discover-iso_isos-name.htm. Retrieved 7 September 2007.

- ^ [1] A Brief History of ISO

- ^ a b c d e The ISO directives are published in two distinct parts:
* "ISO Directives, Part 2: Rules for the structure and drafting of International Standards. 5th Edition" (pdf). ISO/IEC. 2004. Archived from the original on 10 July 2007. http://web.archive.org/web/20070710135001/http://www.iec.ch/tiss/iec/Directives-Part1-Ed5.pdf. Retrieved 7 September 2007.

- ^ ISO. "ISO/IEC Directives and ISO supplement". http://www.iso.org/directives. Retrieved 1 January 2010.

- ^ a b c "About MPEG". chiariglione.org. http://mpeg.chiariglione.org/about_mpeg.htm. Retrieved 13 December 2009.

- ^ a b c ISO. "International harmonized stage codes". http://www.iso.org/iso/standards_development/processes_and_procedures/stages_description/stages_table.htm#s90. Retrieved 31 December 2009.

- ^ a b c d e f ISO. "Stages of the development of International Standards". http://www.iso.org/iso/standards_development/processes_and_procedures/stages_description.htm. Retrieved 31 December 2009.

- ^ a b c "The ISO27k FAQ - ISO/IEC acronyms and committees". IsecT Ltd.. http://www.iso27001security.com/html/faq.html#Acronyms. Retrieved 31 December 2009.

- ^ a b c ISO (2007). "ISO/IEC Directives Supplement — Procedures specific to ISO" (PDF). http://www.astm.org/COMMIT/1st_Supplement.pdf. Retrieved 31 December 2009.

- ^ a b c ISO (2007). "List of abbreviations used throughout ISO Online". http://www.iso.org/iso/support/faqs/faqs_list_abbreviations.htm. Retrieved 31 December 2009.

- ^ a b c "US TAG COMMITTEE HANDBOOK" (DOC). 2008-03. http://www.sae.org/exdomains/standardsdev/global_resources/US%20TAG%20Committe%20Handbook%206March2008.doc. Retrieved 1 January 2010.

- ^ a b c ISO/IEC JTC1 (2 November 2009), Letter Ballot on the JTC 1 Standing Document on Technical Specifications and Technical Reports, http://isotc.iso.org/livelink/livelink/JTC001-N-9876.pdf?func=doc.Fetch&nodeId=8498789&docTitle=JTC001-N-9876, retrieved 1 January 2010

- ^ ISO. "ISO deliverables". http://www.iso.org/iso/standards_development/processes_and_procedures/deliverables.htm. Retrieved 9 April 2010.

- ^ a b ISO (2008) (PDF), ISO/IEC Directives, Part 1 - Procedures for the technical work, Sixth edition, 2008, http://www.iec.ch/tiss/iec/Directives-Part1-Ed6.pdf, retrieved 1 January 2010

- ^ ISO, IEC (5 November 2009). "ISO/IEC JTC 1/SC 29, SC 29/WG 11 Structure (ISO/IEC JTC 1/SC 29/WG 11 - Coding of Moving Pictures and Audio)". http://www.itscj.ipsj.or.jp/sc29/29w12911.htm. Retrieved 7 November 2009.

- ^ "Freely Available ISO Standards". ISO. Last updated 2007-08-08. http://isotc.iso.org/livelink/livelink/fetch/2000/2489/Ittf_Home/PubliclyAvailableStandards.htm. Retrieved 7 September 2007.

- ^ "Free ANSI Standards". Archived from the original on 3 April 2007. http://web.archive.org/web/20070403014457/http://webstore.ansi.org/ansidocstore/free_standards.asp. Retrieved 19 June 2007.

- ^ "General information on ISO". ISO. © 2009. http://www.iso.org/iso/support/faqs/faqs_general_information_on_iso.htm. Retrieved 29 January 2009.

- ^ "ISO/IEC/JTC 2 - Joint Project Committee - Energy efficiency and renewable energy sources - Common terminology". http://www.iso.org/iso/standards_development/technical_committees/list_of_iso_technical_committees/iso_technical_committee.htm?commid=585141. Retrieved 1 January 2010.

- ^ "Freely Available Standards". ISO. http://standards.iso.org/ittf/PubliclyAvailableStandards/index.html. Retrieved 26 April 2008.

- ^ "Shopping FAQs". ISO. http://www.iso.org/iso/store/shopping_faqs.htm. Retrieved 26 April 2008.

- ^ Jelliffe, Rick (1 August 2007). "Where to get ISO Standards on the Internet free". oreillynet.com. http://www.oreillynet.com/xml/blog/2007/08/where_to_get_iso_standards_on.html. Retrieved 26 April 2008. "The lack of free online availability has effectively made ISO standard irrelevant to the (home/hacker section of the) Open Source community"

- ^ "Report on WG1 activity for December 2007 Meeting of ISO/IEC JTC1/SC34/WG1 in Kyoto". iso/jtc1 sc34. 29 November 2007. http://www.jtc1sc34.org/repository/0940.htm.

- ^ "Ubuntu’s Shuttleworth blames ISO for OOXML’s win". ZDNet.com. 1 April 2008. http://blogs.zdnet.com/open-source/?p=2222.

- [edit] External links

- Official website (free access to the catalogue of standards only, not to the contents)

- Publicly Available Standards (free access to a small subset of the standards)

- ISO/TC 37 "Terminology and other language and content resources", a fundamental committee for all ISO standardization projects

- ISO/IEC JTC1

- ISO Advanced search for standards and/or projects

- ISO Concept Database (terminology database of ISO standards)

From Wikipedia, the free encyclopedia

Jump to: navigation, search

This article is about the public worldwide computer network system. For other uses, see Internet (disambiguation).

Visualization of the various routes through a portion of the Internet. From 'The Opte Project'

The Internet is a global system of interconnected computer networks that use the standard Internet Protocol Suite (TCP/IP) to serve billions of users worldwide. It is a network of networks that consists of millions of private, public, academic, business, and government networks, of local to global scope, that are linked by a broad array of electronic and optical networking technologies. The Internet carries a vast range of information resources and services, such as the inter-linked hypertext documents of the World Wide Web (WWW) and the infrastructure to support electronic mail.

Most traditional communications media including telephone, music, film, and television are being reshaped or redefined by the Internet. Newspaper, book and other print publishing are having to adapt to Web sites and blogging. The Internet has enabled or accelerated new forms of human interactions through instant messaging, Internet forums, and social networking. Online shopping has boomed both for major retail outlets and small artisans and traders. Business-to-business and financial services on the Internet affect supply chains across entire industries.

The origins of the Internet reach back to the 1960s with both private and United States military research into robust, fault-tolerant, and distributed computer networks. The funding of a new U.S. backbone by the National Science Foundation, as well as private funding for other commercial backbones, led to worldwide participation in the development of new networking technologies, and the merger of many networks. The commercialization of what was by then an international network in the mid 1990s resulted in its popularization and incorporation into virtually every aspect of modern human life. As of 2009, an estimated quarter of Earth's population used the services of the Internet.

The Internet has no centralized governance in either technological implementation or policies for access and usage; each constituent network sets its own standards. Only the overreaching definitions of the two principal name spaces in the Internet, the Internet Protocol address space and the Domain Name System, are directed by a maintainer organization, the Internet Corporation for Assigned Names and Numbers (ICANN). The technical underpinning and standardization of the core protocols (IPv4 and IPv6) is an activity of the Internet Engineering Task Force (IETF), a non-profit organization of loosely affiliated international participants that anyone may associate with by contributing technical expertise.

- Contents [hide] - 1 Terminology - 2 History - 3 Technology - 3.1 Protocols - 3.2 Structure - 4 Governance - 5 Modern uses - 6 Services - 6.1 Information - 6.2 Communication - 6.3 Data transfer - 7 Access - 8 Social impact - 9 See also - 10 Notes - 11 References - 12 External links

- Terminology

See also: Internet capitalization conventions

The terms Internet and World Wide Web are often used in everyday speech without much distinction. However, the Internet and the World Wide Web are not one and the same. The Internet is a global data communications system. It is a hardware and software infrastructure that provides connectivity between computers. In contrast, the Web is one of the services communicated via the Internet. It is a collection of interconnected documents and other resources, linked by hyperlinks and URLs.[1]

The term the Internet, when referring to the entire global system of IP networks, has traditionally been treated as a proper noun and written with an initial capital letter. In the media and popular culture a trend has developed to regard it as a generic term or common noun and thus write it as "the internet", without capitalization.

The Internet is also often simply referred to as the net. In many technical illustrations when the precise location or interrelation of Internet resources is not important, the Internet is often referred as the cloud, and literally depicted as such.

- History

Main article: History of the Internet

The USSR's launch of Sputnik spurred the United States to create the Advanced Research Projects Agency (ARPA or DARPA) in February 1958 to regain a technological lead.[2][3] ARPA created the Information Processing Technology Office (IPTO) to further the research of the Semi Automatic Ground Environment (SAGE) program, which had networked country-wide radar systems together for the first time. The IPTO's purpose was to find ways to address the US Military's concern about survivability of their communications networks, and as a first step interconnect their computers at the Pentagon, Cheyenne Mountain, and SAC HQ. J. C. R. Licklider, a promoter of universal networking, was selected to head the IPTO. Licklider moved from the Psycho-Acoustic Laboratory at Harvard University to MIT in 1950, after becoming interested in information technology. At MIT, he served on a committee that established Lincoln Laboratory and worked on the SAGE project. In 1957 he became a Vice President at BBN, where he bought the first production PDP-1 computer and conducted the first public demonstration of time-sharing.

Professor Leonard Kleinrock with one of the first ARPANET Interface Message Processors at UCLA

At the IPTO, Licklider's successor Ivan Sutherland in 1965 got Lawrence Roberts to start a project to make a network, and Roberts based the technology on the work of Paul Baran,[4] who had written an exhaustive study for the United States Air Force that recommended packet switching (opposed to circuit switching) to achieve better network robustness and disaster survivability. Roberts had worked at the MIT Lincoln Laboratory originally established to work on the design of the SAGE system. UCLA professor Leonard Kleinrock had provided the theoretical foundations for packet networks in 1962, and later, in the 1970s, for hierarchical routing, concepts which have been the underpinning of the development towards today's Internet.

Sutherland's successor Robert Taylor convinced Roberts to build on his early packet switching successes and come and be the IPTO Chief Scientist. Once there, Roberts prepared a report called Resource Sharing Computer Networks which was approved by Taylor in June 1968 and laid the foundation for the launch of the working ARPANET the following year.

After much work, the first two nodes of what would become the ARPANET were interconnected between Kleinrock's Network Measurement Center at the UCLA's School of Engineering and Applied Science and Douglas Engelbart's NLS system at SRI International (SRI) in Menlo Park, California, on 29 October 1969. The third site on the ARPANET was the Culler-Fried Interactive Mathematics center at the University of California at Santa Barbara, and the fourth was the University of Utah Graphics Department. In an early sign of future growth, there were already fifteen sites connected to the young ARPANET by the end of 1971.

The ARPANET was one of the eve networks of today's Internet. In an independent development, Donald Davies at the UK National Physical Laboratory also discovered the concept of packet switching in the early 1960s, first giving a talk on the subject in 1965, after which the teams in the new field from two sides of the Atlantic ocean first became acquainted. It was actually Davies' coinage of the wording "packet" and "packet switching" that was adopted as the standard terminology. Davies also built a packet switched network in the UK called the Mark I in 1970. [5] Bolt Beranek and Newman (BBN), the private contractors for ARPANET, set out to create a separate commercial version after establishing "value added carriers" was legalized in the U.S.[6]. The network they established was called Telenet and began operation in 1975, installing free public dial-up access in cities throughout the U.S. Telenet was the first packet-switching network open to the general public.[7]

Following the demonstration that packet switching worked on the ARPANET, the British Post Office, Telenet, DATAPAC and TRANSPAC collaborated to create the first international packet-switched network service. In the UK, this was referred to as the International Packet Switched Service (IPSS), in 1978. The collection of X.25-based networks grew from Europe and the US to cover Canada, Hong Kong and Australia by 1981. The X.25 packet switching standard was developed in the CCITT (now called ITU-T) around 1976.

A plaque commemorating the birth of the Internet at Stanford University

X.25 was independent of the TCP/IP protocols that arose from the experimental work of DARPA on the ARPANET, Packet Radio Net and Packet Satellite Net during the same time period.

The early ARPANET ran on the Network Control Program (NCP), implementing the host-to-host connectivity and switching layers of the protocol stack, designed and first implemented in December 1970 by a team called the Network Working Group (NWG) led by Steve Crocker. To respond to the network's rapid growth as more and more locations connected, Vinton Cerf and Robert Kahn developed the first description of the now widely used TCP protocols during 1973 and published a paper on the subject in May 1974. Use of the term "Internet" to describe a single global TCP/IP network originated in December 1974 with the publication of RFC 675, the first full specification of TCP that was written by Vinton Cerf, Yogen Dalal and Carl Sunshine, then at Stanford University. During the next nine years, work proceeded to refine the protocols and to implement them on a wide range of operating systems. The first TCP/IP-based wide-area network was operational by 1 January 1983 when all hosts on the ARPANET were switched over from the older NCP protocols. In 1985, the United States' National Science Foundation (NSF) commissioned the construction of the NSFNET, a university 56 kilobit/second network backbone using computers called "fuzzballs" by their inventor, David L. Mills. The following year, NSF sponsored the conversion to a higher-speed 1.5 megabit/second network. A key decision to use the DARPA TCP/IP protocols was made by Dennis Jennings, then in charge of the Supercomputer program at NSF.

The opening of the NSFNET to other networks began in 1988.[citation needed] The US Federal Networking Council approved the interconnection of the NSFNET to the commercial MCI Mail system in that year and the link was made in the summer of 1989. Other commercial electronic mail services were soon connected, including OnTyme, Telemail and Compuserve. In that same year, three commercial Internet service providers (ISPs) began operations: UUNET, PSINet, and CERFNET. Important, separate networks that offered gateways into, then later merged with, the Internet include Usenet and BITNET. Various other commercial and educational networks, such as Telenet (by that time renamed to Sprintnet), Tymnet, Compuserve and JANET were interconnected with the growing Internet in the 1980s as the TCP/IP protocol became increasingly popular. The adaptability of TCP/IP to existing communication networks allowed for rapid growth. The open availability of the specifications and reference code permitted commercial vendors to build interoperable network components, such as routers, making standardized network gear available from many companies. This aided in the rapid growth of the Internet and the proliferation of local-area networking. It seeded the widespread implementation and rigorous standardization of TCP/IP on UNIX and virtually every other common operating system.

This NeXT Computer was used by Sir Tim Berners-Lee at CERN and became the world's first Web server.

Although the basic applications and guidelines that make the Internet possible had existed for almost two decades, the network did not gain a public face until the 1990s. On 6 August 1991, CERN, a pan European organization for particle research, publicized the new World Wide Web project. The Web was invented by British scientist Tim Berners-Lee in 1989. An early popular web browser was ViolaWWW, patterned after HyperCard and built using the X Window System. It was eventually replaced in popularity by the Mosaic web browser. In 1993, the National Center for Supercomputing Applications at the University of Illinois released version 1.0 of Mosaic, and by late 1994 there was growing public interest in the previously academic, technical Internet. By 1996 usage of the word Internet had become commonplace, and consequently, so had its use as a synecdoche in reference to the World Wide Web.

Meanwhile, over the course of the decade, the Internet successfully accommodated the majority of previously existing public computer networks (although some networks, such as FidoNet, have remained separate). During the 1990s, it was estimated that the Internet grew by 100 percent per year, with a brief period of explosive growth in 1996 and 1997.[8] This growth is often attributed to the lack of central administration, which allows organic growth of the network, as well as the non-proprietary open nature of the Internet protocols, which encourages vendor interoperability and prevents any one company from exerting too much control over the network.[9] The estimated population of Internet users is 1.97 billion as of 30 June 2010.[10]

From 2009 onward, the Internet is expected to grow significantly in Brazil, Russia, India, China, and Indonesia (BRICI countries). These countries have large populations and moderate to high economic growth, but still low Internet penetration rates. In 2009, the BRICI countries represented about 45 percent of the world's population and had approximately 610 million Internet users, but by 2015, Internet users in BRICI countries will double to 1.2 billion, and will triple in Indonesia.[11][12]

- Technology

- Protocols

Main article: Internet Protocol Suite

The complex communications infrastructure of the Internet consists of its hardware components and a system of software layers that control various aspects of the architecture. While the hardware can often be used to support other software systems, it is the design and the rigorous standardization process of the software architecture that characterizes the Internet and provides the foundation for its scalability and success. The responsibility for the architectural design of the Internet software systems has been delegated to the Internet Engineering Task Force (IETF).[13] The IETF conducts standard-setting work groups, open to any individual, about the various aspects of Internet architecture. Resulting discussions and final standards are published in a series of publications, each called a Request for Comments (RFC), freely available on the IETF web site. The principal methods of networking that enable the Internet are contained in specially designated RFCs that constitute the Internet Standards. Other less rigorous documents are simply informative, experimental, or historical, or document the best current practices (BCP) when implementing Internet technologies.

The Internet Standards describe a framework known as the Internet Protocol Suite. This is a model architecture that divides methods into a layered system of protocols (RFC 1122, RFC 1123). The layers correspond to the environment or scope in which their services operate. At the top is the Application Layer, the space for the application-specific networking methods used in software applications, e.g., a web browser program. Below this top layer, the Transport Layer connects applications on different hosts via the network (e.g., client–server model) with appropriate data exchange methods. Underlying these layers are the core networking technologies, consisting of two layers. The Internet Layer enables computers to identify and locate each other via Internet Protocol (IP) addresses, and allows them to connect to one-another via intermediate (transit) networks. Lastly, at the bottom of the architecture, is a software layer, the Link Layer, that provides connectivity between hosts on the same local network link, such as a local area network (LAN) or a dial-up connection. The model, also known as TCP/IP, is designed to be independent of the underlying hardware which the model therefore does not concern itself with in any detail. Other models have been developed, such as the Open Systems Interconnection (OSI) model, but they are not compatible in the details of description, nor implementation, but many similarities exist and the TCP/IP protocols are usually included in the discussion of OSI networking.

The most prominent component of the Internet model is the Internet Protocol (IP) which provides addressing systems (IP addresses) for computers on the Internet. IP enables internetworking and essentially establishes the Internet itself. IP Version 4 (IPv4) is the initial version used on the first generation of the today's Internet and is still in dominant use. It was designed to address up to ~4.3 billion (109) Internet hosts. However, the explosive growth of the Internet has led to IPv4 address exhaustion which is estimated to enter its final stage in approximately 2011.[14] A new protocol version, IPv6, was developed in the mid 1990s which provides vastly larger addressing capabilities and more efficient routing of Internet traffic. IPv6 is currently in commercial deployment phase around the world and Internet address registries (RIRs) have begun to urge all resource managers to plan rapid adoption and conversion.[15]

IPv6 is not interoperable with IPv4. It essentially establishes a "parallel" version of the Internet not directly accessible with IPv4 software. This means software upgrades or translator facilities are necessary for every networking device that needs to communicate on the IPv6 Internet. Most modern computer operating systems are already converted to operate with both versions of the Internet Protocol. Network infrastructures, however, are still lagging in this development. Aside from the complex physical connections that make up its infrastructure, the Internet is facilitated by bi- or multi-lateral commercial contracts (e.g., peering agreements), and by technical specifications or protocols that describe how to exchange data over the network. Indeed, the Internet is defined by its interconnections and routing policies.

- Structure

The Internet structure and its usage characteristics have been studied extensively. It has been determined that both the Internet IP routing structure and hypertext links of the World Wide Web are examples of scale-free networks. Similar to the way the commercial Internet providers connect via Internet exchange points, research networks tend to interconnect into large subnetworks such as GEANT, GLORIAD, Internet2 (successor of the Abilene Network), and the UK's national research and education network JANET. These in turn are built around smaller networks (see also the list of academic computer network organizations).

Many computer scientists describe the Internet as a "prime example of a large-scale, highly engineered, yet highly complex system".[16] The Internet is extremely heterogeneous; for instance, data transfer rates and physical characteristics of connections vary widely. The Internet exhibits "emergent phenomena" that depend on its large-scale organization. For example, data transfer rates exhibit temporal self-similarity. The principles of the routing and addressing methods for traffic in the Internet reach back to their origins the 1960s when the eventual scale and popularity of the network could not be anticipated. Thus, the possibility of developing alternative structures is investigated.[17]

- Governance

Main article: Internet governance

ICANN headquarters in Marina Del Rey, California, United States

The Internet is a globally distributed network comprising many voluntarily interconnected autonomous networks. It operates without a central governing body. However, to maintain interoperability, all technical and policy aspects of the underlying core infrastructure and the principal name spaces are administered by the Internet Corporation for Assigned Names and Numbers (ICANN), headquartered in Marina del Rey, California. ICANN is the authority that coordinates the assignment of unique identifiers for use on the Internet, including domain names, Internet Protocol (IP) addresses, application port numbers in the transport protocols, and many other parameters. Globally unified name spaces, in which names and numbers are uniquely assigned, are essential for the global reach of the Internet. ICANN is governed by an international board of directors drawn from across the Internet technical, business, academic, and other non-commercial communities. The US government continues to have the primary role in approving changes to the DNS root zone that lies at the heart of the domain name system. ICANN's role in coordinating the assignment of unique identifiers distinguishes it as perhaps the only central coordinating body on the global Internet. On 16 November 2005, the World Summit on the Information Society, held in Tunis, established the Internet Governance Forum (IGF) to discuss Internet-related issues.

- Modern uses

The Internet is allowing greater flexibility in working hours and location, especially with the spread of unmetered high-speed connections and web applications.

The Internet can now be accessed almost anywhere by numerous means, especially through mobile Internet devices. Mobile phones, datacards, handheld game consoles and cellular routers allow users to connect to the Internet from anywhere there is a wireless network supporting that device's technology. Within the limitations imposed by small screens and other limited facilities of such pocket-sized devices, services of the Internet, including email and the web, may be available. Service providers may restrict the services offered and wireless data transmission charges may be significantly higher than other access methods.

Educational material at all levels from pre-school to post-doctoral is available from websites. Examples range from CBeebies, through school and high-school revision guides, virtual universities, to access to top-end scholarly literature through the likes of Google Scholar. In distance education, help with homework and other assignments, self-guided learning, whiling away spare time, or just looking up more detail on an interesting fact, it has never been easier for people to access educational information at any level from anywhere. The Internet in general and the World Wide Web in particular are important enablers of both formal and informal education.

The low cost and nearly instantaneous sharing of ideas, knowledge, and skills has made collaborative work dramatically easier, with the help of collaborative software. Not only can a group cheaply communicate and share ideas, but the wide reach of the Internet allows such groups to easily form in the first place. An example of this is the free software movement, which has produced, among other programs, Linux, Mozilla Firefox, and OpenOffice.org. Internet "chat", whether in the form of IRC chat rooms or channels, or via instant messaging systems, allow colleagues to stay in touch in a very convenient way when working at their computers during the day. Messages can be exchanged even more quickly and conveniently than via e-mail. Extensions to these systems may allow files to be exchanged, "whiteboard" drawings to be shared or voice and video contact between team members.

Version control systems allow collaborating teams to work on shared sets of documents without either accidentally overwriting each other's work or having members wait until they get "sent" documents to be able to make their contributions. Business and project teams can share calendars as well as documents and other information. Such collaboration occurs in a wide variety of areas including scientific research, software development, conference planning, political activism and creative writing. Social and political collaboration is also becoming more widespread as both Internet access and computer literacy grow. From the flash mob 'events' of the early 2000s to the use of social networking in the 2009 Iranian election protests, the Internet allows people to work together more effectively and in many more ways than was possible without it.

The Internet allows computer users to remotely access other computers and information stores easily, wherever they may be across the world. They may do this with or without the use of security, authentication and encryption technologies, depending on the requirements. This is encouraging new ways of working from home, collaboration and information sharing in many industries. An accountant sitting at home can audit the books of a company based in another country, on a server situated in a third country that is remotely maintained by IT specialists in a fourth. These accounts could have been created by home-working bookkeepers, in other remote locations, based on information e-mailed to them from offices all over the world. Some of these things were possible before the widespread use of the Internet, but the cost of private leased lines would have made many of them infeasible in practice. An office worker away from their desk, perhaps on the other side of the world on a business trip or a holiday, can open a remote desktop session into his normal office PC using a secure Virtual Private Network (VPN) connection via the Internet. This gives the worker complete access to all of his or her normal files and data, including e-mail and other applications, while away from the office. This concept has been referred to among system administrators as the Virtual Private Nightmare,[18] because it extends the secure perimeter of a corporate network into its employees' homes.

- Services

- Information

Many people use the terms Internet and World Wide Web, or just the Web, interchangeably, but the two terms are not synonymous. The World Wide Web is a global set of documents, images and other resources, logically interrelated by hyperlinks and referenced with Uniform Resource Identifiers (URIs). URIs allow providers to symbolically identify services and clients to locate and address web servers, file servers, and other databases that store documents and provide resources and access them using the Hypertext Transfer Protocol (HTTP), the primary carrier protocol of the Web. HTTP is only one of the hundreds of communication protocols used on the Internet. Web services may also use HTTP to allow software systems to communicate in order to share and exchange business logic and data.

World Wide Web browser software, such as Microsoft's Internet Explorer, Mozilla Firefox, Opera, Apple's Safari, and Google Chrome, let users navigate from one web page to another via hyperlinks embedded in the documents. These documents may also contain any combination of computer data, including graphics, sounds, text, video, multimedia and interactive content including games, office applications and scientific demonstrations. Through keyword-driven Internet research using search engines like Yahoo! and Google, users worldwide have easy, instant access to a vast and diverse amount of online information. Compared to printed encyclopedias and traditional libraries, the World Wide Web has enabled the decentralization of information.

The Web has also enabled individuals and organizations to publish ideas and information to a potentially large audience online at greatly reduced expense and time delay. Publishing a web page, a blog, or building a website involves little initial cost and many cost-free services are available. Publishing and maintaining large, professional web sites with attractive, diverse and up-to-date information is still a difficult and expensive proposition, however. Many individuals and some companies and groups use web logs or blogs, which are largely used as easily updatable online diaries. Some commercial organizations encourage staff to communicate advice in their areas of specialization in the hope that visitors will be impressed by the expert knowledge and free information, and be attracted to the corporation as a result. One example of this practice is Microsoft, whose product developers publish their personal blogs in order to pique the public's interest in their work. Collections of personal web pages published by large service providers remain popular, and have become increasingly sophisticated. Whereas operations such as Angelfire and GeoCities have existed since the early days of the Web, newer offerings from, for example, Facebook and MySpace currently have large followings. These operations often brand themselves as social network services rather than simply as web page hosts.

Advertising on popular web pages can be lucrative, and e-commerce or the sale of products and services directly via the Web continues to grow.

When the Web began in the 1990s, a typical web page was stored in completed form on a web server, formatted with HTML, ready to be sent to a user's browser in response to a request. Over time, the process of creating and serving web pages has become more automated and more dynamic. Websites are often created using content management or wiki software with, initially, very little content. Contributors to these systems, who may be paid staff, members of a club or other organization or members of the public, fill underlying databases with content using editing pages designed for that purpose, while casual visitors view and read this content in its final HTML form. There may or may not be editorial, approval and security systems built into the process of taking newly entered content and making it available to the target visitors.

- Communication

E-mail is an important communications service available on the Internet. The concept of sending electronic text messages between parties in a way analogous to mailing letters or memos predates the creation of the Internet. Today it can be important to distinguish between internet and internal e-mail systems. Internet e-mail may travel and be stored unencrypted on many other networks and machines out of both the sender's and the recipient's control. During this time it is quite possible for the content to be read and even tampered with by third parties, if anyone considers it important enough. Purely internal or intranet mail systems, where the information never leaves the corporate or organization's network, are much more secure, although in any organization there will be IT and other personnel whose job may involve monitoring, and occasionally accessing, the e-mail of other employees not addressed to them. Pictures, documents and other files can be sent as e-mail attachments. E-mails can be cc-ed to multiple e-mail addresses.

Internet telephony is another common communications service made possible by the creation of the Internet. VoIP stands for Voice-over-Internet Protocol, referring to the protocol that underlies all Internet communication. The idea began in the early 1990s with walkie-talkie-like voice applications for personal computers. In recent years many VoIP systems have become as easy to use and as convenient as a normal telephone. The benefit is that, as the Internet carries the voice traffic, VoIP can be free or cost much less than a traditional telephone call, especially over long distances and especially for those with always-on Internet connections such as cable or ADSL. VoIP is maturing into a competitive alternative to traditional telephone service. Interoperability between different providers has improved and the ability to call or receive a call from a traditional telephone is available. Simple, inexpensive VoIP network adapters are available that eliminate the need for a personal computer.

Voice quality can still vary from call to call but is often equal to and can even exceed that of traditional calls. Remaining problems for VoIP include emergency telephone number dialing and reliability. Currently, a few VoIP providers provide an emergency service, but it is not universally available. Traditional phones are line-powered and operate during a power failure; VoIP does not do so without a backup power source for the phone equipment and the Internet access devices. VoIP has also become increasingly popular for gaming applications, as a form of communication between players. Popular VoIP clients for gaming include Ventrilo and Teamspeak. Wii, PlayStation 3, and Xbox 360 also offer VoIP chat features.

- Data transfer

File sharing is an example of transferring large amounts of data across the Internet. A computer file can be e-mailed to customers, colleagues and friends as an attachment. It can be uploaded to a website or FTP server for easy download by others. It can be put into a "shared location" or onto a file server for instant use by colleagues. The load of bulk downloads to many users can be eased by the use of "mirror" servers or peer-to-peer networks. In any of these cases, access to the file may be controlled by user authentication, the transit of the file over the Internet may be obscured by encryption, and money may change hands for access to the file. The price can be paid by the remote charging of funds from, for example, a credit card whose details are also passed—usually fully encrypted—across the Internet. The origin and authenticity of the file received may be checked by digital signatures or by MD5 or other message digests. These simple features of the Internet, over a worldwide basis, are changing the production, sale, and distribution of anything that can be reduced to a computer file for transmission. This includes all manner of print publications, software products, news, music, film, video, photography, graphics and the other arts. This in turn has caused seismic shifts in each of the existing industries that previously controlled the production and distribution of these products.

Streaming media refers to the act that many existing radio and television broadcasters promote Internet "feeds" of their live audio and video streams (for example, the BBC). They may also allow time-shift viewing or listening such as Preview, Classic Clips and Listen Again features. These providers have been joined by a range of pure Internet "broadcasters" who never had on-air licenses. This means that an Internet-connected device, such as a computer or something more specific, can be used to access on-line media in much the same way as was previously possible only with a television or radio receiver. The range of available types of content is much wider, from specialized technical webcasts to on-demand popular multimedia services. Podcasting is a variation on this theme, where—usually audio—material is downloaded and played back on a computer or shifted to a portable media player to be listened to on the move. These techniques using simple equipment allow anybody, with little censorship or licensing control, to broadcast audio-visual material worldwide.

Webcams can be seen as an even lower-budget extension of this phenomenon. While some webcams can give full-frame-rate video, the picture is usually either small or updates slowly. Internet users can watch animals around an African waterhole, ships in the Panama Canal, traffic at a local roundabout or monitor their own premises, live and in real time. Video chat rooms and video conferencing are also popular with many uses being found for personal webcams, with and without two-way sound. YouTube was founded on 15 February 2005 and is now the leading website for free streaming video with a vast number of users. It uses a flash-based web player to stream and show video files. Registered users may upload an unlimited amount of video and build their own personal profile. YouTube claims that its users watch hundreds of millions, and upload hundreds of thousands of videos daily.[19]

- Access

See also: Internet access worldwide, List of countries by number of Internet users, English on the Internet, Global Internet usage, and Unicode

Graph of Internet users per 100 inhabitants between 1997 and 2007 by International Telecommunication Union

The prevalent language for communication on the Internet is English. This may be a result of the origin of the Internet, as well as English's role as a lingua franca. It may also be related to the poor capability of early computers, largely originating in the United States, to handle characters other than those in the English variant of the Latin alphabet. After English (28% of Web visitors) the most requested languages on the World Wide Web are Chinese (23%), Spanish (8%), Japanese (5%), Portuguese and German (4% each), Arabic, French and Russian (3% each), and Korean (2%).[20] By region, 42% of the world's Internet users are based in Asia, 24% in Europe, 14% in North America, 10% in Latin America and the Caribbean taken together, 5% in Africa, 3% in the Middle East and 1% in Australia/Oceania.[21] The Internet's technologies have developed enough in recent years, especially in the use of Unicode, that good facilities are available for development and communication in the world's widely used languages. However, some glitches such as mojibake (incorrect display of some languages' characters) still remain.

Common methods of Internet access in homes include dial-up, landline broadband (over coaxial cable, fiber optic or copper wires), Wi-Fi, satellite and 3G technology cell phones. Public places to use the Internet include libraries and Internet cafes, where computers with Internet connections are available. There are also Internet access points in many public places such as airport halls and coffee shops, in some cases just for brief use while standing. Various terms are used, such as "public Internet kiosk", "public access terminal", and "Web payphone". Many hotels now also have public terminals, though these are usually fee-based. These terminals are widely accessed for various usage like ticket booking, bank deposit, online payment etc. Wi-Fi provides wireless access to computer networks, and therefore can do so to the Internet itself. Hotspots providing such access include Wi-Fi cafes, where would-be users need to bring their own wireless-enabled devices such as a laptop or PDA. These services may be free to all, free to customers only, or fee-based. A hotspot need not be limited to a confined location. A whole campus or park, or even an entire city can be enabled. Grassroots efforts have led to wireless community networks. Commercial Wi-Fi services covering large city areas are in place in London, Vienna, Toronto, San Francisco, Philadelphia, Chicago and Pittsburgh. The Internet can then be accessed from such places as a park bench.[22] Apart from Wi-Fi, there have been experiments with proprietary mobile wireless networks like Ricochet, various high-speed data services over cellular phone networks, and fixed wireless services. High-end mobile phones such as smartphones generally come with Internet access through the phone network. Web browsers such as Opera are available on these advanced handsets, which can also run a wide variety of other Internet software. More mobile phones have Internet access than PCs, though this is not as widely used.[citation needed] An Internet access provider and protocol matrix differentiates the methods used to get online.

- Social impact

Main article: Sociology of the Internet

The Internet has enabled entirely new forms of social interaction, activities, and organizing, thanks to its basic features such as widespread usability and access. Social networking websites such as Facebook, Twitter and MySpace have created new ways to socialize and interact. Users of these sites are able to add a wide variety of information to pages, to pursue common interests, and to connect with others. It is also possible to find existing acquaintances, to allow communication among existing groups of people. Sites like LinkedIn foster commercial and business connections. YouTube and Flickr specialize in users' videos and photographs.

In the first decade of the 21st century the first generation is raised with widespread availability of Internet connectivity, bringing consequences and concerns in areas such as personal privacy and identity, and distribution of copyrighted materials. These "digital natives" face a variety of challenges that were not present for prior generations.

The Internet has achieved new relevance as a political tool, leading to Internet censorship by some states. The presidential campaign of Howard Dean in 2004 in the United States was notable for its success in soliciting donation via the Internet. Many political groups use the Internet to achieve a new method of organizing in order to carry out their mission, having given rise to Internet activism. Some governments, such as those of Iran, North Korea, Myanmar, the People's Republic of China, and Saudi Arabia, restrict what people in their countries can access on the Internet, especially political and religious content.[citation needed] This is accomplished through software that filters domains and content so that they may not be easily accessed or obtained without elaborate circumvention.[original research?]

In Norway, Denmark, Finland[23] and Sweden, major Internet service providers have voluntarily, possibly to avoid such an arrangement being turned into law, agreed to restrict access to sites listed by authorities. While this list of forbidden URLs is only supposed to contain addresses of known child pornography sites, the content of the list is secret.[citation needed] Many countries, including the United States, have enacted laws against the possession or distribution of certain material, such as child pornography, via the Internet, but do not mandate filtering software. There are many free and commercially available software programs, called content-control software, with which a user can choose to block offensive websites on individual computers or networks, in order to limit a child's access to pornographic materials or depiction of violence.

The Internet has been a major outlet for leisure activity since its inception, with entertaining social experiments such as MUDs and MOOs being conducted on university servers, and humor-related Usenet groups receiving much traffic. Today, many Internet forums have sections devoted to games and funny videos; short cartoons in the form of Flash movies are also popular. Over 6 million people use blogs or message boards as a means of communication and for the sharing of ideas. The pornography and gambling industries have taken advantage of the World Wide Web, and often provide a significant source of advertising revenue for other websites.[citation needed] Although many governments have attempted to restrict both industries' use of the Internet, this has generally failed to stop their widespread popularity.[citation needed]

One main area of leisure activity on the Internet is multiplayer gaming. This form of recreation creates communities, where people of all ages and origins enjoy the fast-paced world of multiplayer games. These range from MMORPG to first-person shooters, from role-playing games to online gambling. This has revolutionized the way many people interact[citation needed] while spending their free time on the Internet. While online gaming has been around since the 1970s,[citation needed] modern modes of online gaming began with subscription services such as GameSpy and MPlayer. Non-subscribers were limited to certain types of game play or certain games. Many people use the Internet to access and download music, movies and other works for their enjoyment and relaxation. Free and fee-based services exist for all of these activities, using centralized servers and distributed peer-to-peer technologies. Some of these sources exercise more care with respect to the original artists' copyrights than others.

Many people use the World Wide Web to access news, weather and sports reports, to plan and book vacations and to find out more about their interests. People use chat, messaging and e-mail to make and stay in touch with friends worldwide, sometimes in the same way as some previously had pen pals. The Internet has seen a growing number of Web desktops, where users can access their files and settings via the Internet.

Cyberslacking can become a drain on corporate resources; the average UK employee spent 57 minutes a day surfing the Web while at work, according to a 2003 study by Peninsula Business Services.[24] Internet addiction disorder is excessive computer use that interferes with daily life. Some psychologists believe that Internet use has other effects on individuals for instance interfering with the deep thinking that leads to true creativity.[citation needed]

- See also

|  | Book:Internet |
| --- | --- |
| Books are collections of articles that can be downloaded or ordered in print. |  |

|  | Internet portal |
| --- | --- |

- All pages with titles containing "Internet"

- Internet organizations

- Internet Strategy Forum

- List of Internet topics

- Network Neutrality

- Outline of the Internet

- Notes

- ^ "Links". HTML 4.01 Specification. World Wide Web Consortium. HTML 4.01 Specification. http://www.w3.org/TR/html401/struct/links.html#h-12.1. Retrieved 2008-08-13. "[T]he link (or hyperlink, or Web link) [is] the basic hypertext construct. A link is a connection from one Web resource to another. Although a simple concept, the link has been one of the primary forces driving the success of the Web."

- ^ "ARPA/DARPA". Defense Advanced Research Projects Agency. Archived from the original on April 7, 2007. http://web.archive.org/web/20070407064829/http://www.darpa.mil/body/arpa_darpa.html. Retrieved 2007-05-21.

- ^ "DARPA: History". Defense Advanced Research Projects Agency. http://www.darpa.mil/history.html. Retrieved 2009-12-07.

- ^ Baran, Paul (1964). On Distributed Communications. http://www.rand.org/pubs/research_memoranda/RM3767. Retrieved 2010-07-05.

- ^ "Internet History". Living Internet site. http://www.livinginternet.com/i/ii.htm. Retrieved 2010-03-10.

- ^ "Electronic post for switching data." Timothy Johnson. New Scientist. May 13, 1976

- ^ Stephen Segaller, NERDS 2.0.1:115 (TV Books Publisher 1998)

- ^ Coffman, K. G; Odlyzko, A. M. (1998-10-02) (PDF). The size and growth rate of the Internet. AT&T Labs. http://www.dtc.umn.edu/~odlyzko/doc/internet.size.pdf. Retrieved 2007-05-21.

- ^ Comer, Douglas (2006). The Internet book. Prentice Hall. p. 64. ISBN 0132335530.

- ^ "World Internet Users and Population Stats". Internet World Stats. Miniwatts Marketing Group. 2010-06-30. http://www.internetworldstats.com/stats.htm. Retrieved 2010-08-04.

- ^ http://www.ibtimes.com/art/services/print.php?articleid=48062

- ^ http://www.thejakartaglobe.com/business/internet-users-in-indonesia-to-triple-by-2015-report/394066

- ^ "IETF Home Page". Ietf.org. http://www.ietf.org/. Retrieved 2009-06-20.

- ^ Huston, Geoff. "IPv4 Address Report, daily generated". http://www.potaroo.net/tools/ipv4/index.html. Retrieved 2009-05-20.

- ^ "Notice of Internet Protocol version 4 (IPv4) Address Depletion" (PDF). https://www.arin.net/knowledge/about_resources/ceo_letter.pdf. Retrieved 2009-08-07.

- ^ Walter Willinger, Ramesh Govindan, Sugih Jamin, Vern Paxson, and Scott Shenker (2002). Scaling phenomena in the Internet, in Proceedings of the National Academy of Sciences, 99, suppl. 1, 2573–2580

- ^ "Internet Makeover? Some argue it's time". The Seattle Times, 2007-04-16.

- ^ "The Virtual Private Nightmare: VPN". Librenix. 2004-08-04. http://librenix.com/?inode=5013. Retrieved 2010-07-21.

- ^ "YouTube Fact Sheet". YouTube, LLC. http://www.youtube.com/t/fact_sheet. Retrieved 2009-01-20.

- ^ Internet World Stats, updated for 2009-12-31. Retrieved 2010-07-17.

- ^ World Internet Usage Statistics News and Population Stats updated for 31 December 2009. Retrieved 17 July 2010.

- ^ "Toronto Hydro to Install Wireless Network in Downtown Toronto". Bloomberg.com. Retrieved 19-Mar-2006.

- ^ "Finland censors anti-censorship site". The Register. 2008-02-18. http://www.theregister.co.uk/2008/02/18/finnish_policy_censor_activist/. Retrieved 2008-02-19.

- ^ "Scotsman.com News - Net abuse hits small city firms". News.scotsman.com. http://news.scotsman.com/topics.cfm?tid=914&id=1001802003. Retrieved 2009-08-07.

- References

- Media Freedom Internet Cookbook by the OSCE Representative on Freedom of the Media Vienna, 2004

- Living Internet—Internet history and related information, including information from many creators of the Internet

- First Monday peer-reviewed journal on the Internet

- How Much Does The Internet Weigh? by Stephen Cass, Discover 2007

- Rehmeyer, Julie J. 2007. Mapping a medusa: The Internet spreads its tentacles. Science News 171(June 23):387-388. Available at Sciencenews.org

- Castells, M. 1996. Rise of the Network Society. 3 vols. Vol. 1. Cambridge, MA: Blackwell Publishers.

- Castells, M. (2001), “Lessons from the History of Internet”, in “The Internet Galaxy”, Ch. 1, pp 9–35. Oxford University Press.

- RFC 1122, Requirements for Internet Hosts—Communication Layers, IETF, R. Braden (Ed.), October 1989

- RFC 1123, Requirements for Internet Hosts—Application and Support, IETF, R. Braden (Ed.), October 1989

- External links

| Find more about Internet on Wikipedia's sister projects: |  |
| --- | --- |
|  | Definitions from Wiktionary |
|  | Textbooks from Wikibooks |
|  | Quotations from Wikiquote |
|  | Source texts from Wikisource |
|  | Images and media from Commons |
|  | News stories from Wikinews |
|  | Learning resources from Wikiversity |

- The Internet (National Science Foundation)

- "10 Years that changed the world" — Wired looks back at the evolution of the Internet over last 10 years

- Berkman Center for Internet and Society at Harvard

- CBC Digital Archives—Inventing the Internet Age

- History of the Internet

- How the Internet Came to Be

- The Internet Society History Page

- RFC 801, planning the TCP/IP switchover

- Preparing Europe’s digital future - i2010 Mid-Term Review

- Manjoo, Farhad - The Unrecognizable Internet of 1996 - Slate

Retrieved from "http://en.wikipedia.org/wiki/Internet"

The Telecommunication Standardization Sector (ITU-T) coordinates standards for telecommunications on behalf of the International Telecommunication Union (ITU) and is based in Geneva, Switzerland.

The standardization work of ITU dates back to 1865, with the birth of the International Telegraph Union. It became a United Nations specialized agency in 1947, and the International Telegraph and Telephone Consultative Committee (CCITT), (from the French name "Comité Consultatif International Téléphonique et Télégraphique") was created in 1956. It was renamed ITU-T in 1993.[1]

ITU has been an intergovernmental public-private partnership organization since its inception and now has a membership of 191 countries (Member States) and over 700 public and private sector companies as well as international and regional telecommunication entities, known as Sector Members and Associates, which undertake most of the work of the Sector.[2]

ITU-T has a permanent secretariat, the Telecommunication Standardization Bureau (TSB), based at the ITU HQ in Geneva, Switzerland. The elected Director of the Bureau is Mr. Malcolm Johnson of the UK. Mr. Johnson was elected by the ITU Membership to the Directorship for a 4-year term in November 2006.

- Contents [hide] - 1 Primary function - 2 History - 3 "Real time" standardization - 3.1 Development of Recommendations - 3.2 Approval of Recommendations - 4 Series and Recommendations - 4.1 Series of ITU Recommendations - 4.2 International Telecommunication Regulations (ITRs) - 4.3 Key standards published by ITU - 5 Hot topics - 6 See also - 7 References - 8 External links

- [edit] Primary function

The ITU-T mission is to ensure the efficient and timely production of standards covering all fields of telecommunications on a worldwide basis, as well as defining tariff and accounting principles for international telecommunication services.[3]

The international standards that are produced by the ITU-T are referred to as "Recommendations" (with the word ordinarily capitalized to distinguish its meaning from the ordinary sense of the word "recommendation"), as they become mandatory only when adopted as part of a national law.

Since the ITU-T is part of the ITU, which is a United Nations specialized agency, its standards carry more formal international weight than those of most other standards development organizations that publish technical specifications of a similar form.[4]

- [edit] History

Although the ITU itself dates back to 1865[5], the formal standardization processes are more recent.

Two consultative committees were created by the ITU’s 1925 Paris conference to deal with the complexities of the international telephone services (known as CCIF, as the French acronym) and long-distance telegraphy (CCIT).[6]

In view of the basic similarity of many of the technical problems faced by the CCIF and CCIT, a decision was taken in 1956 to merge them to become the single International Telegraph and Telephone Consultative Committee (CCITT, in the French acronym).[6]

In 1992, the Plenipotentiary Conference (the top policy-making conference of ITU) saw a reform of ITU, giving the Union greater flexibility to adapt to an increasingly complex, interactive and competitive environment. It was at this time that CCITT was renamed the Telecommunication Standardization Sector (ITU-T), as one of three Sectors of the Union alongside the Radiocommunication Sector (ITU-R) and the Telecommunication Development Sector (ITU-D).[7]

Historically, the Recommendations of the CCITT were presented to four-yearly plenary assemblies for endorsement, and the full set of Recommendations were published after each plenary assembly. However, the delays in producing texts, and translating them into other working languages, did not suit the fast pace of change in the telecommunications industry.[8]

- [edit] "Real time" standardization

The rise of the personal computer industry in the early 1980s created a new common practice among both consumers and businesses of adopting "bleeding edge" communications technology even if it was not yet standardized. Thus, standards organizations had to put forth standards much faster, or find themselves ratifying de facto standards after the fact.

The ITU-T now operates under much more streamlined processes. The time between an initial proposal of a draft document by a member company and the final approval of a full-status ITU-T Recommendation can now be as short as a few months (or less in some cases). This makes the standardization approval process in the ITU-T much more responsive to the needs of rapid technology development than in the ITU's historical past.[9] New and updated Recommendations are published on an almost daily basis, and much of the library of over 3,270 Recommendations is now free of charge online[10][11][12]. (Specifications jointly maintained by the ITU-T and ISO/IEC are not free[13].)

ITU-T has moreover tried to facilitate cooperation between the various forums and standard-developing organizations (SDOs). This collaboration is necessary to avoid duplication of work and the consequent risk of conflicting standards in the market place.[14]

In the work of standardization, ITU-T cooperates with other SDOs, e.g., the International Organization for Standardization (ISO) and the Internet Engineering Task Force (IETF)[15]

- [edit] Development of Recommendations

Most of the work of ITU-T is carried out by its Sector Members and Associates, while the Telecommunication Standardization Bureau (TSB) is the executive arm of ITU-T and coordinator for a number of workshops and seminars to progress existing work areas and explore new ones. The events cover a wide array of topics in the field of information and communication technologies (ICT) and attract high-ranking experts as speakers, and attendees from engineers to high-level management from all industry sectors.[16]

The technical work, the development of Recommendations, of ITU-T is managed by Study Groups (SGs). The people involved in these SGs are experts in telecommunications from all over the world. There are currently 13 SGs. Study groups meet face to face according to a calendar issued by the TSB.[17] SGs are augmented by Focus Groups (FGs), an instrument created by ITU-T, providing a way to quickly react to ICT standardization needs and allowing great flexibility in terms of participation and working methods. The key difference between SGs and FGs is that the latter have greater freedom to organize and finance themselves, and to involve non-members in their work. Focus Groups can be created very quickly, are usually short-lived and can choose their own working methods, leadership, financing, and types of deliverables.[18] Recent examples include work on Next Generation Networking, Internet Protocol Television (IPTV) and digital identity management.[19]

- [edit] Approval of Recommendations

The “Alternative Approval Process” (AAP) is a fast-track approval procedure that was developed to allow standards to be brought to market in the timeframe that industry now demands.

This dramatic overhaul of standards-making by streamlining approval procedures was implemented in 2001 and is estimated to have cut the time involved in this critical aspect of the standardization process by 80 to 90 per cent. This means that an average standard which took around four years to approve and publish until the mid nineties, and two years until 1997, can now be approved in an average of two months, or as little as five weeks.

Besides streamlining the underlying procedures involved in the approval process, an important contributory factor to the use of AAP is electronic document handling. Once the approval process has begun the rest of the process can be completed electronically, in the vast majority of cases, with no further physical meetings.

The introduction of AAP also formalizes public/private partnership in the approval process by providing equal opportunities for both Sector Members and Member States in the approval of technical standards.

Once the text of a draft Recommendation prepared by SG experts is considered mature, it is submitted for review to an SG meeting. If agreed by the meeting it is given Consent. This means that the SG has given its consent that the text is sufficiently mature to initiate a final review process leading to approval of the draft Recommendation.

After this Consent has been achieved, TSB announces the start of the AAP procedure by posting the draft text to the ITU-T web site and calling for comments. This gives the opportunity for all members to review the text. This phase, called Last Call, is a four-week period in which comments can be submitted by Member States and Sector Members.

If no comments other than editorial corrections are received, the Recommendation is considered approved since no issues were identified that might need any further work. However, if there are any comments, the SG chairman, in consultation with TSB, sets up a comment resolution process by the concerned experts. The revised text is then posted on the web for an Additional Review period of three weeks.

Similar to the Last Call phase, in Additional Review the Recommendation is considered as approved if no comments are received. If comments are received, it is apparent that there are some issues that still need more work, and the draft text and all comments are sent to the next Study Group meeting for further discussion and possible approval.[20]

Those Recommendations considered as having policy or regulatory implications are approved through what is known as the “Traditional Approval Process” (TAP), which allows a longer period for reflection and commenting by Member States. TAP Recommendations are also translated into the six working languages of ITU (Arabic, Chinese, English, French, Russian and Spanish).[21]

- [edit] Series and Recommendations

ITU-T Recommendations are the names given to telecommunications and computer protocol specification documents published by ITU-T.

Many of the Recommendations that define OSI are also ISO standards.

Standards for Internet protocols are typically developed in the IETF, and standards for mobile telephone systems are developed in ETSI and other forums.

- [edit] Series of ITU Recommendations

ITU-T issues Recommendations that have names like X.500, where X is the series and 500 is an identifying number. When a Recommendation is updated, it will (mostly) keep the same number, so the year of issue may be necessary to identify a specific version of a Recommendation. The term "X.500" is used both to refer to the specific X.500 Recommendation, and to the entire family of Recommendations named X.5xx, where the specific X.500 Recommendation forms the introduction and overview to the set.

| Series | Description |
| --- | --- |
| A | Organization of the work of ITU-T |
| B | Means of expression: definitions, symbols, classification |
| C | General telecommunication statistics |
| D | General tariff principles |
| E | Overall network operation, telephone service, service operation and human factors |
| F | Non-telephone telecommunication services |
| G | Transmission systems and media, digital systems and networks |
| H | Audiovisual and multimedia systems |
| I | Integrated services digital network |
| J | Cable networks and transmission of television, sound programme and other multimedia signals |
| K | Protection against interference |
| L | Construction, installation and protection of cables and other elements of outside plant |
| M | Telecommunication management, including TMN and network maintenance |
| N | Maintenance: international sound programme and television transmission circuits |
| O | Specifications of measuring equipment |
| P | Telephone transmission quality, telephone installations, local line networks |
| Q | Switching and signalling |
| R | Telegraph transmission |
| S | Telegraph services terminal equipment |
| T | Terminals for telematic services |
| U | Telegraph switching |
| V | Data communication over the telephone network |
| X | Data networks, open system communications and security |
| Y | Global information infrastructure, Internet protocol aspects and next-generation networks |
| Z | Languages and general software aspects for telecommunication systems |

G Series Recommendations
Transmission systems and media, digital systems and networks

| Specification Range | Specification Type |
| --- | --- |
| G.100 - G.199 | INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS |
| G.200 - G.299 | GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIERTRANSMISSION SYSTEMS |
| G.300 - G.399 | INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES |
| G.400 - G.449 | GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES |
| G.450 - G.499 | COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY |
| G.600 - G.699 | TRANSMISSION MEDIA AND OPTICAL SYSTEMS CHARACTERISTICS |
| G.700 - G.799 | DIGITAL TERMINAL EQUIPMENTS |
| G.800 - G.899 | DIGITAL NETWORKS |
| G.900 - G.999 | DIGITAL SECTIONS AND DIGITAL LINE SYSTEM |
| G.1000 - G.1999 | QUALITY OF SERVICE AND PERFORMANCE – GENERIC AND USER-RELATED ASPECTS |
| G.6000 - G.6999 | TRANSMISSION MEDIA CHARACTERISTICS |
| G.7000 - G.7999 | DATA OVER TRANSPORT – GENERIC ASPECTS |
| G.8000 - G.8999 | PACKET OVER TRANSPORT ASPECTS |
| G.8000 - G.8099 | Ethernet over Transport aspects |
| G.8100 - G.8199 | MPLS over Transport aspects |
| G.8200 - G.8299 | Quality and availability targets |
| G.8600 - G.8699 | Service Management |
| G.9000 - G.9999 | ACCESS NETWORKS |

Y Series Recommendations
Global Information Infrastructure, Internet Protocol Aspects And Nextgeneration Networks

| Specification Range | Specification Type |
| --- | --- |
| GLOBAL INFORMATION INFRASTRUCTURE |  |
| Y.100–Y.199 | General |
| Y.200–Y.299 | Services, applications and middleware |
| Y.300–Y.399 | Network aspects |
| Y.400–Y.499 | Interfaces and protocols |
| Y.500–Y.599 | Numbering, addressing and naming |
| Y.600–Y.699 | Operation, administration and maintenance |
| Y.700–Y.799 | Security |
| Y.800–Y.899 | Performances |
| INTERNET PROTOCOL ASPECTS |  |
| Y.1000–Y.1099 | General |
| Y.1100–Y.1199 | Services and applications |
| Y.1200–Y.1299 | Architecture, access, network capabilities and resource management |
| Y.1300–Y.1399 | Transport |
| Y.1400–Y.1499 | Interworking |
| Y.1500–Y.1599 | Quality of service and network performance |
| Y.1600–Y.1699 | Signalling |
| Y.1700–Y.1799 | Operation, administration and maintenance |
| Y.1800–Y.1899 | Charging |
| NEXT GENERATION NETWORKS |  |
| Y.2000–Y.2099 | Frameworks and functional architecture models |
| Y.2100–Y.2199 | Quality of Service and performance |
| Y.2200–Y.2249 | Service aspects: Service capabilities and service architecture |
| Y.2250–Y.2299 | Service aspects: Interoperability of services and networks in NGN |
| Y.2300–Y.2399 | Numbering, naming and addressing |
| Y.2400–Y.2499 | Network management |
| Y.2500–Y.2599 | Network control architectures and protocols |
| Y.2700–Y.2799 | Security |
| Y.2800–Y.2899 | Generalized mobility |

- [edit] International Telecommunication Regulations (ITRs)

In addition to the ITU-T Recommendations, which have non-mandatory status until they are adopted in national laws, ITU-T is also the custodian of a binding international treaty, the International Telecommunication Regulations. The ITRs go back to the earliest days of the ITU when there were two separate treaties, dealing with telegraph and telephone. The ITRs were adopted, as a single treaty, at the World Administrative Telegraphy and Telephone Conference held in Melbourne, 1988 (WATTC-88).[22]

In line with the current Constitution and Convention of ITU, the ITRs can be amended through a World Conference on International Telecommunications (WCIT), and the next is scheduled for 2012. Before then a process of review of the ITRs, which began in 1998, will continue.[23]

The ITRs comprise ten articles which deal, inter alia, with the definition of international telecommunication services, cooperation between countries and national administrations, safety of life and priority of telecommunications and charging and accounting principles. The adoption of the ITRs in 1988 is often taken as the start of the wider liberalization process in international telecommunications, though a few countries, including United States and United Kingdom, had made steps to liberalize their markets before 1988.[24]

- [edit] Key standards published by ITU

- ASN.1 (Abstract Syntax Notation One)

- Coding of audio G.711 and G.72x series

- Coding of still images JPEG T.80 and JPEG 2000 T.800 series

- Coding of video coding H.262/MPEG2-Video and H.264/MPEG-4 AVC

- Construction, installation and protection of cables and other elements of outside plant, L-series

- Data communication over the telephone network, V-series

- Fax standards T.2 – T.4, T.30, T.37, T.38

- G.hn (Next generation wired home networking over powerlines, phonelines and coaxial cable)

- H.323 family of standards for multimedia and VoIP

- Interconnection rate harmonization, D-series

- International Emergency Preference Scheme E.106

- IMSI codes used in SIM cards E.212

- ISDN and PSTN/3G videoconferencing systems, H.320 and H.324

- ISDN (Integrated Services Digital Network) Q.931

- Open Systems Interconnection

- Optical Transport Network (OTN) G.709, G.798

- Passive optical networks (PON) G.983, G.984

- Public Key Infrastructure (PKI) X.509

- Public telecommunication numbering plan, E.164

- Security framework X.805

- Signalling System 7 Q.7xx series

- Standards relating to Quality of Service (QoS)

- Specification and Description Language

- Synchronous Digital Hierarchy (SDH) G.707 – G.803

- Wavelength-division multiplexing (WDM)

- X.25

- (x)DSL (Digital Subscriber Line) series of standards for broadband telecoms

- [edit] Hot topics

- ITU-T is committed to “bridging the standardization gap” – disparities in the ability of developing countries, relative to developed ones, to access, implement, contribute to and influence international ICT standards. Find out more about the work here.

- The ICT Security Standards Roadmap has been developed to assist in the development of security standards by bringing together information about existing standards and current standards work in key standards development organizations.

- The Next Generation Networks (NGN) concept takes into consideration new realities in the telecommunication industry characterized by factors such as; the need to converge and optimize the operating networks and the extraordinary expansion of digital traffic (i.e., increasing demand for new multimedia services, mobility, etc.).

- [edit] See also

- International Telecommunication Union

- Category:ITU-T recommendations

- Category:ITU-R recommendations

- World Summit on the Information Society

- Global Standards Collaboration

- [edit] References

- ^ About ITU

- ^ About ITU

- ^ ITU-T and TSB General Information

- ^ http://www.apdip.net/publications/iespprimers/eprimer-igov.pdf (p13)

- ^ About ITU

- ^ a b http://www.itu.int/ITU-T/50/docs/ITU-T_50.pdf (p8)

- ^ http://www.itu.int/ITU-T/50/docs/ITU-T_50.pdf (p14)

- ^ CCITT - 50 YEARS OF EXCELLENCE - 1956-2006

- ^ http://www.itu.int/ITU-T/50/docs/ITU-T_50.pdf (p16)

- ^ ITU-T Standards now freely available online

- ^ Telecom Standards Newsletter, September 1 2007, Free access for all to ITU-T standards

- ^ Free access for all to ITU-T standards

- ^ ITU-T Recommendations overview

- ^ http://www.itu.int/ITU-T/50/docs/ITU-T_50.pdf (p17)

- ^ http://www.itu.int/dms_pub/itu-t/oth/0A/0F/T0A0F0000090001PDFE.pdf (s10)

- ^ http://www.itu.int/dms_pub/itu-t/oth/0A/0F/T0A0F0000090001PDFE.pdf (s13-16), http://www.itu.int/ITU-T/info/tsb/info.html

- ^ http://www.itu.int/dms_pub/itu-t/oth/0A/0F/T0A0F0000090001PDFE.pdf (s16), http://www.itu.int/ITU-T/studygroups/index.html

- ^ http://www.itu.int/dms_pub/itu-t/oth/0A/0F/T0A0F0000090001PDFE.pdf (s23-24), http://www.itu.int/ITU-T/focusgroups/index.html

- ^ ITU Telecommunication Standardization Sector (ITU-T) - Focus Groups

- ^ http://www.itu.int/dms_pub/itu-t/oth/0A/0F/T0A0F0000090001PDFE.pdf (s28-29), http://www.itu.int/ITU-T/e-flash/022-jan06.html

- ^ http://www.itu.int/dms_pub/itu-t/oth/0A/0F/T0A0F0000090001PDFE.pdf (s27), http://www.itu.int/ITU-T/e-flash/022-jan06.html

- ^ http://www.itu.int/ITU-T/itr/files/ITR-e.doc

- ^ ITU-T - Review of the International Telecommunication Regulations (ITRs)

- ^ http://www.itu.int/osg/spu/stratpol/ITRs/auth/itr-03.doc

- [edit] External links

- International Telecommunication Union (ITU) (official site)

- ITU Standardization Sector (ITU-T)

- ITU Radiocommunication Sector (ITU-R)

- ITU Development Sector (ITU-D)

- ITU-T Study Groups

- ITU-T Focus Groups

- ITU-T Recommendations

- ITU-T Newslog (RSS available)

- ITU-T Guide for beginners (PDF)

- 50 years of CCITT/ITU-T (PDF)

[hide] v • d • e Multimedia compression formats. Video compression; ISO/IEC; MJPEG · Motion JPEG 2000 · MPEG-1 · MPEG-2 (Part 2) · MPEG-4 (Part 2/ASP · Part 10/AVC) · HEVC. ITU-T; H.120 · H.261 · H.262 · H.263 · H.264 · HEVC. Others; AVS · Bink · CineForm · Cinepak · Dirac · DV · Indeo · Microsoft Video 1 · OMS Video · Pixlet · RealVideo · RTVideo · SheerVideo · Smacker · Sorenson Video & Sorenson Spark · Theora · VC-1 · VC-2 · VC-3 · VP3 · VP6 · VP7 · VP8 · WMV.. Audio compression; ISO/IEC; MPEG-1 Layer III (MP3) · MPEG-1 Layer II (Multichannel) · MPEG-1 Layer I · AAC · HE-AAC · MPEG Surround · MPEG-4 ALS · MPEG-4 SLS · MPEG-4 DST · MPEG-4 HVXC · MPEG-4 CELP. ITU-T; G.711 · G.718 · G.719 · G.722 · G.722.1 · G.722.2 · G.723 · G.723.1 · G.726 · G.728 · G.729 · G.729.1. Others; AC-3 · AMR · AMR-WB · AMR-WB+ · Apple Lossless · ATRAC · CELT · DRA · DTS · EVRC · EVRC-B · FLAC · GSM-HR · GSM-FR · GSM-EFR · iLBC · iSAC · Monkey's Audio · TTA (True Audio) · MT9 · μ-law · Musepack · Nellymoser · OptimFROG · OSQ · QCELP · RealAudio · RTAudio · SD2 · SHN · SILK · Siren · SMV · Speex · SVOPC · TwinVQ · VMR-WB · Vorbis · WavPack · WMA.. Image compression; ISO/IEC/ITU-T; JPEG · JPEG 2000 · JPEG XR · lossless JPEG · JBIG · JBIG2 · PNG. Others; APNG · BMP · DjVu · EXR · GIF · ICER · ILBM · MNG · PCX · PGF · TGA · QTVR · TIFF · WBMP · WebP.. Media containers; ISO/IEC; MPEG-PS · MPEG-TS · ISO base media file format · MPEG-4 Part 14 · Motion JPEG 2000 · MPEG-21 Part 9. ITU-T; H.222.0 · T.802. Others; 3GP and 3G2 · AMV · ASF · AIFF · AVI · AU · Bink · DivX Media Format · DPX · EVO · Flash Video · GXF · M2TS · Matroska · MXF · Ogg · QuickTime File Format · RealMedia · REDCODE RAW · RIFF · Smacker · MOD and TOD · VOB · WAV · WebM.. See Compression methods for methods and Compression software implementations for codecs.

Retrieved from "http://en.wikipedia.org/wiki/ITU-T"

Categories: International Telecommunication Union | Standards organizations

From Wikipedia, the free encyclopedia

Jump to: navigation, search

|  | This article relies largely or entirely upon a single source. Please help improve this article by introducing appropriate citations to additional sources. (April 2010) |
| --- | --- |

Nanoscale network, short for Nanoscale communication network, is a telecommunications technology that provides networked communication among objects that are the size of biological cells and smaller[1][2]. The goal is to achieve efficient network communication at the smallest possible scale. The communication channels are on the order of individual molecules.

- [edit] Approaches

Nanoscale and molecular networks are being developed by a variety of means, for example, leveraging biology including molecular motors, calcium signaling, and gap junctions as well as non-biological mechanisms such as diffusion, carbon nanotubes, and quantum mechanical phenomena.

- [edit] External links

- IEEE NanoCom Subcommittee

- [edit] Notes

- ^ Nanoscale Communication Networks, Bush, S. F., ISBN 978-1-60807-003-9, Artech House, 2010 [1]

- ^ UCI Netgroup Wiki on Molecular Communication

Retrieved from "http://en.wikipedia.org/wiki/Nanoscale_network"

Categories: Nanotechnology | Network architecture | Computer networks | Telecommunications

Hidden categories: Articles needing additional references from April 2010 | All articles needing additional references

From Wikipedia, the free encyclopedia

Jump to: navigation, search

The term network monitoring describes the use of a system that constantly monitors a computer network for slow or failing components and that notifies the network administrator (via email, pager or other alarms) in case of outages. It is a subset of the functions involved in network management.

- Contents [hide] - 1 Details - 2 Network Tomography - 3 Route Analytics - 4 Various types of protocols - 5 Servers around the globe - 6 See also - 7 Notes and references - 8 External links

- [edit] Details

While an intrusion detection system monitors a network for threats from the outside, a network monitoring system monitors the network for problems caused by overloaded and/or crashed servers, network connections or other devices.

For example, to determine the status of a webserver, monitoring software may periodically send an HTTP request to fetch a page. For email servers, a test message might be sent through SMTP and retrieved by IMAP or POP3.

Commonly measured metrics are response time, availability and uptime, although both consistency and reliability metrics are starting to gain popularity. The widespread addition of WAN optimization devices is having an adverse effect on most network monitoring tools -- especially when it comes to measuring accurate end-to-end response time because they limit round trip visibility.[1]

Status request failures - such as when a connection cannot be established, it times-out, or the document or message cannot be retrieved - usually produce an action from the monitoring system. These actions vary -- an alarm may be sent (via SMS, email, etc.) to the resident sysadmin, automatic failover systems may be activated to remove the troubled server from duty until it can be repaired, etc.

Monitoring the performance of a network uplink is also known as network traffic measurement, and more software is listed there.

- [edit] Network Tomography

Network tomography is an important area of network measurement, which deals with monitoring the health of various links in a network using end-to-end probes sent by agents located at vantage points in the network/Internet.

- [edit] Route Analytics

Route analytics is another important area of network measurement. It includes the methods, systems, algorithms and tools to monitor the routing posture of networks. Incorrect routing or routing issues cause undesirable performance degradation or downtime.

- [edit] Various types of protocols

Website monitoring service can check HTTP pages, HTTPS, SNMP, FTP, SMTP, POP3, IMAP, DNS, SSH, TELNET, SSL, TCP, ping, SIP, UDP, Media Streaming and a range of other ports with a variety of check intervals ranging from every four hours to every one minute. Typically, most network monitoring services test your server anywhere between once-per-hour to once-per-minute.

- [edit] Servers around the globe

Network monitoring services usually have a number of servers around the globe - for example in America, Europe, Asia, Australia and other locations. By having multiple servers in different geographic locations, a monitoring service can determine if a Web server is available across different networks worldwide. The more the locations used, the more complete is the picture on network availability.

- [edit] See also

- Comparison of network monitoring systems

- Intrusion detection system

- Myth of the nines

- Network management

- Network Monitoring Interface Card

- Network tap

- Website monitoring

- Business service management

- [edit] Notes and references

- ^ The impact of WAN Optimization on NetFlow/IPFIX measurements

- [edit] External links

- Network Management at the Open Directory Project

- WiseGeek Brief Definition of Network Monitoring

- Article in CIO Magazine "An Introduction to Network Monitoring"

- Categorization of Network Monitoring Tools

- Article on Network Monitoring in the Enterprise

- Network Management Basics

- 12 most Recommended Network Monitoring Tools

From Wikipedia, the free encyclopedia

Jump to: navigation, search

Network tomography is the study of a network's internal characteristics using information derived from end point data. The word tomography is used to link the field, in concept, to other processes that infer the internal characteristics of an object from external observation, as is done in magnetic resonance imaging or positron emission tomography (even though the term tomography strictly refers to imaging by slicing). The field is a recent development in electrical engineering and computer science, founded in 1996.[1] Network tomography advocates that it is possible to map the path data takes through the Internet by examining information from "edge nodes," the computers where data is originated and requested from.

The field is useful for engineers attempting to develop more efficient computer networks. Data derived from network tomography studies can be used to increase quality of service by limiting link packet loss and increasing routing optimization.

- Contents [hide] - 1 Recent developments - 1.1 Loss tomography - 1.2 Delay tomography - 2 More applications - 3 See also - 4 References

- [edit] Recent developments

There have been many published papers and tools in the area of network tomography, which aim to monitor the health of various links in a network in real-time. These can be classified into loss and delay tomography. A summary can be found in [2] and [3].

- [edit] Loss tomography

Loss tomography aims to find "lossy" links in a network by sending active "probes" from various vantage points in the network or the Internet. Published work in this area (not exhaustive) includes [4], and [5].

- [edit] Delay tomography

The area of delay tomography has also attracted attention in the recent past. It aims to find link delays using end-to-end probes sent from vantage points. This can potentially help isolate links with large queueing delays caused by congestion. Related papers include one by Tsang et al.[6].

- [edit] More applications

The area of network tomography also includes that of inferring network topology using end-to-end probes. Topology discovery is a tradeoff between accuracy vs overhead. In network tomography, the emphasis is to achieve as accurate a picture of the network with minimal overhead. In comparison, other network topology discovery techniques using SNMP or Route analytics aim for greater accuracy with less emphasis on overhead reduction.

Other example applications of network tomography include finding links which are shared by multiple paths (and can thus become potential bottlenecks in the future)[7].

- [edit] See also

- Network science

- Computer network

- [edit] References

- ^ Yardi, Y. (1996). "Network Tomography: estimating source-destination traffic intensities from link data". J. Am. Statistics Association (Journal of the American Statistical Association, Vol. 91, No. 433) 91 (433): 365–377. doi:10.2307/2291416. http://jstor.org/stable/2291416.

- ^ Castro, R.; Coates, Mark; Liang, Gang; Nowak, Robert; Yu, Bin (2004). "Network Tomography: Recent Developments". Stat. Science 19: 499–517. doi:10.1214/088342304000000422.

- ^ Coates, M.; Hero Iii, A.O.; Nowak, R.; Bin Yu (2002). "Internet tomography". IEEE Signal Processing Magazine 19: 47–65. doi:10.1109/79.998081.

- ^ Coates, M. (2000). "Network loss inference using unicast end-to-end measurement". Proc. ITC Seminar on IP Traffic, Measurement, and Modeling 28.

- ^ Duffield, N. (2001). "Inferring link loss using striped unicast probes". Ieee Infocom 2: 915–923.

- ^ Tsang, Y.; Coates, M.; Nowak, R.D. (2003). "Network Delay Tomography". IEEE Trans. Signal Processing 51: 2125–2136. doi:10.1109/TSP.2003.814520.

- ^ Rubenstein, D.; Kurose, J.; Towsley, D. (2002). "Detecting shared congestion of flows via end-to-end measurement.". IEEE/ACM Transactions on Networking 10: 381–395. doi:10.1109/TNET.2002.1012369.

From Wikipedia, the free encyclopedia

Jump to: navigation, search

|  | This article has multiple issues. Please help improve it or discuss these issues on the talk page. - It needs additional references or sources for verification. Tagged since October 2008. - It appears to contradict itself. Tagged since May 2009. - It may need reorganization to meet Wikipedia's quality standards. Tagged since May 2009. - It may be confusing or unclear for some readers. Tagged since May 2009. - It is in need of attention from an expert on the subject. Tagged since March 2010. - It may need a complete rewrite to meet Wikipedia's quality standards. Tagged since May 2009. |
| --- | --- |

Diagram of different network topologies.

Network topology is the layout pattern of interconnections of the various elements (links, nodes, etc.) of a computer network.[1][2] Network topologies may be physical or logical. Physical topology means the physical design of a network including the devices, location and cable installation. Logical topology refers to how data is actually transferred in a network as opposed to its physical design.

Topology can be considered as a virtual shape or structure of a network. This shape does not correspond to the actual physical design of the devices on the computer network. The computers on a home network can be arranged in a circle but it does not necessarily mean that it represents a ring topology.

Any particular network topology is determined only by the graphical mapping of the configuration of physical and/or logical connections between nodes. The study of network topology uses graph theory. Distances between nodes, physical interconnections, transmission rates, and/or signal types may differ in two networks and yet their topologies may be identical.

A local area network (LAN) is one example of a network that exhibits both a physical topology and a logical topology. Any given node in the LAN has one or more links to one or more nodes in the network and the mapping of these links and nodes in a graph results in a geometric shape that may be used to describe the physical topology of the network. Likewise, the mapping of the data flow between the nodes in the network determines the logical topology of the network. The physical and logical topologies may or may not be identical in any particular network.

## Contents [hide] - 1 Basic topology types - 2 Classification of network topologies - 2.1 Physical topologies - 2.1.1 Classification of physical topologies - 2.1.1.1 Point-to-point - 2.1.1.2 Bus - 2.1.1.3 Star - 2.2 Notes - 2.3 Extended star - 2.4 Distributed Star - 2.4.1 Ring - 2.4.2 Mesh - 2.4.3 Tree - 2.5 Signal topology - 2.6 Logical topology - 2.6.1 Classification of logical topologies - 3 Daisy chains - 4 Centralization - 5 Decentralization - 6 Hybrids - 7 See also - 8 References - 9 External links

## [edit] Basic topology types

The study of network topology recognizes seven basic topologies:[citation needed]

- Single Node Topology

- Bus topology

- Star topology

- Ring topology

- Tree topology

- Mesh topology

- Hybrid topology

This classification is based on the interconnection between computers — be it physical or logical.

## [edit] Classification of network topologies

There are also three basic categories of network topologies:

- Physical topologies

- Signal topologies

- Logical topologies

The terms Signal topology and logical topology are often used interchangeably, though there may be a subtle or substantial difference between the two. Example IEEE 802.3 1BASE-5 was a logical bus, although the signal topology was a tree structure.[citation needed]

### [edit] Physical topologies

The mapping of the nodes of a network and the physical connections between them – the layout of wiring, cables, the locations of nodes, and the interconnections between the nodes and the cabling or wiring system.[1]

#### [edit] Classification of physical topologies

##### [edit] Point-to-point

The simplest topology is a permanent link between two endpoints (the line in the illustration above). Switched point-to-point topologies are the basic model of conventional telephony. The value of a permanent point-to-point network is the value of guaranteed, or nearly so, communications between the two endpoints. The value of an on-demand point-to-point connection is proportional to the number of potential pairs of subscribers, and has been expressed as Metcalfe's Law.

Permanent (dedicated)

Easiest to understand, of the variations of point-to-point topology, is a point-to-point communications channel that appears, to the user, to be permanently associated with the two endpoints. Children's "tin-can telephone" is one example, with a microphone to a single public address speaker is another. These are examples of physical dedicated channels.

Within many switched telecommunications systems, it is possible to establish a permanent circuit. One example might be a telephone in the lobby of a public building, which is programmed to ring only the number of a telephone dispatcher. "Nailing down" a switched connection saves the cost of running a physical circuit between the two points. The resources in such a connection can be released when no longer needed, for example, a television circuit from a parade route back to the studio.

Switched:

Using circuit-switching or packet-switching technologies, a point-to-point circuit can be set up dynamically, and dropped when no longer needed. This is the basic mode of conventional telephony.

##### [edit] Bus

Main article: Bus network

Bus network topology

In local area networks where bus topology is used, each machine is connected to a single cable. Each computer or server is connected to the single bus cable through some kind of connector. A terminator is required at each end of the bus cable to prevent the signal from bouncing back and forth on the bus cable. A signal from the source travels in both directions to all machines connected on the bus cable until it finds the MAC address or IP address on the network that is the intended recipient. If the machine address does not match the intended address for the data, the machine ignores the data. Alternatively, if the data does match the machine address, the data is accepted. Since the bus topology consists of only one wire, it is rather inexpensive to implement when compared to other topologies. However, the low cost of implementing the technology is offset by the high cost of managing the network. Additionally, since only one cable is utilized, it can be the single point of failure. If the network cable breaks, the entire network will be down.

Linear bus

The type of network topology in which all of the nodes of the network are connected to a common transmission medium which has exactly two endpoints (this is the 'bus', which is also commonly referred to as the backbone, or trunk) – all data that is transmitted between nodes in the network is transmitted over this common transmission medium and is able to be received by all nodes in the network virtually simultaneously (disregarding propagation delays)[1].

Note: The two endpoints of the common transmission medium are normally terminated with a device called a terminator that exhibits the characteristic impedance of the transmission medium and which dissipates or absorbs the energy that remains in the signal to prevent the signal from being reflected or propagated back onto the transmission medium in the opposite direction, which would cause interference with and degradation of the signals on the transmission medium (See Electrical termination).

Distributed bus

The type of network topology in which all of the nodes of the network are connected to a common transmission medium which has more than two endpoints that are created by adding branches to the main section of the transmission medium – the physical distributed bus topology functions in exactly the same fashion as the physical linear bus topology (i.e., all nodes share a common transmission medium).

Notes:

1.) All of the endpoints of the common transmission medium are normally terminated with a device called a 'terminator' (see the note under linear bus).

2.) The physical linear bus topology is sometimes considered to be a special case of the physical distributed bus topology – i.e., a distributed bus with no branching segments.

3.) The physical distributed bus topology is sometimes incorrectly referred to as a physical tree topology – however, although the physical distributed bus topology resembles the physical tree topology, it differs from the physical tree topology in that there is no central node to which any other nodes are connected, since this hierarchical functionality is replaced by the common bus.

##### [edit] Star

Main article: Star network

Star network topology

In local area networks with a star topology, each network host is connected to a central hub. In contrast to the bus topology, the star topology connects each node to the hub with a point-to-point connection. All traffic that traverses the network passes through the central hub. The hub acts as a signal booster or repeater. The star topology is considered the easiest topology to design and implement. An advantage of the star topology is the simplicity of adding additional nodes. The primary disadvantage of the star topology is that the hub represents a single point of failure.

### [edit] Notes

- A point-to-point link (described above) is sometimes categorized as a special instance of the physical star topology – therefore, the simplest type of network that is based upon the physical star topology would consist of one node with a single point-to-point link to a second node, the choice of which node is the 'hub' and which node is the 'spoke' being arbitrary[1].

- After the special case of the point-to-point link, as in note 1.) above, the next simplest type of network that is based upon the physical star topology would consist of one central node – the 'hub' – with two separate point-to-point links to two peripheral nodes – the 'spokes'.

- Although most networks that are based upon the physical star topology are commonly implemented using a special device such as a hub or switch as the central node (i.e., the 'hub' of the star), it is also possible to implement a network that is based upon the physical star topology using a computer or even a simple common connection point as the 'hub' or central node – however, since many illustrations of the physical star network topology depict the central node as one of these special devices, some confusion is possible, since this practice may lead to the misconception that a physical star network requires the central node to be one of these special devices, which is not true because a simple network consisting of three computers connected as in note 2.) above also has the topology of the physical star.

- Star networks may also be described as either broadcast multi-access or nonbroadcast multi-access (NBMA), depending on whether the technology of the network either automatically propagates a signal at the hub to all spokes, or only addresses individual spokes with each communication.

### [edit] Extended star

A type of network topology in which a network that is based upon the physical star topology has one or more repeaters between the central node (the 'hub' of the star) and the peripheral or 'spoke' nodes, the repeaters being used to extend the maximum transmission distance of the point-to-point links between the central node and the peripheral nodes beyond that which is supported by the transmitter power of the central node or beyond that which is supported by the standard upon which the physical layer of the physical star network is based.

If the repeaters in a network that is based upon the physical extended star topology are replaced with hubs or switches, then a hybrid network topology is created that is referred to as a physical hierarchical star topology, although some texts make no distinction between the two topologies.

### [edit] Distributed Star

A type of network topology that is composed of individual networks that are based upon the physical star topology connected together in a linear fashion – i.e., 'daisy-chained' – with no central or top level connection point (e.g., two or more 'stacked' hubs, along with their associated star connected nodes or 'spokes').

##### [edit] Ring

Main article: Ring network

Ring network topology

In local area networks where the ring topology is used, each computer is connected to the network in a closed loop or ring. Each machine or computer has a unique address that is used for identification purposes. The signal passes through each machine or computer connected to the ring in one direction. Ring topologies typically utilize a token passing scheme, used to control access to the network. By utilizing this scheme, only one machine can transmit on the network at a time. The machines or computers connected to the ring act as signal boosters or repeaters which strengthen the signals that traverse the network. The primary disadvantage of ring topology is the failure of one machine will cause the entire network to fail.[citation needed]

##### [edit] Mesh

Main article: Mesh networking

The value of fully meshed networks is proportional to the exponent of the number of subscribers, assuming that communicating groups of any two endpoints, up to and including all the endpoints, is approximated by Reed's Law.

Fully connected mesh topology

The number of connections in a full mesh = n(n - 1) / 2

Fully connected

Note: The physical fully connected mesh topology is generally too costly and complex for practical networks, although the topology is used when there are only a small number of nodes to be interconnected.

Partially connected mesh topology

Partially connected

The type of network topology in which some of the nodes of the network are connected to more than one other node in the network with a point-to-point link – this makes it possible to take advantage of some of the redundancy that is provided by a physical fully connected mesh topology without the expense and complexity required for a connection between every node in the network.

Note: In most practical networks that are based upon the physical partially connected mesh topology, all of the data that is transmitted between nodes in the network takes the shortest path (or an approximation of the shortest path) between nodes, except in the case of a failure or break in one of the links, in which case the data takes an alternative path to the destination. This requires that the nodes of the network possess some type of logical 'routing' algorithm to determine the correct path to use at any particular time.

##### [edit] Tree

Tree network topology

Also known as a hierarchical network.

The type of network topology in which a central 'root' node (the top level of the hierarchy) is connected to one or more other nodes that are one level lower in the hierarchy (i.e., the second level) with a point-to-point link between each of the second level nodes and the top level central 'root' node, while each of the second level nodes that are connected to the top level central 'root' node will also have one or more other nodes that are one level lower in the hierarchy (i.e., the third level) connected to it, also with a point-to-point link, the top level central 'root' node being the only node that has no other node above it in the hierarchy (The hierarchy of the tree is symmetrical.) Each node in the network having a specific fixed number, of nodes connected to it at the next lower level in the hierarchy, the number, being referred to as the 'branching factor' of the hierarchical tree.This tree has individual peripheral nodes.

1.) A network that is based upon the physical hierarchical topology must have at least three levels in the hierarchy of the tree, since a network with a central 'root' node and only one hierarchical level below it would exhibit the physical topology of a star.

2.) A network that is based upon the physical hierarchical topology and with a branching factor of 1 would be classified as a physical linear topology.

3.) The branching factor, f, is independent of the total number of nodes in the network and, therefore, if the nodes in the network require ports for connection to other nodes the total number of ports per node may be kept low even though the total number of nodes is large – this makes the effect of the cost of adding ports to each node totally dependent upon the branching factor and may therefore be kept as low as required without any effect upon the total number of nodes that are possible.

4.) The total number of point-to-point links in a network that is based upon the physical hierarchical topology will be one less than the total number of nodes in the network.

5.) If the nodes in a network that is based upon the physical hierarchical topology are required to perform any processing upon the data that is transmitted between nodes in the network, the nodes that are at higher levels in the hierarchy will be required to perform more processing operations on behalf of other nodes than the nodes that are lower in the hierarchy. Such a type of network topology is very useful and highly recommended.

### [edit] Signal topology

The mapping of the actual connections between the nodes of a network, as evidenced by the path that the signals take when propagating between the nodes.

Note: The term 'signal topology' is often used synonymously with the term 'logical topology', however, some confusion may result from this practice in certain situations since, by definition, the term 'logical topology' refers to the apparent path that the data takes between nodes in a network while the term 'signal topology' generally refers to the actual path that the signals (e.g., optical, electrical, electromagnetic, etc.) take when propagating between nodes.

### [edit] Logical topology

The logical topology, in contrast to the "physical", is the way that the signals act on the network media, or the way that the data passes through the network from one device to the next without regard to the physical interconnection of the devices. A network's logical topology is not necessarily the same as its physical topology. For example, twisted pair Ethernet is a logical bus topology in a physical star topology layout. While IBM's Token Ring is a logical ring topology, it is physically set up in a star topology.

#### [edit] Classification of logical topologies

The logical classification of network topologies generally follows the same classifications as those in the physical classifications of network topologies, the path that the data takes between nodes being used to determine the topology as opposed to the actual physical connections being used to determine the topology

Notes:

1.) Logical topologies are often closely associated with media access control (MAC) methods and protocols.

2.) The logical topologies are generally determined by network protocols as opposed to being determined by the physical layout of cables, wires, and network devices or by the flow of the electrical signals, although in many cases the paths that the electrical signals take between nodes may closely match the logical flow of data, hence the convention of using the terms 'logical topology' and 'signal topology' interchangeably.

3.) Logical topologies are able to be dynamically reconfigured by special types of equipment such as routers and switches.

## [edit] Daisy chains

Except for star-based networks, the easiest way to add more computers into a network is by daisy-chaining, or connecting each computer in series to the next. If a message is intended for a computer partway down the line, each system bounces it along in sequence until it reaches the destination. A daisy-chained network can take two basic forms: linear and ring.

- A linear topology puts a two-way link between one computer and the next. However, this was expensive in the early days of computing, since each computer (except for the ones at each end) required two receivers and two transmitters.

- By connecting the computers at each end, a ring topology can be formed. An advantage of the ring is that the number of transmitters and receivers can be cut in half, since a message will eventually loop all of the way around. When a node sends a message, the message is processed by each computer in the ring. If a computer is not the destination node, it will pass the message to the next node, until the message arrives at its destination. If the message is not accepted by any node on the network, it will travel around the entire ring and return to the sender. This potentially results in a doubling of travel time for data.

## [edit] Centralization

The star topology reduces the probability of a network failure by connecting all of the peripheral nodes (computers, etc.) to a central node. When the physical star topology is applied to a logical bus network such as Ethernet, this central node (traditionally a hub) rebroadcasts all transmissions received from any peripheral node to all peripheral nodes on the network, sometimes including the originating node. All peripheral nodes may thus communicate with all others by transmitting to, and receiving from, the central node only. The failure of a transmission line linking any peripheral node to the central node will result in the isolation of that peripheral node from all others, but the remaining peripheral nodes will be unaffected. However, the disadvantage is that the failure of the central node will cause the failure of all of the peripheral nodes also.

If the central node is passive, the originating node must be able to tolerate the reception of an echo of its own transmission, delayed by the two-way round trip transmission time (i.e. to and from the central node) plus any delay generated in the central node. An active star network has an active central node that usually has the means to prevent echo-related problems.

A tree topology (a.k.a. hierarchical topology) can be viewed as a collection of star networks arranged in a hierarchy. This tree has individual peripheral nodes (e.g. leaves) which are required to transmit to and receive from one other node only and are not required to act as repeaters or regenerators. Unlike the star network, the functionality of the central node may be distributed.

As in the conventional star network, individual nodes may thus still be isolated from the network by a single-point failure of a transmission path to the node. If a link connecting a leaf fails, that leaf is isolated; if a connection to a non-leaf node fails, an entire section of the network becomes isolated from the rest.

In order to alleviate the amount of network traffic that comes from broadcasting all signals to all nodes, more advanced central nodes were developed that are able to keep track of the identities of the nodes that are connected to the network. These network switches will "learn" the layout of the network by "listening" on each port during normal data transmission, examining the data packets and recording the address/identifier of each connected node and which port it's connected to in a lookup table held in memory. This lookup table then allows future transmissions to be forwarded to the intended destination only.

## [edit] Decentralization

In a mesh topology (i.e., a partially connected mesh topology), there are at least two nodes with two or more paths between them to provide redundant paths to be used in case the link providing one of the paths fails. This decentralization is often used to advantage to compensate for the single-point-failure disadvantage that is present when using a single device as a central node (e.g., in star and tree networks). A special kind of mesh, limiting the number of hops between two nodes, is a hypercube. The number of arbitrary forks in mesh networks makes them more difficult to design and implement, but their decentralized nature makes them very useful. This is similar in some ways to a grid network, where a linear or ring topology is used to connect systems in multiple directions. A multi-dimensional ring has a toroidal topology, for instance.

A fully connected network, complete topology or full mesh topology is a network topology in which there is a direct link between all pairs of nodes. In a fully connected network with n nodes, there are n(n-1)/2 direct links. Networks designed with this topology are usually very expensive to set up, but provide a high degree of reliability due to the multiple paths for data that are provided by the large number of redundant links between nodes. This topology is mostly seen in military applications. However, it can also be seen in the file sharing protocol BitTorrent in which users connect to other users in the "swarm" by allowing each user sharing the file to connect to other users also involved. Often in actual usage of BitTorrent any given individual node is rarely connected to every single other node as in a true fully connected network but the protocol does allow for the possibility for any one node to connect to any other node when sharing files.

## [edit] Hybrids

Hybrid networks use a combination of any two or more topologies in such a way that the resulting network does not exhibit one of the standard topologies (e.g., bus, star, ring, etc.). For example, a tree network connected to a tree network is still a tree network, but two star networks connected together exhibit a hybrid network topology. A hybrid topology is always produced when two different basic network topologies are connected. Two common examples for Hybrid network are: star ring network and star bus network

- A Star ring network consists of two or more star topologies connected using a multistation access unit (MAU) as a centralized hub.

- A Star Bus network consists of two or more star topologies connected using a bus trunk (the bus trunk serves as the network's backbone).

While grid networks have found popularity in high-performance computing applications, some systems have used genetic algorithms to design custom networks that have the fewest possible hops in between different nodes. Some of the resulting layouts are nearly incomprehensible, although they function quite well.

## [edit] See also

- Shared mesh

- Switched mesh

- Expander graph

- Scale-free network

- Network diagram

- Computer network

- Relay network

- Cube-connected cycles

- Redundant topologies

- Tree structure

- Hierarchical clustering

- Internet topology

## [edit] References

- ^ a b c d Groth, David; Toby Skandier (2005). Network+ Study Guide, Fourth Edition'. Sybex, Inc.. ISBN 0-7821-4406-3.

- ^ ATIS committee PRQC. "network topology". ATIS Telecom Glossary 2007. Alliance for Telecommunications Industry Solutions. http://www.atis.org/glossary/definition.aspx?id=3516. Retrieved 2008-10-10.

## [edit] External links

|  | Wikimedia Commons has media related to: Topology (Network) |
| --- | --- |

- A Guide to Network Topology

- Research network topology

- Types of topology

Retrieved from "http://en.wikipedia.org/wiki/Network_topology"

From Wikipedia, the free encyclopedia

Jump to: navigation, search

For other uses, see Node (disambiguation).

In communication networks, a node (Latin nodus, ‘knot’) is a connection point, either a redistribution point or a communication endpoint (some terminal equipment). The definition of a node depends on the network and protocol layer referred to. A physical network node is an active electronic device that is attached to a network, and is capable of sending, receiving, or forwarding information over a communications channel.[1] A passive distribution point such as a distribution frame is consequently not a node.

In network theory or graph theory, the term node refers to a point in a network topology at which lines intersect or branch[2].

- Contents [hide] - 1 Computer network nodes - 2 Telecommunication network nodes - 3 Distributed system nodes - 3.1 End node in cloud computing - 4 See also - 5 References

- [edit] Computer network nodes

In data communication, a physical network node may either be a data circuit-terminating equipment (DCE) such as a modem, hub, bridge or switch; or a data terminal equipment (DTE) such as a digital telephone handset, a printer or a host computer, for example a router, a workstation or a server.

If the network in question is a LAN or WAN, every LAN or WAN node (that are at least data link layer devices) must have a MAC address. Examples are computers, packet switches and ADSL modem (with Ethernet interface). Note that a hub constitutes a physical network node, but not a LAN node in this sense, since a hubbed network logically is a bus network. Analogously, a repeater or PSTN modem (with serial interface) are physical network nodes but not LAN nodes in this sense.

If the network in question is the Internet, many physical network nodes are host computers, also known as Internet nodes, identified by an IP address, and all hosts are physical network nodes. However, datalink layer devices such as switches, bridges and WLAN access points do not have an IP host address (except sometimes for administrative purposes), and are not considered as Internet nodes, but as physical network nodes or LAN nodes.

- [edit] Telecommunication network nodes

In the fixed telephone network, a node may be a public or private telephone exchange, a remote concentrator or a computer providing some intelligent network service. In cellular communication, switching points and databases such as the Base station controller, Home Location Register, Gateway GPRS Support Node (GGSN) and Serving GPRS Support Node (SGSN) are examples of nodes. Cellular network base stations are not considered as nodes in this context.

In cable television systems (CATV), this term has assumed a broader context and is generally associated with a fiber optic node. This can be defined as those homes or businesses within a specific geographic area that are served from a common fiber optic receiver. A fiber optic node is generally described in terms of the number of "homes passed" that are served by that specific fiber node.

- [edit] Distributed system nodes

If the network in question is a distributed system, the nodes are clients, servers or peers. A peer may sometimes serve as client, sometimes server. In a peer-to-peer or overlay network, nodes that actively route data for the other networked devices as well as themselves are called supernodes.

- [edit] End node in cloud computing

Within a vast computer network, the individual computers on the periphery of the network, those that do not also connect other networks, and those that often connect transiently to one or more clouds are called end nodes. Typically, within the cloud computing construct, the individual user / customer computer that connects into one well-managed cloud is called an end node. Since these computers are a part of yet unmanaged by the cloud's host, they present significant risks to the entire cloud. This is called the End Node Problem. [3] There are several means to remedy this problem but all require instilling trust in the end node computer. [4]

- [edit] See also

- Host (network)

- [edit] References

- ^ "encarta". Archived from the original on 2009-11-01. http://www.webcitation.org/5kx5kPIKV.

- ^ AskOxford

- ^ [www.nets-find.net/Meetings/S09Meeting/Talks/clark.ppt]

- ^ "LPS-Public". http://www.spi.dod.mil/lipose.htm.

Retrieved from "http://en.wikipedia.org/wiki/Node_(networking)"

From Wikipedia, the free encyclopedia

(Redirected from Protocol (computing))

Jump to: navigation, search

For other senses of this word, see protocol.

A communications protocol is a formal description of digital message formats and the rules for exchanging those messages in or between computing systems and in telecommunications. Protocols may include signaling, authentication and error detection and correction capabilities. A protocol describes the syntax, semantics, and synchronization of communication and may be implemented in hardware or software, or both.

- Contents [hide] - 1 Introduction - 2 Description - 3 Protocol design - 3.1 A basis for protocol design - 3.1.1 Protocols are to computer communication what programming languages are to computation - 3.1.2 Protocol layering - 3.1.3 Software layering - 3.1.4 Strict layering - 4 Protocol development - 5 Protocol standardization - 6 Protocol testing - 7 Social impact of protocols - 8 Common types of protocols - 9 See also - 10 Notes - 11 References - 12 External links

- [edit] Introduction

The protocols in human communication are rules about appearance, speaking, listening and understanding. These rules, also called protocols of conversation, represent different layers of communication. They work together to help people communicate successfully. The need for protocols also applies to computing systems. Network engineers have written rules for communication that must be strictly followed for successful host-to-host communication. These rules apply to different layers of sophistication such as which physical connections to use, how hosts listen, how to interrupt, how to terminate communications, which language to use and many others. These rules, or protocols, that work together to ensure successful communication are grouped into what is known as a protocol suite or family. Some of the best known protocol suites include: IPX/SPX, X.25, AX.25, AppleTalk and TCP/IP.

- [edit] Description

The information exchanged between devices on a network or other communications medium is governed by rules that can be set out in a technical specification called a communication protocol standard. The nature of the communication, the actual data exchanged and any state-dependent behaviors are defined by the specification. This approach is often taken for protocols in use by telecommunications.

In digital computing systems, the rules can be expressed by algorithms and datastructures, raising the opportunity of hardware independency. Expressing the algorithms in a portable programming language, makes the protocolsoftware operating system independent.

To implement a protocol, the protocol software modules are to be interfaced with a framework assumed to be implemented on the machine's operating system. This framework implements the networking functionality of the operating system. Obviously, the framework needs to be as simple as it can be, to allow for an easier incorporation into the operating systems.

At the time the Internet was formed, layering had proven to be a successful design approach for both compiler and operating system design and given the similarities between programming languages and communication protocols, it was intuitively felt that layering should be applied to the protocols as well.[1] This gave rise to the concept of layered protocols which nowadays forms the basis of protocoldesign. [2]

Systems do not use a single protocol to handle a transmission. Instead they use a set of cooperating protocols, sometimes called a protocol family or protocol suite.[3] To cooperate the protocols have to communicate with each other, so there is an unnamed 'protocol' to do this. A technique used by this 'protocol' is called encapsulation, which makes it possible to pass messages from layer to layer. [4]

The protocols can be arranged on functionality in groups, for instance there is a group of transport protocols. The functionalities are mapped on the layers, each layer solving a distinct class of problems relating to, for instance: application-, transport-, internet- and network interface-functions.[5] To transmit a message, a protocol has to be selected from each layer, so some sort of multiplexing/demultiplexing takes place. The selection of the next protocol, also part of the aforementioned 'protocol' is accomplished by extending the message with a protocolselector for each layer.[6]

There's a myriad of protocols, but they all only differ in the details. For this reason the TCP/IP protocol suite can be studied to get the overall protocol picture. [7] The Internet Protocol (IP) and the Transmission Control Protocol (TCP) are the most important of these, and the term Internet Protocol Suite, or TCP/IP, refers to a collection of its most used protocols. Most of the communication protocols in use on the Internet are described in the Request for Comments (RFC) documents of the Internet Engineering Task Force (IETF). RCF1122, in particular, documents the suite itself.

- [edit] Protocol design

The design of the protocolsoftware belongs to the discipline of 'concurrent systems design'.[8] Communicating systems operate in parallel. The programming tools and techniques for dealing with parallel processes are collectively called concurrent programming. Concurrent programming has traditionally been a topic in operating systems theorie texts. [9] A mathematical approach to the study of concurrency and communication is referred to as Communicating Sequential Processes. Concurrency can also be modelled using finite state machines like Mealy- and Moore Machines. Mealy- and Moore machines are in use as design tools in digital electronics systems, which we encounter in the form of hardware used in telecommunications or electronic devices in general.[10]

This kind of design can be a bit of a challenge to say the least, so it is important to keep things simple. For the Internet protocols, in particular and in retrospect, this meant a basis for protocol design was needed to allow decomposition of protocols into much simpler, cooperating protocols.

- [edit] A basis for protocol design

Systems do not use a single protocol to handle a transmission. Instead they use a set of cooperating protocols, sometimes called a protocol family or protocol suite.[11] To cooperate the protocols have to communicate with each other, so some kind of conceptual framework is needed to make this communication possible. Also note that software is needed to implement both the 'xfer-mechanism' and a protocol (no protocol, no communication).

In literature there are numerous references to the analogies between computer communication and programming. By analogy we could say that the aforementioned 'xfer-mechanism' is comparable to a cpu; a 'xfer-mechanism' performs communications and a cpu performs computations and the 'framework' introduces something that allows the protocols to be designed independent of one and another by providing separate execution environments for the protocols. Furthermore, it is repeatedly stated that protocols are to computer communication what programming languages are to computation.[12][13]

- [edit] Protocols are to computer communication what programming languages are to computation

In programming languages the association of identifiers to a value is termed a definition. Program text is structured using block constructs and definitions can be local to a block. The localized association of an identifier to a value established by a definition is termed a binding and the region of program text in which a binding is effective is known as its scope.[14] The computational state is kept using two components: the environment, used as a record of identifier bindings, and the store, which is used as a record of the effects of assignments. [15]

In communications, message values are transferred using transmission media. By analogy, the equivalent of a store would be a collection of transmission media, instead of a collection of memory locations. A valid assignment in a protocol (as an analog of programming language) could be Ethernet:='message' , meaning a message is to be broadcast on the local ethernet.
On a transmission medium there can be many receivers. For instance a mac-address identifies an ether network card on the transmission medium (the 'ether'). In our imaginary protocol, the assignment ethernet[mac-address]:=message value could therefore make sense. [16]

Programming like this would require knowledge about the hardware, like the details of the Ethernet frame format, network access policy, and frame error handling. Such a protocol is called a low-level protocol. The Internet Protocol specifies higher level abstractions like IP addressing, datagram format, and the concept of unreliable, connectionless delivery, and is therefore called a high-level protocol.
It should be apparent by now how closely the analogy fits. [17]

- [edit] Protocol layering

Figure 1. Message flows using a protocol suite

Protocol layering now forms the basis of protocol design.[18] It allows the decomposition of single, complex protocols into simpler, cooperating protocols, but it is also a functional decomposition, because each protocol belongs to a functional class, called a protocol layer. [19] The protocol layers each solve a distinct class of communications problems. The Internet protocol suite consists of the following layers: application-, transport-, internet- and network interface-functions. [20] Together, the layers make up a layering scheme or model.
In computations, we have algorithms and data, and in communications, we have protocols and messages, so the analog of a data flow diagram would be some kind of message flow diagram. To visualize protocol layering and protocol suites, a diagram of the message flows in and between two systems, A and B, is shown in figure 1.
The systems both make use of the same protocol suite. The vertical flows (and protocols) are in system and the horizontal message flows (and protocols) are between systems. The message flows are governed by rules, and dataformats specified by protocols. The blue lines therefore mark the boundaries of the (horizontal) protocol layers.

The vertical protocols are not layered because they don't obey the protocol layering principle which states that a layered protocol is designed so that layer n at the destination receives exactly the same object sent by layer n at the source. The horizontal protocols are layered protocols and all belong to the protocol suite. Layered protocols allow the protocol designer to concentrate on one layer at a time, without worrying about how other layers perform. [21]
The vertical protocols neednot be the same protocols on both systems, but they have to satisfy some minimal assumptions to ensure the protocol layering principle holds for the layered protocols. This can be achieved using a technique called Encapsulation.[22]
Usually, a message or a stream of data is divided into small pieces, called messages or streams, packets, IP datagrams or network frames depending on the layer in which the pieces are to be transmitted. The pieces contain a header area and a data area. The data in the header area identifies the source and the destination on the network of the packet, the protocol, and other data meaningful to the protocol like CRC's of the data to be send, data length, and a timestamp.[23] [24]
The rule enforced by the vertical protocols is that the pieces for transmission are to be encapsulated in the data area of all lower protocols on the sending side and the reverse is to happen on the receiving side. The result is that at the lowest level the piece looks like this: 'Header1,Header2,Header3,data' and in the layer directly above it: 'Header2,Header3,data' and in the top layer: 'Header3,data', both on the sending and receiving side. This rule therefore ensures that the protocol layering principle holds and effectively virtualizes all but the lowest transmission lines, so for this reason some message flows are coloured red in figure 1.
To ensure both sides use the same protocol, the pieces also carry data identifying the protocol in their header.

The design of the protocol layering and the network (or Internet) architecture are interrelated, so one cannot be designed without the other. [25]

- [edit] Software layering

- [edit] Strict layering

Strictly adhering to a layered model, a practice known as strict layering, is not always the best approach to networking.[26] Strict layering, can have a serious impact on the performance of the implementation, so there is at least a trade-off between simplicity and performance.[27] Another, perhaps more important point can be shown by considering the fact that some of the protocols in the Internet Protocol Suite cannot be expressed using the TCP/IP model, in other words some of the protocols behave in ways not described by the model.[28] To improve on the model, an offending protocol could, perhaps be split up into two protocols, at the cost of one or two extra layers, but there is a hidden caveat, because the model is also used to provide a conceptual view on the suite for the intended users. There is a trade-off to be made here between preciseness for the designer and clarity for the intended user. [29]

- [edit] Protocol development

For communication to take place, protocols have to be agreed upon. Recall that in digital computing systems, the rules can be expressed by algorithms and datastructures, raising the opportunity of hardware independency. Expressing the algorithms in a portable programming language, makes the protocolsoftware operating system independent. The sourcecode could be considered a protocol specification. This form of specification, however is not suitable for the parties involved.
For one thing, this would enforce a source on all parties and for another, proprietary software producers would not accept this. By describing the software interfaces of the modules on paper and agreeing on the interfaces, implementers are free to do it their way. This is referred to as source independency. By specifying the algorithms on paper, development continues to become a standard. It should be noted though that in some cases protocol standards are not sufficient to gain wide-spread acceptance i.e. sometimes the sourcecode needs to be disclosed enforced by law or the government.

- [edit] Protocol standardization

Communications protocols are often designed, developed and promulgated by groups of engineering professionals organized by standards organizations, such as the International Organization for Standardization (ISO), the International Telecommunications Union (ITU), the Institute of Electrical and Electronics Engineers (IEEE), or the Internet Engineering Task Force (IETF). The IETF maintains the protocols in use on the Internet. The IEEE controls many software and hardware protocols in the electronics industry for commercial and consumer devices. The ITU is an umbrella organization of telecommunications engineers designing the public switched telephone network (PSTN), as well as many radio communication systems. For marine electronics the NMEA standards are used.

Many communication protocols are also designed by commercial entities, such as small and large corporations in the software and data communications industries. Xerox Corporation (Xerox PARC), Microsoft, Intel, and many others have developed communications protocols that are in wide-spread use. Some were developed initially in proprietary fashion, but were published and were accepted later by the broader public. Some remain in proprietary status.

- [edit] Protocol testing

In general, protocol testers or protocol analyzers work by capturing the information exchanged between a device under test (DUT) and a reference device known to operate properly. In the example of a manufacturer producing a new keyboard for a personal computer, the DUT would be the keyboard and the reference device, the PC.

Protocol testing is an essential step towards commercialization of standards-based products. It helps to ensure that products from different manufacturers will interoperate.

- [edit] Social impact of protocols

- [edit] Common types of protocols

Main article: List of network protocols

The Internet Protocol is used in concert with with other protocols within the Internet Protocol Suite. Prominent members of which include:

- Transmission Control Protocol (TCP)

- User Datagram Protocol (UDP)

- Internet Control Message Protocol (ICMP)

- Hypertext Transfer Protocol (HTTP)

- Post Office Protocol (POP3)

- File Transfer Protocol (FTP)

- Internet Message Access Protocol (IMAP)

Other instances of high level interaction protocols are:

- IIOP

- RMI

- DCOM

- DDE

- SOAP

- [edit] See also

- Application programming interface

- Calling convention

- Connection-oriented protocol

- Connectionless protocol

- Digital radio

- Tunneling protocol

- [edit] Notes

- ^ Comer 2000, Sect. 11.2 - The Need For Multiple Protocols, p. 177, explains this by drawing analogies between computer communication and programming languages.

- ^ Sect. 11.10 - The Disadvantage Of Layering, p. 192, states: layering forms the basis for protocol design.

- ^ Comer 2000, Sect. 11.2 - The Need For Multiple Protocols, p. 177, states the same.

- ^ Comer 2000, Glossary of Internetworking terms, p.686: term encapsulation.

- ^ Comer 2000, Sect. 11.3 - The Conceptual Layers Of Protocol Software, p. 178, "Each layer takes responsibility for handling one part of the problem."

- ^ Comer 2000, Sect. 11.11 - The Basic Idea Behind Multiplexing And Demultiplexing, p. 192, states the same.

- ^ Comer 2000,Foreword To The First Edition By The Late Jon Postel, xxv, "The principles of architecture, layering, multiplexing, encapsulation, adressing and address mapping, routing, and naming are quite similar in any protocol suite, though of course, different in detail.".

- ^ Holzmann 1991, Bookoverview.

- ^ Ben-Ari 1982, in his preface, pg. xiii.

- ^ S. Srinivasan, NPTEL courses:::: Electronics & Communication Engineering :: Digital Circuits and Systems, available online: http://nptel.iitm.ac.in/video.php?courseId=1005&p=3

- ^ Comer 2000, Sect. 11.2 - The Need For Multiple Protocols, p. 177, states the same.

- ^ Comer 2000, Sect. 11.2 - The Need For Multiple Protocols, p. 177, states more or less the same, using other analogies.

- ^ Comer 2000, Sect. 11.7 - The Protocol Layering Principle, p. 187, explains layered protocols.

- ^ Tennent 1981, Section 2.3.1 - Definitions, p.15, defines scope and binding.

- ^ Tennent 1981, Section 2.3.2 Environments and stores, pg.16, the semantics of blocks and definitions are described using environments and stores.

- ^ Hoare (1985), Ch. 4 - Communication, pg. 133, In the introduction: a communication is an event described by a pair c.v where c is the name of the communication channel and v is the value of the message.

- ^ Comer (2000), Sect. 11.2 - The Need For Multiple Protocols, pg. 177, first paragraph.

- ^ Sect. 11.10 - The Disadvantage Of Layering, p. 192, states: layering forms the basis for protocol design.

- ^ Comer 2000, Sect. 11.2 - The Need For Multiple Protocols, p. 177, introduces the decomposition in layers.

- ^ Comer 2000, Sect. 11.3 - The Conceptual Layers Of Protocol Software, p. 178, "Each layer takes responsibility for handling one part of the problem."

- ^ Comer 2000, Sect. 11.7 - The Protocol Layering Principle, p. 187, explains layered protocols.

- ^ Comer 2000, Glossary of Internetworking terms, p.686: term encapsulation.

- ^ Comer 2000, Sect. 11.5.1 - The TCP/IP 5-Layer Reference Model, pg. 184, Describes the transformations of messages or streams that can be observed in the protocol layers.

- ^ Comer 2000, Sect. 2.4.10 - Ethernet Frame Format, pg. 30, Ethernet frames are used as an example for administrative data for the protocol itself.

- ^ Comer 2000, Sect. 11.4 - Functionality Of The Layers, p. 181, states the same about the software organization.

- ^ IETF 1989, Sect 1.3.1 - Organization, p. 15, 2nd paragraph: many design choices involve creative "breaking" of strict layering.

- ^ Comer 2000, Sect. 11.10 - The Disadvantage Of Layering, p. 192, explains why "strict layering can be extremely inefficient" giving examples of optimizations.

- ^ IETF 1989, Sect 1.3.1 - Organization, p. 15, 2nd paragraph, explaining why "strict layering is an imperfect model"

- ^ IETF 1989, Sect 1.3.1 - Organization, p. 15, states: This layerist organization was chosen for simplicity and clarity.

- [edit] References

- Radia Perlman: Interconnections: Bridges, Routers, Switches, and Internetworking Protocols. 2nd Edition. Addison-Wesley 1999, ISBN 0-201-63448-1. In particular Ch. 18 on "network design folklore", which is also available online at http://www.informit.com/articles/article.aspx?p=20482

- Gerard J. Holzmann: Design and Validation of Computer Protocols. Prentice Hall, 1991, ISBN 0-13-539925-4. Also available online at http://spinroot.com/spin/Doc/Book91.html

- Douglas E. Comer (2000): Internetworking with TCP/IP - Principles, Protocols and Architecture 4th Edition. Prentice Hall, ISBN 0-13-018380-6. In particular Ch.11 Protocol layering. Also has a RFC guide and a Glossary of Internetworking Terms and Abbreviations.

- Internet Engineering Task Force abbr. IETF (1989): RFC1122, Requirements for Internet Hosts -- Communication Layers, R.Braden (ed.), Available online at http://tools.ietf.org/html/rfc1122. Describes TCP/IP to the implementors of protocolsoftware. In particular the introduction gives an overview of the design goals of the suite.

- M. Ben-Ari (1982): Principles of concurrent programming 10th Print. ISBN 0-13-701078-8.

- C.A.R. Hoare (1985): Communicating sequential processes 10th Print. ISBN 0-13-153271-5.

- R.D. Tennent (1981): Principles of programming languages 10th Print. ISBN 0-13-709873-1.

- [edit] External links

- Javvin's Protocol Dictionary

- Network Protocol Index

- Overview of protocols in telecontrol field with OSI Reference Model

- List of Data Communication Protocols

- PDF-Chart showing the Protocols and the OSI reference layer

From Wikipedia, the free encyclopedia

Jump to: navigation, search

A scale-free network is a network whose degree distribution follows a power law, at least asymptotically. That is, the fraction P(k) of nodes in the network having k connections to other nodes goes for large values of k as

where γ is a constant whose value is typically in the range 2 < γ < 3, although occasionally it may lie outside these bounds.

Scale-free networks are noteworthy because many empirically observed networks appear to be scale-free, including the world wide web, the Internet, citation networks, and some social networks.

- Contents [hide] - 1 Highlights - 2 History - 3 Characteristics and examples - 4 Generative models - 5 See also - 6 References - 7 External Links

- [edit] Highlights

- Scale-free networks show a power law degree distribution like many real networks.

- The mechanism of preferential attachment has been proposed as a mechanism to explain power law degree distributions in some networks.

- [edit] History

In studies of the networks of citations between scientific papers, Derek de Solla Price showed in 1965 that the number of links to papers—i.e., the number of citations they receive—had a heavy-tailed distribution following a Pareto distribution or power law, and thus that the citation network was scale-free. He did not however use the term "scale-free network", which was not coined until some decades later. In a later paper in 1976, Price also proposed a mechanism to explain the occurrence of power laws in citation networks, which he called "cumulative advantage" but which is today more commonly known under the name preferential attachment.

Recent interest in scale-free networks started in 1999 with work by Albert-László Barabási and colleagues at the University of Notre Dame who mapped the topology of a portion of the Web (Barabási and Albert 1999), finding that some nodes, which they called "hubs", had many more connections than others and that the network as a whole had a power-law distribution of the number of links connecting to a node.

After finding that a few other networks, including some social and biological networks, also had heavy-tailed degree distributions, Barabási and collaborators coined the term "scale-free network" to describe the class of networks that exhibit a power-law degree distribution. Soon after, Amaral et al. showed that most of the real-world networks can be classified into two large categories according to the decay of P(k) for large k.

Barabási and Albert proposed a mechanism to explain the appearance of the power-law distribution, which they called "preferential attachment" and which is essentially the same as that proposed by Price. Analytic solutions for this mechanism (also similar to the solution of Price) were presented in 2000 by Dorogovtsev, Mendes and Samukhin and independently by Krapivsky, Redner, and Leyvraz, and later rigorously proved by mathematician Béla Bollobás. Notably, however, this mechanism only produces a specific subset of networks in the scale-free class, and many alternative mechanisms have been discovered since.

Although the scientific community is still debating the usefulness of the scale-free term in reference to networks, Li et al. (2005) recently offered a potentially more precise "scale-free metric". Briefly, let g be a graph with edge-set ε, and let the degree (number of edges) at a vertex i be di. Define

This is maximised when high-degree nodes are connected to other high-degree nodes. Now define

where smax is the maximum value of s(h) for h in the set of all graphs with an identical degree distribution to g. This gives a metric between 0 and 1, such that graphs with low S(g) are "scale-rich", and graphs with S(g) close to 1 are "scale-free". This definition captures the notion of self-similarity implied in the name "scale-free".

- [edit] Characteristics and examples

Random network (a) and scale-free network (b). In the scale-free network, the larger hubs are highlighted.

As with all systems characterized by a power law distribution, the most notable characteristic in a scale-free network is the relative commonness of vertices with a degree that greatly exceeds the average. The highest-degree nodes are often called "hubs", and are thought to serve specific purposes in their networks, although this depends greatly on the domain.

The power law distribution highly influences the network topology. It turns out that the major hubs are closely followed by smaller ones. These ones, in turn, are followed by other nodes with an even smaller degree and so on. This hierarchy allows for a fault tolerant behavior. Since failures occur at random and the vast majority of nodes are those with small degree, the likelihood that a hub would be affected is almost negligible. Even if such event occurs, the network will not lose its connectedness, which is guaranteed by the remaining hubs. On the other hand, if we choose a few major hubs and take them out of the network, it simply falls apart and is turned into a set of rather isolated graphs. Thus hubs are both the strength of scale-free networks and their Achilles' heel.

Another important characteristic of scale-free networks is the clustering coefficient distribution, which decreases as the node degree increases. This distribution also follows a power law. That means that the low-degree nodes belong to very dense sub-graphs and those sub-graphs are connected to each other through hubs. Consider a social network in which nodes are people and links are acquaintance relationships between people. It is easy to see that people tend to form communities, i.e., small groups in which everyone knows everyone (one can think of such community as a complete graph). In addition, the members of a community also have a few acquaintance relationships to people outside that community. Some people, however, are so related to other people (e.g., celebrities, politicians) that they are connected to a large number of communities. Those people may be considered the hubs responsible for the small world phenomenon.

At present, the more specific characteristics of scale-free networks can only be discussed in either the context of the generative mechanism used to create them, or the context of a particular real-world network thought to be scale-free. For instance, networks generated by preferential attachment typically place the high-degree vertices in the middle of the network, connecting them together to form a core, with progressively lower-degree nodes making up the regions between the core and the periphery. Many interesting results are known for this subclass of scale-free networks. For instance, the random removal of even a large fraction of vertices impacts the overall connectedness of the network very little, suggesting that such topologies could be useful for security, while targeted attacks destroys the connectedness very quickly. Other scale-free networks, which place the high-degree vertices at the periphery, do not exhibit these properties; notably, the structure of the Internet is more like this latter kind of network than the kind built by preferential attachment. Indeed, many of the results about scale-free networks have been claimed to apply to the Internet, but are disputed by Internet researchers and engineers.

As with most disordered networks, such as the small world network model, the average distance between two vertices in the network is very small relative to a highly ordered network such as a lattice. The clustering coefficient of scale-free networks can vary significantly depending on other topological details, and there are now generative mechanisms that allow one to create such networks that have a high density of triangles.

It is interesting that Cohen and Havlin proved that uncorrelated power-law graph having 2 < γ < 3 will also have ultrasmall diameter d ~ ln ln N. So from the practical point of view, the diameter of a growing scale-free network might be considered almost constant.

Although many real-world networks are thought to be scale-free, the evidence remains inconclusive, primarily because the generative mechanisms proposed have not been rigorously validated against the real-world data. As such, it is too early to rule out alternative hypotheses. A few examples of networks claimed to be scale-free include:

- Social networks, including collaboration networks. An example that has been studied extensively is the collaboration of movie actors in films.

- Protein-Protein interaction networks.

- Sexual partners in humans, which affects the dispersal of sexually transmitted diseases.

- Many kinds of computer networks, including the World Wide Web.

- Semantic networks. [1]

- [edit] Generative models

These scale-free networks do not arise by chance alone. Erdős and Rényi (1960) studied a model of growth for graphs in which, at each step, two nodes are chosen uniformly at random and a link is inserted between them. The properties of these random graphs are not consistent with the properties observed in scale-free networks, and therefore a model for this growth process is needed.

The scale-free properties of the Web have been studied, and its distribution of links is very close to a power law, because there are a few Web sites with huge numbers of links, which benefit from a good placement in search engines and an established presence on the Web. Those sites are the ones that attract more of the new links. This has been called the winner takes all phenomenon.

The mostly widely known generative model for a subset of scale-free networks is Barabási and Albert's (1999) rich get richer generative model in which each new Web page creates links to existing Web pages with a probability distribution which is not uniform, but proportional to the current in-degree of Web pages. This model was originally discovered by Derek J. de Solla Price in 1965 under the term cumulative advantage, but did not reach popularity until Barabási rediscovered the results under its current name (BA Model). According to this process, a page with many in-links will attract more in-links than a regular page. This generates a power-law but the resulting graph differs from the actual Web graph in other properties such as the presence of small tightly connected communities. More general models and networks characteristics have been proposed and studied (for a review see the book by Dorogovtsev and Mendes).

A different generative model is the copy model studied by Kumar et al. (2000), in which new nodes choose an existent node at random and copy a fraction of the links of the existent node. This also generates a power law.

However, if we look at communities of interests in a specific topic, discarding the major hubs of the Web, the distribution of links is no longer a power law but resembles more a normal distribution, as observed by Pennock et al. (2002) in the communities of the home pages of universities, public companies, newspapers and scientists. Based on these observations, the authors propose a generative model that mixes preferential attachment with a baseline probability of gaining a link.

The growth of the networks (adding new nodes) is not a necessary condition for creating a scale-free topology. Dangalchev (2004) gives examples of generating static scale-free networks. Another possibility (Caldarelli et al. 2002) is to consider the structure as static and draw a link between vertices according to a particular property of the two vertices involved. Once specified the statistical distribution for these vertices properties (fitnesses), it turns out that in some circumstances also static networks develop scale-free properties.

Scale free topology has been also found in high temperature supercondcutors [2]. The qualities of a high-temperature superconductor — a compound in which electrons obey the spooky laws of quantum physics, and flow in perfect synchrony, without friction — appear linked to the fractal arrangements of seemingly random oxygen atoms.

Recently, Manev and Manev (Med. Hypotheses, 2005) proposed that small world networks may be operative in adult brain neurogenesis. Adult neurogenesis has been observed in mammalian brains, including those of humans, but a question remains: how do new neurons become functional in the adult brain? It is proposed that the random addition of only a few new neurons functions as a maintenance system for the brain's "small-world" networks. Randomly added to an orderly network, new links enhance signal propagation speed and synchronizability. Newly generated neurons are ideally suited to become such links: they are immature, form more new connections compared to mature ones, and their number but not their precise location may be maintained by continuous proliferation and dying off. Similarly, it is envisaged that the treatment of brain pathologies by cell transplantation would also create new random links in small-world networks and that even a small number of successfully incorporated new neurons may be functionally important.

- [edit] See also

- Social-circles network model - a more generalized generative model for many "real-world networks" of which the scale-free network is a special case

- Random graph

- Bose-Einstein condensation: a network theory approach

- [edit] References

- Albert R. and Barabási A.-L., "Statistical mechanics of complex networks", Rev. Mod. Phys. 74, 47–97 (2002).

- Amaral, LAN, Scala, A., Barthelemy, M., Stanley, HE., "Classes of behavior of small-world networks". Proceedings of the National Academy of Sciences (USA) 97:11149-11152 (2000).

- Barabási, Albert-László Linked: How Everything is Connected to Everything Else, 2004 ISBN 0-452-28439-2

- Barabási, Albert-László "Scale-Free Networks". Scientific American, 288:60-69, May 2003.

- Barabási, Albert-László and Albert, Réka. "Emergence of scaling in random networks". Science, 286:509-512, October 15, 1999.

- Dan Braha and Yaneer Bar-Yam, "Topology of Large-Scale Engineering Problem-Solving Networks" Phys. Rev. E 69, 016113 (2004)

- Caldarelli G. "Scale-Free Networks" Oxford University Press, Oxford (2007).

- Caldarelli G., Capocci A., De Los Rios P., Muñoz M.A., "Scale-free networks from varying vertex intrinsic fitness" Physical Review Letters 89, 258702 (2002).

- Dangalchev, Ch., Generation models for scale-free networks, Physica A, 338,(2004)

- Dorogovtsev, S.N. and Mendes, J.F.F., Evolution of Networks: from biological networks to the Internet and WWW, Oxford University Press, 2003, ISBN 0-19-851590-1

- Dorogovtsev, S.N. and Mendes, J.F.F. and Samukhin, A.N., "Structure of Growing Networks: Exact Solution of the Barabási--Albert's Model", Phys. Rev. Lett. 85, 4633 (2000)

- Dorogovtsev, S.N. and Mendes, J.F.F., Evolution of networks, Advances in Physics 51, 1079-1187 (2002)

- Erdős, P. and Rényi, A. Random graphs. Publication of the Mathematical Institute of the Hungarian Academy of Science, 5, pages 17-61, 1960.

- Faloutsos, M., Faloutsos, P. and Faloutsos, C., On power-law relationships of the internet topology Comp. Comm. rev. 29, 251, 1999.

- Li, L., Alderson, D., Tanaka, R., Doyle, J.C., Willinger, W., Towards a Theory of Scale-Free Graphs: Definition, Properties, and Implications (Extended Version). Internet Mathematics, 2005.

- Kumar, R., Raghavan, P., Rajagopalan, S., Sivakumar, D., Tomkins, A., and Upfal, E.: Stochastic models for the web graph. In Proceedings of the 41st Annual Symposium on Foundations of Computer Science (FOCS), pages 57-65, Redondo Beach, CA, USA. IEEE CS Press, 2000.

- Manev R., Manev H.; Med. Hypotheses 2005;64(1):114-7 [1]

- Matlis, Jan. Scale-Free Networks. ComputerWorld. November 4, 2002.

- Newman, Mark E. J. The structure and function of complex networks, 2003.

- Pastor-Satorras, R.,Vespignani, A.,"Evolution and Structure of the Internet: A Statistical Physics Approach",Cambridge University Press, 2004, ISBN 0521826985

- Pennock, D. M., Flake, G. W., Lawrence, S., Glover, E. J., and Giles, C. L.: Winners don't take all: Characterizing the competition for links on the web. Proceedings of the National Academy of Sciences, 99(8):5207-5211, 2002.

- Robb, John. Scale-Free Networks and Terrorism, 2004.

- Keller, E.F. Revisiting "scale-free" networks, BioEssays 27(10) 1060-1068, 2005.

- R. N. Onody and P. A. de Castro, Complex Network Study of Brazilian Soccer Player, Phys. Rev. E 70, 037103 (2004)

- Reuven Cohen, Shlomo Havlin, "Scale-Free Networks are Ultrasmall" Phys. Rev. Lett. 90, 058701 (2003)

- ^ Steyvers, Mark; Joshua B. Tenenbaum (2005). "The Large-Scale Structure of Semantic Networks: Statistical Analyses and a Model of Semantic Growth". Cognitive Science 29 (1): 41–78. doi:10.1207/s15516709cog2901_3. http://www.leaonline.com/doi/abs/10.1207/s15516709cog2901_3.

- ^ Fratini, Michela, Poccia, Nicola, Ricci, Alessandro, Campi, Gaetano, Burghammer, Manfred, Aeppli, Gabriel Bianconi, Antonio, 'Scale-free structural organization of oxygen interstitials in La2CuO4+y', Nature , vol. 466, no. 7308, 841-844 (2010).

- [edit] External Links

From Wikipedia, the free encyclopedia

Jump to: navigation, search

|  | This article needs attention from an expert on the subject. See the talk page for details. Consider associating this request with a WikiProject. (September 2010) |
| --- | --- |

Wireless network refers to any type of computer network that is wireless, and is commonly associated with a telecommunications network whose interconnections between nodes are implemented without the use of wires.[1] Wireless telecommunications networks are generally implemented with some type of remote information transmission system that uses electromagnetic waves, such as radio waves, for the carrier and this implementation usually takes place at the physical level or "layer" of the network.[2]

- Contents [hide] - 1 Types of wireless connections - 1.1 Wireless PAN - 1.2 Wireless LAN - 1.3 Wireless MAN - 1.4 Wireless WAN - 1.5 Mobile devices networks - 2 Uses - 3 Environmental concerns and health hazard - 4 See also - 5 References - 6 Further reading - 7 External links

- [edit] Types of wireless connections

- [edit] Wireless PAN

Wireless Personal Area Networks (WPANs) interconnect devices within a relatively small area, generally within reach of a person. For example, Bluetooth provides a WPAN for interconnecting a headset to a laptop. ZigBee also supports WPAN applications.[3] Wi-Fi PANs are also getting popular as vendors have started integrating Wi-Fi in variety of consumer electronic devices. Intel My WiFi and Windows 7 virtual Wi-Fi capabilities have made Wi-Fi PANs simpler and easier to set up and configure.[4]

- [edit] Wireless LAN

Main article: Wireless LAN

A wireless local area network (WLAN) links two or more devices using a wireless distribution method (typically spread-spectrum or OFDM radio), and usually providing a connection through an access point to the wider internet. This gives users the mobility to move around within a local coverage area and still be connected to the network.

- Wi-Fi: Wi-Fi is increasingly used as a synonym for 802.11 WLANs, although it is technically a certification of interoperability between 802.11 devices.

- Fixed Wireless Data: This implements point to point links between computers or networks at two locations, often using dedicated microwave or laser beams over line of sight paths. It is often used in cities to connect networks in two or more buildings without physically wiring the buildings together.

- [edit] Wireless MAN

Wireless Metropolitan area networks are a type of wireless network that connects several Wireless LANs.

- WiMAX is the term used to refer to wireless MANs and is covered in IEEE 802.16d/802.16e.

- [edit] Wireless WAN

wireless wide area networks are wireless networks that typically cover large outdoor areas. These networks can be used to connect branch offices of business or as a public internet access system. They are usually deployed on the 2.4 GHz band. A typical system contains base station gateways, access points and wireless bridging relays. Other configurations are mesh systems where each access point acts as a relay also. When combined with renewable energy systems such as photo-voltaic solar panels or wind systems they can be stand alone systems.

- [edit] Mobile devices networks

Further information: mobile telecommunications

With the development of smart phones, cellular telephone networks routinely carry data in addition to telephone conversations:

- Global System for Mobile Communications (GSM): The GSM network is divided into three major systems: the switching system, the base station system, and the operation and support system. The cell phone connects to the base system station which then connects to the operation and support station; it then connects to the switching station where the call is transferred to where it needs to go. GSM is the most common standard and is used for a majority of cell phones.[5]

- Personal Communications Service (PCS): PCS is a radio band that can be used by mobile phones in North America and South Asia. Sprint happened to be the first service to set up a PCS.

- D-AMPS: Digital Advanced Mobile Phone Service, an upgraded version of AMPS, is being phased out due to advancement in technology. The newer GSM networks are replacing the older system.

- [edit] Uses

|  | This section is written like a personal reflection or essay and may require cleanup. Please help improve it by rewriting it in an encyclopedic style. (September 2010) |
| --- | --- |

An embedded RouterBoard 112 with U.FL-RSMA pigtail and R52 mini PCI Wi-Fi card widely used by wireless Internet service providers (WISPs) in the Czech Republic.

Wireless networks have continued to develop and their uses have grown significantly. Cellular phones are part of huge wireless network systems. People use these phones daily to communicate with one another. Sending information overseas is possible through wireless network systems using satellites and other signals to communicate across the world. Emergency services such as the police department utilize wireless networks to communicate important information quickly. People and businesses use wireless networks to send and share data quickly whether it be in a small office building or across the world.

Another important use for wireless networks is as an inexpensive and rapid way to be connected to the Internet in countries and regions where the telecom infrastructure is poor or there is a lack of resources, as in most developing countries.

Compatibility issues also arise when dealing with wireless networks. Different components not made by the same company may not work together, or might require extra work to fix these issues. Wireless networks are typically slower than those that are directly connected through an Ethernet cable.

A wireless network is more vulnerable, because anyone can try to break into a network broadcasting a signal.[citation needed] Many networks offer WEP - Wired Equivalent Privacy - security systems which have been found to be vulnerable to intrusion. Though WEP does block some intruders, the security problems have caused some businesses to stick with wired networks until security can be improved. Another type of security for wireless networks is WPA - Wi-Fi Protected Access. WPA provides more security to wireless networks than a WEP security set up. The use of firewalls will help with security breaches which can help to fix security problems in some wireless networks that are more vulnerable.

- [edit] Environmental concerns and health hazard

In recent times,[when?] there have been increased concerns about the safety of wireless communications, despite little evidence of health risks so far.[6] The president of Lakehead University refused to agree to installation of a wireless network citing a California Public Utilities Commission study which said that the possible risk of tumors and other diseases due to exposure to electromagnetic fields (EMFs) needs to be further investigated.[7]

- [edit] See also

- Exposed terminal problem

- Physical layer

- Public Safety Network

- Wireless community network

- Wireless access point

- Wireless LAN client comparison

- [edit] References

- ^ "Overview of Wireless Communications". cambridge.org. http://www.cambridge.org/us/catalogue/catalogue.asp?isbn=0521837162&ss=exc. Retrieved 2008-02-08.

- ^ "Getting to Know Wireless Networks and Technology". informit.com. http://www.informit.com/articles/printerfriendly.aspx?p=98132. Retrieved 2008-02-08.

- ^ "Wireless Network Industry Report". http://www.wireless-nets.com/resources/downloads/wireless_industry_report_2007.html. Retrieved 2008-07-08.

- ^ "Wi-Fi Personal Area Networks get a boost with Windows 7 and Intel My WiFi". http://www.informationweek.in/Software/10-04-27/Wi-Fi_personal_area_networks_get_a_boost_with_Windows_7.aspx. Retrieved 2010-04-27.

- ^ "Global System for Mobile Communication (GSM)". iec.org. Archived from the original on 2008-02-07. http://web.archive.org/web/20080207170512/http://www.iec.org/online/tutorials/gsm/topic03.html. Retrieved 2008-02-08.

- ^ Lean, Geoffrey (2007-04-22). "Wi-Fi: Children at risk from 'electronic smog'". London: news.independent.co.uk. http://news.independent.co.uk/uk/health_medical/article2472133.ece. Retrieved 2008-02-08.

- ^ "Canadian university says no to WiFi over health concerns". arstechnica.com. http://arstechnica.com/news.ars/post/20060222-6235.html. Retrieved 2008-02-08.

- [edit] Further reading

- Wireless Networking in the Developing World: A practical guide to planning and building low-cost telecommunications infrastructure (2nd ed.). Hacker Friendly LLC. 2007. pp. 425. http://wndw.net/pdf/wndw2-en/wndw2-ebook.pdf.

- Geier, Jim (2001). Wireless LANs. Sams;. ISBN 0672320584.

- Goldsmith, Andrea (2005). Wireless Communications. Cambridge University Press. ISBN 0521837162.

- Molisch, Andreas (2005). Wireless Communications. Wiley-IEEE Press. ISBN 047084888X.

- Rappaport, Theodore (2002). Wireless Communications: Principles and Practice. Prentice Hall. ISBN 0130422320.

- Rhoton, John (2001). The Wireless Internet Explained. Digital Press. ISBN 1555582575.

- Tse, David; Viswanath, Pramod (2005). Fundamentals of Wireless Communication. Cambridge University Press. ISBN 0521845270.

- [edit] External links

- Wireless at the Open Directory Project

- Internet Protocol Journal article on Wireless Data Networks

- Wireless Networking on Microsoft TechNet

Retrieved from "http://en.wikipedia.org/wiki/Wireless_network"

From Wikipedia, the free encyclopedia

Jump to: navigation, search

|  | This article does not cite any references or sources. Please help improve this article by adding citations to reliable sources. Unsourced material may be challenged and removed. (December 2009) (Find sources: Network complexity – news, books, scholar) |
| --- | --- |

|  | This article is an orphan, as few or no other articles link to it. Please introduce links to this page from related articles; suggestions are available. (December 2009) |
| --- | --- |

|  | This article may need to be wikified to meet Wikipedia's quality standards. Please help by adding relevant internal links, or by improving the article's layout. (December 2009) |
| --- | --- |

Network complexity is the number of nodes and alternative paths that exist within a computer network, as well as the variety of communication media, communications equipment, protocols, and hardware and software platforms found in the network.

Simple network: A small LAN with no alternative paths, a single communication protocol, and identical hardware and software platforms across nodes would be classified as a simple network.

Complex network: an enterprise-wide network that uses multiple communication media and communication protocols to interconnect geographically distributed networks with dissimilar hardware and software platforms would be classified as a complex network.

Retrieved from "http://en.wikipedia.org/wiki/Network_complexity"

Cloud computing is Internet-based computing, whereby shared resources, software, and information are provided to computers and other devices on demand, like the electricity grid.

Cloud computing is a paradigm shift following the shift from mainframe to client–server in the early 1980s. Details are abstracted from the users, who no longer have need for expertise in, or control over, the technology infrastructure "in the cloud" that supports them.[1] Cloud computing describes a new supplement, consumption, and delivery model for IT services based on the Internet, and it typically involves over-the-Internet provision of dynamically scalable and often virtualized resources.[2][3] It is a byproduct and consequence of the ease-of-access to remote computing sites provided by the Internet.[4] This frequently takes the form of web-based tools or applications that users can access and use through a web browser as if it were a program installed locally on their own computer.[5] NIST provides a somewhat more objective and specific definition here.[6] The term "cloud" is used as a metaphor for the Internet, based on the cloud drawing used in the past to represent the telephone network,[7] and later to depict the Internet in computer network diagrams as an abstraction of the underlying infrastructure it represents.[8] Typical cloud computing providers deliver common business applications online that are accessed from another Web service or software like a Web browser, while the software and data are stored on servers. A key element of cloud computing is customization and the creation of a user-defined experience.

Most cloud computing infrastructures consist of services delivered through common centers and built on servers. Clouds often appear as single points of access for all consumers' computing needs. Commercial offerings are generally expected to meet quality of service (QoS) requirements of customers, and typically include service level agreements (SLAs).[9] The major cloud service providers include Microsoft, Hewlett Packard,[10] IBM,[11] Salesforce, Amazon and Google.[12][13]

- Contents [hide] - 1 Overview - 1.1 Comparisons - 1.2 Characteristics - 1.3 Economics - 1.4 Architecture - 2 History - 3 Key features - 4 Layers - 4.1 Client - 4.2 Application - 4.3 Platform - 4.4 Infrastructure - 4.5 Server - 5 Deployment models - 5.1 Public cloud - 5.2 Community cloud - 5.3 Hybrid cloud - 5.4 Private cloud - 6 Cloud storage - 7 The Intercloud - 8 Issues - 8.1 Privacy - 8.2 Compliance - 8.3 Legal - 8.4 Open source - 8.5 Open standards - 8.6 Security - 8.7 Availability and performance - 8.8 Sustainability and siting - 9 Research - 10 Criticism of the term - 11 See also - 12 References

- [edit] Overview

- [edit] Comparisons

Cloud computing derives characteristics from, but should not be confused with:

- Autonomic computing — "computer systems capable of self-management"[14]

- Client–server model – client–server computing refers broadly to any distributed application that distinguishes between service providers (servers) and service requesters (clients)[15]

- Grid computing — "a form of distributed computing and parallel computing, whereby a 'super and virtual computer' is composed of a cluster of networked, loosely coupled computers acting in concert to perform very large tasks"

- Mainframe computer — powerful computers used mainly by large organizations for critical applications, typically bulk data-processing such as census, industry and consumer statistics, enterprise resource planning, and financial transaction processing.[16]

- Utility computing — the "packaging of computing resources, such as computation and storage, as a metered service similar to a traditional public utility, such as electricity";[17]

- Peer-to-peer – a distributed architecture without the need for central coordination, with participants being at the same time both suppliers and consumers of resources (in contrast to the traditional client–server model)

- [edit] Characteristics

In general, cloud computing customers do not own the physical infrastructure, instead avoiding capital expenditure by renting usage from a third-party provider. They consume resources as a service and pay only for resources that they use. Many cloud-computing offerings employ the utility computing model, which is analogous to how traditional utility services (such as electricity) are consumed, whereas others bill on a subscription basis. Sharing "perishable and intangible" computing power among multiple tenants can improve utilization rates, as servers are not unnecessarily left idle (which can reduce costs significantly while increasing the speed of application development). A side-effect of this approach is that overall computer usage rises dramatically, as customers do not have to engineer for peak load limits.[18] In addition, "increased high-speed bandwidth" makes it possible to receive the same. The cloud is becoming increasingly associated with small and medium enterprises (SMEs) as in many cases they cannot justify or afford the large capital expenditure of traditional IT. SMEs also typically have less existing infrastructure, less bureaucracy, more flexibility, and smaller capital budgets for purchasing in-house technology. Similarly, SMEs in emerging markets are typically unburdened by established legacy infrastructures, thus reducing the complexity of deploying cloud solutions.[19]

- [edit] Economics

Cloud computing users avoid capital expenditure (CapEx) on hardware, software, and services when they pay a provider only for what they use. Consumption is usually billed on a utility (resources consumed, like electricity) or subscription (time-based, like a newspaper) basis with little or no upfront cost. Other benefits of this approach are low barriers to entry, shared infrastructure and costs, low management overhead, and immediate access to a broad range of applications. In general, users can terminate the contract at any time (thereby avoiding return on investment risk and uncertainty), and the services are often covered by service level agreements (SLAs) with financial penalties.[20][21]

According to Nicholas Carr, the strategic importance of information technology is diminishing as it becomes standardized and less expensive. He argues that the cloud computing paradigm shift is similar to the displacement of electricity generators by electricity grids early in the 20th century.[22]

Although companies might be able to save on upfront capital expenditures, they might not save much and might actually pay more for operating expenses. In situations where the capital expense would be relatively small, or where the organization has more flexibility in their capital budget than their operating budget, the cloud model might not make great fiscal sense. Other factors impacting the scale of any potential cost savings include the efficiency of a company's data center as compared to the cloud vendor's, the company's existing operating costs, the level of adoption of cloud computing, and the type of functionality being hosted in the cloud.[23][24]

Among the items that some cloud hosts charge for are instances (often with extra charges for high-memory or high-CPU instances); data transfer in and out; storage (measured by the GB-month); I/O requests; PUT requests and GET requests; IP addresses; and load balancing. In some cases, users can bid on instances, with pricing dependent on demand for available instances.[citation needed]

- [edit] Architecture

Cloud computing sample architecture

Cloud architecture,[25] the systems architecture of the software systems involved in the delivery of cloud computing, typically involves multiple cloud components communicating with each other over application programming interfaces, usually web services. This resembles the Unix philosophy of having multiple programs each doing one thing well and working together over universal interfaces. Complexity is controlled and the resulting systems are more manageable than their monolithic counterparts.

The two most significant components of cloud computing architecture are known as the front end and the back end. The front end is the part seen by the client, i.e. the computer user. This includes the client’s network (or computer) and the applications used to access the cloud via a user interface such as a web browser. The back end of the cloud computing architecture is the ‘cloud’ itself, comprising various computers, servers and data storage devices.

- [edit] History

The underlying concept of cloud computing dates back to the 1960s, when John McCarthy opined that "computation may someday be organized as a public utility". Almost all the modern day characteristics of cloud computing (elastic provision, provided as a utility, online, illusion of infinite supply), the comparison to the electricity industry and the use of public, private, government and community forms was thoroughly explored in Douglas Parkhill's, 1966 book, "The Challenge of the Computer Utility".

The actual term "cloud" borrows from telephony in that telecommunications companies, who until the 1990s primarily offered dedicated point-to-point data circuits, began offering Virtual Private Network (VPN) services with comparable quality of service but at a much lower cost. By switching traffic to balance utilization as they saw fit they were able to utilise their overall network bandwidth more effectively. The cloud symbol was used to denote the demarcation point between that which was the responsibility of the provider from that of the user. Cloud computing extends this boundary to cover servers as well as the network infrastructure.[26]

Amazon played a key role in the development of cloud computing by modernizing their data centers after the dot-com bubble, which, like most computer networks, were using as little as 10% of their capacity at any one time just to leave room for occasional spikes. Having found that the new cloud architecture resulted in significant internal efficiency improvements whereby small, fast-moving "two-pizza teams" could add new features faster and easier, Amazon initiated a new product development effort to provide cloud computing to external customers and launched Amazon Web Service (AWS) on a utility computing basis in 2006.[27][28]

In 2007, Google, IBM, and a number of universities embarked on a large scale cloud computing research project.[29] In early 2008, Eucalyptus became the first open source AWS API compatible platform for deploying private clouds. By mid-2008, Gartner saw an opportunity for cloud computing "to shape the relationship among consumers of IT services, those who use IT services and those who sell them",[30] and observed that "[o]rganisations are switching from company-owned hardware and software assets to per-use service-based models" so that the "projected shift to cloud computing ... will result in dramatic growth in IT products in some areas and significant reductions in other areas."[31]

In March 2010, Microsoft's CEO, Steve Ballmer, made his strongest statement of betting the company's future in the cloud by proclaiming "For the cloud, we're all in" and further stating "About 75 percent of our folks are doing entirely cloud based or entirely cloud inspired, a year from now that will be 90 percent."[32]

Microsoft has also offered details on cloud services for government agencies [33]

- [edit] Key features

- Agility improves with users' ability to rapidly and inexpensively re-provision technological infrastructure resources.[34]

- Cost is claimed to be greatly reduced and capital expenditure is converted to operational expenditure.[35] This ostensibly lowers barriers to entry, as infrastructure is typically provided by a third-party and does not need to be purchased for one-time or infrequent intensive computing tasks. Pricing on a utility computing basis is fine-grained with usage-based options and fewer IT skills are required for implementation (in-house).[36]

- Device and location independence[37] enable users to access systems using a web browser regardless of their location or what device they are using (e.g., PC, mobile). As infrastructure is off-site (typically provided by a third-party) and accessed via the Internet, users can connect from anywhere.[36]

- Multi-tenancy enables sharing of resources and costs across a large pool of users thus allowing for:

  - Centralization of infrastructure in locations with lower costs (such as real estate, electricity, etc.)

  - Peak-load capacity increases (users need not engineer for highest possible load-levels)

  - Utilization and efficiency improvements for systems that are often only 10–20% utilized.[27]

- Reliability is improved if multiple redundant sites are used, which makes well designed cloud computing suitable for business continuity and disaster recovery.[38] Nonetheless, many major cloud computing services have suffered outages, and IT and business managers can at times do little when they are affected.[39][40]

- Scalability via dynamic ("on-demand") provisioning of resources on a fine-grained, self-service basis near real-time, without users having to engineer for peak loads. Performance is monitored, and consistent and loosely coupled architectures are constructed using web services as the system interface.[36] One of the most important new methods for overcoming performance bottlenecks for a large class of applications is data parallel programming on a distributed data grid.[41]

- Security could improve due to centralization of data,[42] increased security-focused resources, etc., but concerns can persist about loss of control over certain sensitive data, and the lack of security for stored kernels.[43] Security is often as good as or better than under traditional systems, in part because providers are able to devote resources to solving security issues that many customers cannot afford.[44] Providers typically log accesses, but accessing the audit logs themselves can be difficult or impossible. Furthermore, the complexity of security is greatly increased when data is distributed over a wider area and / or number of devices.

- Maintenance of cloud computing applications is easier, since they don't have to be installed on each user's computer. They are easier to support and to improve since the changes reach the clients instantly.

- Metering means that cloud computing resources usage should be measurable and should be metered per client and application on a daily, weekly, monthly, and yearly basis.

- [edit] Layers

- [edit] Client

See also: Category:Cloud clients

A cloud client consists of computer hardware and/or computer software that relies on cloud computing for application delivery, or that is specifically designed for delivery of cloud services and that, in either case, is essentially useless without it. Examples include some computers, phones and other devices, operating systems and browsers.[45][46][47][48][49]

- [edit] Application

See also: Category:Cloud applications

Cloud application services or "Software as a Service (SaaS)" deliver software as a service over the Internet, eliminating the need to install and run the application on the customer's own computers and simplifying maintenance and support. People tend to use the terms ‘SaaS’ and ‘cloud’ interchangeably, when in fact they are 2 different things.[50] Key characteristics include:[51]

- Network-based access to, and management of, commercially available (i.e., not custom) software

- Activities that are managed from central locations rather than at each customer's site, enabling customers to access applications remotely via the Web

- Application delivery that typically is closer to a one-to-many model (single instance, multi-tenant architecture) than to a one-to-one model, including architecture, pricing, partnering, and management characteristics

- Centralized feature updating, which obviates the need for downloadable patches and upgrades.

- [edit] Platform

See also: Category:Cloud platforms

Cloud platform services or "Platform as a Service (PaaS)" deliver a computing platform and/or solution stack as a service, often consuming cloud infrastructure and sustaining cloud applications.[52] It facilitates deployment of applications without the cost and complexity of buying and managing the underlying hardware and software layers.[53][54]

- [edit] Infrastructure

See also: Category:Cloud infrastructure

Cloud infrastructure services, also known as "Infrastructure as a Service (IaaS)", delivers computer infrastructure - typically a platform virtualization environment - as a service. Rather than purchasing servers, software, data-center space or network equipment, clients instead buy those resources as a fully outsourced service. Suppliers typically bill such services on a utility computing basis and amount of resources consumed (and therefore the cost) will typically reflect the level of activity. IaaS evolved from virtual private server offerings.[55]

- [edit] Server

The servers layer consists of computer hardware and/or computer software products that are specifically designed for the delivery of cloud services, including multi-core processors, cloud-specific operating systems and combined offerings.[45][56][57][58]

- [edit] Deployment models

Cloud computing types

- [edit] Public cloud

Public cloud or external cloud describes cloud computing in the traditional mainstream sense, whereby resources are dynamically provisioned on a fine-grained, self-service basis over the Internet, via web applications/web services, from an off-site third-party provider who bills on a fine-grained utility computing basis.[36]

- [edit] Community cloud

A community cloud may be established where several organizations have similar requirements and seek to share infrastructure so as to realize some of the benefits of cloud computing. With the costs spread over fewer users than a public cloud (but more than a single tenant) this option is more expensive but may offer a higher level of privacy, security and/or policy compliance. Examples of community cloud include Google's "Gov Cloud".[59]

- [edit] Hybrid cloud

See also: Cloud storage gateway

A hybrid cloud environment consisting of multiple internal and/or external providers[60] "will be typical for most enterprises".[61] By integrating multiple cloud services users may be able to ease the transition to public cloud services while avoiding issues such as PCI compliance.[62]

Another perspective on deploying a web application in the cloud is using Hybrid Web Hosting, where the hosting infrastructure is a mix between Cloud Hosting for the web server, and Managed dedicated server for the database server.

A hybrid storage cloud uses a combination of public and private storage clouds. Hybrid storage clouds are often useful for archiving and backup functions, allowing local data to be replicated to a public cloud. [63]

- [edit] Private cloud

The concept of a Private Computer Utility was first described by Douglas Parkhill in his 1966 book "The Challenge of the Computer Utility". The idea was based upon direct comparison with other industries (e.g. the electricity industry) and the extensive use of hybrid supply models to balance and mitigate risks.

Private cloud and internal cloud have been described as neologisms, however the concepts themselves pre-date the term cloud by 40 years. Even within modern utility industries, hybrid models still exist despite the formation of reasonably well functioning markets and the ability to combine multiple providers.

Some vendors have used the terms to describe offerings that emulate cloud computing on private networks. These (typically virtualisation automation) products offer the ability to deliver some benefits of cloud computing whilst mitigating some of the pitfalls. These offerings capitalise on data security, corporate governance, and reliability concerns during this time of transition from a product to a functioning service based industry supported by competitive marketplaces.

They have been criticized on the basis that users "still have to buy, build, and manage them" and as such do not benefit from lower up-front capital costs and less hands-on management,[61] essentially "[lacking] the economic model that makes cloud computing such an intriguing concept".[64][65]

- [edit] Cloud storage

Main article: Cloud storage

Cloud Storage is a model of networked computer data storage where data is stored on multiple virtual servers, generally hosted by third parties, rather than being hosted on dedicated servers. Hosting companies operate large data centers; and people who require their data to be hosted buy or lease storage capacity from them and use it for their storage needs. The data center operators, in the background, virtualize the resources according to the requirements of the customer and expose them as virtual servers, which the customers can themselves manage. Physically, the resource may span across multiple servers.

- [edit] The Intercloud

Main article: Intercloud

The Intercloud[66] is an interconnected global "cloud of clouds"[67][68] and an extension of the Internet "network of networks" on which it is based.[69] The term was first used in the context of cloud computing in 2007 when Kevin Kelly stated that "eventually we'll have the intercloud, the cloud of clouds. This Intercloud will have the dimensions of one machine comprising all servers and attendant cloudbooks on the planet.".[67] It became popular in 2009[70] and has also been used to describe the datacenter of the future.[71]

The Intercloud scenario is based on the key concept that each single cloud does not have infinite physical resources. If a cloud saturates the computational and storage resources of its virtualization infrastructure, it could not be able to satisfy further requests for service allocations sent from its clients. The Intercloud scenario aims to address such situation, and in theory, each cloud can use the computational and storage resources of the virtualization infrastructures of other clouds. Such form of pay-for-use may introduce new business opportunities among cloud providers if they manage to go beyond theoretical framework. Nevertheless, the Intercloud raises many more challenges than solutions concerning cloud federation, security, interoperability, QoS, vendor's lock-ins, trust, legal issues, monitoring and billing.[citation needed]

The concept of a competitive utility computing market which combined many computer utilities together was originally described by Douglas Parkhill in his 1966 book, the "Challenge of the Computer Utility". This concept has been subsequently used many times over the last 40 years and is identical to the Intercloud.

- [edit] Issues

- [edit] Privacy

The Cloud model has been criticized by privacy advocates for the greater ease in which the companies hosting the Cloud services control, and thus, can monitor at will, lawfully or unlawfully, the communication and data stored between the user and the host company. Instances such as the secret NSA program, working with AT&T, and Verizon, which recorded over 10 million phone calls between American citizens, causes uncertainty among privacy advocates, and the greater powers it gives to telecommunication companies to monitor user activity.[72] While there have been efforts (such as US-EU Safe Harbor) to "harmonise" the legal environment, providers such as Amazon still cater to major markets (typically the United States and the European Union) by deploying local infrastructure and allowing customers to select "availability zones."[73]

- [edit] Compliance

In order to obtain compliance with regulations including FISMA, HIPAA and SOX in the United States, the Data Protection Directive in the EU and the credit card industry's PCI DSS, users may have to adopt community or hybrid deployment modes which are typically more expensive and may offer restricted benefits. This is how Google is able to "manage and meet additional government policy requirements beyond FISMA"[74][75] and Rackspace Cloud are able to claim PCI compliance.[76] Customers in the EU contracting with Cloud Providers established outside the EU/EEA have to adhere to the EU regulations on export of personal data.[77]

Many providers also obtain SAS 70 Type II certification (e.g. Amazon,[78] Salesforce.com,[79] Google[80] and Microsoft[81]), but this has been criticised on the grounds that the hand-picked set of goals and standards determined by the auditor and the auditee are often not disclosed and can vary widely.[82] Providers typically make this information available on request, under non-disclosure agreement.[83]

- [edit] Legal

In March 2007, Dell applied to trademark the term "cloud computing" (U.S. Trademark 77,139,082) in the United States. The "Notice of Allowance" the company received in July 2008 was cancelled in August, resulting in a formal rejection of the trademark application less than a week later.

Since 2007, the number of trademark filings covering cloud computing brands, goods and services has increased at an almost exponential rate. As companies sought to better position themselves for cloud computing branding and marketing efforts, cloud computing trademark filings increased by 483% between 2008 and 2009. In 2009, 116 cloud computing trademarks were filed, and trademark analysts predict that over 500 such marks could be filed during 2010.[84]

- [edit] Open source

Open source software has provided the foundation for many cloud computing implementations.[85] In November 2007, the Free Software Foundation released the Affero General Public License, a version of GPLv3 intended to close a perceived legal loophole associated with free software designed to be run over a network.[86]

- [edit] Open standards

See also: Category:Cloud standards

Most cloud providers expose APIs which are typically well-documented (often under a Creative Commons license[87]) but also unique to their implementation and thus not interoperable. Some vendors have adopted others' APIs[88] and there are a number of open standards under development, including the OGF's Open Cloud Computing Interface. The Open Cloud Consortium (OCC) [89] is working to develop consensus on early cloud computing standards and practices.

- [edit] Security

Main article: Cloud computing security

The relative security of cloud computing services is a contentious issue which may be delaying its adoption.[90] Some argue that customer data is more secure when managed internally, while others argue that cloud providers have a strong incentive to maintain trust and as such employ a higher level of security.[91]

The Cloud Security Alliance is a non-profit organization formed to promote the use of best practices for providing security assurance within Cloud Computing.[92]

- [edit] Availability and performance

In addition to concerns about security, businesses are also worried about acceptable levels of availability and performance of applications hosted in the cloud.[93]

There are also concerns about a cloud provider shutting down for financial or legal reasons, which has happened in a number of cases.[94]

- [edit] Sustainability and siting

Although cloud computing is often assumed to be a form of "green computing", there is as of yet no published study to substantiate this assumption.[95] Siting the servers affects the environmental effects of cloud computing. In areas where climate favors natural cooling and renewable electricity is readily available, the environmental effects will be more moderate. Thus countries with favorable conditions, such as Finland,[96] Sweden and Switzerland,[97] are trying to attract cloud computing data centers.

- [edit] Research

A number of universities, vendors and government organizations are investing in research around the topic of cloud computing.[98] Academic institutions include University of Melbourne (Australia), Georgia Tech, Yale, Wayne State, Virginia Tech, University of Wisconsin–Madison, Boston University, Carnegie Mellon, MIT, Indiana University, University of Massachusetts, University of Maryland, North Carolina State, Purdue, University of California, University of Washington, University of Virginia, University of Utah, University of Minnesota, among others.[99]

Joint government, academic and vendor collaborative research projects include the IBM/Google Academic Cloud Computing Initiative (ACCI). In October 2007 IBM and Google announced the multi- university project designed to enhance students' technical knowledge to address the challenges of cloud computing.[100] In April 2009, the National Science Foundation joined the ACCI and awarded approximately $5 million in grants to 14 academic institutions.[101]

In July 2008, HP, Intel Corporation and Yahoo! announced the creation of a global, multi-data center, open source test bed, called Open Cirrus,[102] designed to encourage research into all aspects of cloud computing, service and data center management.[103] Open Cirrus partners include the NSF, the University of Illinois (UIUC), Karlsruhe Institute of Technology, the Infocomm Development Authority (IDA) of Singapore, the Electronics and Telecommunications Research Institute (ETRI) in Korea, the Malaysian Institute for Microelectronic Systems(MIMOS), and the Institute for System Programming at the Russian Academy of Sciences (ISPRAS).[104] In Sept. 2010, more researchers joined the HP/Intel/Yahoo Open Cirrus project for cloud computing research. The new researchers are China Mobile Research Institute (CMRI), Spain's Supercomputing Center of Galicia (CESGA by its Spanish acronym), Georgia Tech's Center for Experimental Research in Computer Systems (CERCS) and China Telecom.[105][106]

In July 2010, HP Labs India announced a new cloud-based technology designed to simplify taking content and making it mobile-enabled, even from low-end devices.[107] Called SiteonMobile, the new technology is designed for emerging markets where people are more likely to access the internet via mobile phones rather than computers.[108]

The IEEE Technical Committee on Services Computing[109] in IEEE Computer Society sponsors the IEEE International Conference on Cloud Computing (CLOUD).[110] CLOUD 2010 was held on July 5–10, 2010 in Miami, Florida.

- [edit] Criticism of the term

|  | This article's Criticism or Controversy section(s) may mean the article does not present a neutral point of view of the subject. It may be better to integrate the material in those sections into the article as a whole. (March 2010) |
| --- | --- |

During a video interview, Forrester Research VP Frank Gillett expresses criticism about the nature of and motivations behind the push for cloud computing. He describes what he calls "cloud washing" in the industry whereby companies relabel their products as cloud computing resulting in a lot of marketing innovation on top of real innovation. The result is a lot of overblown hype surrounding cloud computing. Gillett sees cloud computing as revolutionary in the long term but over-hyped and misunderstood in the short term, representing more of a gradual shift in our thinking about computer systems and not a sudden transformational change.[111][112]

Larry Ellison, CEO of Oracle Corporation has stated that cloud computing has been defined as "everything that we already do" and that it will have no effect except to "change the wording on some of our ads".[113][114] Oracle Corporation has since launched a cloud computing center and worldwide tour. Forrester Research Principal Analyst John Rymer dismisses Ellison's remarks by stating that his "comments are complete nonsense and he knows it".[115][116][117]

Richard Stallman said that cloud computing was simply a trap aimed at forcing more people to buy into locked, proprietary systems that would cost them more and more over time. "It's stupidity. It's worse than stupidity: it's a marketing hype campaign", he told The Guardian. "Somebody is saying this is inevitable – and whenever you hear somebody saying that, it's very likely to be a set of businesses campaigning to make it true." [118]

- [edit] See also

- Cloud backup

- Cloud gaming

- [edit] References

- ^ Danielson, Krissi (2008-03-26). "Distinguishing Cloud Computing from Utility Computing". Ebizq.net. http://www.ebizq.net/blogs/saasweek/2008/03/distinguishing_cloud_computing/. Retrieved 2010-08-22.

- ^ "Gartner Say's Cloud Computing Will Be As Influential As E-business". Gartner.com. http://www.gartner.com/it/page.jsp?id=707508. Retrieved 2010-08-22.

- ^ Gruman, Galen (2008-04-07). "What cloud computing really means". InfoWorld. http://www.infoworld.com/d/cloud-computing/what-cloud-computing-really-means-031. Retrieved 2009-06-02.

- ^ "Cloud Computing: Clash of the clouds". The Economist. 2009-10-15. http://www.economist.com/displaystory.cfm?story_id=14637206. Retrieved 2009-11-03.

- ^ Cloud Computing Defined 17 July 2010. Retrieved 26 July 2010.

- ^ "NIST.gov - Computer Security Division - Computer Security Resource Center". Csrc.nist.gov. http://csrc.nist.gov/groups/SNS/cloud-computing/. Retrieved 2010-08-22.

- ^ "Writing & Speaking". Sellsbrothers.com. http://www.sellsbrothers.com/writing/intro2tapi/default.aspx?content=pstn.htm. Retrieved 2010-08-22.

- ^ "The Internet Cloud". Thestandard.com. http://www.thestandard.com/article/0,1902,5466,00.html. Retrieved 2010-08-22.

- ^ Buyya, Rajkumar; Chee Shin Yeo, Srikumar Venugopal (PDF). Market-Oriented Cloud Computing: Vision, Hype, and Reality for Delivering IT Services as Computing Utilities. Department of Computer Science and Software Engineering, University of Melbourne, Australia. pp. 9. http://www.gridBus.org/~raj/papers/hpcc2008_keynote_cloudcomputing.pdf. Retrieved 2008-07-31.

- ^ Poeter, Damon (2010-07-12). "Channel Web: “HP, Microsoft Promise Windows Azure Cloud Platform By Year's End.” July 2010. Poeter". Crn.com. http://www.crn.com/225702946. Retrieved 2010-08-22.

- ^ Goldman, Alex (2010-09-01). "Channel Web: “IBM Outlines an Aggressive Cloud Computing Strategy” September 2010. Goldman". internet.com. http://itmanagement.earthweb.com/features/article.php/3901656/IBM-Outlines-an-Aggressive-Cloud-Computing-Strategy.htm. Retrieved 2010-09-22.

- ^ ""The Rise of Cloud Computing." Michael Otey. April 2010". windowsITpro.com. 2010-04-26. http://www.windowsitpro.com/article/cloud-computing2/The-Rise-of-Cloud-Computing/2.aspx. Retrieved 2010-08-22.

- ^ Lai, Eric (2009-08-27). "Google, Amazon, Microsoft beef up cloud services". Infoworld.com. http://www.infoworld.com/d/cloud-computing/woo-partners-cloud-computing-vendors-show-them-money-426. Retrieved 2010-08-22.

- ^ "What's In A Name? Utility vs. Cloud vs Grid". Datacenterknowledge.com. http://www.datacenterknowledge.com/archives/2008/Mar/25/whats_in_a_name_utility_vs_cloud_vs_grid.html. Retrieved 2010-08-22.

- ^ "Distributed Application Architecture". Sun Microsystem. http://java.sun.com/developer/Books/jdbc/ch07.pdf. Retrieved 2009-06-16.

- ^ "Sun CTO: Cloud computing is like the mainframe". Itknowledgeexchange.techtarget.com. 2009-03-11. http://itknowledgeexchange.techtarget.com/mainframe-blog/sun-cto-cloud-computing-is-like-the-mainframe/. Retrieved 2010-08-22.

- ^ "It's probable that you've misunderstood 'Cloud Computing' until now". TechPluto. http://portal.acm.org/citation.cfm?id=1496091.1496100&coll=&dl=ACM&CFID=21518680&CFTOKEN=18800807. Retrieved 2010-09-14.

- ^ "Cloud Computing: The Evolution of Software-as-a-Service". Knowledge.wpcarey.asu.edu. http://knowledge.wpcarey.asu.edu/article.cfm?articleid=1614. Retrieved 2010-08-22.

- ^ SMBs cloud computing adoption: the future’s bright -Bime - SAAS Business Intelligence (BI)

- ^ http://www.eweek.com/c/a/Enterprise-Applications/Forresters-Advice-to-CFOs-Embrace-Cloud-Computing-to-Cut-Costs/

- ^ "Five cloud computing questions". Networkworld.com. http://www.networkworld.com/columnists/2008/080508-dzubeck.html. Retrieved 2010-08-22.

- ^ "Nicholas Carr on 'The Big Switch' to cloud computing". Computerworlduk.com. http://www.computerworlduk.com/technology/internet/applications/instant-expert/index.cfm?articleid=1610. Retrieved 2010-08-22.

- ^ Paul, Fredric. "1 Midsize Organization Busts 5 Cloud Computing Myths". Bmighty.com. http://www.bmighty.com/services/showArticle.jhtml?articleID=211600030. Retrieved 2010-08-22.

- ^ "Cloud Computing Savings – Real or Imaginary?". Appirio.com. 2009-04-16. http://www.appirio.com/blog/2009/04/cloud-computing-savings-real-or.php. Retrieved 2010-08-22.

- ^ "Building GrepTheWeb in the Cloud, Part 1: Cloud Architectures". Developer.amazonwebservices.com. http://developer.amazonwebservices.com/connect/entry.jspa?externalID=1632&categoryID=100. Retrieved 2010-08-22.

- ^ "July, 1993 meeting report from the IP over ATM working group of the IETF". http://mirror.switch.ch/ftp/doc/ietf/ipatm/atm-minutes-93jul.txt. Retrieved 2010-08-22.

- ^ a b Jeff Bezos' Risky Bet.

- ^ [1].

- ^ Google and I.B.M. Join in 'Cloud Computing' Research

- ^ Keep an eye on cloud computing, Amy Schurr, Network World, 2008-07-08, citing the Gartner report, "Cloud Computing Confusion Leads to Opportunity". Retrieved 2009-09-11.

- ^ Gartner Says Worldwide IT Spending On Pace to Surpass $3.4 Trillion in 2008, Gartner, 2008-08-18. Retrieved 2009-09-11.

- ^ Steve Ballmer speech at UW: "We're all in" for cloud computing,

- ^ Microsoft Cloud Computing Guide for government,

- ^ "Infrastructure Agility: Cloud Computing as a Best Practice". Edgewatertech.wordpress.com. 2010-01-15. http://edgewatertech.wordpress.com/2009/04/24/best-practice-cloud-computing/. Retrieved 2010-08-22.

- ^ "Recession Is Good For Cloud Computing – Microsoft Agrees". Cloudave.com. http://www.cloudave.com/link/recession-is-good-for-cloud-computing-microsoft-agrees. Retrieved 2010-08-22.

- ^ a b c d "Defining "Cloud Services" and "Cloud Computing"". Blogs.idc.com. 2008-09-23. http://blogs.idc.com/ie/?p=190. Retrieved 2010-08-22.

- ^ Farber, Dan (2008-06-25). "The new geek chic: Data centers". News.cnet.com. http://news.cnet.com/8301-13953_3-9977049-80.html. Retrieved 2010-08-22.

- ^ King, Rachael (2008-08-04). "Cloud Computing: Small Companies Take Flight". Businessweek.com. http://www.businessweek.com/technology/content/aug2008/tc2008083_619516.htm. Retrieved 2010-08-22.

- ^ "Google Apps Admins Jittery About Gmail, Hopeful About Future". Pcworld.com. 2008-08-15. http://www.pcworld.com/businesscenter/article/149892/google_apps_admins_jittery_about_gmail_hopeful_about_future.html. Retrieved 2010-08-22.

- ^ "New Resource, Born of a Cloud Feud". Datacenterknowledge.com. 2009-06-03. http://www.datacenterknowledge.com/archives/2008/09/22/a-new-resource-born-of-a-cloud-feud/A. Retrieved 2010-08-22.

- ^ "Scaling Storage and Analysis of Data Using Distributed Data Grids". Nubifer.wordpress.com. 2010-02-16. http://nubifer.wordpress.com/2010/02/16/scaling-storage-and-analysis-of-data-using-distributed-data-grids/. Retrieved 2010-08-22.

- ^ "Exari: Death By Laptop". Exari.blogspot.com. 2006-05-08. http://exari.blogspot.com/2006/05/death-by-laptop.html. Retrieved 2010-08-22.

- ^ "Encrypted Storage and Key Management for the cloud". Cryptoclarity.com. 2009-07-30. http://www.cryptoclarity.com/CryptoClarityLLC/Welcome/Entries/2009/7/23_Encrypted_Storage_and_Key_Management_for_the_cloud.html. Retrieved 2010-08-22.

- ^ Mills, Elinor (2009-01-27). "Cloud computing security forecast: Clear skies". News.cnet.com. http://news.cnet.com/8301-1009_3-10150569-83.html. Retrieved 2010-08-22.

- ^ a b Nimbus Cloud Guide[dead link]

- ^ "Microsoft's cloud operating system, Windows Azure, to go live in January". Geek.com. 2009-11-18. http://www.geek.com/articles/news/microsofts-cloud-operating-system-windows-azure-to-go-live-in-january-20091118/. Retrieved 2010-08-22.

- ^ Claburn, Thomas. "Google Reveals Nexus One 'Super Phone'". Informationweek.com. http://www.informationweek.com/news/software/web_services/showArticle.jhtml?articleID=222200331. Retrieved 2010-08-22.

- ^ "What Makes a Cloud Computer?". Gigaom.com. 2008-06-22. http://gigaom.com/2008/06/22/what-makes-a-good-cloud-computer/. Retrieved 2010-08-22.

- ^ by Brian BraikerSeptember 02, 2008 (2008-09-02). "The Cloud's Chrome Lining". Newsweek.com. http://www.newsweek.com/id/156911. Retrieved 2010-08-22.

- ^ What’s the Difference Between SaaS and the Cloud? -Bime - SAAS Business Intelligence (BI)

- ^ 2005 Software as a Service Taxonomy and Research Guide[dead link]

- ^ "An example of a 'Cloud Platform' for building applications". Eccentex.com. http://www.eccentex.com/platform/workflow.html. Retrieved 2010-08-22.

- ^ Jack Schofield (2008-04-17). "Google angles for business users with 'platform as a service'". London: Guardian. http://www.guardian.co.uk/technology/2008/apr/17/google.software. Retrieved 2010-08-22.

- ^ "The Emerging Cloud Service Architecture". Aws.typepad.com. 2008-06-03. http://aws.typepad.com/aws/2008/06/the-forthcoming.html. Retrieved 2010-08-22.

- ^ "EMC buys Pi and forms a cloud computing group". Searchstorage.techtarget.com. 2008-02-21. http://searchstorage.techtarget.com/news/article/0,289142,sid5_gci1301852,00.html. Retrieved 2010-08-22.

- ^ Myslewski, Rik (2009-12-02). "Intel puts cloud on single megachip". Theregister.co.uk. http://www.theregister.co.uk/2009/12/02/intel_scc/. Retrieved 2010-08-22.

- ^ Duffy, Jim (2009-05-12). "Cisco unveils cloud computing platform for service providers". Infoworld.com. http://www.infoworld.com/d/cloud-computing/cisco-unveils-cloud-computing-platform-service-providers-113. Retrieved 2010-08-22.

- ^ Microsoft Plans 'Cloud' Operating System

- ^ Claburn, Thomas. "Google's "Gov Cloud" Wins $7.2 Million Los Angeles Contract". Informationweek.com. http://www.informationweek.com/news/services/saas/showArticle.jhtml?articleID=221100129. Retrieved 2010-08-22.

- ^ Eric Krangel (2009-02-10). "IBM Embraces Juniper For Its Smart 'Hybrid Cloud', Disses Cisco (IBM)". Businessinsider.com. http://www.businessinsider.com/2009/2/ibm-embraces-juniper-for-its-smart-hybrid-cloud-disses-cisco-ibm. Retrieved 2010-08-22.

- ^ a b Foley, John. "Private Clouds Take Shape". Informationweek.com. http://www.informationweek.com/news/services/business/showArticle.jhtml?articleID=209904474. Retrieved 2010-08-22.

- ^ "Forecast for 2010: The Rise of Hybrid Clouds". Gigaom.com. 2010-01-01. http://gigaom.com/2010/01/01/on-the-rise-of-hybrid-clouds/. Retrieved 2010-08-22.

- ^ Managing Private and Hybrid Clouds for Data Storage, SNIA, Jan 2010

- ^ Haff, Gordon (2009-01-27). "Just don't call them private clouds". News.cnet.com. http://news.cnet.com/8301-13556_3-10150841-61.html. Retrieved 2010-08-22.

- ^ "There's No Such Thing As A Private Cloud". Informationweek.com. 2010-06-30. http://www.informationweek.com/cloud-computing/blog/archives/2009/01/theres_no_such.html. Retrieved 2010-08-22.

- ^ Bernstein, David; Ludvigson, Erik; Sankar, Krishna; Diamond, Steve; Morrow, Monique (2009-05-24). Blueprint for the Intercloud - Protocols and Formats for Cloud Computing Interoperability. IEEE Computer Society. pp. 328–336. doi:10.1109/ICIW.2009.55. http://www2.computer.org/portal/web/csdl/doi/10.1109/ICIW.2009.55.

- ^ a b "Kevin Kelly: A Cloudbook for the Cloud". Kk.org. http://www.kk.org/thetechnium/archives/2007/11/a_cloudbook_for.php. Retrieved 2010-08-22.

- ^ "Intercloud is a global cloud of clouds". Samj.net. 2009-06-22. http://samj.net/2009/06/intercloud-is-global-cloud-of-clouds.html. Retrieved 2010-08-22.

- ^ "Vint Cerf: Despite Its Age, The Internet is Still Filled with Problems". Readwriteweb.com. http://www.readwriteweb.com/archives/vint_cerf_despite_its_age_the.php?mtcCampaign=2765. Retrieved 2010-08-22.

- ^ "SP360: Service Provider: From India to Intercloud". Blogs.cisco.com. http://blogs.cisco.com/sp/comments/from_india_to_intercloud/. Retrieved 2010-08-22.

- ^ Canada. "Head in the clouds? Welcome to the future". Theglobeandmail.com. http://www.theglobeandmail.com/servlet/story/LAC.20071129.TWLINKS29/TPStory/Business. Retrieved 2010-08-22.

- ^ Cauley, Leslie (2006-05-11). "NSA has massive database of Americans' phone calls". Usatoday.com. http://www.usatoday.com/news/washington/2006-05-10-nsa_x.htm. Retrieved 2010-08-22.

- ^ "Feature Guide: Amazon EC2 Availability Zones". Developer.amazonwebservices.com. http://developer.amazonwebservices.com/connect/entry.jspa?externalID=1347&categoryID=112. Retrieved 2010-08-22.

- ^ "FISMA compliance for federal cloud computing on the horizon in 2010". Searchcompliance.techtarget.com. http://searchcompliance.techtarget.com/news/article/0,289142,sid195_gci1377298,00.html. Retrieved 2010-08-22.

- ^ "Google Apps and Government". Googleenterprise.blogspot.com. 2009-09-15. http://googleenterprise.blogspot.com/2009/09/google-apps-and-government.html. Retrieved 2010-08-22.

- ^ "Cloud Hosting is Secure for Take-off: Mosso Enables The Spreadsheet Store, an Online Merchant, to become PCI Compliant". Rackspacecloud.com. 2009-03-14. http://www.rackspacecloud.com/blog/2009/03/05/cloud-hosting-is-secure-for-take-off-mosso-enables-the-spreadsheet-store-an-online-merchant-to-become-pci-compliant/. Retrieved 2010-08-22.

- ^ "How the New EU Rules on Data Export Affect Companies in and Outside the EU | Dr. Thomas Helbing - Kanzlei für Datenschutz-, Online- und IT-Recht". Thomashelbing.com. http://www.thomashelbing.com/en/how-new-eu-rules-data-export-affect-companies-and-outside-eu. Retrieved 2010-08-22.

- ^ "AWS Completes SAS70 Type II Audit". Aws.amazon.com. http://aws.amazon.com/about-aws/whats-new/2009/11/11/aws-completes-sas70-type-ii-audit/. Retrieved 2010-08-22.

- ^ "Trust Salesforce.com". Trust Salesforce.com. http://trust.salesforce.com/. Retrieved 2010-08-22.

- ^ Feigenbaum, Eran (2008-11-04). "SAS 70 Type II for Google Apps". Googleenterprise.blogspot.com. http://googleenterprise.blogspot.com/2008/11/sas-70-type-ii-for-google-apps.html. Retrieved 2010-08-22.

- ^ "Securing Microsoft's Cloud Infrastructure". Blogs.technet.com. 2009-05-28. http://blogs.technet.com/gfs/archive/2009/05/27/securing-microsoft-s-cloud-infrastructure.aspx. Retrieved 2010-08-22.

- ^ "Amazon gets SAS 70 Type II audit stamp, but analysts not satisfied". Searchcloudcomputing.techtarget.com. 2009-11-17. http://searchcloudcomputing.techtarget.com/news/article/0,289142,sid201_gci1374629,00.html. Retrieved 2010-08-22.

- ^ "Assessing Cloud Computing Agreements and Controls". Wistechnology.com. http://wistechnology.com/articles/6954/. Retrieved 2010-08-22.

- ^ "Inside Trademarks March 24, 2010". Insidetrademarks.com. 2010-03-24. http://insidetrademarks.com/2010/03/24/cloud-computing-trademark-trends/. Retrieved 2010-08-22.

- ^ "Open source fuels growth of cloud computing, software-as-a-service". Networkworld.com. http://www.networkworld.com/news/2008/072808-open-source-cloud-computing.html. Retrieved 2010-08-22.

- ^ "AGPL: Open Source Licensing in a Networked Age". Redmonk.com. 2009-04-15. http://redmonk.com/sogrady/2009/04/15/open-source-licensing-in-a-networked-age/. Retrieved 2010-08-22.

- ^ GoGrid Moves API Specification to Creative Commons[dead link]

- ^ "Eucalyptus Completes Amazon Web Services Specs with Latest Release". Ostatic.com. http://ostatic.com/blog/eucalyptus-completes-amazon-web-services-specs-with-latest-release. Retrieved 2010-08-22.

- ^ "Open Cloud Consortium.org". Open Cloud Consortium.org. 1999-02-22. http://opencloudconsortium.org. Retrieved 2010-08-22.

- ^ "Are security issues delaying adoption of cloud computing?". Networkworld.com. http://www.networkworld.com/news/2009/042709-burning-security-cloud-computing.html. Retrieved 2010-08-22.

- ^ "Security of virtualization, cloud computing divides IT and security pros". Networkworld.com. 2010-02-22. http://www.networkworld.com/news/2010/022210-virtualization-cloud-security-debate.html. Retrieved 2010-08-22.

- ^ "Cloud Security Alliance Official web page". Cloudsecurityalliance.org. http://www.cloudsecurityalliance.org/. Retrieved 2010-08-22.

- ^ Mullins, Robert (2010-06-16). "IDC Survey: Risk In The Cloud". Network Computing. http://www.networkcomputing.com/cloud-computing/cloud-minuses-outweigh-pluses-for-businesses.php. Retrieved 2010-08-22.

- ^ Scheier, Robert L. (2009-04-20). "What to do if your cloud provider disappears | Cloud Computing". InfoWorld. http://www.infoworld.com/d/cloud-computing/what-do-if-your-cloud-provider-disappears-508. Retrieved 2010-08-22.

- ^ James Urquhart (January 7, 2010). "Cloud computing's green paradox". CNET News. http://news.cnet.com/8301-19413_3-10428065-240.html. Retrieved March 12, 2010. "...there is some significant evidence that the cloud is encouraging more compute consumption"

- ^ Finland - First Choice for Siting Your Cloud Computing Data Center.. Retrieved 4 August 2010.

- ^ Swiss Carbon-Neutral Servers Hit the Cloud.. Retrieved 4 August 2010.

- ^ "Cloud Net Directory. Retrieved 2010-03-01". Cloudbook.net. http://www.cloudbook.net/directories/research-clouds. Retrieved 2010-08-22.

- ^ nsf.gov - National Science Foundation (NSF) News - National Science Foundation Awards Millions to Fourteen Universities for Cloud Computing Research - US National Science Foun...

- ^ ""IBM, Google Team on an Enterprise Cloud." May 2008. Rich Miller Retrieved 2010-04-01". DataCenterKnowledge.com. 2008-05-02. http://www.datacenterknowledge.com/archives/2008/05/02/ibm-google-team-on-an-enterprise-cloud/. Retrieved 2010-08-22.

- ^ "National Science Foundation press release. September 2008. "National Science Foundation Awards Millions to Fourteen Universities for Cloud Computing Research." Retrieved 2010-03-01". Nsf.gov. http://www.nsf.gov/news/news_summ.jsp?cntn_id=114686. Retrieved 2010-08-22.

- ^ "HP News Release. "HP, Intel and Yahoo! Create Global Cloud Computing Research Test Bed." July 2008". Hp.com. 2008-07-29. http://www.hp.com/hpinfo/newsroom/press_kits/2008/cloudresearch/index.html?jumpid=reg_R1002_USEN. Retrieved 2010-08-22.

- ^ "HP News Release. "HP, Intel and Yahoo! Attract Leading Research Organizations to Collaborative Cloud Computing Test Bed. June 2009". Hp.com. http://www.hp.com/hpinfo/newsroom/press/2009/090608a.html?jumpid=reg_R1002_USEN. Retrieved 2010-08-22.

- ^ "Open Cirrus: the Open Cloud Computing Research Testbed."

- ^ IDG News Service. “More join HP, Intel and Yahoo on Open Cirrus cloud test bed.” Perez. Sept. 2010

- ^ Open Cirrus Video

- ^ "Information Week: “HP Labs India offers SiteonMobile for emerging markets.” Ribeiro, IDG News Service\Bangalore Bureau July 2010". Informationweek.in. 2010-07-09. http://www.informationweek.in/Mobile/10-07-09/HP_Labs_India_offers_SiteonMobile_for_emerging_markets.aspx. Retrieved 2010-08-22.

- ^ "IDG: San Francisco Chronicle: “HP Labs uses cloud technology to simplify mobile web access.” Ribeiro. July 2010". Sfgate.com. http://www.sfgate.com/cgi-bin/article.cgi?f=/g/a/2010/07/08/urnidgns852573C4006938800025775A001A2D85.DTL#ixzz0tB7X8Iti. Retrieved 2010-08-22. [dead link]

- ^ "IEEE Technical Committee on Services Computing". Tab.computer.org. http://tab.computer.org/tcsc. Retrieved 2010-08-22.

- ^ "IEEE International Conference on Cloud Computing (CLOUD)". Thecloudcomputing.org. http://www.thecloudcomputing.org. Retrieved 2010-08-22.

- ^ “” (2008-09-29). "Cloud Computing is Hyped and Overblown". Youtube.com. http://www.youtube.com/watch?v=f7wv1i8ubng. Retrieved 2010-08-22.

- ^ "Cloud Computing is Hyped and Overblown, Forrester's Frank Gillett Big Tech Companies Have "Cloud Envy"". Beet.tv. http://www.beet.tv/2008/09/cloud-computing.html. Retrieved 2010-08-22.

- ^ “”. "Larry Ellison – What The Hell Is Cloud Computing?". Youtube.com. http://www.youtube.com/watch?v=0FacYAI6DY0. Retrieved 2010-08-22.

- ^ Farber, Dan (2008-09-26). "Oracle's Ellison nails cloud computing". News.cnet.com. http://news.cnet.com/8301-13953_3-10052188-80.html. Retrieved 2010-08-22.

- ^ "Oracle desperately seeking cloud cred". Searchcloudcomputing.techtarget.com. 2010-02-16. http://searchcloudcomputing.techtarget.com/news/article/0,289142,sid201_gci1384202,00.html. Retrieved 2010-08-22.

- ^ "Oracle launches worldwide cloud-computing tour". Computerworld.com.au. 2010-02-10. http://www.computerworld.com.au/article/335674/oracle_launches_worldwide_cloud-computing_tour/. Retrieved 2010-08-22.

- ^ By James Rogers (2009-12-18). "Oracle's Ellison Issues Cloud Challenge". Thestreet.com. http://www.thestreet.com/story/10649557/1/oracles-ellison-issues-cloud-challenge.html. Retrieved 2010-08-22.

From Wikipedia, the free encyclopedia

Jump to: navigation, search

This article is about computer networking, the discipline of engineering computer networks. For the article on computer networks, see Computer network.

"Datacom" redirects here. For other uses, see Datacom (disambiguation).

Network cards such as this one can transmit and receive data at high rates over various types of network cables. This card is a 'Combo' card which supports three cabling standards.

Computer networking or Data communications (Datacom) is the engineering discipline concerned with the communication between computer systems or devices. A computer network is any set of computers or devices connected to each other with the ability to exchange data.[1] Computer networking is sometimes considered a sub-discipline of telecommunications, computer science, information technology and/or computer engineering since it relies heavily upon the theoretical and practical application of these scientific and engineering disciplines. The three types of networks are: the Internet, the intranet, and the extranet. Examples of different network methods are:

- Local area network (LAN), which is usually a small network constrained to a small geographic area. An example of a LAN would be a computer network within a building.

- Metropolitan area network (MAN), which is used for medium size area. examples for a city or a state.

- Wide area network (WAN) that is usually a larger network that covers a large geographic area.

- Wireless LANs and WANs (WLAN & WWAN) are the wireless equivalent of the LAN and WAN.

All networks are interconnected to allow communication with a variety of different kinds of media, including twisted-pair copper wire cable, coaxial cable, optical fiber, power lines and various wireless technologies.[2] The devices can be separated by a few meters (e.g. via Bluetooth) or nearly unlimited distances (e.g. via the interconnections of the Internet[3]). Networking, routers, routing protocols, and networking over the public Internet have their specifications defined in documents called RFCs.[4]

- Contents [hide] - 1 Views of networks - 2 History of computer networks - 3 Networking methods - 3.1 Local area network (LAN) - 3.2 Wide area network (WAN) - 3.3 Wireless networks (WLAN, WWAN) - 4 Network topology - 5 See also - 6 References - 7 External links

- [edit] Views of networks

Users and network administrators typically have different views of their networks. Users can share printers and some servers from a workgroup, which usually means they are in the same geographic location and are on the same LAN. A [community-of-interest network community of interest]has less of a connection of being in a local area, and should be thought of as a set of arbitrarily located users who share a set of servers , and possibly also communicate via peer-to-peer technologies.

Network administrators can see networks from both physical and logical perspectives. The physical perspective involves geographic locations, physical cabling, and the network elements (e.g., routers, bridges and application layer gateways that interconnect the physical media. Logical networks, called, in the TCP/IP architecture, subnets, map onto one or more physical media. For example, a common practice in a campus of buildings is to make a set of LAN cables in each building appear to be a common subnet, using virtual LAN (VLAN) technology.

Both users and administrators will be aware, to varying extents, of the trust and scope characteristics of a network. Again using TCP/IP architectural terminology, an intranet is a community of interest under private administration usually by an enterprise, and is only accessible by authorized users (e.g. employees).[5] Intranets do not have to be connected to the Internet, but generally have a limited connection. An extranet is an extension of an intranet that allows secure communications to users outside of the intranet (e.g. business partners, customers).[5]

Unofficially, the Internet is the set of users, enterprises, and content providers that are interconnected by Internet Service Providers (ISP). From an engineering viewpoint, the Internet is the set of subnets, and aggregates of subnets, which share the registered IP address space and exchange information about the reachability of those IP addresses using the Border Gateway Protocol. Typically, the human-readable names of servers are translated to IP addresses, transparently to users, via the directory function of the Domain Name System (DNS).

Over the Internet, there can be business-to-business (B2B), business-to-consumer (B2C) and consumer-to-consumer (C2C) communications. Especially when money or sensitive information is exchanged, the communications are apt to be secured by some form of communications security mechanism. Intranets and extranets can be securely superimposed onto the Internet, without any access by general Internet users, using secure Virtual Private Network (VPN) technology.

When used for gaming one computer will need to be the server while the others play through it.

- [edit] History of computer networks

Before the advent of computer networks that were based upon some type of telecommunications system, communication between calculation machines and early computers was performed by human users by carrying instructions between them. Many of the social behaviors seen in today's Internet were demonstrably present in the nineteenth century and arguably in even earlier networks using visual signals.

In September 1940 George Stibitz used a teletype machine to send instructions for a problem set from his Model at Dartmouth College in New Hampshire to his Complex Number Calculator in New York and received results back by the same means. Linking output systems like teletypes to computers was an interest at the Advanced Research Projects Agency (ARPA) when, in 1962, J.C.R. Licklider was hired and developed a working group he called the "Intergalactic Network"Bold text, a precursor to the ARPANet.

In 1964, researchers at Dartmouth developed the Dartmouth Time Sharing System for distributed users of large computer systems. The same year, at MIT, a research group supported by General Electric and Bell Labs used a computer DEC's to route and manage telephone connections.

Throughout the 1960s Leonard Kleinrock, Paul Baran and Donald Davies independently conceptualized and developed network systems which used datagrams or packets that could be used in a network between computer systems.

1965 Thomas Merrill and Lawrence G. Roberts created the first wide area network (WAN).

The first widely used PSTN switch that used true computer control was the Western Electric introduced in 1965.

In 1969 the University of California at Los Angeles, SRI (in Stanford), University of California at Santa Barbara, and the University of Utah were connected as the beginning of the ARPANET network using 50 kbit/s circuits. Commercial services using X.25 were deployed in 1972, and later used as an underlying infrastructure for expanding TCP/IP networks.

Computer networks, and the technologies needed to connect and communicate through and between them, continue to drive computer hardware, software, and peripherals industries. This expansion is mirrored by growth in the numbers and types of users of networks from the researcher to the home user.

Today, computer networks are the core of modern communication. All modern aspects of the Public Switched Telephone Network (PSTN) are computer-controlled, and telephony increasingly runs over the Internet Protocol, although not necessarily the public Internet. The scope of communication has increased significantly in the past decade, and this boom in communications would not have been possible without the progressively advancing computer network.

- [edit] Networking methods

One way to categorize computer networks is by their geographic scope, although many real-world networks interconnect Local Area Networks (LAN) via Wide Area Networks (WAN) and wireless wide area networks (WWAN). These three (broad) types are:

- [edit] Local area network (LAN)

A local area network is a network that spans a relatively small space and provides services to a small number of people.

A peer-to-peer or client-server method of networking may be used. A peer-to-peer network is where each client shares their resources with other workstations in the network. Examples of peer-to-peer networks are: Small office networks where resource use is minimal and a home network. A client-server network is where every client is connected to the server and each other. Client-server networks use servers in different capacities. These can be classified into two types:

1. Single-service servers

2. Print servers

The server performs one task such as file server, while other servers can not only perform in the capacity of file servers and print servers, but also can conduct calculations and use them to provide information to clients (Web/Intranet Server). Computers may be connected in many different ways, including Ethernet cables, Wireless networks, or other types of wires such as power lines or phone lines.

The ITU-T G.hn standard is an example of a technology that provides high-speed (up to 1 Gbit/s) local area networking over existing home wiring (power lines, phone lines and coaxial cables).

- [edit] Wide area network (WAN)

A wide area network is a network where a wide variety of resources are deployed across a large domestic area or internationally. An example of this is a multinational business that uses a WAN to interconnect their offices in different countries. The largest and best example of a WAN is the Internet, which is a network composed of many smaller networks. The Internet is considered the largest network in the world.[6]. The PSTN (Public Switched Telephone Network) also is an extremely large network that is converging to use Internet technologies, although not necessarily through the public Internet.

A Wide Area Network involves communication through the use of a wide range of different technologies. These technologies include Point-to-Point WANs such as Point-to-Point Protocol (PPP) and High-Level Data Link Control (HDLC), Frame Relay, ATM (Asynchronous Transfer Mode) and Sonet (Synchronous Optical Network). The difference between the WAN technologies is based on the switching capabilities they perform and the speed at which sending and receiving bits of information (data) occur.

- [edit] Wireless networks (WLAN, WWAN)

A wireless network is basically the same as a LAN or a WAN but there are no wires between hosts and servers. The data is transferred over sets of radio transceivers. These types of networks are beneficial when it is too costly or inconvenient to run the necessary cables. For more information, see Wireless LAN and Wireless wide area network. The media access protocols for LANs come from the IEEE.

The most common IEEE 802.11 WLANs cover, depending on antennas, ranges from hundreds of meters to a few kilometers. For larger areas, either communications satellites of various types, cellular radio, or wireless local loop (IEEE 802.16) all have advantages and disadvantages. Depending on the type of mobility needed, the relevant standards may come from the IETF or the ITU.

- [edit] Network topology

The network topology defines the way in which computers, printers, and other devices are connected, physically and logically. A network topology describes the layout of the wire and devices as well as the paths used by data transmissions.

Network topology has two types:

- Physical

- Logical

Commonly used topologies include:

- Bus

- Star

- Tree (hierarchical)

- Linear

- Ring

- Mesh

  - partially connected

  - fully connected (sometimes known as fully redundant)

The network topologies mentioned above are only a general representation of the kinds of topologies used in computer network and are considered basic topologies

- [edit] See also

|  | Book:Computer Networking |
| --- | --- |
| Books are collections of articles that can be downloaded or ordered in print. |  |

- Data transmission

- Digital communications

- Communication network

- Network architecture

- Data interventions

- [edit] References

- ^ http://www.atis.org/tg2k/_computer_network.html Computer network definition

- ^ http://www.bellevuelinux.org/network.html Computer networks defined.

- ^ Interplanetary Internet, 2000 Third Annual International Symposium on Advanced Radio Technologies, A. Hooke,September 2000

- ^ The Internet Standards Process -- Revision 3, RFC 2026, rushawn o wright, October 1996.

- ^ a b RFC 2547

- ^ http://www.pcmag.com/encyclopedia_term/0,2542,t=internet&j=54184,00.asp "internet" defined

- Andrew S. Tanenbaum, Computer Networks (ISBN 0-13-349945-6).

- Important publications in computer networks

- Vinton G. Cerf "Software: Global Infrastructure for the 21st Century"

- Meyers, Mike, "Mike Meyers' Certifcation Passport: Network+" ISBN 0072253487"

- Odom, Wendall, "CCNA Certification Guide"

- Network Communication Architecture and Protocols: OSI Network Architecture 7 Layers Model.

- [edit] External links

- Easy Network Concepts (Linux kernel specific)

- Computer Networks and Protocol (Research document, 2006)

- Computer Networking Glossary

- Networking at the Open Directory Project

Retrieved from "http://en.wikipedia.org/wiki/Computer_networking"