From Wikipedia, the free encyclopedia

Jump to: navigation, search

|  | This article includes a list of references, but its sources remain unclear because it has insufficient inline citations. Please help to improve this article by introducing more precise citations where appropriate. (July 2008) |
| --- | --- |

Picture of the internals of an ADSL modem/router. A modern example of an embedded system. Labelled parts include a microprocessor (4), RAM (6), and flash memory (7).

An embedded system is a computer system designed to perform one or a few dedicated functions[1][2] often with real-time computing constraints. It is embedded as part of a complete device often including hardware and mechanical parts. By contrast, a general-purpose computer, such as a personal computer (PC), is designed to be flexible and to meet a wide range of end-user needs. Embedded systems control many devices in common use today.[3]

Embedded systems are controlled by one or more main processing cores that are typically either microcontrollers or digital signal processors (DSP).[4] The key characteristic, however, is being dedicated to handle a particular task, which may require very powerful processors. For example, air traffic control systems may usefully be viewed as embedded, even though they involve mainframe computers and dedicated regional and national networks between airports and radar sites (each radar probably includes one or more embedded systems of its own).

Since the embedded system is dedicated to specific tasks, design engineers can optimize it to reduce the size and cost of the product and increase the reliability and performance. Some embedded systems are mass-produced, benefiting from economies of scale.

Physically, embedded systems range from portable devices such as digital watches and MP3 players, to large stationary installations like traffic lights, factory controllers, or the systems controlling nuclear power plants. Complexity varies from low, with a single microcontroller chip, to very high with multiple units, peripherals and networks mounted inside a large chassis or enclosure.

In general, "embedded system" is not a strictly definable term, as most systems have some element of extensibility or programmability. For example, handheld computers share some elements with embedded systems such as the operating systems and microprocessors which power them, but they allow different applications to be loaded and peripherals to be connected. Moreover, even systems which don't expose programmability as a primary feature generally need to support software updates. On a continuum from "general purpose" to "embedded", large application systems will have subcomponents at most points even if the system as a whole is "designed to perform one or a few dedicated functions", and is thus appropriate to call "embedded".

- Contents [hide] - 1 Variety of embedded systems - 2 History - 3 Characteristics - 3.1 User interface - 3.2 Processors in embedded systems - 3.2.1 Ready made computer boards - 3.2.2 ASIC and FPGA solutions - 3.3 Peripherals - 3.4 Tools - 3.5 Debugging - 3.6 Reliability - 3.7 High vs low volume - 4 Embedded software architectures - 4.1 Simple control loop - 4.2 Interrupt controlled system - 4.3 Cooperative multitasking - 4.4 Preemptive multitasking or multi-threading - 4.5 Microkernels and exokernels - 4.6 Monolithic kernels - 4.7 Exotic custom operating systems - 4.8 Additional software components - 5 See also - 6 References - 7 External links

- [edit] Variety of embedded systems

PC Engines' ALIX.1C Mini-ITX embedded board with an x86 AMD Geode LX 800 together with Compact Flash, miniPCI and PCI slots, 44-pin IDE interface, audio, USB and 256MB RAM

An embedded RouterBoard 112 with U.FL-RSMA pigtail and R52 miniPCI Wi-Fi card widely used by wireless Internet service providers (WISPs) in the Czech Republic.

Embedded systems span all aspects of modern life and there are many examples of their use.

Telecommunications systems employ numerous embedded systems from telephone switches for the network to mobile phones at the end-user. Computer networking uses dedicated routers and network bridges to route data.

Consumer electronics include personal digital assistants (PDAs), mp3 players, mobile phones, videogame consoles, digital cameras, DVD players, GPS receivers, and printers. Many household appliances, such as microwave ovens, washing machines and dishwashers, are including embedded systems to provide flexibility, efficiency and features. Advanced HVAC systems use networked thermostats to more accurately and efficiently control temperature that can change by time of day and season. Home automation uses wired- and wireless-networking that can be used to control lights, climate, security, audio/visual, surveillance, etc., all of which use embedded devices for sensing and controlling.

Transportation systems from flight to automobiles increasingly use embedded systems. New airplanes contain advanced avionics such as inertial guidance systems and GPS receivers that also have considerable safety requirements. Various electric motors — brushless DC motors, induction motors and DC motors — are using electric/electronic motor controllers. Automobiles, electric vehicles, and hybrid vehicles are increasingly using embedded systems to maximize efficiency and reduce pollution. Other automotive safety systems include anti-lock braking system (ABS), Electronic Stability Control (ESC/ESP), traction control (TCS) and automatic four-wheel drive.

Medical equipment is continuing to advance with more embedded systems for vital signs monitoring, electronic stethoscopes for amplifying sounds, and various medical imaging (PET, SPECT, CT, MRI) for non-invasive internal inspections.

In addition to commonly described embedded systems based on small computers, a new class of miniature wireless devices called motes are quickly gaining popularity as the field of wireless sensor networking rises. Wireless sensor networking, WSN, makes use of miniaturization made possible by advanced IC design to couple full wireless subsystems to sophisticated sensors, enabling people and companies to measure a myriad of things in the physical world and act on this information through IT monitoring and control systems. These motes are completely self contained, and will typically run off a battery source for many years before the batteries need to be changed or charged.

- [edit] History

In the earliest years of computers in the 1940–50s, computers were sometimes dedicated to a single task, but were far too large and expensive for most kinds of tasks performed by embedded computers of today. Over time however, the concept of programmable controllers evolved from traditional electromechanical sequencers, via solid state devices, to the use of computer technology.

One of the first recognizably modern embedded systems was the Apollo Guidance Computer, developed by Charles Stark Draper at the MIT Instrumentation Laboratory. At the project's inception, the Apollo guidance computer was considered the riskiest item in the Apollo project as it employed the then newly developed monolithic integrated circuits to reduce the size and weight. An early mass-produced embedded system was the Autonetics D-17 guidance computer for the Minuteman missile, released in 1961. It was built from transistor logic and had a hard disk for main memory. When the Minuteman II went into production in 1966, the D-17 was replaced with a new computer that was the first high-volume use of integrated circuits. This program alone reduced prices on quad nand gate ICs from $1000/each to $3/each[citation needed], permitting their use in commercial products.

Since these early applications in the 1960s, embedded systems have come down in price and there has been a dramatic rise in processing power and functionality. The first microprocessor for example, the Intel 4004, was designed for calculators and other small systems but still required many external memory and support chips. In 1978 National Engineering Manufacturers Association released a "standard" for programmable microcontrollers, including almost any computer-based controllers, such as single board computers, numerical, and event-based controllers.

As the cost of microprocessors and microcontrollers fell it became feasible to replace expensive knob-based analog components such as potentiometers and variable capacitors with up/down buttons or knobs read out by a microprocessor even in some consumer products. By the mid-1980s, most of the common previously external system components had been integrated into the same chip as the processor and this modern form of the microcontroller allowed an even more widespread use, which by the end of the decade were the norm rather than the exception for almost all electronics devices.

The integration of microcontrollers has further increased the applications for which embedded systems are used into areas where traditionally a computer would not have been considered. A general purpose and comparatively low-cost microcontroller may often be programmed to fulfill the same role as a large number of separate components. Although in this context an embedded system is usually more complex than a traditional solution, most of the complexity is contained within the microcontroller itself. Very few additional components may be needed and most of the design effort is in the software. The intangible nature of software makes it much easier to prototype and test new revisions compared with the design and construction of a new circuit not using an embedded processor.

- [edit] Characteristics

Gumstix Overo COM, a tiny, OMAP-based embedded computer-on-module with Wifi and Bluetooth.

- Embedded systems are designed to do some specific task, rather than be a general-purpose computer for multiple tasks. Some also have real-time performance constraints that must be met, for reasons such as safety and usability; others may have low or no performance requirements, allowing the system hardware to be simplified to reduce costs.

- Embedded systems are not always standalone devices. Many embedded systems consist of small, computerized parts within a larger device that serves a more general purpose. For example, the Gibson Robot Guitar features an embedded system for tuning the strings, but the overall purpose of the Robot Guitar is, of course, to play music.[5] Similarly, an embedded system in an automobile provides a specific function as a subsystem of the car itself.

- The program instructions written for embedded systems are referred to as firmware, and are stored in read-only memory or Flash memory chips. They run with limited computer hardware resources: little memory, small or non-existent keyboard and/or screen.

- [edit] User interface

Embedded system text user interface using MicroVGA

Embedded systems range from no user interface at all — dedicated only to one task — to complex graphical user interfaces that resemble modern computer desktop operating systems. Simple embedded devices use buttons, LEDs, graphic or character LCDs (for example popular HD44780 LCD) with a simple menu system.

A more sophisticated devices use graphical screen with touch sensing or screen-edge buttons provide flexibility while minimizing space used: the meaning of the buttons can change with the screen, and selection involves the natural behavior of pointing at what's desired. Handheld systems often have a screen with a "joystick button" for a pointing device.

Some systems provide user interface remotely with the help of a serial (e.g. RS-232, USB, I²C, etc.) or network (e.g. Ethernet) connection. In spite of the potentially necessary proprietary client software and/or specialist cables that are needed, this approach usually gives a lot of advantages: extends the capabilities of embedded system, avoids the cost of a display, simplifies BSP, allows to build rich user interface on the PC. A good example of this is the combination of an embedded web server running on an embedded device (such as an IP camera or a network routers. The user interface is displayed in a web browser on a PC connected to the device, therefore needing no bespoke software to be installed.

- [edit] Processors in embedded systems

Embedded processors can be broken into two broad categories: ordinary microprocessors (μP) and microcontrollers (μC), which have many more peripherals on chip, reducing cost and size. Contrasting to the personal computer and server markets, a fairly large number of basic CPU architectures are used; there are Von Neumann as well as various degrees of Harvard architectures, RISC as well as non-RISC and VLIW; word lengths vary from 4-bit to 64-bits and beyond (mainly in DSP processors) although the most typical remain 8/16-bit. Most architectures come in a large number of different variants and shapes, many of which are also manufactured by several different companies.

A long but still not exhaustive list of common architectures are: 65816, 65C02, 68HC08, 68HC11, 68k, 8051, ARM, AVR, AVR32, Blackfin, C167, Coldfire, COP8, Cortus APS3, eZ8, eZ80, FR-V, H8, HT48, M16C, M32C, MIPS, MSP430, PIC, PowerPC, R8C, SHARC, SPARC, ST6, SuperH, TLCS-47, TLCS-870, TLCS-900, Tricore, V850, x86, XE8000, Z80, AsAP etc.

- [edit] Ready made computer boards

PC/104 and PC/104+ are examples of standards for ready made computer boards intended for small, low-volume embedded and ruggedized systems, mostly x86-based. These often use DOS, Linux, NetBSD, or an embedded real-time operating system such as MicroC/OS-II, QNX or VxWorks. Sometimes these boards use non-x86 processors.

In certain applications, where small size or power efficiency are not primary concerns, the components used may be compatible with those used in general purpose x86 personal computers. Boards such as the VIA EPIA range help to bridge the gap by being PC-compatible but highly integrated, physically smaller or have other attributes making them attractive to embedded engineers. The advantage of this approach is that low-cost commodity components may be used along with the same software development tools used for general software development. Systems built in this way are still regarded as embedded since they are integrated into larger devices and fulfill a single role. Examples of devices that may adopt this approach are ATMs and arcade machines, which contain code specific to the application.

However, most ready-made embedded systems boards are not PC-centered and do not use the ISA or PCI busses. When a System-on-a-chip processor is involved, there may be little benefit to having a standarized bus connecting discrete compontents, and the environment for both hardware and software tools may be very different.

One common design style uses a small system module, perhaps the size of a business card, holding high density BGA chips such as an ARM-based System-on-a-chip processor and peripherals, external flash memory for storage, and DRAM for runtime memory. The module vendor will usually provide boot software and make sure there is a selection of operating systems, usually including Linux and some real time choices. These modules can be manufactured in high volume, by organizations familiar with their specialized testing issues, and combined with much lower volume custom mainboards with application-specific external peripherals. Gumstix product lines are a Linux-centric example of this model.

- [edit] ASIC and FPGA solutions

A common configuration for very-high-volume embedded systems is the system on a chip (SoC) which contains a complete system consisting of multiple processors, multipliers, caches and interfaces on a single chip. SoCs can be implemented as an application-specific integrated circuit (ASIC) or using a field-programmable gate array (FPGA).

- [edit] Peripherals

Embedded Systems talk with the outside world via peripherals, such as:

- Serial Communication Interfaces (SCI): RS-232, RS-422, RS-485 etc.

- Synchronous Serial Communication Interface: I2C, SPI, SSC and ESSI (Enhanced Synchronous Serial Interface)

- Universal Serial Bus (USB)

- Multi Media Cards (SD Cards, Compact Flash etc.)

- Networks: Ethernet, Controller Area Network, LonWorks, etc.

- Timers: PLL(s), Capture/Compare and Time Processing Units

- Discrete IO: aka General Purpose Input/Output (GPIO)

- Analog to Digital/Digital to Analog (ADC/DAC)

- Debugging: JTAG, ISP, ICSP, BDM Port, BITP, and DP9 ports.

- [edit] Tools

As for other software, embedded system designers use compilers, assemblers, and debuggers to develop embedded system software. However, they may also use some more specific tools:

- In circuit debuggers or emulators (see next section).

- Utilities to add a checksum or CRC to a program, so the embedded system can check if the program is valid.

- For systems using digital signal processing, developers may use a math workbench such as Scilab / Scicos, MATLAB / Simulink, EICASLAB, MathCad, Mathematica,or FlowStone DSP to simulate the mathematics. They might also use libraries for both the host and target which eliminates developing DSP routines as done in DSPnano RTOS and Unison Operating System.

- Custom compilers and linkers may be used to improve optimisation for the particular hardware.

- An embedded system may have its own special language or design tool, or add enhancements to an existing language such as Forth or Basic.

- Another alternative is to add a real-time operating system or embedded operating system, which may have DSP capabilities like DSPnano RTOS.

Software tools can come from several sources:

- Software companies that specialize in the embedded market

- Ported from the GNU software development tools

- Sometimes, development tools for a personal computer can be used if the embedded processor is a close relative to a common PC processor

As the complexity of embedded systems grows, higher level tools and operating systems are migrating into machinery where it makes sense. For example, cellphones, personal digital assistants and other consumer computers often need significant software that is purchased or provided by a person other than the manufacturer of the electronics. In these systems, an open programming environment such as Linux, NetBSD, OSGi or Embedded Java is required so that the third-party software provider can sell to a large market.

- [edit] Debugging

Embedded debugging may be performed at different levels, depending on the facilities available. From simplest to most sophisticated they can be roughly grouped into the following areas:

- Interactive resident debugging, using the simple shell provided by the embedded operating system (e.g. Forth and Basic)

- External debugging using logging or serial port output to trace operation using either a monitor in flash or using a debug server like the Remedy Debugger which even works for heterogeneous multicore systems.

- An in-circuit debugger (ICD), a hardware device that connects to the microprocessor via a JTAG or Nexus interface. This allows the operation of the microprocessor to be controlled externally, but is typically restricted to specific debugging capabilities in the processor.

- An in-circuit emulator replaces the microprocessor with a simulated equivalent, providing full control over all aspects of the microprocessor.

- A complete emulator provides a simulation of all aspects of the hardware, allowing all of it to be controlled and modified, and allowing debugging on a normal PC.

Unless restricted to external debugging, the programmer can typically load and run software through the tools, view the code running in the processor, and start or stop its operation. The view of the code may be as assembly code or source-code.

Because an embedded system is often composed of a wide variety of elements, the debugging strategy may vary. For instance, debugging a software- (and microprocessor-) centric embedded system is different from debugging an embedded system where most of the processing is performed by peripherals (DSP, FPGA, co-processor). An increasing number of embedded systems today use more than one single processor core. A common problem with multi-core development is the proper synchronization of software execution. In such a case, the embedded system design may wish to check the data traffic on the busses between the processor cores, which requires very low-level debugging, at signal/bus level, with a logic analyzer, for instance.

- [edit] Reliability

Embedded systems often reside in machines that are expected to run continuously for years without errors, and in some cases recover by themselves if an error occurs. Therefore the software is usually developed and tested more carefully than that for personal computers, and unreliable mechanical moving parts such as disk drives, switches or buttons are avoided.

Specific reliability issues may include:

- The system cannot safely be shut down for repair, or it is too inaccessible to repair. Examples include space systems, undersea cables, navigational beacons, bore-hole systems, and automobiles.

- The system must be kept running for safety reasons. "Limp modes" are less tolerable. Often backups are selected by an operator. Examples include aircraft navigation, reactor control systems, safety-critical chemical factory controls, train signals, engines on single-engine aircraft.

- The system will lose large amounts of money when shut down: Telephone switches, factory controls, bridge and elevator controls, funds transfer and market making, automated sales and service.

A variety of techniques are used, sometimes in combination, to recover from errors—both software bugs such as memory leaks, and also soft errors in the hardware:

- watchdog timer that resets the computer unless the software periodically notifies the watchdog

- subsystems with redundant spares that can be switched over to

- software "limp modes" that provide partial function

- Designing with a Trusted Computing Base (TCB) architecture[6] ensures a highly secure & reliable system environment

- An Embedded Hypervisor is able to provide secure encapsulation for any subsystem component, so that a compromised software component cannot interfere with other subsystems, or privileged-level system software. This encapsulation keeps faults from propagating from one subsystem to another, improving reliability. This may also allow a subsystem to be automatically shut down and restarted on fault detection.

- Immunity Aware Programming

- [edit] High vs low volume

For high volume systems such as portable music players or mobile phones, minimizing cost is usually the primary design consideration. Engineers typically select hardware that is just “good enough” to implement the necessary functions.

For low-volume or prototype embedded systems, general purpose computers may be adapted by limiting the programs or by replacing the operating system with a real-time operating system.

- [edit] Embedded software architectures

There are several different types of software architecture in common use.

- [edit] Simple control loop

In this design, the software simply has a loop. The loop calls subroutines, each of which manages a part of the hardware or software.

- [edit] Interrupt controlled system

Some embedded systems are predominantly interrupt controlled. This means that tasks performed by the system are triggered by different kinds of events. An interrupt could be generated for example by a timer in a predefined frequency, or by a serial port controller receiving a byte.

These kinds of systems are used if event handlers need low latency and the event handlers are short and simple.

Usually these kinds of systems run a simple task in a main loop also, but this task is not very sensitive to unexpected delays.

Sometimes the interrupt handler will add longer tasks to a queue structure. Later, after the interrupt handler has finished, these tasks are executed by the main loop. This method brings the system close to a multitasking kernel with discrete processes.

- [edit] Cooperative multitasking

A nonpreemptive multitasking system is very similar to the simple control loop scheme, except that the loop is hidden in an API. The programmer defines a series of tasks, and each task gets its own environment to “run” in. When a task is idle, it calls an idle routine, usually called “pause”, “wait”, “yield”, “nop” (stands for no operation), etc.

The advantages and disadvantages are very similar to the control loop, except that adding new software is easier, by simply writing a new task, or adding to the queue-interpreter.

- [edit] Preemptive multitasking or multi-threading

In this type of system, a low-level piece of code switches between tasks or threads based on a timer (connected to an interrupt). This is the level at which the system is generally considered to have an "operating system" kernel. Depending on how much functionality is required, it introduces more or less of the complexities of managing multiple tasks running conceptually in parallel.

As any code can potentially damage the data of another task (except in larger systems using an MMU) programs must be carefully designed and tested, and access to shared data must be controlled by some synchronization strategy, such as message queues, semaphores or a non-blocking synchronization scheme.

Because of these complexities, it is common for organizations to use a real-time operating system (RTOS), allowing the application programmers to concentrate on device functionality rather than operating system services, at least for large systems; smaller systems often cannot afford the overhead associated with a generic real time system, due to limitations regarding memory size, performance, and/or battery life.The choice that a RTOS is required brings in its own issues however as the selection must be done prior to starting to the application development process. This timing forces developers to choose the embedded operating system for their device based upon current requirements and so restricts future options to a large extent[7].The restriction of future options becomes more of an issue as product life decreases. Additionally the level of complexity is continuously growing as devices are required to manage many variables such as serial, USB, TCP/IP, Bluetooth, Wireless LAN, trunk radio, multiple channels, data and voice, enhanced graphics, multiple states, multiple threads, numerous wait states and so on. These trends are leading to the uptake of embedded middleware in addition to a real time operating system.

- [edit] Microkernels and exokernels

A microkernel is a logical step up from a real-time OS. The usual arrangement is that the operating system kernel allocates memory and switches the CPU to different threads of execution. User mode processes implement major functions such as file systems, network interfaces, etc.

In general, microkernels succeed when the task switching and intertask communication is fast, and fail when they are slow.

Exokernels communicate efficiently by normal subroutine calls. The hardware, and all the software in the system are available to, and extensible by application programmers.

- [edit] Monolithic kernels

In this case, a relatively large kernel with sophisticated capabilities is adapted to suit an embedded environment. This gives programmers an environment similar to a desktop operating system like Linux or Microsoft Windows, and is therefore very productive for development; on the downside, it requires considerably more hardware resources, is often more expensive, and because of the complexity of these kernels can be less predictable and reliable.

Common examples of embedded monolithic kernels are Embedded Linux and Windows CE.

Despite the increased cost in hardware, this type of embedded system is increasing in popularity, especially on the more powerful embedded devices such as Wireless Routers and GPS Navigation Systems. Here are some of the reasons:

- Ports to common embedded chip sets are available.

- They permit re-use of publicly available code for Device Drivers, Web Servers, Firewalls, and other code.

- Development systems can start out with broad feature-sets, and then the distribution can be configured to exclude unneeded functionality, and save the expense of the memory that it would consume.

- Many engineers believe that running application code in user mode is more reliable, easier to debug and that therefore the development process is easier and the code more portable.

- Many embedded systems lack the tight real time requirements of a control system. Although a system such as Embedded Linux may be fast enough in order to respond to many other applications.

- Features requiring faster response than can be guaranteed can often be placed in hardware.

- Many RTOS systems have a per-unit cost. When used on a product that is or will become a commodity, that cost is significant.

- [edit] Exotic custom operating systems

A small fraction of embedded systems require safe, timely, reliable or efficient behavior unobtainable with the one of the above architectures. In this case an organization builds a system to suit. In some cases, the system may be partitioned into a "mechanism controller" using special techniques, and a "display controller" with a conventional operating system. A communication system passes data between the two.

- [edit] Additional software components

In addition to the core operating system, many embedded systems have additional upper-layer software components. These components consist of networking protocol stacks like CAN, TCP/IP, FTP, HTTP, and HTTPS, and also included storage capabilities like FAT and flash memory management systems. If the embedded devices has audio and video capabilities, then the appropriate drivers and codecs will be present in the system. In the case of the monolithic kernels, many of these software layers are included. In the RTOS category, the availability of the additional software components depends upon the commercial offering.

- [edit] See also

- Communications server

- Cyber-physical system

- DSP

- Electronic Control Unit

- Embedded Hypervisor

- Embedded operating systems

- Embedded software

- Firmware

- FPGA

- Information appliance

- Microprocessor

- Microcontroller

- Programming languages

- Real-time operating system

- Software engineering

- System on a chip

- System on module

- Ubiquitous computing

- [edit] References

- ^ Michael Barr. "Embedded Systems Glossary". Netrino Technical Library. http://www.netrino.com/Embedded-Systems/Glossary. Retrieved 2007-04-21.

- ^ Heath, Steve (2003). Embedded systems design. EDN series for design engineers (2 ed.). Newnes. p. 2. ISBN 9780750655460. http://books.google.com/books?id=BjNZXwH7HlkC&pg=PA2. "An embedded system is a microprocessor based system that is built to control a function or a range of functions."

- ^ Michael Barr; Anthony J. Massa (2006). "Introduction". Programming embedded systems: with C and GNU development tools. O'Reilly. pp. 1–2. ISBN 9780596009830. http://books.google.com/books?id=nPZaPJrw_L0C&pg=PA1.

- ^ Giovino, Bill. "Microcontroller.com - Embedded Systems supersite". http://www.microcontroller.com/.

- ^ Embedded.com - Under the Hood: Robot Guitar embeds autotuning By David Carey, TechOnline EE Times (04/22/08, 11:10:00 AM EDT)Embedded Systems Design - Embedded.com

- ^ Your System is secure? Prove it!, Gernot Heiser, December 2007, Vol. 2 No. 6 Page 35-38, ;login: The USENIX Magazine

- ^ "Working across Multiple Embedded Platforms". clarinox. http://www.clarinox.com/docs/whitepapers/Whitepaper_06_CrossPlatformDiscussion.pdf. Retrieved 2010-8-17.

- [edit] External links

- Electronic control unit

From Wikipedia, the free encyclopedia

(Redirected from Electronic Control Unit)

Jump to: navigation, search

An ECU from a Geo Storm.

In automotive electronics, electronic control unit (ECU) is a generic term for any embedded system that controls one or more of the electrical systems or subsystems in a motor vehicle.

Other terms for ECU include electronic control module (ECM), central control module (CCM), control unit, or control module. Taken together, these systems are sometimes referred to as the car's computer. (Technically there is no single computer but multiple ones.)

Some modern motor vehicles have up to 80 ECUs. Embedded software in ECUs continue to increase in line count, complexity, and sophistication[1]. Managing the increasing complexity and number of ECUs in a vehicle has become a key challenge for original equipment manufacturers (OEMs).

- [edit] Types of electronic control units

- Airbag Control Unit (ACU)

- Body Control Module controls door locks, electric windows, courtesy lights, etc.

- Convenience Control Unit (CCU)

- Door Control Unit

- Engine Control Unit (ECU)—not to be confused with electronic control unit, the generic term for all these devices

- Man Machine Interface (MMI)

- On-Board Diagnostics (OBD)

- Powertrain Control Module (PCM): Sometimes the functions of the Engine Control Unit and Transmission Control Unit are combined into a single unit called the Powertrain Control Module.

- Seat Control Unit

- Speed Control Unit

- Telephone Control Unit (TCU)

- Transmission Control Unit (TCU)

- [edit] References

- ^ Ebert, Christof; Jones, Capers (2009-04-01). "Embedded Software: Facts, Figures, and Future". IEEE Computer Society Press. http://portal.acm.org/citation.cfm?id=1550393.1550433. Retrieved 2010-09-15.

|  | This article about an automotive technology is a stub. You can help Wikipedia by expanding it. v • d • e |
| --- | --- |

|  | This article about an automotive part or component is a stub. You can help Wikipedia by |
| --- | --- |

- Embedded Hypervisor

From Wikipedia, the free encyclopedia

Jump to: navigation, search

Software virtualization has been a major topic in the enterprise space for quite some time, but has become an important technology for embedded systems only in the last few years. The application of the technology to the enterprise space has been well described elsewhere by companies such as Citrix, IBM & VMware. The situation is less well defined for developers of embedded systems. Most importantly, the use of virtualization and its implementation in the form of a hypervisor in embedded systems are very different from enterprise applications. An effective implementation of an Embedded Hypervisor must deal with a number of issues specific to such applications. These issues include the highly integrated nature of embedded systems, the requirement for isolated functional blocks within the system to communicate rapidly, the need for real-time/deterministic performance, the resource-constrained target environment and the wide range of security and reliability requirements.

- Contents [hide] - 1 Definitions - 2 Implementation - 3 Use Cases - 4 References

- [edit] Definitions

Hypervisor
A hypervisor provides a software virtualization environment in which other software, including operating systems, can run with the appearance of full access to the underlying system hardware, but in fact such access is under the complete control of the hypervisor. The software running in such a hypervisor managed environment is said to be executing within a virtual machine (VM). Multiple VMs may be managed simultaneously by a hypervisor. Hypervisors are generally classed as type 1 or type 2, depending on whether the hypervisor is running in supervisor mode or privileged mode on “bare metal” (type 1) or is itself hosted by an operating system (type 2).

Embedded Hypervisor
An embedded hypervisor is a type 1 hypervisor that supports the requirements of embedded systems development and is designed into the embedded device from the outset, rather than loaded subsequent to device deployment. The requirements for a hypervisor architecture suitable for embedded applications is quite distinct from hypervisors aimed at enterprise applications, which have a fundamentally different set of requirements. See references [1] & [2] for a more detailed discussion. These requirements are summarized below.

- A small, fast type 1 hypervisor with support for multiple VMs;

- Support for lightweight but secure encapsulation of medium-grain subsystem components that interact strongly;

- High-bandwidth, low-latency communication between system components, subject to a configurable, system-wide security policy;

- Minimal impact on system resources & real-time performance;

- Ability to implement a scheduling policy between VMs and provide support for real-time system components;

- [edit] Implementation

To meet the requirements typical of most embedded applications, an embedded hypervisor must provide multiple VMs, each of which is essentially indistinguishable from the underlying hardware to which the virtualised software needs access. This ensures that the embedded code that runs on the real machine will run on the virtual machine and vice versa. The size and efficiency of the implementation is also an issue for an embedded hypervisor, as it needs to maintain as closely as possible the native speed, real-time response and determinism and power efficiency of the underlying hardware platform.
Implementations for embedded systems applications have been most commonly based on the use of a microkernel or small separation kernel, with virtualization built-in as an integral capability. Examples of these approaches have been produced by companies such as OK Labs (microkernel) and LynuxWorks (small separation kernel). VirtualLogix appears to take the position that an approach based on a dedicated Virtual Machine Monitor (VMM) would be even smaller and more efficient. This issue is the subject of some ongoing debate; see references [3], [4] & [5]. However, the main point at issue is the same on all sides of the discussion – the speed and size of the implementation (for a given level of functionality) are of major importance. For example: " . . . hypervisors for embedded use must be real-time capable, as well as resource-miserly." [6]

The small code size generated by these approaches is of such great importance since system resources such as memory are typically highly constrained in embedded applications, compared to enterprise systems. Thus embedded hypervisors usually have a memory footprint from several tens to several hundred Kbytes, depending on the efficiency of the implementation and the level of functionality provided. An implementation requiring several Mbytes of memory (or more) is generally not acceptable.
The small size of the hypervisor also provides the basis for a minimal trusted computing base (TCB). This TCB is the only part of the entire system code which is allowed to run in privileged mode, thus enhancing the inherent security of the system. The amount of trusted user-mode code can be kept to an absolute minimum with this approach. This is in contrast to type 2 hypervisors, which are commonly designed to be used with a guest OS underneath any other software.
This approach also means that the TCB can be made highly secure & reliable [7]. Standard software-engineering techniques, such as code inspections and systematic testing, can be used to reduce the number of bugs in such a small code base to a tiny fraction of the defects that must be expected for a hypervisor and guest OS combination that may be 100,000–300,000 lines in total. [8]
One of the most important functions required in an embedded hypervisor is a secure message-passing mechanism, which is needed to support real-time communication between processes. In the embedded environment, a system will typically have a number of closely coupled tasks, some of which may require secure isolation from each other. In a virtualized environment, the embedded hypervisor will support and enforce this isolation between multiple VMs. These VMs will therefore require access to a mechanism that provides low-latency communication between the tasks.
An inter-process communication (IPC) mechanism can be used to provide these functions, as well as invoking all system services, and implemented in a manner which ensures that the desired level of VM isolation is maintained. Also, due to its significant impact on system performance, such an IPC mechanism should be highly optimised for minimal latency. [9]
Lastly, an embedded hypervisor needs to be in complete control of system resources, including memory accesses, to ensure that software cannot break out of the VM. So an embedded hypervisor requires the target CPU to provide memory management support (typically using an MMU). However, unlike the enterprise space, a number of processors aimed at embedded applications do not provide such support, so selection of a suitable target architecture able to support a Type 1 hypervisor may be an issue.

- [edit] Use Cases

Some of the most common use cases for an embedded hypervisor are:

1. OS Independence
Designers of embedded systems may have many hardware drivers and system services which are specific to a target platform. If support for more than one OS is required on the platform, either concurrently or consecutively using a common hardware design, an embedded hypervisor can greatly simplify the task. Such drivers and system services can be implemented just once for the virtualized environment; these services are then available to any hosted OS. This level of abstraction also allows the embedded developer to implement or change a driver or service in either hardware or software at any point, without this being apparent to the hosted OS.

2. Support for multiple operating systems on a single processor
Typically this is used to run a RTOS for low-level real-time functionality (such as the communication stack) while at the same time running a high-level OS, (HLOS) like Linux or Windows, to support user applications, such as a web browser or calendar. The objective might be to upgrade an existing design without the added complexity of a second processor, or simply to minimize the Bill of Materials (BoM).

3. System Security
An embedded hypervisor is able to provide secure encapsulation for any subsystem defined by the developer, so that a compromised subsystem cannot interfere with other subsystems. For example, an encryption subsystem needs to be strongly shielded from attack to prevent leaking the information the encryption is supposed to protect. As the embedded hypervisor can encapsulate a subsystem in a VM, it can then enforce the required security policies for communication to and from that subsystem.

4. System Reliability
The encapsulation of a subsystem components into a VM ensures that failure of any subsystem cannot impact other subsystems. This encapsulation keeps faults from propagating from a subsystem in one VM to a subsystem in another VM, improving reliability. This may also allow a subsystem to be automatically shut down and restarted on fault detection. This can be particularly important for embedded device drivers, as this is where the highest density of fault conditions is seen to occur, and is thus the most common cause of OS failure and system instability. It also allows the encapsulation of operating systems that were not necessarily built to the reliability standards demanded of the new system design.

5. Dynamic Update of System Software
Subsystem software or applications can be securely updated and tested for integrity, by downloading to a secure VM before “going live” in an executing system. Even if this process then fails, the system can revert to its former state by restarting the original software subsystem/application, without halting system operation.

6. Legacy Code re-use
Virtualization allows legacy embedded code to be used with the OS environment it has been developed and validated with, while freeing the developer to use a different OS environment in a separate VM for new services and applications. Legacy embedded code, written for a particular system configuration may assume exclusive control of all system resources of memory, I/O and processor. This code base can be re-used unchanged on alternative system configurations of I/O and memory through the use of a VM to present a resource map and functionality that is consistent with the original system configuration, effectively de-coupling the legacy code from the specifics of a new or modified hardware design.
Where access to the operating system source code is available, paravirtualization is commonly used to virtualize the OS’s on processors without hardware virtualization support, and thus the applications supported by the OS can also run unmodified and without re-compilation in new hardware platform designs.
Even without source access, legacy binary code can be executed in systems running on processors with hardware virtualization support such as the AMD-V or Intel VT technologies. The legacy binary code could run completely unmodified in a VM with all resource mapping handled by the embedded hypervisor, assuming the system hardware provides equivalent functionality.

7. IP Protection
Valuable proprietary IP may need protection from theft or misuse when an embedded platform is being shipped for further development work by (for example) an OEM customer. An embedded hypervisor makes it possible to restrict access by other system software components to a specific part of the system containing IP that needs to be protected.

8. Software Licence Segregation
Software IP operating under one licensing scheme can be separated from other software IP operating under a different scheme. For example, the embedded hypervisor can provide an isolated execution environment for proprietary software sharing the processor with open source software subject to the GPL [10].

9. Migration of applications from uni-core to multi-core systems
As new processors utilise multi-core architectures to increase performance, the embedded hypervisor can manage the underlying architecture and present a uni-processor environment to legacy applications and operating systems while efficiently using the new multiprocessor system design. In this way a change in hardware environment does not require a change to the existing software.

- [edit] References

- ^ [1] Role of Virtualization in Embedded Systems

- ^ [2] Virtualization and hypervisors aid embedded design

- ^ [3] Small kernels versus virtual-machine monitors

- ^ [4] Are Virtual Machine Monitors Microkernels Done Right?

- ^ [5] (Response to) Are Virtual-Machine Monitors Microkernels Done Right?

- ^ [6] Embedded hypervisor performance & benchmarking

- ^ [7] Is Your System Secure?

- ^ [8] Trustworthy Computing Systems

- ^ [9] Improving IPC by Kernel Design

- ^ [10] GPL FAQ

Retrieved from "http://en.wikipedia.org/wiki/Embedded_Hypervisor"

- Embedded operating system

From Wikipedia, the free encyclopedia

Jump to: navigation, search

An embedded operating system is an operating system for embedded computer systems. These operating systems are designed to be very compact and efficient, forsaking many functions that non-embedded computer operating systems provide, and which may not be used by the specialized applications they run. They are frequently also real-time operating systems.

Examples of embedded operating systems could include the software used in Automated Teller Machines, Cash Registers, CCTV systems, a TV box set, a GPS, jukeboxes, missiles, etc.

- [edit] See also

- Embedded software

From Wikipedia, the free encyclopedia

Jump to: navigation, search

Embedded software is computer software which plays an integral role in the electronics it is supplied with.

Embedded software's principal role is not Information technology[clarification needed], but rather the interaction with the physical world. It's written for machines that are not, first and foremost, computers. Embedded software is 'built in' to the electronics in cars, telephones, audio equipment, robots, appliances, toys, security systems, pacemakers, televisions and digital watches, for example. This software can become very sophisticated in applications like airplanes, missiles, process control systems, and so on.[1]

Embedded software is usually written for special purpose hardware: that is computer chips that are different from general purpose CPUs[dubious – discuss], sometimes using Real-time operating system such as LynxOS, VxWorks, Linux (with patched kernel), PikeOS, eCos, ThreadX, Windows CE, Fusion RTOS, Nucleus RTOS, RTEMS, Integrity and QNX.

Communications protocols designed for use in embedded systems are available as closed source from companies including InterNiche Technologies and CMX Systems, and are also published as open source from uIP, lwip and others.

- [edit] References

- Edward A. Lee, "Embedded Software", Advances in Computers (M. Zelkowitz, editor) 56, Academic Press, London, 2002.

- ^ http://ptolemy.eecs.berkeley.edu/publications/papers/02/embsoft/embsoftwre.pdf

- [edit] See also

- Firmware

From Wikipedia, the free encyclopedia

Jump to: navigation, search

A typical firmware-controlled device, a television remote control.

In electronics and computing, firmware is a term often used to denote the fixed, usually rather small, programs and/or data structures that internally control various electronic devices. Typical examples of devices containing firmware range from end-user products such as remote controls or calculators, through computer parts and devices like hard disks, keyboards, TFT screens or memory cards, all the way to scientific instrumentation and industrial robotics. Also more complex consumer devices, such as mobile phones, digital cameras, synthesizers, etc., contain firmware to enable the device's basic operation as well as implementing higher-level functions.

There are no strict boundaries between firmware and software, as both are quite loose descriptive terms. However, the term firmware was originally coined in order to contrast to higher level software which could be changed without replacing a hardware component, and firmware is typically involved with very basic low-level operations without which a device would be completely non-functional. Firmware is also a relative term, as most embedded devices contain firmware at more than one level. Subsystems such as CPUs, flash chips, communication controllers, LCD modules, and so on, have their own (usually fixed) program code and/or microcode, regarded as "part of the hardware" by the higher-level(s) firmware.

Low-level firmware typically resides in a PLA structure or in a ROM (or OTP/PROM), while higher level firmware (often on the border to software) typically employs flash memory to allow for updates, at least in modern devices. (Common reasons for updating firmware include fixing bugs or adding features to the device. Doing so usually involves loading a binary image file provided by the manufacturer into the device, according to a specific procedure; this is sometimes intended to be done by the end user.)

Thus, while high-level firmware (or software) typically is stored as a configuration of charges, low-level firmware may instead often be regarded as actual hardware in itself. For instance, older firmware was often implemented as a discrete semiconductor diode matrix. The modern equivalent is an integrated matrix of field effect transistors where 0's and 1's are represented by whether a particular component in the ROM and/or PLA matrices is present or not.

- Contents [hide] - 1 Origin of the term - 2 Firmware as of 2010[update] - 2.1 Personal computers - 2.1.1 Computer peripherals - 2.2 Consumer products - 2.3 Automobiles - 2.4 IEEE Definition - 3 Examples - 4 Firmware "hacking" - 5 See also - 6 References

- [edit] Origin of the term

Ascher Opler coined the term "firmware" in a 1967 Datamation article.[1] Originally, it meant the microcode – contents of a writable control store (a small specialized high speed memory), which defined and implemented the computer's instruction set. If necessary, one could re-load the firmware to specialize or modify the instructions that the central processing unit (CPU) could execute. As originally used, firmware contrasted with hardware (the CPU itself) and software (normal instructions executing on a CPU). It was not composed of CPU machine instructions, but of lower-level microcode involved in the implementation of machine instructions. It existed on the boundary between hardware and software, thus the name "firmware".

Later[when?] application of the term broadened to include any type of microcode, whether in RAM or ROM.

Still later, popular usage extended the word "firmware" to denote anything ROM-resident, including processor machine-instructions for BIOS, bootstrap loaders, or specialized applications.

Until the mid 1990s, updating firmware to a new version typically involved replacing a storage-medium containing firmware, usually a socketed ROM. As of 2009[update] firmware-upgraders have largely abandoned this approach in favor of using firmware's capability to overwrite itself in a convenient, purely electronic operation.

- [edit] Firmware as of 2010[update]

The concept of "firmware" has evolved to mean almost any programmable content of a hardware device, not only machine code for a processor, but also configurations and data for application-specific integrated circuits (ASICs), programmable logic devices, etc.

- [edit] Personal computers

ROM BIOS firmware on a Baby AT motherboard

In some respects, the various firmware components are as important as the operating system in a working computer. However, unlike most modern operating systems, firmware rarely has a well-evolved automatic mechanism of updating itself to fix any functionality issues detected after shipping the unit.

Currently, one can fairly easily update the BIOS in a modern PC; devices like video cards or modems often rely on firmware dynamically loaded by a device driver and may thus get transparently updated through the operating system update mechanisms. In contrast, firmware in storage devices rarely gets updated, even when flash (rather than ROM) storage is used; there are no standardized mechanisms for detecting and updating firmware versions. However, in practice, such devices have a low rate of functionality issues compared to parts where the firmware may be updated.[citation needed]

- [edit] Computer peripherals

Most computer peripherals are themselves special-purpose computers. While external devices have firmware stored internally, as of 2010 modern graphics cards and peripheral expansion cards often have parts of the firmware loaded by the host system at start-up, as this provides greater flexibility. Such hardware may therefore fail to function fully until the host computer has "fed" it the requisite firmware, typically via a specific device driver (more exactly: via a start-up subsystem within a device driver package). Modern device drivers, whether for internal or external "peripheral" devices, may also expose a direct graphical user-interface for configuration, often using parts of a normal application program interface in addition to lower level operating system calls, hooks, and/or other interfaces designed for device drivers.

- [edit] Consumer products

See also: Mobile phone firmware

As of 2010[update] most modern portable music players support firmware upgrades. Some companies use firmware updates to add new playable file formats (encodings); iriver added the Vorbis format this way, for instance. Other features that may change with firmware updates include the GUI or even the battery life. Most mobile phones have a firmware upgrade capability for much the same reasons; some may even be upgraded to enhance reception or sound quality, illustrating the fact that firmware is used at more than one level in complex products (in a CPU-like microcontroller versus in a digital signal processor in this particular case).

- [edit] Automobiles

Since 1996 most automobiles have employed an on-board computer and various sensors to detect mechanical problems. As of 2010[update] modern vehicles also employ computer-controlled ABS systems and computer-operated Transmission Control Systems (TCS). The driver can also get in-dash information while driving in this manner, such as real-time fuel-economy and tire-pressure readings. Local dealers can update most vehicle firmware.

- [edit] IEEE Definition

Institute of Electrical and Electronics Engineers (IEEE) Standard Glossary of Software Engineering Terminology, Std 610.12-1990, defines firmware as follows:

"The combination of a hardware device and computer instructions and data that reside as read-only software on that device.

Notes: (1) This term is sometimes used to refer only to the hardware device or only to the computer instructions or data, but these meanings are deprecated. (2) The confusion surrounding this term has led some to suggest that it be avoided altogether."

- [edit] Examples

Examples of firmware include:

- Timing and control systems for washing machines

- The BIOS found in IBM-compatible personal computers

- The platform code found on Itanium systems, Intel-based Mac OS X machines, and many Intel desktop boards is EFI compliant firmware

- Open Firmware, used in computers from Sun Microsystems, Apple Computer, and Genesi

- ARCS (computing), used in computers from Silicon Graphics

- Kickstart used in Amiga line of computers: a unique example of a firmware mixing BIOS capabilities (POST, hardware init + Plug and Play auto-configuration of peripherals, etc.), the Kernel itself of the AmigaOS and parts of code used in the OS (Such as Graphical Interface primitives and libraries)

- RTAS (Run-Time Abstraction Services), used in computers from IBM

- EPROM chips used in the Eventide H-3000 series of digital music processors

- The Common Firmware Environment (CFE)

- Controlling sound and video attributes as well as the channel list in modern TVs

- [edit] Firmware "hacking"

Sometimes third parties may write an unofficial new or modified version of firmware to provide new features or to unlock hidden functionality. Examples include:

- Rockbox for digital audio players

- CHDK[2] and Magic Lantern[2][3] for Canon digital cameras

- OpenWRT for wireless routers[2],

- RouterTech - for ADSL modem/routers based on the Texas Instruments AR7 chipset (with the Pspboot or Adam2 bootloader)

- region free firmware for DVD-drives, which are not region free with the official firmware

- SamyGO, a replacement firmware for Samsung televisions[4]

- many homebrew projects for gaming consoles. These can often unlock general purpose computing functionality in previously limited devices (e.g., running Doom on iPods).

Most firmware hacks are free and open source software as well.

These hacks usually take advantage of the firmware update facility on many devices to install or run themselves. Some, however, must resort to exploits in order to run, because the manufacturer has attempted to lock the hardware to stop it from running unlicensed code.

- [edit] See also

- ROM image

- BIOS

- Coreboot

- Microcode

- Binary blob

- Software

- Hardware

- The Adam2 bootloader

- The Pspboot bootloader

- [edit] References

- ^ Opler, Ascher (January 1967). "Fourth-Generation Software". Datamation 13 (1): 22–24.

- ^ a b c "Custom Firmware Rocks!". 2009-08-05. http://www.maximumpc.com/article/features/frimware_hacks?page=0%2C1. Retrieved 2009-08-13.

- ^ "Magic Lantern firmware for Canon 5D Mark II". http://magiclantern.wikia.com. Retrieved 2009-08-13.

- ^ "SamyGO: replacing television firmware". LWN.net. 2009-11-14. http://lwn.net/Articles/361445/. Retrieved 2009-12-11.

Retrieved from "http://en.wikipedia.org/wiki/Firmware"

Categories: Embedded systems | Firmware

Hidden categories: Vague or ambiguous time | Articles containing potentially dated statements from 2009 | All articles containing potentially dated statements | Articles containing potentially dated statements from 2010 | All articles with unsourced statements | Articles with unsourced statements from May 2010

- Personal tools

- New features

- Log in / create account

- Namespaces

- Article

- Discussion

- Variants

- Views

- Read

- Edit

- View history

- Actions

- Search

Top of Form

Bottom of Form

- Navigation

- Main page

- Contents

- Featured content

- Current events

- Random article

- Donate

- Interaction

- About Wikipedia

- Community portal

- Recent changes

- Contact Wikipedia

- Help

- Toolbox

- What links here

- Related changes

- Upload file

- Special pages

- Permanent link

- Cite this page

- Print/export

- Create a book

- Download as PDF

- Printable version

- Languages

- العربية

- বাংলা

- Bosanski

- Català

- Česky

- Dansk

- Deutsch

- Ελληνικά

- Español

- Euskara

- فارسی

- Français

- Galego

- 한국어

- Bahasa Indonesia

- Italiano

- עברית

- Lietuvių

- Magyar

- മലയാളം

- Nederlands

- 日本語

- ‪Norsk (bokmål)‬

- Plattdüütsch

- Polski

- Português

- Română

- Русский

- සිංහල

- Simple English

- Slovenčina

- Српски / Srpski

- Suomi

- Svenska

- Tagalog

- ไทย

- Türkçe

- Українська

- Tiếng Việt

- 中文

- This page was last modified on 30 September 2010 at 10:36.

- Text is available under the Creative Commons Attribution-ShareAlike License; additional terms may apply. See Terms of Use for details.
Wikipedia® is a registered trademark of the Wikimedia Foundation, Inc., a non-profit organization.

- Contact us

- Privacy policy

- About Wikipedia

- Disclaimers

- Digital signal processing

From Wikipedia, the free encyclopedia

Jump to: navigation, search

|  | This article needs additional citations for verification. Please help improve this article by adding reliable references. Unsourced material may be challenged and removed. (May 2008) |
| --- | --- |

Digital signal processing (DSP) is concerned with the representation of signals by a sequence of numbers or symbols and the processing of these signals. Digital signal processing and analog signal processing are subfields of signal processing. DSP includes subfields like: audio and speech signal processing, sonar and radar signal processing, sensor array processing, spectral estimation, statistical signal processing, digital image processing, signal processing for communications, control of systems, biomedical signal processing, seismic data processing, etc.

The goal of DSP is usually to measure, filter and/or compress continuous real-world analog signals. The first step is usually to convert the signal from an analog to a digital form, by sampling it using an analog-to-digital converter (ADC), which turns the analog signal into a stream of numbers. However, often, the required output signal is another analog output signal, which requires a digital-to-analog converter (DAC). Even if this process is more complex than analog processing and has a discrete value range, the application of computational power to digital signal processing allows for many advantages over analog processing in many applications, such as error detection and correction in transmission as well as data compression.[1]

DSP algorithms have long been run on standard computers, on specialized processors called digital signal processors (DSPs), or on purpose-built hardware such as application-specific integrated circuit (ASICs). Today there are additional technologies used for digital signal processing including more powerful general purpose microprocessors, field-programmable gate arrays (FPGAs), digital signal controllers (mostly for industrial apps such as motor control), and stream processors, among others.[2]

- Contents [hide] - 1 DSP domains - 2 Signal sampling - 3 Time and space domains - 4 Frequency domain - 5 Z-domain analysis - 6 Applications - 7 Implementation - 8 Techniques - 9 Related fields - 10 References - 11 Further reading

- [edit] DSP domains

In DSP, engineers usually study digital signals in one of the following domains: time domain (one-dimensional signals), spatial domain (multidimensional signals), frequency domain, autocorrelation domain, and wavelet domains. They choose the domain in which to process a signal by making an informed guess (or by trying different possibilities) as to which domain best represents the essential characteristics of the signal. A sequence of samples from a measuring device produces a time or spatial domain representation, whereas a discrete Fourier transform produces the frequency domain information, that is the frequency spectrum. Autocorrelation is defined as the cross-correlation of the signal with itself over varying intervals of time or space.

- [edit] Signal sampling

Main article: Sampling (signal processing)

With the increasing use of computers the usage of and need for digital signal processing has increased. In order to use an analog signal on a computer it must be digitized with an analog-to-digital converter. Sampling is usually carried out in two stages, discretization and quantization. In the discretization stage, the space of signals is partitioned into equivalence classes and quantization is carried out by replacing the signal with representative signal of the corresponding equivalence class. In the quantization stage the representative signal values are approximated by values from a finite set.

The Nyquist–Shannon sampling theorem states that a signal can be exactly reconstructed from its samples if the sampling frequency is greater than twice the highest frequency of the signal. In practice, the sampling frequency is often significantly more than twice the required bandwidth.

A digital-to-analog converter is used to convert the digital signal back to analog. The use of a digital computer is a key ingredient in digital control systems.

- [edit] Time and space domains

Main article: Time domain

The most common processing approach in the time or space domain is enhancement of the input signal through a method called filtering. Digital filtering generally consists of some linear transformation of a number of surrounding samples around the current sample of the input or output signal. There are various ways to characterize filters; for example:

- A "linear" filter is a linear transformation of input samples; other filters are "non-linear". Linear filters satisfy the superposition condition, i.e. if an input is a weighted linear combination of different signals, the output is an equally weighted linear combination of the corresponding output signals.

- A "causal" filter uses only previous samples of the input or output signals; while a "non-causal" filter uses future input samples. A non-causal filter can usually be changed into a causal filter by adding a delay to it.

- A "time-invariant" filter has constant properties over time; other filters such as adaptive filters change in time.

- Some filters are "stable", others are "unstable". A stable filter produces an output that converges to a constant value with time, or remains bounded within a finite interval. An unstable filter can produce an output that grows without bounds, with bounded or even zero input.

- A "finite impulse response" (FIR) filter uses only the input signals, while an "infinite impulse response" filter (IIR) uses both the input signal and previous samples of the output signal. FIR filters are always stable, while IIR filters may be unstable.

Filters can be represented by block diagrams which can then be used to derive a sample processing algorithm to implement the filter using hardware instructions. A filter may also be described as a difference equation, a collection of zeroes and poles or, if it is an FIR filter, an impulse response or step response.

The output of a digital filter to any given input may be calculated by convolving the input signal with the impulse response.

- [edit] Frequency domain

Main article: Frequency domain

Signals are converted from time or space domain to the frequency domain usually through the Fourier transform. The Fourier transform converts the signal information to a magnitude and phase component of each frequency. Often the Fourier transform is converted to the power spectrum, which is the magnitude of each frequency component squared.

The most common purpose for analysis of signals in the frequency domain is analysis of signal properties. The engineer can study the spectrum to determine which frequencies are present in the input signal and which are missing.

In addition to frequency information, phase information is often needed. This can be obtained from the fourier transform. With some applications, how the phase varies with frequency can be a significant consideration.

Filtering, particularly in non-realtime work can also be achieved by converting to the frequency domain, applying the filter and then converting back to the time domain. This is a fast, O(n log n) operation, and can give essentially any filter shape including excellent approximations to brickwall filters.

There are some commonly used frequency domain transformations. For example, the cepstrum converts a signal to the frequency domain through Fourier transform, takes the logarithm, then applies another Fourier transform. This emphasizes the frequency components with smaller magnitude while retaining the order of magnitudes of frequency components.

Frequency domain analysis is also called spectrum- or spectral analysis.

- [edit] Z-domain analysis

Whereas analog filters are usually analysed on the s-plane; digital filters are analysed on the z-plane or z-domain in terms of z-transforms.

Most filters can be described in Z-domain (a complex number superset of the frequency domain) by their transfer functions. A filter may be analysed in the z-domain by its characteristic collection of zeroes and poles.

- [edit] Applications

The main applications of DSP are audio signal processing, audio compression, digital image processing, video compression, speech processing, speech recognition, digital communications, RADAR, SONAR, seismology, and biomedicine. Specific examples are speech compression and transmission in digital mobile phones, room matching equalization of sound in Hifi and sound reinforcement applications, weather forecasting, economic forecasting, seismic data processing, analysis and control of industrial processes, computer-generated animations in movies, medical imaging such as CAT scans and MRI, MP3 compression, image manipulation, high fidelity loudspeaker crossovers and equalization, and audio effects for use with electric guitar amplifiers.

- [edit] Implementation

Digital signal processing is often implemented using specialised microprocessors such as the DSP56000, the TMS320, or the SHARC. These often process data using fixed-point arithmetic, although some versions are available which use floating point arithmetic and are more powerful. For faster applications FPGAs[3] might be used. Beginning in 2007, multicore implementations of DSPs have started to emerge from companies including Freescale and Stream Processors, Inc. For faster applications with vast usage, ASICs might be designed specifically. For slow applications, a traditional slower processor such as a microcontroller may be adequate. Also a growing number of DSP applications are now being implemented on Embedded Systems using powerful PCs with a Multi-core processor.

- [edit] Techniques

- Bilinear transform

- Discrete Fourier transform

- Discrete-time Fourier transform

- Filter design

- LTI system theory

- Minimum phase

- Transfer function

- Z-transform

- Goertzel algorithm

- s-plane

- [edit] Related fields

- Analog signal processing

- Automatic control

- Computer Engineering

- Computer Science

- Data compression

- Dataflow programming

- Electrical engineering

- Information theory

- Telecommunication

- Machine Learning

- Stream processing

- Wavelet

- Fourier Analysis

- Time series

- [edit] References

- ^ James D. Broesch, Dag Stranneby and William Walker. Digital Signal Processing: Instant access. Butterworth-Heinemann. p. 3.

- ^ Dag Stranneby and William Walker (2004). Digital Signal Processing and Applications (2nd ed. ed.). Elsevier. ISBN 0750663448. http://books.google.com/books?id=NKK1DdqcDVUC&pg=PA241&dq=called+digital+signal+processor+hardware+application-specific+integrated+circuit+general-purpose+microprocessors+field-programmable+gate+arrays+dsp+asic+fpga+stream&as_brr=3&ei=cA0mSLWwB4qIswOEvsC5DQ&sig=EwJs3lv92Kai7Xw0i2XG9UpBVuA.

- ^ JpFix. "FPGA-Based Image Processing Accelerator". http://www.jpfix.com/About_Us/Articles/FPGA-Based_Image_Processing_Ac/fpga-based_image_processing_ac.html. Retrieved 2008-05-10.

- [edit] Further reading

|  | Wikibooks has a book on the topic of Digital Signal Processing |
| --- | --- |

- Alan V. Oppenheim, Ronald W. Schafer, John R. Buck : Discrete-Time Signal Processing, Prentice Hall, ISBN 0-13-754920-2

- Boaz Porat: A Course in Digital Signal Processing, Wiley, ISBN 0471149616

- Richard G. Lyons: Understanding Digital Signal Processing, Prentice Hall, ISBN 0-13-108989-7

- Jonathan Yaakov Stein, Digital Signal Processing, a Computer Science Perspective, Wiley, ISBN 0-471-29546-9

- Sen M. Kuo, Woon-Seng Gan: Digital Signal Processors: Architectures, Implementations, and Applications, Prentice Hall, ISBN 0-13-035214-4

- Bernard Mulgrew, Peter Grant, John Thompson: Digital Signal Processing - Concepts and Applications, Palgrave Macmillan, ISBN 0-333-96356-3

- Steven W. Smith: Digital Signal Processing - A Practical Guide for Engineers and Scientists, Newnes, ISBN 0-7506-7444-X

- Paul A. Lynn, Wolfgang Fuerst: Introductory Digital Signal Processing with Computer Applications, John Wiley & Sons, ISBN 0-471-97984-8

- James D. Broesch: Digital Signal Processing Demystified, Newnes, ISBN 1-878707-16-7

- John G. Proakis, Dimitris Manolakis: Digital Signal Processing - Principles, Algorithms and Applications, Pearson, ISBN 0-13-394289-9

- Hari Krishna Garg: Digital Signal Processing Algorithms, CRC Press, ISBN 0-8493-7178-3

- P. Gaydecki: Foundations Of Digital Signal Processing: Theory, Algorithms And Hardware Design, Institution of Electrical Engineers, ISBN 0-85296-431-5

- Gibson, John. “Spectral Delay as a Compositional Resource.” eContact! 11.4 — Toronto Electroacoustic Symposium 2009 (TES) / Symposium Électroacoustique 2009 de Toronto (December 2009). Montréal: CEC.

- Paul M. Embree, Damon Danieli: C++ Algorithms for Digital Signal Processing, Prentice Hall, ISBN 0-13-179144-3

- Anthony Zaknich: Neural Networks for Intelligent Signal Processing, World Scientific Pub Co Inc, ISBN 981-238-305-0

- Vijay Madisetti, Douglas B. Williams: The Digital Signal Processing Handbook, CRC Press, ISBN 0-8493-8572-5

- Stergios Stergiopoulos: Advanced Signal Processing Handbook: Theory and Implementation for Radar, Sonar, and Medical Imaging Real-Time Systems, CRC Press, ISBN 0-8493-3691-0

- Joyce Van De Vegte: Fundamentals of Digital Signal Processing, Prentice Hall, ISBN 0-13-016077-6

- Ashfaq Khan: Digital Signal Processing Fundamentals, Charles River Media, ISBN 1-58450-281-9

- Jonathan M. Blackledge, Martin Turner: Digital Signal Processing: Mathematical and Computational Methods, Software Development and Applications, Horwood Publishing, ISBN 1-898563-48-9

- Bimal Krishna, K. Y. Lin, Hari C. Krishna: Computational Number Theory & Digital Signal Processing, CRC Press, ISBN 0-8493-7177-5

- Doug Smith: Digital Signal Processing Technology: Essentials of the Communications Revolution, American Radio Relay League, ISBN 0-87259-819-5

- Henrique S. Malvar: Signal Processing with Lapped Transforms, Artech House Publishers, ISBN 0-89006-467-9

- Charles A. Schuler: Digital Signal Processing: A Hands-On Approach, McGraw-Hill, ISBN 0-07-829744-3

- James H. McClellan, Ronald W. Schafer, Mark A. Yoder: Signal Processing First, Prentice Hall, ISBN 0-13-090999-8

- Artur Krukowski, Izzet Kale: DSP System Design: Complexity Reduced Iir Filter Implementation for Practical Applications, Kluwer Academic Publishers, ISBN 1-4020-7558-8

- Kainam Thomas Wong [1]: Statistical Signal Processing lecture notes [2] at the University of Waterloo, Canada.

- John G. Proakis: A Self-Study Guide for Digital Signal Processing, Prentice Hall, ISBN 0-13-143239-7

- Communications server

From Wikipedia, the free encyclopedia

Jump to: navigation, search

For Microsoft's enterprise real-time communications server product, see Microsoft Office Communications Server.

Communications servers are open, standards-based computing systems that operate as a carrier-grade common platform for a wide range of communications applications and allow equipment providers to add value at many levels of the system architecture.

Based on industry-managed standards such as AdvancedTCA, MicroTCATM, Carrier Grade Linux and Service AvailabilityTM Forum specifications, communications servers are the foundational platform upon which equipment providers build network infrastructure elements for deployments such as IP Multimedia Subsystem (IMS), IPTV and wireless broadband (e.g. WiMAX).

Support for communications servers as a category of server is developing rapidly throughout the communications industry. Standards bodies, industry associations, vendor alliance programs, hardware and software manufacturers, communications server vendors and users are all part of an increasingly robust communications server ecosystem.

Regardless of their specific, differentiated features, communications servers have the following attributes: open, flexible, carrier-grade, and communications-focused.

- Contents [hide] - 1 Attributes - 1.1 Open - 1.2 Flexible - 1.3 Carrier Grade - 1.4 Communications Focus - 2 Industry-managed standards - 2.1 AdvancedTCA - 2.2 AdvancedMC - 2.3 MicroTCA - 2.4 Carrier Grade Linux - 2.5 HPI and AIS - 3 Industry associations - 3.1 SCOPE Alliance - 3.2 Communications Platforms Trade Association - 4 Vendor alliance programs - 4.1 Intel Communications Alliance - 4.2 Motorola Communications Server Alliance

- [edit] Attributes

- [edit] Open

- Based on industry-managed open standards

- Broad, multi-vendor ecosystem

- Industry certified interoperability

- Availability of tools that facilitate development and integration of applications at the standardized interfaces

- Multiple competitive options for standards-based modules

- [edit] Flexible

- Designed to easily incorporate application-specific added value at all levels of the solution

- Can be rapidly repurposed as needs change to protect customer investment

- Multi-level, scalable, bladed architecture

- Meets needs of multiple industries beyond telecommunications, such as medical imaging, defense and aerospace

- [edit] Carrier Grade

- Designed for

  - Longevity of supply

  - Extended lifecycle (>10 years) support

  - High availability (>5NINES)

- “Non-disruptively” upgradeable and updateable

- Hard real time capability to ensure quality of service for critical traffic

- Meets network building regulations

- [edit] Communications Focus

- Designed as common platform for a wide range of communications applications

- Optimized configurations for:

  - Control plane processing

    - E.g. radio network controller (RNC), IP Multimedia Subsystem (IMS) CSCF

  - Applications Servers and network embedded databases

    - Including HSS, HLR/VLR

  - Packet termination and routing

    - Including signaling gateways, session border controllers

  - Multimedia servers and translators

    - Including media servers, media gateways, video encoders, video on demand servers

- Extensive range of telecom I/O options

- Including ATM, Ethernet, SONET, T1/E1, T3

- Sourced as integrated and pre-validated systems to streamline supply chain and simplify integration and testing

- [edit] Industry-managed standards

Several industry-managed standards are critical to the success of communications servers, including:

- [edit] AdvancedTCA

The Advanced Telecommunications Computing Architecture (ATCA) is a series of PCI Industrial Computers Manufacturers Group (PICMG) specifications, targeted to meet the requirements for carrier grade communications equipment. This series of specifications incorporates the latest trends in high speed interconnect technologies, next generation processors and improved reliability, manageability and serviceability.

- [edit] AdvancedMC

The PICMG Advanced Mezzanine Card specification defines the base-level requirements for a wide-range of high-speed mezzanine cards optimized for, but not limited to, AdvancedTCA Carriers. AdvancedMC enhances AdvancedTCA’s flexibility by extending its high-bandwidth, multi-protocol interface to individual hot-swappable modules.

- [edit] MicroTCA

This PICMG specification provides a framework for combining AdvancedMC modules directly, without the need for an AdvancedTCA or custom carrier. MicroTCA is aimed at smaller equipment – such as wireless base stations, Wi-Fi and WiMAX radios, and VoIP access gateways where small physical size low entry cost, and scalability are key requirements.

- [edit] Carrier Grade Linux

An enhanced version of Linux for use in a highly available, secure, scalable, and maintainable carrier grade system. The specification is managed by the CGL Working Group of the Open Source Development Labs.

- [edit] HPI and AIS

These Service Availability Forum (SA Forum) specifications define standard interfaces for telecom platform management and high-availability software.

The Hardware Platform Interface (HPI) specification defines the interface between high availability middleware and the underlying hardware and operating system.

At a higher layer than HPI, the Application Interface Specification (AIS) defines the application programming interface between the high availability middleware and the application. AIS allows an application to run on multiple computing modules, and applications that support AIS can migrate more easily between computing platforms from different manufacturers that support the standard.

In addition to the standards development organizations mentioned above, four industry associations / vendor alliance programs are playing key roles in the development of the communications server ecosystem.

- [edit] Industry associations

- [edit] SCOPE Alliance

SCOPE Alliance is an industry alliance committed to accelerating the deployment of carrier grade base platforms for service provider applications. Its mission is to help, enable and promote the availability of open carrier grade base platforms based on Commercial-Off-The-Shelf hardware / software and Free Open Source Software building blocks, and to promote interoperability to better serve Service Providers and consumers.

- [edit] Communications Platforms Trade Association

The Communications Platforms Trade Association (CP-TA) is an association of communications platforms and building block providers dedicated to accelerating the adoption of SIG-governed, open specification-based communications platforms through interoperability certification. With industry collaboration, the CP-TA plans to drive a mainstream market for open industry standards-based communications platforms by certifying interoperable products.

- [edit] Vendor alliance programs

- [edit] Intel Communications Alliance

The Intel Communications Alliance is a community of communications and embedded developers and solutions providers committed to the development of modular, standards-based solutions on Intel technologies.

- [edit] Motorola Communications Server Alliance

The Motorola Communications Server Alliance is an ecosystem of technology, service and solution providers aligned to provide standards-based solution elements validated with Motorola’s communications servers. Alliance participants receive access to Motorola embedded communications computing product roadmaps, development systems, and participate in marketing activities with Motorola.

Retrieved from "http://en.wikipedia.org/wiki/Communications_server"

- Cyber-physical system

From Wikipedia, the free encyclopedia

Jump to: navigation, search

A cyber-physical system (CPS) is a system featuring a tight combination of, and coordination between, the system’s computational and physical elements. Today, a pre-cursor generation of cyber-physical systems can be found in areas as diverse as aerospace, automotive, chemical processes, civil infrastructure, energy, healthcare, manufacturing, transportation, entertainment, and consumer appliances. This generation is often referred to as embedded systems. In embedded systems the emphasis tends to be more on the computational elements, and less on an intense link between the computational and physical elements.

Unlike more traditional embedded systems, a full-fledged CPS is typically designed as a network of interacting elements with physical input and output instead of as standalone devices.[1] The notion is closely tied to concepts of robotics and sensor networks. The expectation is that in the coming years ongoing advances in science and engineering will improve the link between computational and physical elements, dramatically increasing the adaptability, autonomy, efficiency, functionality, reliability, safety, and usability of cyber-physical systems. The advances will broaden the potential of cyber-physical systems in several dimensions, including: intervention (e.g., collision avoidance); precision (e.g., robotic surgery and nano-level manufacturing); operation in dangerous or inaccessible environments (e.g., search and rescue, firefighting, and deep-sea exploration); coordination (e.g., air traffic control, war fighting); efficiency (e.g., zero-net energy buildings); and augmentation of human capabilities (e.g., healthcare monitoring and delivery).[2]

The US National Science Foundation (NSF) has identified cyber-physical systems as a key area of research.[3] Starting in late 2006, the NSF and other United States federal agencies sponsored several workshops on cyber-physical systems.[4][5][6][7][8][9][10]

- Contents [hide] - 1 Examples - 2 References - 3 Further reading - 4 External links

- [edit] Examples

Common applications of CPS typically fall under sensor-based systems and autonomous systems. For example, many wireless sensor networks monitor some aspect of the environment and relay the processed information to a central node. Other types of CPS include autonomous automotive systems, medical monitoring, process control systems, distributed robotics, and automatic pilot avionics.

A real-world example of such a system is the Distributed Robot Garden at MIT in which a team of robots tend a garden of tomato plants. This system combines distributed sensing (each plant is equipped with a sensor node monitoring its status), navigation, Manipulation and wireless networking.

Another example is MIT's ongoing CarTel project where a fleet of taxis collecting real-time traffic information in the Boston area. Together with historical data, this information is then used for calculating fastest routes for a given time of the day.

- [edit] References

- ^ Lee, Edward (January 23, 2008). Cyber Physical Systems: Design Challenges. University of California, Berkeley Technical Report No. UCB/EECS-2008-8. http://www.eecs.berkeley.edu/Pubs/TechRpts/2008/EECS-2008-8.html. Retrieved 2008-06-07.

- ^ "Cyber-physical systems". Program Announcements & Information. The National Science Foundation, 4201 Wilson Boulevard, Arlington, Virginia 22230, USA. 2008-09-30. http://www.nsf.gov/publications/pub_summ.jsp?ods_key=nsf08611. Retrieved 2009-07-21.

- ^ Wolf, Wayne (November 2007). "The Good News and the Bad News (Embedded Computing Column". IEEE Computer. http://www.computer.org/portal/site/computer/menuitem.5d61c1d591162e4b0ef1bd108bcd45f3/index.jsp?&pName=computer_level1_article&TheCat=1085&path=computer/homepage/Nov07&file=embedded.xml&xsl=article.xsl. Retrieved 2008-06-09.

- ^ "NSF Workshop On Cyber-Physical Systems". http://varma.ece.cmu.edu/cps. Retrieved 2008-06-09.

- ^ "Beyond SCADA: Networked Embedded Control for Cyber Physical Systems". http://www.truststc.org/scada/. Retrieved 2008-06-09.

- ^ "NSF Cyber-Physical Systems Summit". http://varma.ece.cmu.edu/Summit. Retrieved 2008-08-01.

- ^ "National Workshop on High-Confidence Automotive Cyber-Physical Systems". http://varma.ece.cmu.edu/Auto-CPS. Retrieved 2008-08-03.

- ^ "National Workshop on Composable and Systems Technologies for High-Confidence Cyber-Physical Systems,". http://www.isis.vanderbilt.edu/CST-HCCPS/. Retrieved 2008-08-04.

- ^ "National Workshop on High-Confidence Software Platforms for Cyber-Physical Systems (HCSP-CPS),". http://www.isis.vanderbilt.edu/HCSP-CPS/. Retrieved 2008-08-04.

- ^ "New Research Directions for Future Cyber-Physical Energy Systems". http://www.ece.cmu.edu/~nsf-cps/. Retrieved 2009-06-05.

- [edit] Further reading

NSF Workshop On Cyber-Physical Systems

- Edward A. Lee, Cyber-Physical Systems - Are Computing Foundations Adequate?

- Paulo Tabuada, Cyber-Physical Systems: Position Paper

- Rajesh Gupta, Programming Models and Methods for Spatio-Temporal Actions and Reasoning in Cyber-Physical Systems

- CPS Community, CPS Workshops

- [edit] External links

- The CPS Virtual Organization

|  | This computer engineering-related article is a stub. You can help Wikipedia by expanding it. v • d • e |
| --- | --- |

Retrieved from "http://en.wikipedia.org/wiki/Cyber-physical_system"

- Field-programmable gate array

From Wikipedia, the free encyclopedia

(Redirected from FPGA)

Jump to: navigation, search

"FPGA" redirects here. It is not to be confused with Flip-chip pin grid array.

An Altera Stratix IV GX FPGA

An example of an FPGA programming/evaluation board

A field-programmable gate array (FPGA) is an integrated circuit designed to be configured by the customer or designer after manufacturing—hence "field-programmable". The FPGA configuration is generally specified using a hardware description language (HDL), similar to that used for an application-specific integrated circuit (ASIC) (circuit diagrams were previously used to specify the configuration, as they were for ASICs, but this is increasingly rare). FPGAs can be used to implement any logical function that an ASIC could perform. The ability to update the functionality after shipping, partial re-configuration of the portion of the design[1] and the low non-recurring engineering costs relative to an ASIC design (not withstanding the generally higher unit cost), offer advantages for many applications.[2]

FPGAs contain programmable logic components called "logic blocks", and a hierarchy of reconfigurable interconnects that allow the blocks to be "wired together"—somewhat like a one-chip programmable breadboard. Logic blocks can be configured to perform complex combinational functions, or merely simple logic gates like AND and XOR. In most FPGAs, the logic blocks also include memory elements, which may be simple flip-flops or more complete blocks of memory.[2]

In addition to digital functions, some FPGAs have analog features. The most common analog feature is programmable slew rate and drive strength on each output pin, allowing the engineer to set slow rates on lightly loaded pins that would otherwise ring unacceptably, and to set stronger, faster rates on heavily loaded pins on high-speed channels that would otherwise run too slow.[3][4] Another relatively common analog feature is differential comparators on input pins designed to be connected to differential signaling channels. A few "mixed signal FPGAs" have integrated peripheral ADCs and DACs and analog signal conditioning blocks allowing them to operate as a system-on-a-chip.[5] Such devices blur the line between an FPGA, which carries digital ones and zeros on its internal programmable interconnect fabric, and field-programmable analog array (FPAA), which carries analog values on its internal programmable interconnect fabric.

- Contents [hide] - 1 History - 1.1 Modern developments - 1.2 Gates - 1.3 Market size - 1.4 FPGA design starts - 2 FPGA comparisons - 2.1 Versus complex programmable logic devices - 2.2 Security considerations - 3 Applications - 4 Architecture - 5 FPGA design and programming - 6 Basic process technology types - 7 Major manufacturers - 8 See also - 9 References - 10 External links

- [edit] History

The FPGA industry sprouted from programmable read-only memory (PROM) and programmable logic devices (PLDs). PROMs and PLDs both had the option of being programmed in batches in a factory or in the field (field programmable), however programmable logic was hard-wired between logic gates.[6]

In the late 1980s the Naval Surface Warfare Department funded an experiment proposed by Steve Casselman to develop a computer that would implement 600,000 reprogrammable gates. Casselman was successful and a patent related to the system was issued in 1992.[6]

Some of the industry’s foundational concepts and technologies for programmable logic arrays, gates, and logic blocks are founded in patents awarded to David W. Page and LuVerne R. Peterson in 1985.[7][8]

Xilinx Co-Founders, Ross Freeman and Bernard Vonderschmitt, invented the first commercially viable field programmable gate array in 1985 – the XC2064.[9] The XC2064 had programmable gates and programmable interconnects between gates, the beginnings of a new technology and market.[10] The XC2064 boasted a mere 64 configurable logic blocks (CLBs), with two 3-input lookup tables (LUTs).[11] More than 20 years later, Freeman was entered into the National Inventor's Hall of Fame for his invention.[12]

Xilinx continued unchallenged and quickly growing from 1985 to the mid-1990s, when competitors sprouted up, eroding significant market-share. By 1993, Actel was serving about 18 percent of the market.[10]

The 1990s were an explosive period of time for FPGAs, both in sophistication and the volume of production. In the early 1990s, FPGAs were primarily used in telecommunications and networking. By the end of the decade, FPGAs found their way into consumer, automotive, and industrial applications.[13]

FPGAs got a glimpse of fame in 1997, when Adrian Thompson, a researcher working at the University of Sussex, merged genetic algorithm technology and FPGAs to create a sound recognition device. Thomson’s algorithm configured an array of 10 x 10 cells in a Xilinx FPGA chip to discriminate between two tones, utilising analogue features of the digital chip. The application of genetic algorithms to the configuration of devices like FPGA's is now referred to as Evolvable hardware[6][14]

- [edit] Modern developments

A recent trend has been to take the coarse-grained architectural approach a step further by combining the logic blocks and interconnects of traditional FPGAs with embedded microprocessors and related peripherals to form a complete "system on a programmable chip". This work mirrors the architecture by Ron Perlof and Hana Potash of Burroughs Advanced Systems Group which combined a reconfigurable CPU architecture on a single chip called the SB24. That work was done in 1982. Examples of such hybrid technologies can be found in the Xilinx Virtex-II PRO and Virtex-4 devices, which include one or more PowerPC processors embedded within the FPGA's logic fabric. The Atmel FPSLIC is another such device, which uses an AVR processor in combination with Atmel's programmable logic architecture. The Actel SmartFusion devices incorporate an ARM architecture Cortex-M3 hard processor core (with up to 512kB of flash and 64kB of RAM) and analog peripherals such as a multi-channel ADC and DACs to their flash-based FPGA fabric.

An alternate approach to using hard-macro processors is to make use of soft processor cores that are implemented within the FPGA logic.

As previously mentioned, many modern FPGAs have the ability to be reprogrammed at "run time," and this is leading to the idea of reconfigurable computing or reconfigurable systems — CPUs that reconfigure themselves to suit the task at hand. The Mitrion Virtual Processor from Mitrionics is an example of a reconfigurable soft processor, implemented on FPGAs. However, it does not support dynamic reconfiguration at runtime, but instead adapts itself to a specific program.

Additionally, new, non-FPGA architectures are beginning to emerge. Software-configurable microprocessors such as the Stretch S5000 adopt a hybrid approach by providing an array of processor cores and FPGA-like programmable cores on the same chip.

- [edit] Gates

- 1987: 9,000 gates, Xilinx[10]

- 1992: 600,000, Naval Surface Warfare Department[6]

- Early 2000s: Millions [13]

- [edit] Market size

- 1985: First commercial FPGA technology invented by Xilinx[10]

- 1987: $14 million[10]

- ~1993: >$385 million[10]

- 2005: $1.9 billion[15]

- 2010 estimates: $2.75 billion [15]

- [edit] FPGA design starts

- 10,000[16]

- 2005: 80,000[17]

- 2008: 90,000[18]

- [edit] FPGA comparisons

Historically, FPGAs have been slower, less energy efficient and generally achieved less functionality than their fixed ASIC counterparts. A study has shown that designs implemented on FPGAs need on average 18 times as much area, draw 7 times as much dynamic power, and are 3 times slower than the corresponding ASIC implementations.[19]

An Altera Cyclone II FPGA, on an Altera teraSIC DE1 Prototyping board.

Advantages include the ability to re-program in the field to fix bugs, and may include a shorter time to market and lower non-recurring engineering costs.[citation needed] Vendors can also take a middle road by developing their hardware on ordinary FPGAs, but manufacture their final version so it can no longer be modified after the design has been committed.

Xilinx claims that several market and technology dynamics are changing the ASIC/FPGA paradigm:[20]

- Integrated circuit costs are rising aggressively

- ASIC complexity has lengthened development time

- R&D resources and headcount are decreasing

- Revenue losses for slow time-to-market are increasing

- Financial constraints in a poor economy are driving low-cost technologies

These trends make FPGAs a better alternative than ASICs for a larger number of higher-volume applications than they have been historically used for, to which the company attributes the growing number of FPGA design starts (see History).[20]

Some FPGAs have the capability of partial re-configuration that lets one portion of the device be re-programmed while other portions continue running.

- [edit] Versus complex programmable logic devices

The primary differences between CPLDs and FPGAs are architectural. A CPLD has a somewhat restrictive structure consisting of one or more programmable sum-of-products logic arrays feeding a relatively small number of clocked registers. The result of this is less flexibility, with the advantage of more predictable timing delays and a higher logic-to-interconnect ratio. The FPGA architectures, on the other hand, are dominated by interconnect. This makes them far more flexible (in terms of the range of designs that are practical for implementation within them) but also far more complex to design for.

Another notable difference between CPLDs and FPGAs is the presence in most FPGAs of higher-level embedded functions (such as adders and multipliers) and embedded memories, as well as to have logic blocks implement decoders or mathematical functions.

- [edit] Security considerations

With respect to security, FPGAs have both advantages and disadvantages as compared to ASICs or secure microprocessors. FPGAs' flexibility makes malicious modifications during fabrication a lower risk.[21] For many FPGAs, the loaded design is exposed while it is loaded (typically on every power-on). To address this issue, some FPGAs support bitstream encryption.

- [edit] Applications

Applications of FPGAs include digital signal processing, software-defined radio, aerospace and defense systems, ASIC prototyping, medical imaging, computer vision, speech recognition, cryptography, bioinformatics, computer hardware emulation, radio astronomy, metal detection and a growing range of other areas.

FPGAs originally began as competitors to CPLDs and competed in a similar space, that of glue logic for PCBs. As their size, capabilities, and speed increased, they began to take over larger and larger functions to the state where some are now marketed as full systems on chips (SoC). Particularly with the introduction of dedicated multipliers into FPGA architectures in the late 1990s, applications which had traditionally been the sole reserve of DSPs began to incorporate FPGAs instead.[22][23]

FPGAs especially find applications in any area or algorithm that can make use of the massive parallelism offered by their architecture. One such area is code breaking, in particular brute-force attack, of cryptographic algorithms.

FPGAs are increasingly used in conventional high performance computing applications where computational kernels such as FFT or Convolution are performed on the FPGA instead of a microprocessor.

The inherent parallelism of the logic resources on an FPGA allows for considerable computational throughput even at a low MHz clock rates. The flexibility of the FPGA allows for even higher performance by trading off precision and range in the number format for an increased number of parallel arithmetic units. This has driven a new type of processing called reconfigurable computing, where time intensive tasks are offloaded from software to FPGAs.

The adoption of FPGAs in high performance computing is currently limited by the complexity of FPGA design compared to conventional software and the turn-around times of current design tools.

Traditionally, FPGAs have been reserved for specific vertical applications where the volume of production is small. For these low-volume applications, the premium that companies pay in hardware costs per unit for a programmable chip is more affordable than the development resources spent on creating an ASIC for a low-volume application. Today, new cost and performance dynamics have broadened the range of viable applications.

- [edit] Architecture

The most common FPGA architecture[24] consists of an array of logic blocks (called Configurable Logic Block, CLB, or Logic Array Block, LAB, depending on vendor), I/O pads, and routing channels. Generally, all the routing channels have the same width (number of wires). Multiple I/O pads may fit into the height of one row or the width of one column in the array.

An application circuit must be mapped into an FPGA with adequate resources. While the number of CLBs/LABs and I/Os required is easily determined from the design, the number of routing tracks needed may vary considerably even among designs with the same amount of logic. For example, a crossbar switch requires much more routing than a systolic array with the same gate count. Since unused routing tracks increase the cost (and decrease the performance) of the part without providing any benefit, FPGA manufacturers try to provide just enough tracks so that most designs that will fit in terms of LUTs and IOs can be routed. This is determined by estimates such as those derived from Rent's rule or by experiments with existing designs.

In general, a logic block (CLB or LAB) consists of a few logical cells (called ALM, LE, Slice etc). A typical cell consists of a 4-input Lookup table (LUT), a Full adder (FA) and a D-type flip-flop, as shown below. The LUT are in this figure split into two 3-input LUTs. In normal mode those are combined into a 4-input LUT through the left mux. In arithmetic mode, their outputs are fed to the FA. The selection of mode are programmed into the middle mux. The output can be either synchronous or asynchronous, depending on the programming of the mux to the right, in the figure example. In practice, entire or parts of the FA are put as functions into the LUTs in order to save space.[25][26][27]

Simplified example illustration of a logic cell

ALMs and Slices usually contains 2 or 4 structures similar to the example figure, with some shared signals.

CLBs/LABs typically contains a few ALMs/LEs/Slices.

In recent years, manufacturers have started moving to 6-input LUTs in their high performance parts, claiming increased performance.[28]

Since clock signals (and often other high-fanout signals) are normally routed via special-purpose dedicated routing networks in commercial FPGAs, they and other signals are separately managed.

For this example architecture, the locations of the FPGA logic block pins are shown below.

Logic Block Pin Locations

Each input is accessible from one side of the logic block, while the output pin can connect to routing wires in both the channel to the right and the channel below the logic block.

Each logic block output pin can connect to any of the wiring segments in the channels adjacent to it.

Similarly, an I/O pad can connect to any one of the wiring segments in the channel adjacent to it. For example, an I/O pad at the top of the chip can connect to any of the W wires (where W is the channel width) in the horizontal channel immediately below it.

Generally, the FPGA routing is unsegmented. That is, each wiring segment spans only one logic block before it terminates in a switch box. By turning on some of the programmable switches within a switch box, longer paths can be constructed. For higher speed interconnect, some FPGA architectures use longer routing lines that span multiple logic blocks.

Whenever a vertical and a horizontal channel intersect, there is a switch box. In this architecture, when a wire enters a switch box, there are three programmable switches that allow it to connect to three other wires in adjacent channel segments. The pattern, or topology, of switches used in this architecture is the planar or domain-based switch box topology. In this switch box topology, a wire in track number one connects only to wires in track number one in adjacent channel segments, wires in track number 2 connect only to other wires in track number 2 and so on. The figure below illustrates the connections in a switch box.

Switch box topology

Modern FPGA families expand upon the above capabilities to include higher level functionality fixed into the silicon. Having these common functions embedded into the silicon reduces the area required and gives those functions increased speed compared to building them from primitives. Examples of these include multipliers, generic DSP blocks, embedded processors, high speed IO logic and embedded memories.

FPGAs are also widely used for systems validation including pre-silicon validation, post-silicon validation, and firmware development. This allows chip companies to validate their design before the chip is produced in the factory, reducing the time-to-market.

- [edit] FPGA design and programming

To define the behavior of the FPGA, the user provides a hardware description language (HDL) or a schematic design. The HDL form is more suited to work with large structures because it's possible to just specify them numerically rather than having to draw every piece by hand. However, schematic entry can allow for easier visualisation of a design.

Then, using an electronic design automation tool, a technology-mapped netlist is generated. The netlist can then be fitted to the actual FPGA architecture using a process called place-and-route, usually performed by the FPGA company's proprietary place-and-route software. The user will validate the map, place and route results via timing analysis, simulation, and other verification methodologies. Once the design and validation process is complete, the binary file generated (also using the FPGA company's proprietary software) is used to (re)configure the FPGA.

Going from schematic/HDL source files to actual configuration: The source files are fed to a software suite from the FPGA/CPLD vendor that through different steps will produce a file. This file is then transferred to the FPGA/CPLD via a serial interface (JTAG) or to an external memory device like an EEPROM.

The most common HDLs are VHDL and Verilog, although in an attempt to reduce the complexity of designing in HDLs, which have been compared to the equivalent of assembly languages, there are moves to raise the abstraction level through the introduction of alternative languages. National Instrument's LabVIEW graphical programming language ( sometimes referred to as "G" ) has an FPGA add-in module available to target and program FPGA hardware. The LabVIEW approach drastically simplifies the FPGA programming process[citation needed].

To simplify the design of complex systems in FPGAs, there exist libraries of predefined complex functions and circuits that have been tested and optimized to speed up the design process. These predefined circuits are commonly called IP cores, and are available from FPGA vendors and third-party IP suppliers (rarely free, and typically released under proprietary licenses). Other predefined circuits are available from developer communities such as OpenCores (typically released under free and open source licenses such as the GPL, BSD or similar license), and other sources.

In a typical design flow, an FPGA application developer will simulate the design at multiple stages throughout the design process. Initially the RTL description in VHDL or Verilog is simulated by creating test benches to simulate the system and observe results. Then, after the synthesis engine has mapped the design to a netlist, the netlist is translated to a gate level description where simulation is repeated to confirm the synthesis proceeded without errors. Finally the design is laid out in the FPGA at which point propagation delays can be added and the simulation run again with these values back-annotated onto the netlist.

- [edit] Basic process technology types

- SRAM - based on static memory technology. In-system programmable and re-programmable. Requires external boot devices. CMOS.

- Antifuse - One-time programmable. CMOS.

- PROM - Programmable Read-Only Memory technology. One-time programmable because of plastic packaging.

- EPROM - Erasable Programmable Read-Only Memory technology. One-time programmable but with window, can be erased with ultraviolet (UV) light. CMOS.

- EEPROM - Electrically Erasable Programmable Read-Only Memory technology. Can be erased, even in plastic packages. Some but not all EEPROM devices can be in-system programmed. CMOS.

- Flash - Flash-erase EPROM technology. Can be erased, even in plastic packages. Some but not all flash devices can be in-system programmed. Usually, a flash cell is smaller than an equivalent EEPROM cell and is therefore less expensive to manufacture. CMOS.

- Fuse - One-time programmable. Bipolar.

- [edit] Major manufacturers

Xilinx and Altera are the current FPGA market leaders and long-time industry rivals. Together, they control over 80 percent of the market,[29] with Xilinx alone representing over 50 percent.

Both Xilinx and Altera provide free Windows and Linux design software.[30][31]

Other competitors include Lattice Semiconductor (SRAM based with integrated configuration Flash, instant-on, low power, live reconfiguration), Actel (antifuse, flash-based, mixed-signal), SiliconBlue Technologies (low power), Achronix (RAM based, 1.5 GHz fabric speed), and QuickLogic (handheld focused CSSP, no general purpose FPGAs).

In March 2010, two FPGA companies that had previously worked secretly,[citation needed] announced their new FPGA technology: Tabula and Tier Logic; Tabula uses time-multiplexing and Tier Logic 3D-FPGA technology.

- [edit] See also

- Gate array

- PSoC

- Application-specific integrated circuit (ASIC)

- Application-specific instruction-set processor (ASIP)

- Impulse CoDeveloper (Impulse C)

- Combinational logic

- Complex programmable logic device (CPLD)

- Software Defined Silicon (SDS)

- Soft processor

- FPGA prototype

- VHDL: VHSIC (Very High Speed Integrated Circuit) Hardware Description Language

- Verilog: Hardware Description Language

- JHDL: Just-Another Hardware Description Language

- Reconfigurable Computing

- Partial re-configuration

- Hybrid-core computing

- Configware

- MyHDL Python based HDL—generates Verilog or VHDL

- SystemC System Description Language—C like

- Handel-C Extended C based description language designed for FPGAs

- Minimig open source re-implementation of an Amiga 500 using an FPGA

- Computing with Memory A time-multiplexed reconfigurable architecture using 2-D memory array

- Digital Clock Manager DCM - Digital Clock Management

- [edit] References

- ^ Wiśniewski, Remigiusz (2009). Synthesis of compositional microprogram control units for programmable devices. Zielona Góra: University of Zielona Góra. pp. 153. ISBN 978-83-7481-293-1.

- ^ a b FPGA Architecture for the Challenge

- ^ FPGA Signal Integrity tutorial

- ^ NASA: FPGA drive strength

- ^ Mike Thompson. "Mixed-signal FPGAs provide GREEN POWER". EE Times, 2007-07-02.

- ^ a b c d History of FPGAs

- ^ Google Patent Search, "Re-programmable PLA". Retrieved February 5, 2009.

- ^ Google Patent Search, "Dynamic data re-programmable PLA". Retrieved February 5, 2009.

- ^ Peter Clarke, EE Times, "Xilinx, ASIC Vendors Talk Licensing." June 22, 2001. Retrieved February 10, 2009.

- ^ a b c d e f Funding Universe. “Xilinx, Inc.” Retrieved January 15, 2009.

- ^ Clive Maxfield, Programmable Logic DesignLine, "Xilinx unveil revolutionary 65nm FPGA architecture: the Virtex-5 family. May 15, 2006. Retrieved February 5, 2009.

- ^ Press Release, "Xilinx Co-Founder Ross Freeman Honored as 2009 National Inventors Hall of Fame Inductee for Invention of FPGA"

- ^ a b Clive Maxfield, book, "The Design Warrior's Guide to FPGAs".Published by Elsevier, 2004. ISBN 0750676043, 9780750676045. Retrieved February 5, 2009

- ^ "Original Paper on Work, Thompson". http://www.informatics.sussex.ac.uk/users/adrianth/ices96/paper.ps.

- ^ a b Dylan McGrath, EE Times, "FPGA Market to Pass $2.7 Billion by '10, In-Stat Says". May 24, 2006. Retrieved February 5, 2009.

- ^ Narinder Lall, eASIC Corporation, "FPGA Judgment Day:Rise of Second Generation Structured ASICs. March, 2008. Retrieved February 5, 2009.

- ^ Dylan McGrath, EE Times, "Gartner Dataquest Analyst Gives ASIC, FPGA Markets Clean Bill of Health". June 13, 2005. Retrieved February 5, 2009.

- ^ Virtex-4 Family Overview

- ^ Kuon, I.; Rose, J. (2006). Measuring the gap between FPGAs and ASICs. pp. 21. doi:10.1145/1117201.1117205.  edit

- ^ a b Tim Erjavec, White Paper, "Introducing the Xilinx Targeted Design Platform: Fulfilling the Programmable Imperative." February 2, 2009. Retrieved February 2, 2009

- ^ Huffmire Paper "Managing Security in FPGA-Based Embedded Systems." Nov-Dec 2008. Retrieved Sept 22, 2009

- ^ FPGA/DSP Blend Tackles Telecom Apps

- ^ Xilinx aims 65-nm FPGAs at DSP applications

- ^ http://www.eecg.toronto.edu/~vaughn/challenge/fpga_arch.html

- ^ http://www.altera.com/literature/hb/cyc2/cyc2_cii51002.pdf

- ^ http://www.altera.com/literature/hb/stratix-iv/stx4_5v1_01.pdf

- ^ http://www.xilinx.com/support/documentation/user_guides/ug070.pdf

- ^ http://www.xilinx.com/bvdocs/whitepapers/wp245.pdf

- ^ Seeking Alpha, "Altera and Xilinx Report: The Battle Continues". July 17, 2008. Retrieved February 5, 2009.

- ^ "Xilinx ISE WebPACK". http://www.xilinx.com/ise/logic_design_prod/webpack.htm.

- ^ "Quartus II Web edition software". https://www.altera.com/support/software/download/altera_design/quartus_we/dnl-quartus_we.jsp.

- [edit] External links

- University of North Carolina at Charlotte's Reconfigurable Computing Laboratory

- FPGA Database

Retrieved from "http://en.wikipedia.org/wiki/Field-programmable_gate_array"

- Information appliance

From Wikipedia, the free encyclopedia

Jump to: navigation, search

Not to be confused with computer appliance or Internet appliance.

A Newton PDA

In general terms, an information appliance or information device is any machine or device that is usable for the purposes of computing, telecommunicating, reproducing, and presenting encoded information in myriad forms and applications.[citation needed] The common technical usage of "information appliance" (IA) is more specific — i.e., an appliance that is specially designed to perform a specific user-friendly function —such as playing music, photography, or editing text.[1][2]

Typical examples are smartphones and personal digital assistants (PDAs). Information appliances partially overlap in definition with, or are sometimes referred to as smart devices, embedded systems, mobile devices or wireless devices.

- Contents [hide] - 1 Appliance vs computer - 2 Walled gardens versus open standards - 3 See also - 4 References - 5 External links

- [edit] Appliance vs computer

The term information appliance was coined by Jef Raskin around 1979.[3][4] As later explained by an influential Donald Norman's The Invisible Computer,[5] the main characteristics of IA, as opposed to any normal computer, were:

- designed and pre-configured for a single application (like a toaster appliance, which is designed only to make toast),

- so easy to use for untrained people, that it effectively becomes unnoticeable, "invisible" to them,

- able to automatically share information with any other IAs.

This definition of IA was different from today's. Jef Raskin initially tried to include such features in the Apple Macintosh, which he designed, but eventually the project went a quite different way. For a short while during the mid- and late 1980s, there were a few models of simple electronic typewriters with screens and some form of memory storage. These dedicated word processor machines had some of the attributes of an information appliance, and Raskin designed one of them, the Canon Cat. He described some properties of his definition of information appliance in his book The Humane Interface.

Larry Ellison, Oracle Corporation CEO, predicted that information appliances and network computers would supersede personal computers (PCs)[6]. This prediction has not yet come true.

- [edit] Walled gardens versus open standards

In an ideal world, any true information appliance would be able to communicate with any other information appliance using open standard protocols and technologies, regardless of the maker of the software or the hardware. The communications aspects and all user interface elements would be designed together so that a user could switch seamlessly from one information appliance to another.

Some vendors are attempting to create "walled gardens" of closed proprietary content for information appliances, leveraging existing proprietary technologies. However, with the exception of NTT DoCoMo's i-mode, these efforts have been less successful than predicted, due to the willingness of most vendors to work together within open standards frameworks,[citation needed] and the pre-existing widespread adoption of open standards such as GSM, IP, SMS and SMTP.

- [edit] See also

- Computer appliance

- Internet appliance

- Embedded system

- Ubiquitous computing

- Archy

- Mobile web

- [edit] References

- ^ Pirhonen, A.; Isomäki, H.; Roast, C.; Saariluoma, Pertti. Future Interaction Design. Springer. pp. 129. ISBN 1-85233-791-5. http://books.google.com/books?id=vzaBpX9JVu0C&pg=PA129&dq=computer+IT+appliance+date:1970-2005&lr=&num=100&as_brr=3&sig=1jKI4yt3p1Pp-agVS_o6motyx7w#PPA129,M1. Retrieved 2008-05-06.

- ^ Benyon, David; Turner, Phil; Turner, Susan. Designing Interactive Systems: People, Activities, Contexts, Technologies. Addison Wesley Publishing Company. pp. 18. ISBN 0-321-11629-1. http://books.google.com/books?id=iWe7VkFW0zMC&pg=PA18&dq=Weiser+appliance+date:2003-2009&lr=&num=100&as_brr=3&sig=ACni7PiLgT8RBhJsyH1YgZkFm-M#PPA18,M1. Retrieved 2008-05-06.

- ^ Bergman, Eric. Information Appliances and Beyond (Interactive Technologies). Morgan Kaufmann. pp. 2–3. ISBN 1-55860-600-9. http://books.google.com/books?id=wtPlin2FNXMC&printsec=frontcover#PPA3,M1. Retrieved 2008-05-06.

- ^ Allan, Roy (2001). A history of the personal computer: the people and the technology. London, Ont.: Allan Pub.. pp. 49. ISBN 0-9689108-0-7. http://books.google.com/books?id=FLabRYnGrOcC&pg=PA12&dq=computing+IT+appliance+date:1970-2005&lr=&num=100&as_brr=1&sig=1Blpya3Kinpo1kHOvWSNIJAF2T4#PPA49,M1. Retrieved 2008-05-06.

- ^ Norman, Donald A. (1998). The invisible computer: why good products can fail, the personal computer is so complex, and information appliances are the solution. Cambridge, Mass.: MIT Press. ISBN 0-262-64041-4. http://isbndb.com/d/book/the_invisible_computer_a01.html.

- ^ Walters, E. Garrison (2001). The essential guide to computing. Upper Saddle River, NJ: Prentice Hall PTR. pp. 13. ISBN 0-13-019469-7. http://books.google.com/books?id=AwrQsOW5SsQC&pg=PA398&dq=information+network+desktop+computer+IT+appliance+date:1970-2005&lr=&num=100&as_brr=0&sig=qXs4I4h6ZEKVisWrGwM81r6m1ig#PPA398,M1. Retrieved 2008-05-06.

- [edit] External links

|  | Look up Appliance in Wiktionary, the free dictionary. |
| --- | --- |

- A blog that covers information appliances including Internet TVs, Media players and widget devices

- Compact HTML for Small Information Appliances — W3C NOTE 09-Feb-1998

- IBM:A universal information appliance

[hide] v • d • e Computer sizes. Classes of computers. Larger; Super · Minisuper · Mainframe · Mini (Midrange) · Supermini · Server. Micro; Personal (Workstation · Desktop · Home · SFF (Nettop)) · Plug · Portable · Arcade system board · Video game console. Mobile; Portable/Mobile data terminal · Electronic organizer · Pocket computer. Laptop; Desktop replacement computer · Subnotebook (Netbook · Smartbook). Tablet computer; Tablet personal computer (Ultra-Mobile PC) · Mobile Internet device (Internet Tablet). Wearable computer; Calculator watch · Wristop · Virtual retinal display · Head-mounted display (Head-up display). Information appliance; PDA (Palm-size PC · Handheld PC · Pocket computer) · EDA · Mobile phone (Smartphone · Feature phone) · PMP · DAP · E-book reader · Handheld game console. Calculators; Scientific · Programmable · Graphing.. Others; Single-board computer · Wireless sensor network · Microcontroller · Smartdust · Nanocomputer · Pizza Box Case.

- Microprocessor

From Wikipedia, the free encyclopedia

Jump to: navigation, search

Intel 4004, the first general-purpose, commercial microprocessor

A microprocessor incorporates most or all of the functions of a computer's central processing unit (CPU) on a single integrated circuit (IC, or microchip).[1] The first microprocessors emerged in the early 1970s and were used for electronic calculators, using binary-coded decimal (BCD) arithmetic on 4-bit words. Other embedded uses of 4-bit and 8-bit microprocessors, such as terminals, printers, various kinds of automation etc., followed soon after. Affordable 8-bit microprocessors with 16-bit addressing also led to the first general-purpose microcomputers from the mid-1970s on.

During the 1960s, computer processors were often constructed out of small and medium-scale ICs containing from tens to a few hundred transistors. The integration of a whole CPU onto a single chip greatly reduced the cost of processing power. From these humble beginnings, continued increases in microprocessor capacity have rendered other forms of computers almost completely obsolete (see history of computing hardware), with one or more microprocessors used in everything from the smallest embedded systems and handheld devices to the largest mainframes and supercomputers.

Since the early 1970s, the increase in capacity of microprocessors has been a consequence of Moore's Law, which suggests that the number of transistors that can be fitted onto a chip doubles every two years. Although originally calculated as a doubling every year,[2] Moore later refined the period to two years.[3]

In the late 1990s, and in the high-performance microprocessor segment, heat generation (TDP), due to switching losses, static current leakage, and other factors, emerged as a leading developmental constraint.[4]

- Contents [hide] - 1 Firsts - 1.1 Intel 4004 - 1.2 TMS 1000 - 1.3 Pico/General Instrument - 1.4 CADC - 1.5 Gilbert Hyatt - 1.6 Four-Phase Systems AL1 - 2 8-bit designs - 3 12-bit designs - 4 16-bit designs - 5 32-bit designs - 6 64-bit designs in personal computers - 7 Multicore designs - 8 RISC - 9 Special-purpose designs - 10 Market statistics - 11 See also - 12 Notes and references - 13 External links

- [edit] Firsts

Three projects delivered a microprocessor at about the same time: Intel's 4004, Texas Instruments (TI) TMS 1000, and Garrett AiResearch's Central Air Data Computer (CADC).

- [edit] Intel 4004

The 4004 with cover removed (left) and as actually used (right).

The Intel 4004 is generally regarded as the first microprocessor,[5][6] and cost thousands of dollars.[7] The first known advertisement for the 4004 is dated November 1971 and appeared in Electronic News.[8][dead link] The project that produced the 4004 originated in 1969, when Busicom, a Japanese calculator manufacturer, asked Intel to build a chipset for high-performance desktop calculators. Busicom's original design called for a programmable chip set consisting of seven different chips. Three of the chips were to make a special-purpose CPU with its program stored in ROM and its data stored in shift register read-write memory. Ted Hoff, the Intel engineer assigned to evaluate the project, believed the Busicom design could be simplified by using dynamic RAM storage for data, rather than shift register memory, and a more traditional general-purpose CPU architecture. Hoff came up with a four–chip architectural proposal: a ROM chip for storing the programs, a dynamic RAM chip for storing data, a simple I/O device and a 4-bit central processing unit (CPU). Although not a chip designer, he felt the CPU could be integrated into a single chip. This chip would later be called the 4004 microprocessor.

The architecture and specifications of the 4004 came from the interaction of Hoff with Stanley Mazor, a software engineer reporting to him, and with Busicom engineer Masatoshi Shima, during 1969. In April 1970, Intel hired Federico Faggin to lead the design of the four-chip set. Faggin, who originally developed the silicon gate technology (SGT) in 1968 at Fairchild Semiconductor[9] and designed the world’s first commercial integrated circuit using SGT, the Fairchild 3708, had the correct background to lead the project since it was SGT that made it possible to implement a single-chip CPU with the proper speed, power dissipation and cost. Faggin also developed the new methodology for random logic design, based on silicon gate, that made the 4004 possible. Production units of the 4004 were first delivered to Busicom in March 1971 and shipped to other customers in late 1971.

- [edit] TMS 1000

The Smithsonian Institution says TI engineers Gary Boone and Michael Cochran succeeded in creating the first microcontroller (also called a microcomputer) in 1971. The result of their work was the TMS 1000, which went commercial in 1974.[10]

TI developed the 4-bit TMS 1000 and stressed pre-programmed embedded applications, introducing a version called the TMS1802NC on September 17, 1971 which implemented a calculator on a chip.

TI filed for the patent on the microprocessor. Gary Boone was awarded U.S. Patent 3,757,306 for the single-chip microprocessor architecture on September 4, 1973. It may never be known which company actually had the first working microprocessor running on the lab bench. In both 1971 and 1976, Intel and TI entered into broad patent cross-licensing agreements, with Intel paying royalties to TI for the microprocessor patent. A nice history of these events is contained in court documentation from a legal dispute between Cyrix and Intel, with TI as intervenor and owner of the microprocessor patent.

A computer-on-a-chip is a variation of a microprocessor that combines the microprocessor core (CPU), some program memory and read/write memory, and I/O (input/output) lines onto one chip. The computer-on-a-chip patent, called the "microcomputer patent" at the time, U.S. Patent 4,074,351, was awarded to Gary Boone and Michael J. Cochran of TI. Aside from this patent, the standard meaning of microcomputer is a computer using one or more microprocessors as its CPU(s), while the concept defined in the patent is more akin to a microcontroller.

- [edit] Pico/General Instrument

The PICO1/GI250 chip introduced in 1971. This was designed by Pico Electronics (Glenrothes, Scotland) and manufactured by General Instrument of Hicksville NY

In 1971 Pico Electronics[11] and General Instrument (GI) introduced their first collaboration in ICs, a complete single chip calculator IC for the Monroe/Litton Royal Digital III calculator. This chip could also arguably lay claim to be one of the first microprocessors or microcontrollers having ROM, RAM and a RISC instruction set on-chip. The layout for the four layers of the PMOS process was hand drawn at x500 scale on mylar film, a significant task at the time given the complexity of the chip.

Pico was a spinout by five GI design engineers whose vision was to create single chip calculator ICs. They had significant previous design experience on multiple calculator chipsets with both GI and Marconi-Elliott.[12] The key team members had originally been tasked by Elliott Automation to create an 8 bit computer in MOS and had helped establish a MOS Research Laboratory in Glenrothes, Scotland in 1967.

Calculators were becoming the largest single market for semiconductors and Pico and GI went on to have significant success in this burgeoning market. GI continued to innovate in microprocessors and microcontrollers with products including the PIC1600, PIC1640 and PIC1650. In 1985 the GI Microelectronics business was spun out into the very successful PIC_microcontroller business.

- [edit] CADC

|  | This section needs references that appear in reliable third-party publications. Primary sources or sources affiliated with the subject are generally not sufficient for a Wikipedia article. Please add more appropriate citations from reliable sources. (March 2010) |
| --- | --- |

In 1968, Garrett AiResearch (which employed designers Ray Holt and Steve Geller) was invited to produce a digital computer to compete with electromechanical systems then under development for the main flight control computer in the US Navy's new F-14 Tomcat fighter. The design was complete by 1970, and used a MOS-based chipset as the core CPU. The design was significantly (approximately 20 times) smaller and much more reliable than the mechanical systems it competed against, and was used in all of the early Tomcat models. This system contained "a 20-bit, pipelined, parallel multi-microprocessor". The Navy refused to allow publication of the design until 1997. For this reason the CADC, and the MP944 chipset it used, are fairly unknown.[13] Ray Holt graduated California Polytechnic University in 1968, and began his computer design career with the CADC. From its inception, it was shrouded in secrecy until 1998 when at Holt's request, the US Navy allowed the documents into the public domain. Since then several[who?] have debated if this was the first microprocessor. Holt has stated that no one has compared this microprocessor with those that came later.[14] According to Parab et al. (2007), "The scientific papers and literature published around 1971 reveal that the MP944 digital processor used for the F-14 Tomcat aircraft of the US Navy qualifies as the first microprocessor. Although interesting, it was not a single-chip processor, and was not general purpose – it was more like a set of parallel building blocks you could use to make a special-purpose DSP form. It indicates that today’s industry theme of converging DSP-microcontroller architectures was started in 1971."[15] This convergence of DSP and microcontroller architectures is known as a Digital Signal Controller.[citation needed]

- [edit] Gilbert Hyatt

Gilbert Hyatt was awarded a patent claiming an invention pre-dating both TI and Intel, describing a "microcontroller".[16] The patent was later invalidated, but not before substantial royalties were paid out.[17][18]

- [edit] Four-Phase Systems AL1

The Four-Phase Systems AL1 was an 8-bit bit slice chip containing eight registers and an ALU.[19] It was designed by Lee Boysel in 1969.[20][21][22] At the time, it formed part of a nine-chip, 24-bit CPU with three AL1s, but it was later called a microprocessor when, in response to 1990s litigation by Texas Instruments, a demonstration system was constructed where a single AL1 formed part of a courtroom demonstration computer system, together with RAM, ROM, and an input-output device.[23]

- [edit] 8-bit designs

The Intel 4004 was followed in 1972 by the Intel 8008, the world's first 8-bit microprocessor. According to A History of Modern Computing, (MIT Press), pp. 220–21, Intel entered into a contract with Computer Terminals Corporation, later called Datapoint, of San Antonio TX, for a chip for a terminal they were designing. Datapoint later decided not to use the chip, and Intel marketed it as the 8008 in April, 1972. This was the world's first 8-bit microprocessor. It was the basis for the famous "Mark-8" computer kit advertised in the magazine Radio-Electronics in 1974.

The 8008 was the precursor to the very successful Intel 8080 (1974), Zilog Z80 (1976), and derivative Intel 8-bit processors. The competing Motorola 6800 was released August 1974 and the similar MOS Technology 6502 in 1975 (designed largely by the same people). The 6502 rivaled the Z80 in popularity during the 1980s.

A low overall cost, small packaging, simple computer bus requirements, and sometimes the integration of extra circuitry (e.g. the Z80's built-in memory refresh circuitry) allowed the home computer "revolution" to accelerate sharply in the early 1980s. This delivered such inexpensive machines as the Sinclair ZX-81, which sold for US$99.

The Western Design Center, Inc. (WDC) introduced the CMOS 65C02 in 1982 and licensed the design to several firms. It was used as the CPU in the Apple IIe and IIc personal computers as well as in medical implantable grade pacemakers and defibrilators, automotive, industrial and consumer devices. WDC pioneered the licensing of microprocessor designs, later followed by ARM and other microprocessor Intellectual Property (IP) providers in the 1990s.

Motorola introduced the MC6809 in 1978, an ambitious and thought-through 8-bit design source compatible with the 6800 and implemented using purely hard-wired logic. (Subsequent 16-bit microprocessors typically used microcode to some extent, as CISC design requirements were getting too complex for purely hard-wired logic only.)

Another early 8-bit microprocessor was the Signetics 2650, which enjoyed a brief surge of interest due to its innovative and powerful instruction set architecture.

A seminal microprocessor in the world of spaceflight was RCA's RCA 1802 (aka CDP1802, RCA COSMAC) (introduced in 1976), which was used onboard the Galileo probe to Jupiter (launched 1989, arrived 1995). RCA COSMAC was the first to implement CMOS technology. The CDP1802 was used because it could be run at very low power, and because a variant was available fabricated using a special production process (Silicon on Sapphire), providing much better protection against cosmic radiation and electrostatic discharges than that of any other processor of the era. Thus, the SOS version of the 1802 was said to be the first radiation-hardened microprocessor.

The RCA 1802 had what is called a static design, meaning that the clock frequency could be made arbitrarily low, even to 0 Hz, a total stop condition. This let the Galileo spacecraft use minimum electric power for long uneventful stretches of a voyage. Timers and/or sensors would awaken/improve the performance of the processor in time for important tasks, such as navigation updates, attitude control, data acquisition, and radio communication.

- [edit] 12-bit designs

The Intersil 6100 family consisted of a 12-bit microprocessor (the 6100) and a range of peripheral support and memory ICs. The microprocessor recognised the DEC PDP-8 minicomputer instruction set. As such it was sometimes referred to as the CMOS-PDP8. Since it was also produced by Harris Corporation, it was also known as the Harris HM-6100. By virtue of its CMOS technology and associated benefits, the 6100 was being incorporated into some military designs until the early 1980s.

- [edit] 16-bit designs

The first multi-chip 16-bit microprocessor was the National Semiconductor IMP-16, introduced in early 1973. An 8-bit version of the chipset was introduced in 1974 as the IMP-8.

Other early multi-chip 16-bit microprocessors include one used by Digital Equipment Corporation (DEC) in the LSI-11 OEM board set and the packaged PDP 11/03 minicomputer, and the Fairchild Semiconductor MicroFlame 9440, both of which were introduced in the 1975 to 1976 timeframe.

In 1975, National introduced the first 16-bit single-chip microprocessor, the National Semiconductor PACE, which was later followed by an NMOS version, the INS8900.

Another early single-chip 16-bit microprocessor was TI's TMS 9900, which was also compatible with their TI-990 line of minicomputers. The 9900 was used in the TI 990/4 minicomputer, the TI-99/4A home computer, and the TM990 line of OEM microcomputer boards. The chip was packaged in a large ceramic 64-pin DIP package, while most 8-bit microprocessors such as the Intel 8080 used the more common, smaller, and less expensive plastic 40-pin DIP. A follow-on chip, the TMS 9980, was designed to compete with the Intel 8080, had the full TI 990 16-bit instruction set, used a plastic 40-pin package, moved data 8 bits at a time, but could only address 16 KB. A third chip, the TMS 9995, was a new design. The family later expanded to include the 99105 and 99110.

The Western Design Center, Inc. (WDC) introduced the CMOS 65816 16-bit upgrade of the WDC CMOS 65C02 in 1984. The 65816 16-bit microprocessor was the core of the Apple IIgs and later the Super Nintendo Entertainment System, making it one of the most popular 16-bit designs of all time.

Intel followed a different path, having no minicomputers to emulate, and instead "upsized" their 8080 design into the 16-bit Intel 8086, the first member of the x86 family, which powers most modern PC type computers. Intel introduced the 8086 as a cost effective way of porting software from the 8080 lines, and succeeded in winning much business on that premise. The 8088, a version of the 8086 that used an external 8-bit data bus, was the microprocessor in the first IBM PC, the model 5150. Following up their 8086 and 8088, Intel released the 80186, 80286 and, in 1985, the 32-bit 80386, cementing their PC market dominance with the processor family's backwards compatibility. The 8086 and 80186 had a crude method of segmentation, while the 80286 introduced a full-featured semgented memory management unit (MMU), and the 80386 introduced a flat 32-bit memory model with paged memory management.

- [edit] 32-bit designs

Upper interconnect layers on an Intel 80486DX2 die.

16-bit designs had only been on the market briefly when 32-bit implementations started to appear.

The most significant of the 32-bit designs is the MC68000, introduced in 1979. The 68K, as it was widely known, had 32-bit registers but used 16-bit internal data paths and a 16-bit external data bus to reduce pin count, and supported only 24-bit addresses. Motorola generally described it as a 16-bit processor, though it clearly has 32-bit architecture. The combination of high performance, large (16 megabytes or 224 bytes) memory space and fairly low cost made it the most popular CPU design of its class. The Apple Lisa and Macintosh designs made use of the 68000, as did a host of other designs in the mid-1980s, including the Atari ST and Commodore Amiga.

The world's first single-chip fully-32-bit microprocessor, with 32-bit data paths, 32-bit buses, and 32-bit addresses, was the AT&T Bell Labs BELLMAC-32A, with first samples in 1980, and general production in 1982[24][25] After the divestiture of AT&T in 1984, it was renamed the WE 32000 (WE for Western Electric), and had two follow-on generations, the WE 32100 and WE 32200. These microprocessors were used in the AT&T 3B5 and 3B15 minicomputers; in the 3B2, the world's first desktop supermicrocomputer; in the "Companion", the world's first 32-bit laptop computer; and in "Alexander", the world's first book-sized supermicrocomputer, featuring ROM-pack memory cartridges similar to today's gaming consoles. All these systems ran the UNIX System V operating system.

Intel's first 32-bit microprocessor was the iAPX 432, which was introduced in 1981 but was not a commercial success. It had an advanced capability-based object-oriented architecture, but poor performance compared to contemporary architectures such as Intel's own 80286 (introduced 1982), which was almost four times as fast on typical benchmark tests. However, the results for the iAPX432 was partly due to a rushed and therefore suboptimal Ada compiler.

The ARM first appeared in 1985. This is a RISC processor design, which has since come to dominate the 32-bit embedded systems processor space due in large part to its power efficiency, its licensing model, and its wide selection of system development tools. Semiconductor manufacturers generally license cores such as the ARM11 and integrate them into their own system on a chip products; only a few such vendors are licensed to modify the ARM cores. Most cell phones include an ARM processor, as do a wide variety of other products. There are microcontroller-oriented ARM cores without virtual memory support, as well as SMP applications processors with virtual memory.

Motorola's success with the 68000 led to the MC68010, which added virtual memory support. The MC68020, introduced in 1985 added full 32-bit data and address busses. The 68020 became hugely popular in the Unix supermicrocomputer market, and many small companies (e.g., Altos, Charles River Data Systems) produced desktop-size systems. The MC68030 was introduced next, improving upon the previous design by integrating the MMU into the chip. The continued success led to the MC68040, which included an FPU for better math performance. A 68050 failed to achieve its performance goals and was not released, and the follow-up MC68060 was released into a market saturated by much faster RISC designs. The 68K family faded from the desktop in the early 1990s.

Other large companies designed the 68020 and follow-ons into embedded equipment. At one point, there were more 68020s in embedded equipment than there were Intel Pentiums in PCs.[26] The ColdFire processor cores are derivatives of the venerable 68020.

During this time (early to mid-1980s), National Semiconductor introduced a very similar 16-bit pinout, 32-bit internal microprocessor called the NS 16032 (later renamed 32016), the full 32-bit version named the NS 32032. Later the NS 32132 was introduced which allowed two CPUs to reside on the same memory bus, with built in arbitration. The NS32016/32 outperformed the MC68000/10 but the NS32332 which arrived at approximately the same time the MC68020 did not have enough performance. The third generation chip, the NS32532 was different. It had about double the performance of the MC68030 which was released around the same time. The appearance of RISC processors like the AM29000 and MC88000 (now both dead) influenced the architecture of the final core, the NS32764. Technically advanced, using a superscalar RISC core, internally overclocked, with a 64 bit bus, it was still capable of executing Series 32000 instructions through real time translation. When National Semiconductor decided to leave the Unix market, the chip was redesigned into the Swordfish Embedded processor with a set of on chip peripherals. The chip turned out to be too expensive for the laser printer market and was killed. The design team went to Intel and there designed the Pentium processor which is very similar to the NS32764 core internally The big success of the Series 32000 was in the laser printer market, where the NS32CG16 with microcoded BitBlt instructions had very good price/performance and was adopted by large companies like Canon. By the mid-1980s, Sequent introduced the first symmetric multiprocessor (SMP) server-class computer using the NS 32032. This was one of the design's few wins, and it disappeared in the late 1980s.

The MIPS R2000 (1984) and R3000 (1989) were highly successful 32-bit RISC microprocessors. They were used in high-end workstations and servers by SGI, among others.

Other designs included the interesting Zilog Z80000, which arrived too late to market to stand a chance and disappeared quickly.

In the late 1980s, "microprocessor wars" started killing off some of the microprocessors. Apparently, with only one major design win, Sequent, the NS 32032 just faded out of existence, and Sequent switched to Intel microprocessors.

From 1985 to 2003, the 32-bit x86 architectures became increasingly dominant in desktop, laptop, and server markets, and these microprocessors became faster and more capable. Intel had licensed early versions of the architecture to other companies, but declined to license the Pentium, so AMD and Cyrix built later versions of the architecture based on their own designs. During this span, these processors increased in complexity (transistor count) and capability (instructions/second) by at least three orders of magnitude. Intel's Pentium line is probably the most famous and recognizable 32-bit processor model, at least with the public at large.

- [edit] 64-bit designs in personal computers

While 64-bit microprocessor designs have been in use in several markets since the early 1990s, the early 2000s saw the introduction of 64-bit microprocessors targeted at the PC market.

With AMD's introduction of a 64-bit architecture backwards-compatible with x86, x86-64 (also called AMD64), in September 2003, followed by Intel's near fully compatible 64-bit extensions (first called IA-32e or EM64T, later renamed Intel 64), the 64-bit desktop era began. Both versions can run 32-bit legacy applications without any performance penalty as well as new 64-bit software. With operating systems Windows XP x64, Windows Vista x64, Windows 7 x64, Linux, BSD and Mac OS X that run 64-bit native, the software is also geared to fully utilize the capabilities of such processors. The move to 64 bits is more than just an increase in register size from the IA-32 as it also doubles the number of general-purpose registers.

The move to 64 bits by PowerPC processors had been intended since the processors' design in the early 90s and was not a major cause of incompatibility. Existing integer registers are extended as are all related data pathways, but, as was the case with IA-32, both floating point and vector units had been operating at or above 64 bits for several years. Unlike what happened when IA-32 was extended to x86-64, no new general purpose registers were added in 64-bit PowerPC, so any performance gained when using the 64-bit mode for applications making no use of the larger address space is minimal.

- [edit] Multicore designs

Front of Pentium D dual core processor

Back of Pentium D dual core processor

Main article: Multi-core (computing)

A different approach to improving a computer's performance is to add extra processors, as in symmetric multiprocessing designs, which have been popular in servers and workstations since the early 1990s. Keeping up with Moore's Law is becoming increasingly challenging as chip-making technologies approach the physical limits of the technology.

In response, the microprocessor manufacturers look for other ways to improve performance, in order to hold on to the momentum of constant upgrades in the market.

A multi-core processor is simply a single chip containing more than one microprocessor core, effectively multiplying the potential performance with the number of cores (as long as the operating system and software is designed to take advantage of more than one processor). Some components, such as bus interface and second level cache, may be shared between cores. Because the cores are physically very close they interface at much faster clock rates compared to discrete multiprocessor systems, improving overall system performance.

In 2005, the first personal computer dual-core processors were announced and as of 2009 dual-core and quad-core processors are widely used in servers, workstations and PCs while six and eight-core processors will be available for high-end applications in both the home and professional environments.

Sun Microsystems has released the Niagara and Niagara 2 chips, both of which feature an eight-core design. The Niagara 2 supports more threads and operates at 1.6 GHz.

High-end Intel Xeon processors that are on the LGA771 socket are DP (dual processor) capable, as well as the Intel Core 2 Extreme QX9775 also used in the Mac Pro by Apple and the Intel Skulltrail motherboard. With the transition to the LGA1366 and LGA1156 socket and the Intel i7 and i5 chips, quad core is now considered mainstream, but with the release of the i7-980x, six core processors are now well within reach.

- [edit] RISC

In the mid-1980s to early-1990s, a crop of new high-performance Reduced Instruction Set Computer (RISC) microprocessors appeared, influenced by discrete RISC-like CPU designs such as the IBM 801 and others. RISC microprocessors were initially used in special-purpose machines and Unix workstations, but then gained wide acceptance in other roles.

In 1986, HP released its first system with a PA-RISC CPU. The first commercial RISC microprocessor design was released either by MIPS Computer Systems, the 32-bit R2000 (the R1000 was not released) or by Acorn computers, the 32-bit ARM2 in 1987.[citation needed] The R3000 made the design truly practical, and the R4000 introduced the world's first commercially available 64-bit RISC microprocessor. Competing projects would result in the IBM POWER and Sun SPARC architectures. Soon every major vendor was releasing a RISC design, including the AT&T CRISP, AMD 29000, Intel i860 and Intel i960, Motorola 88000, DEC Alpha.

As of 2007, two 64-bit RISC architectures are still produced in volume for non-embedded applications: SPARC and Power ISA.

- [edit] Special-purpose designs

Though the term "microprocessor" has traditionally referred to a single- or multi-chip CPU or system-on-a-chip (SoC), several types of specialized processing devices have followed from the technology. The most common examples are microcontrollers, digital signal processors (DSP) and graphics processing units (GPU). Many examples of these are either not programmable, or have limited programming facilities. For example, in general GPUs through the 1990s were mostly non-programmable and have only recently gained limited facilities like programmable vertex shaders. There is no universal consensus on what defines a "microprocessor", but it is usually safe to assume that the term refers to a general-purpose CPU of some sort and not a special-purpose processor unless specifically noted.

- [edit] Market statistics

In 2003, about $44 billion (USD) worth of microprocessors were manufactured and sold.[27] Although about half of that money was spent on CPUs used in desktop or laptop personal computers, those count for only about 0.2% of all CPUs sold.[citation needed]

About 55% of all CPUs sold in the world are 8-bit microcontrollers, over two billion of which were sold in 1997.[28]

As of 2002, less than 10% of all the CPUs sold in the world are 32-bit or more. Of all the 32-bit CPUs sold, about 2% are used in desktop or laptop personal computers. Most microprocessors are used in embedded control applications such as household appliances, automobiles, and computer peripherals. Taken as a whole, the average price for a microprocessor, microcontroller, or DSP is just over $6.[29]

About ten billion CPUs were manufactured in 2008. About 98% of new CPUs produced each year are embedded.[30]

- [edit] See also

- Microprocessor chronology

- List of instruction sets

- List of microprocessors

- Comparison of CPU architectures

- Arithmetic logic unit

- Central processing unit

- Floating point unit

- Digital signal processor

- Computer architecture

- Computer engineering

- CPU design

- Instruction set

- Microarchitecture

- Microcode

- Microcontroller

- [edit] Notes and references

- ^ Osborne, Adam (1980). An Introduction to Microcomputers. Volume 1: Basic Concepts (2nd ed.). Berkely, California: Osborne-McGraw Hill. ISBN 0-931988-34-9.

- ^ Moore, Gordon (19 April 1965). "Cramming more components onto integrated circuits" (PDF). Electronics 38 (8). ftp://download.intel.com/museum/Moores_Law/Articles-Press_Releases/Gordon_Moore_1965_Article.pdf. Retrieved 2009-12-23.

- ^ (PDF) Excerpts from A Conversation with Gordon Moore: Moore’s Law. Intel. 2005. ftp://download.intel.com/museum/Moores_Law/Video-Transcripts/Excepts_A_Conversation_with_Gordon_Moore.pdf. Retrieved 2009-12-23.  It is often incorrectly quoted as a doubling of transistors every 18 months.}}

- ^ Hodgin, Rick (3 December 2007). "Six fold reduction in semiconductor power loss, a faster, lower heat process technology". TG Daily (DD&M). http://www.tgdaily.com/content/view/35094/113/. Retrieved 2009-12-23.

- ^ Mack, Pamela E. (30 November 2005). "The Microcomputer Revolution". http://www.clemson.edu/caah/history/FacultyPages/PamMack/lec122/micro.htm. Retrieved 2009-12-23.

- ^ (PDF) History in the Computing Curriculum. http://www.hofstra.edu/pdf/CompHist_9812tla6.PDF. Retrieved 2009-12-23.

- ^ Karam, Andrew P. (2000). "Advances in Microprocessor Technology". In Schlager, Neil; Lauer, Josh. Science and Its Times. Farmington Hills, MI: The Gail Group. pp. 525–528.

- ^ Faggin, Federico; Hoff, Marcian E., Jr.; Mazor, Stanley; Shima, Masatoshi (1996). "The History of the 4004" (PDF). IEEE Micro. http://www.cse.nd.edu/courses/cse30322/www/hw/history_of_4004.pdf. Retrieved 2009-12-23. [dead link]

- ^ Faggin, F.; Klein, T.; L. (23 October 1968). "Insulated Gate Field Effect Transistor Integrated Circuits with Silicon Gates" (JPEG image). International Electronic Devices Meeting. IEEE Electron Devices Group. http://www.intel4004.com/images/iedm_covart.jpg. Retrieved 2009-12-23.

- ^ Augarten, Stan (1983). The Most Widely Used Computer on a Chip: The TMS 1000. New Haven and New York: Ticknor & Fields. ISBN 0-89919-195-9. http://smithsonianchips.si.edu/augarten/p38.htm. Retrieved 2009-12-23.

- ^ McGonigal, James (20 September 2006). "Microprocessor History: Foundations in Glenrothes, Scotland". McGonigal personal website accessdate=2009-12-23. http://www.spingal.plus.com/micro.

- ^ Tout, Nigel. "ANITA at its Zenith". Bell Punch Company and the ANITA calculators. http://anita-calculators.info/html/anita_at_its_zenith.html. Retrieved 2010-07-25.

- ^ Holt, Ray M.. "World’s First Microprocessor Chip Set". Ray M. Holt website. Archived from the original on 2010-07-25. http://web.archive.org/web/20100725060322/http://www.microcomputerhistory.com/index.php. Retrieved 2010-07-25.

- ^ Holt, Ray. "Lecture: Microprocessor Design and Development for the US Navy F14 FighterJet" Room 8220, Wean Hall, Carnegie Mellon University, Pittsburgh, PA, US (27 September 2001). Retrieved on 2010-07-25.

- ^ Parab, Jivan S.; Shelake, Vinod G.; Kamat, Rajanish K.; Naik, Gourish M. (2007) (PDF). Exploring C for Microcontrollers: A Hands on Approach. Springer. p. 4. ISBN 978-1-4020-6067-0. http://ee.sharif.edu/~sakhtar3/books/Exploring%20C%20for%20Microcontrollers.pdf. Retrieved 2010-07-25.

- ^ Hyatt, Gilbert P., "Single chip integrated circuit computer architecture", Patent 4942516, issued July 17, 1990

- ^ "The Gilbert Hyatt Patent". intel4004.com. Federico Faggin. http://www.intel4004.com/hyatt.htm. Retrieved 2009-12-23.

- ^ Crouch, Dennis (1 July 2007). "Written Description: CAFC Finds Prima Facie Rejection(Hyatt v. Dudas (Fed. Cir. 2007)". Patently-O blog. http://www.patentlyo.com/patent/2007/07/hyatt-v-dudas-f.html. Retrieved 2009-12-23.

- ^ Basset, Ross (2003). "When is a Microprocessor not a Microprocessor? The Industrial Construction of Semiconductor Innovation". In Finn, Bernard. Exposing Electronics. Michigan State University Press. p. 121. ISBN 0-87013-658-5. http://books.google.com/books?id=rsRJTiu1h9MC.

- ^ "1971 - Microprocessor Integrates CPU Function onto a Single Chip". The Silicon Engine. Computer History Museum. http://www.computerhistory.org/semiconductor/timeline/1971-MPU.html. Retrieved 2010-07-25.

- ^ Shaller, Robert R. (15 April 2004). "Dissertation: Technological Innovation in the Semiconductor Industry: A Case Study of the International Technology Roadmap for Semiconductors" (PDF). George Mason University. http://web.archive.org/web/20061219012629/http://home.comcast.net/~gordonepeterson2/schaller_dissertation_2004.pdf. Retrieved 2010-07-25.

- ^ RW (3 March 1995). "Interview with Gordon E. Moore". LAIR History of Science and Technology Collections. Los Altos Hills, California: Stanford University. http://www-sul.stanford.edu/depts/hasrg/histsci/silicongenesis/moore-ntb.html.

- ^ Bassett 2003. pp. 115, 122.

- ^ "Shoji, M. Bibliography". Bell Laboratories. 7 October 1998. http://cm.bell-labs.com/cm/cs/bib/shoji.bib. Retrieved 2009-12-23.

- ^ "Timeline: 1982–1984". Physical Sciences & Communications at Bell Labs. Bell Labs, Alcatel-Lucent. 17 January 2001. http://www.bell-labs.com/org/physicalsciences/timeline/span23.html. Retrieved 2009-12-23.

- ^ Turley, Jim (July 1998). "MCore: Does Motorola Need Another Processor Family?". Embedded Systems Design. TechInsights (United Business Media). Archived from the original on 1998-07-02. http://web.archive.org/web/19980702003323/http://www.embedded.com/98/9807sr.htm. Retrieved 2009-12-23.

- ^ World Semiconductor Trade Statistics. "WSTS Semiconductor Market Forecast World Release Date: 1 June 2004 - 6:00 UTC". Press release. Archived from the original on 2004-12-07. http://web.archive.org/web/20041207091926/http://www.wsts.org/press.html.

- ^ Cantrell, Tom (1998). "Microchip on the March". Archived from the original on 2007-02-20. http://web.archive.org/web/20070220134759/http://www.circuitcellar.com/library/designforum/silicon_update/3/index.asp.

- ^ Turley, Jim (18 December 2002). "The Two Percent Solution". Embedded Systems Design. TechInsights (United Business Media). http://www.embedded.com/shared/printableArticle.jhtml?articleID=9900861. Retrieved 2009-12-23.

- ^ Barr, Michael (1 August 2009). "Real men program in C". Embedded Systems Design. TechInsights (United Business Media). p. 2. http://www.embedded.com/columns/barrcode/218600142?pgno=2. Retrieved 2009-12-23.

- Ray, A. K.; Bhurchand, K.M.. Advanced Microprocessors and Peripherals. India: Tata McGraw-Hill.

- [edit] External links

|  | Wikiversity has learning materials about Introduction to Computers/Processor |
| --- | --- |

|  | Wikimedia Commons has media related to: Microprocessors |
| --- | --- |

- Dirk Oppelt. "The CPU Collection". http://www.cpu-collection.de/. Retrieved 2009-12-23.

- Gennadiy Shvets. "CPU-World". http://www.cpu-world.com/. Retrieved 2009-12-23.

- Jérôme Cremet. "The Cecko's CPU Library". http://gecko54000.free.fr/. Retrieved 2009-12-23.

- "How Microprocessors Work". http://computer.howstuffworks.com/microprocessor.htm. Retrieved 2009-12-23.

- William Blair. "IC Die Photography". http://diephotos.blogspot.com/. Retrieved 2009-12-23.

- John Bayko (December 2003). "Great Microprocessors

- Microcontroller

From Wikipedia, the free encyclopedia

Jump to: navigation, search

|  | This article needs additional citations for verification. Please help improve this article by adding reliable references. Unsourced material may be challenged and removed. (November 2009) |
| --- | --- |

The die from an Intel 8742, an 8-bit microcontroller that includes a CPU running at 12 MHz, 128 bytes of RAM, 2048 bytes of EPROM, and I/O in the same chip.

A microcontroller (sometimes abbreviated µC, uC or MCU) is a small computer on a single integrated circuit containing a processor core, memory, and programmable input/output peripherals. Program memory in the form of NOR flash or OTP ROM is also often included on chip, as well as a typically small amount of RAM. Microcontrollers are designed for embedded applications, in contrast to the microprocessors used in personal computers or other general purpose applications.

Microcontrollers are used in automatically controlled products and devices, such as automobile engine control systems, implantable medical devices, remote controls, office machines, appliances, power tools, and toys. By reducing the size and cost compared to a design that uses a separate microprocessor, memory, and input/output devices, microcontrollers make it economical to digitally control even more devices and processes. Mixed signal microcontrollers are common, integrating analog components needed to control non-digital electronic systems.

Some microcontrollers may use four-bit words and operate at clock rate frequencies as low as 4 kHz, for low power consumption (milliwatts or microwatts). They will generally have the ability to retain functionality while waiting for an event such as a button press or other interrupt; power consumption while sleeping (CPU clock and most peripherals off) may be just nanowatts, making many of them well suited for long lasting battery applications. Other microcontrollers may serve performance-critical roles, where they may need to act more like a digital signal processor (DSP), with higher clock speeds and power consumption.

- Contents [hide] - 1 Embedded design - 1.1 Interrupts - 1.2 Programs - 1.3 Other microcontroller features - 2 Higher integration - 3 Volumes - 4 Programming environments - 5 Types of microcontrollers - 6 Interrupt latency - 7 History - 8 Microcontroller embedded memory technology - 8.1 Data - 8.2 Firmware - 9 See also - 10 Notes - 11 External links

- [edit] Embedded design

A microcontroller can be considered a self-contained system with a processor, memory and peripherals and can be used as an embedded system.[1] The majority of microcontrollers in use today are embedded in other machinery, such as automobiles, telephones, appliances, and peripherals for computer systems. These are called embedded systems. While some embedded systems are very sophisticated, many have minimal requirements for memory and program length, with no operating system, and low software complexity. Typical input and output devices include switches, relays, solenoids, LEDs, small or custom LCD displays, radio frequency devices, and sensors for data such as temperature, humidity, light level etc. Embedded systems usually have no keyboard, screen, disks, printers, or other recognizable I/O devices of a personal computer, and may lack human interaction devices of any kind.

- [edit] Interrupts

Microcontrollers must provide real time (predictable, though not necessarily fast) response to events in the embedded system they are controlling. When certain events occur, an interrupt system can signal the processor to suspend processing the current instruction sequence and to begin an interrupt service routine (ISR, or "interrupt handler"). The ISR will perform any processing required based on the source of the interrupt before returning to the original instruction sequence. Possible interrupt sources are device dependent, and often include events such as an internal timer overflow, completing an analog to digital conversion, a logic level change on an input such as from a button being pressed, and data received on a communication link. Where power consumption is important as in battery operated devices, interrupts may also wake a microcontroller from a low power sleep state where the processor is halted until required to do something by a peripheral event.

- [edit] Programs

Microcontroller programs must fit in the available on-chip program memory, since it would be costly to provide a system with external, expandable, memory. Compilers and assemblers are used to convert high-level language and assembler language codes into a compact machine code for storage in the microcontroller's memory. Depending on the device, the program memory may be permanent, read-only memory that can only be programmed at the factory, or program memory may be field-alterable flash or erasable read-only memory.

- [edit] Other microcontroller features

Microcontrollers usually contain from several to dozens of general purpose input/output pins (GPIO). GPIO pins are software configurable to either an input or an output state. When GPIO pins are configured to an input state, they are often used to read sensors or external signals. Configured to the output state, GPIO pins can drive external devices such as LEDs or motors.

Many embedded systems need to read sensors that produce analog signals. This is the purpose of the analog-to-digital converter (ADC). Since processors are built to interpret and process digital data, i.e. 1s and 0s, they are not able to do anything with the analog signals that may be sent to it by a device. So the analog to digital converter is used to convert the incoming data into a form that the processor can recognize. A less common feature on some microcontrollers is a digital-to-analog converter (DAC) that allows the processor to output analog signals or voltage levels.

In addition to the converters, many embedded microprocessors include a variety of timers as well. One of the most common types of timers is the Programmable Interval Timer (PIT). A PIT just counts down from some value to zero. Once it reaches zero, it sends an interrupt to the processor indicating that it has finished counting. This is useful for devices such as thermostats, which periodically test the temperature around them to see if they need to turn the air conditioner on, the heater on, etc.

Time Processing Unit (TPU) is a sophisticated timer. In addition to counting down, the TPU can detect input events, generate output events, and perform other useful operations.

A dedicated Pulse Width Modulation (PWM) block makes it possible for the CPU to control power converters, resistive loads, motors, etc., without using lots of CPU resources in tight timer loops.

Universal Asynchronous Receiver/Transmitter (UART) block makes it possible to receive and transmit data over a serial line with very little load on the CPU. Dedicated on-chip hardware also often includes capabilities to communicate with other devices (chips) in digital formats such as I2C and Serial Peripheral Interface (SPI).

- [edit] Higher integration

In contrast to general-purpose CPUs, micro-controllers may not implement an external address or data bus as they integrate RAM and non-volatile memory on the same chip as the CPU. Using fewer pins, the chip can be placed in a much smaller, cheaper package.

Integrating the memory and other peripherals on a single chip and testing them as a unit increases the cost of that chip, but often results in decreased net cost of the embedded system as a whole. Even if the cost of a CPU that has integrated peripherals is slightly more than the cost of a CPU and external peripherals, having fewer chips typically allows a smaller and cheaper circuit board, and reduces the labor required to assemble and test the circuit board.

A micro-controller is a single integrated circuit, commonly with the following features:

- central processing unit - ranging from small and simple 4-bit processors to complex 32- or 64-bit processors

- discrete input and output bits, allowing control or detection of the logic state of an individual package pin

- serial input/output such as serial ports (UARTs)

- other serial communications interfaces like I²C, Serial Peripheral Interface and Controller Area Network for system interconnect

- peripherals such as timers, event counters, PWM generators, and watchdog

- volatile memory (RAM) for data storage

- ROM, EPROM, EEPROM or Flash memory for program and operating parameter storage

- clock generator - often an oscillator for a quartz timing crystal, resonator or RC circuit

- many include analog-to-digital converters

- in-circuit programming and debugging support

This integration drastically reduces the number of chips and the amount of wiring and circuit board space that would be needed to produce equivalent systems using separate chips. Furthermore, and on low pin count devices in particular, each pin may interface to several internal peripherals, with the pin function selected by software. This allows a part to be used in a wider variety of applications than if pins had dedicated functions. Micro-controllers have proved to be highly popular in embedded systems since their introduction in the 1970s.

Some microcontrollers use a Harvard architecture: separate memory buses for instructions and data, allowing accesses to take place concurrently. Where a Harvard architecture is used, instruction words for the processor may be a different bit size than the length of internal memory and registers; for example: 12-bit instructions used with 8-bit data registers.

The decision of which peripheral to integrate is often difficult. The microcontroller vendors often trade operating frequencies and system design flexibility against time-to-market requirements from their customers and overall lower system cost. Manufacturers have to balance the need to minimize the chip size against additional functionality.

Microcontroller architectures vary widely. Some designs include general-purpose microprocessor cores, with one or more ROM, RAM, or I/O functions integrated onto the package. Other designs are purpose built for control applications. A micro-controller instruction set usually has many instructions intended for bit-wise operations to make control programs more compact.[2] For example, a general purpose processor might require several instructions to test a bit in a register and branch if the bit is set, where a micro-controller could have a single instruction to provide that commonly-required function.

Microcontrollers typically do not have a math coprocessor, so floating point arithmetic is performed by software.

- [edit] Volumes

About 55% of all CPUs sold in the world are 8-bit microcontrollers and microprocessors. According to Semico, over four billion 8-bit microcontrollers were sold in 2006.[3]

A typical home in a developed country is likely to have only four general-purpose microprocessors but around three dozen microcontrollers. A typical mid-range automobile has as many as 30 or more microcontrollers. They can also be found in many electrical device such as washing machines, microwave ovens, and telephones.

A PIC 18F8720 microcontroller in an 80-pin TQFP package.

Manufacturers have often produced special versions of their microcontrollers in order to help the hardware and software development of the target system. Originally these included EPROM versions that have a "window" on the top of the device through which program memory can be erased by ultraviolet light, ready for reprogramming after a programming ("burn") and test cycle. Since 1998, EPROM versions are rare and have been replaced by EEPROM and flash, which are easier to use (can be erased electronically) and cheaper to manufacture.

Other versions may be available where the ROM is accessed as an external device rather than as internal memory, however these are becoming increasingly rare due to the widespread availability of cheap microcontroller programmers.

The use of field-programmable devices on a microcontroller may allow field update of the firmware or permit late factory revisions to products that have been assembled but not yet shipped. Programmable memory also reduces the lead time required for deployment of a new product.

Where hundreds of thousands of identical devices are required, using parts programmed at the time of manufacture can be an economical option. These "mask programmed" parts have the program laid down in the same way as the logic of the chip, at the same time.

- [edit] Programming environments

Microcontrollers were originally programmed only in assembly language, but various high-level programming languages are now also in common use to target microcontrollers. These languages are either designed specially for the purpose, or versions of general purpose languages such as the C programming language. Compilers for general purpose languages will typically have some restrictions as well as enhancements to better support the unique characteristics of microcontrollers. Some microcontrollers have environments to aid developing certain types of applications. Microcontroller vendors often make tools freely available to make it easier to adopt their hardware.

Many microcontrollers are so quirky that they effectively require their own non-standard dialects of C, such as SDCC for the 8051, which prevent using standard tools (such as code libraries or static analysis tools) even for code unrelated to hardware features. Interpreters are often used to hide such low level quirks.

Interpreter firmware is also available for some microcontrollers. For example, BASIC on the early microcontrollers Intel 8052[4]; BASIC and FORTH on the Zilog Z8[5] as well as some modern devices. Typically these interpreters support interactive programming.

Simulators are available for some microcontrollers, such as in Microchip's MPLAB environment. These allow a developer to analyze what the behavior of the microcontroller and their program should be if they were using the actual part. A simulator will show the internal processor state and also that of the outputs, as well as allowing input signals to be generated. While on the one hand most simulators will be limited from being unable to simulate much other hardware in a system, they can exercise conditions that may otherwise be hard to reproduce at will in the physical implementation, and can be the quickest way to debug and analyze problems.

Recent microcontrollers are often integrated with on-chip debug circuitry that when accessed by an in-circuit emulator via JTAG, allow debugging of the firmware with a debugger.

- [edit] Types of microcontrollers

|  | This section requires expansion. |
| --- | --- |

See also: List of common microcontrollers

As of 2008 there are several dozen microcontroller architectures and vendors including:

- Freescale 68HC11 (8-bit)

- Intel 8051

- ARM processors (from many vendors) using ARM7 or Cortex-M3 cores are generally microcontrollers

- STMicroelectronics STM8S (8-bit), ST10 (16-bit) and STM32 (32-bit)

- Atmel AVR (8-bit), AVR32 (32-bit), and AT91SAM (32-bit)

- Freescale ColdFire (32-bit) and S08 (8-bit)

- Hitachi H8, Hitachi SuperH (32-bit)

- Hyperstone E1/E2 (32-bit, First full integration of RISC and DSP on one processor core [1996] [1])

- MIPS (32-bit PIC32)

- NEC V850 (32-bit)

- PIC (8-bit PIC16, PIC18, 16-bit dsPIC33 / PIC24)

- PowerPC ISE

- PSoC (Programmable System-on-Chip)

- Rabbit 2000 (8-bit)

- Texas Instruments Microcontrollers MSP430 (16-bit), C2000 (32-bit), and Stellaris (32-bit)

- Toshiba TLCS-870 (8-bit/16-bit)

- Zilog eZ8 (16-bit), eZ80 (8-bit)

and many others, some of which are used in very narrow range of applications or are more like applications processors than microcontrollers. The microcontroller market is extremely fragmented, with numerous vendors, technologies, and markets. Note that many vendors sell (or have sold) multiple architectures.

- [edit] Interrupt latency

In contrast to general-purpose computers, microcontrollers used in embedded systems often seek to optimize interrupt latency over instruction throughput. Issues include both reducing the latency, and making it be more predictable (to support real-time control).

When an electronic device causes an interrupt, the intermediate results (registers) have to be saved before the software responsible for handling the interrupt can run. They must also be restored after that software is finished. If there are more registers, this saving and restoring process takes more time, increasing the latency. Ways to reduce such context/restore latency include having relatively few registers in their central processing units (undesirable because it slows down most non-interrupt processing substantially), or at least having the hardware not save them all (this fails if the software then needs to compensate by saving the rest "manually"). Another technique involves spending silicon gates on "shadow registers": one or more duplicate registers used only by the interrupt software, perhaps supporting a dedicated stack.

Other factors affecting interrupt latency include:

- Cycles needed to complete current CPU activities. To minimize those costs, microcontrollers tend to have short pipelines (often three instructions or less), small write buffers, and ensure that longer instructions are continuable or restartable. RISC design principles ensure that most instructions take the same number of cycles, helping avoid the need for most such continuation/restart logic.

- The length of any critical section that needs to be interrupted. Entry to a critical section restricts concurrent data structure access. When a data structure must be accessed by an interrupt handler, the critical section must block that interrupt. Accordingly, interrupt latency is increased by however long that interrupt is blocked. When there are hard external constraints on system latency, developers often need tools to measure interrupt latencies and track down which critical sections cause slowdowns.

  - One common technique just blocks all interrupts for the duration of the critical section. This is easy to implement, but sometimes critical sections get uncomfortably long.

  - A more complex technique just blocks the interrupts that may trigger access to that data structure. This often based on interrupt priorities, which tend to not correspond well to the relevant system data structures. Accordingly, this technique is used mostly in very constrained environments.

  - Processors may have hardware support for some critical sections. Examples include supporting atomic access to bits or bytes within a word, or other atomic access primitives like the LDREX/STREX exclusive access primitives introduced in the ARMv6 architecture.

- Interrupt nesting. Some microcontrollers allow higher priority interrupts to interrupt lower priority ones. This allows software to manage latency by giving time-critical interrupts higher priority (and thus lower and more predictable latency) than less-critical ones.

- Trigger rate. When interrupts occur back-to-back, microcontrollers may avoid an extra context save/restore cycle by a form of tail call optimization.

Lower end microcontrollers tend to support fewer interrupt latency controls than higher end ones.

- [edit] History

|  | This section requires expansion. |
| --- | --- |

The first single-chip microprocessor was the 4-bit Intel 4004 released in 1971. With the Intel 8008 and more capable microprocessors available over the next several years.

These however all required external chip(s) to implement a working system, raising total system cost, and making it impossible to economically computerize appliances.

The first computer system on a chip optimized for control applications was the Intel 8048 released in 1975,[citation needed] with both RAM and ROM on the same chip. This chip would find its way into over one billion PC keyboards, and other numerous applications. At this time Intels President, Luke J. Valenter, stated that the (Microcontroller) was one of the most successful in the companies history, and expanded the division's budget over 25%.

Most microcontrollers at this time had two variants. One had an erasable EPROM program memory, which was significantly more expensive than the PROM variant which was only programmable once.

In 1993, the introduction of EEPROM memory allowed microcontrollers (beginning with the Microchip PIC16x84) [2][citation needed]) to be electrically erased quickly without an expensive package as required for EPROM, allowing both rapid prototyping, and In System Programming.

The same year, Atmel introduced the first microcontroller using Flash memory.[6]

Other companies rapidly followed suit, with both memory types.

Cost has plummeted over time, with the cheapest 8-bit microcontrollers being available for under $0.25 in quantity (thousands) in 2009,[citation needed] and some 32-bit microcontrollers around $1 for similar quantities.

Nowadays microcontrollers are low cost and readily available for hobbyists, with large online communities around certain processors.

In the future, MRAM could potentially be used in microcontrollers as it has infinite endurance and its incremental semiconductor wafer process cost is relatively low.

- [edit] Microcontroller embedded memory technology

Since the emergence of microcontrollers, many different memory technologies have been used. Almost all microcontrollers have at least two different kinds of memory, a non-volatile memory for storing firmware and a read-write memory for temporary data.

- [edit] Data

From the earliest microcontrollers to today, six-transistor SRAM is almost always used as the read/write working memory, with a few more transistors per bit used in the register file. MRAM could potentially replace it as it is 4-10 times denser which would make it more cost effective.

In addition to the SRAM, some microcontrollers also have internal EEPROM for data storage; and even ones that do not have any (or not enough) are often connected to external serial EEPROM chip (such as the BASIC Stamp) or external serial flash memory chip.

A few recent microcontrollers beginning in 2003 have "self-programmable" flash memory[6].

- [edit] Firmware

The earliest microcontrollers used hard-wired or mask ROM to store firmware. Later microcontrollers (such as the early versions of the Freescale 68HC11 and early PIC microcontrollers) had quartz windows that allowed ultraviolet light in to erase the EPROM.

The Microchip PIC16C84, introduced in 1993,[7] was the first microcontroller to use EEPROM to store firmware.

Also in 1993, Atmel introduced the first microcontroller using NOR Flash memory to store firmware.[6]

PSoC microcontrollers, introduced in 2002, store firmware in SONOS flash memory.

MRAM could potentially be used to store firmware.

- [edit] See also

- In-circuit emulator

- List of common microcontrollers

- Microarchitecture

- Microbotics

- Programmable logic controller

- PSoC

- Single-board microcontroller

- [edit] Notes

|  | This article includes a list of references, but its sources remain unclear because it has insufficient inline citations. Please help to improve this article by introducing more precise citations where appropriate. (July 2008) |
| --- | --- |

- ^ Heath, Steve (2003). Embedded systems design. EDN series for design engineers (2 ed.). Newnes. pp. 11–12. ISBN 9780750655460. http://books.google.com/books?id=BjNZXwH7HlkC&pg=PA11.

- ^ Easy Way to build a microcontroller project

- ^ http://www.semico.com

- ^ "8052-Basic Microcontrollers" by Jan Axelson 1994

- ^ "Optimizing the Zilog Z8 Forth Microcontroller for Rapid Prototyping" by Robert Edwards 1987, page 3. http://www.ornl.gov/info/reports/1987/3445602791343.pdf

- ^ a b c "Atmel’s Self-Programming Flash Microcontrollers" by Odd Jostein Svendsli 2003

- ^ Microchip unveils PIC16C84, a reprogrammable EEPROM-based 8-bit microcontroller" 1993

- [edit] External links

|  | Wikibooks has a book on the topic of Embedded Systems |
| --- | --- |

|  | Wikimedia Commons has media related to: Microcontrollers |
| --- | --- |

- Microcontroller at the Open Directory Project

- Microcontroller.com - Embedded Systems industry website with tutorials and dedicated resources

- Embedded Systems Design magazine

[show] v • d • e CPU technologies. Architecture; ISA : CISC · EDGE · EPIC · MISC · OISC · RISC · VLIW · ZISC · Harvard architecture · von Neumann architecture · 4-bit · 8-bit · 12-bit · 16-bit · 18-bit · 24-bit · 31-bit · 32-bit · 36-bit · 48-bit · 64-bit · 128-bit. Parallelism; Pipeline; Instruction pipelining · In-order & out-of-order execution · Register renaming · Speculative execution · Hazards. Level; Bit · Instruction · Superscalar · Data · Task. Threads; Multithreading · Simultaneous multithreading · Hyperthreading · Superthreading. Flynn's taxonomy; SISD · SIMD · MISD · MIMD.. Types; Digital signal processor · Microcontroller · System-on-a-chip · Vector processor. Components; Arithmetic logic unit (ALU) · Barrel shifter · Floating-point unit (FPU) · Back-side bus · Multiplexer · Demultiplexer · Registers · Memory management unit (MMU) · Translation lookaside buffer (TLB) · Cache · register file · microcode · control unit · clock rate. Power management; APM · ACPI (states) · Dynamic frequency scaling · Dynamic voltage scaling · Clock gating.

[show] v • d • e 8-bit microcontroller families. AVR · PIC · COP8 · MCS-48 · MCS-51 · Z8 · eZ80 · HC08 · HC11 · H8 · PSoC.

[show] v • d • e Computer sizes. Classes of computers. Larger; Super · Minisuper · Mainframe · Mini (Midrange) · Supermini · Server. Micro; Personal (Workstation · Desktop · Home · SFF (Nettop)) · Plug · Portable · Arcade system board · Video game console. Mobile; Portable/Mobile data terminal · Electronic organizer · Pocket computer. Laptop; Desktop replacement computer · Subnotebook (Netbook · Smartbook). Tablet computer; Tablet personal computer (Ultra-Mobile PC) · Mobile Internet device (Internet Tablet). Wearable computer; Calculator watch · Wristop · Virtual retinal display · Head-mounted display (Head-up display). Information appliance; PDA (Palm-size PC · Handheld PC · Pocket computer) · EDA · Mobile phone (Smartphone · Feature phone) · PMP · DAP · E-book reader · Handheld game console. Calculators; Scientific · Programmable · Graphing.. - Programming language From Wikipedia, the free encyclopedia Jump to: navigation, search A programming language is an artificial language designed to express computations that can be performed by a machine, particularly a computer. Programming languages can be used to create programs that control the behavior of a machine, to express algorithms precisely, or as a mode of human communication. Programming language lists. - Alphabetical - Categorical - Chronological - Generational. Many programming languages have some form of written specification of their syntax (form) and semantics (meaning). Some languages are defined by a specification document. For example, the C programming language is specified by an ISO Standard. Other languages, such as Perl, have a dominant implementation that is used as a reference. The earliest programming languages predate the invention of the computer, and were used to direct the behavior of machines such as Jacquard looms and player pianos. Thousands of different programming languages have been created, mainly in the computer field, with many more being created every year. Most programming languages describe computation in an imperative style, i.e., as a sequence of commands, although some languages, such as those that support functional programming or logic programming, use alternative forms of description. - Contents [hide] - 1 Definitions - 2 Elements - 2.1 Syntax - 2.2 Static semantics - 2.3 Type system - 2.3.1 Typed versus untyped languages - 2.3.2 Static versus dynamic typing - 2.3.3 Weak and strong typing - 2.4 Execution semantics - 2.5 Core library - 3 Design and implementation - 3.1 Specification - 3.2 Implementation - 4 Usage - 4.1 Measuring language usage - 5 Taxonomies - 6 History - 6.1 Early developments - 6.2 Refinement - 6.3 Consolidation and growth - 7 See also - 8 References - 9 Further reading - 10 External links. - [edit] Definitions A programming language is a notation for writing programs, which are specifications of a computation or algorithm.[1] Some, but not all, authors restrict the term "programming language" to those languages that can express all possible algorithms.[1][2] Traits often considered important for what constitutes a programming language include: - Function and target: A computer programming language is a language[3] used to write computer programs, which involve a computer performing some kind of computation[4] or algorithm and possibly control external devices such as printers, disk drives, robots,[5] and so on. For example PostScript programs are frequently created by another program to control a computer printer or display. More generally, a programming language may describe computation on some, possibly abstract, machine. It is generally accepted that a complete specification for a programming language includes a description, possibly idealized, of a machine or processor for that language.[6] In most practical contexts, a programming language involves a computer; consequently programming languages are usually defined and studied this way.[7] Programming languages differ from natural languages in that natural languages are only used for interaction between people, while programming languages also allow humans to communicate instructions to machines. - Abstractions: Programming languages usually contain abstractions for defining and manipulating data structures or controlling the flow of execution. The practical necessity that a programming language support adequate abstractions is expressed by the abstraction principle;[8] this principle is sometimes formulated as recommendation to the programmer to make proper use of such abstractions.[9] - Expressive power: The theory of computation classifies languages by the computations they are capable of expressing. All Turing complete languages can implement the same set of algorithms. ANSI/ISO SQL and Charity are examples of languages that are not Turing complete, yet often called programming languages.[10][11] Markup languages like XML, HTML or troff, which define structured data, are not generally considered programming languages.[12][13][14] Programming languages may, however, share the syntax with markup languages if a computational semantics is defined. XSLT, for example, is a Turing complete XML dialect.[15][16][17] Moreover, LaTeX, which is mostly used for structuring documents, also contains a Turing complete subset.[18][19] The term computer language is sometimes used interchangeably with programming language.[20] However, the usage of both terms varies among authors, including the exact scope of each. One usage describes programming languages as a subset of computer languages.[21] In this vein, languages used in computing that have a different goal than expressing computer programs are generically designated computer languages. For instance, markup languages are sometimes referred to as computer languages to emphasize that they are not meant to be used for programming.[22] Another usage regards programming languages as theoretical constructs for programming abstract machines, and computer languages as the subset thereof that runs on physical computers, which have finite hardware resources.[23] John C. Reynolds emphasizes that formal specification languages are just as much programming languages as are the languages intended for execution. He also argues that textual and even graphical input formats that affect the behavior of a computer are programming languages, despite the fact they are commonly not Turing-complete, and remarks that ignorance of programming language concepts is the reason for many flaws in input formats.[24] - [edit] Elements All programming languages have some primitive building blocks for the description of data and the processes or transformations applied to them (like the addition of two numbers or the selection of an item from a collection). These primitives are defined by syntactic and semantic rules which describe their structure and meaning respectively. - [edit] Syntax Parse tree of Python code with inset tokenization Syntax highlighting is often used to aid programmers in recognizing elements of source code. The language above is Python. Main article: Syntax (programming languages) A programming language's surface form is known as its syntax. Most programming languages are purely textual; they use sequences of text including words, numbers, and punctuation, much like written natural languages. On the other hand, there are some programming languages which are more graphical in nature, using visual relationships between symbols to specify a program. The syntax of a language describes the possible combinations of symbols that form a syntactically correct program. The meaning given to a combination of symbols is handled by semantics (either formal or hard-coded in a reference implementation). Since most languages are textual, this article discusses textual syntax. Programming language syntax is usually defined using a combination of regular expressions (for lexical structure) and Backus–Naur Form (for grammatical structure). Below is a simple grammar, based on Lisp: expression ::= atom ¦ list atom ::= number ¦ symbol number ::= [+-]?['0'-'9']+ symbol ::= ['A'-'Z''a'-'z'].* list ::= '(' expression* ')' This grammar specifies the following: - an expression is either an atom or a list; - an atom is either a number or a symbol; - a number is an unbroken sequence of one or more decimal digits, optionally preceded by a plus or minus sign; - a symbol is a letter followed by zero or more of any characters (excluding whitespace); and - a list is a matched pair of parentheses, with zero or more expressions inside it. The following are examples of well-formed token sequences in this grammar: '12345', '()', '(a b c232 (1))' Not all syntactically correct programs are semantically correct. Many syntactically correct programs are nonetheless ill-formed, per the language's rules; and may (depending on the language specification and the soundness of the implementation) result in an error on translation or execution. In some cases, such programs may exhibit undefined behavior. Even when a program is well-defined within a language, it may still have a meaning that is not intended by the person who wrote it. Using natural language as an example, it may not be possible to assign a meaning to a grammatically correct sentence or the sentence may be false: - "Colorless green ideas sleep furiously." is grammatically well-formed but has no generally accepted meaning. - "John is a married bachelor." is grammatically well-formed but expresses a meaning that cannot be true. The following C language fragment is syntactically correct, but performs an operation that is not semantically defined (because p is a null pointer, the operations p->real and p->im have no meaning): complex *p = NULL; complex abs_p = sqrt (p->real * p->real + p->im * p->im); If the type declaration on the first line were omitted, the program would trigger an error on compilation, as the variable "p" would not be defined. But the program would still be syntactically correct, since type declarations provide only semantic information. The grammar needed to specify a programming language can be classified by its position in the Chomsky hierarchy. The syntax of most programming languages can be specified using a Type-2 grammar, i.e., they are context-free grammars.[25] Some languages, including Perl and Lisp, contain constructs that allow execution during the parsing phase. Languages that have constructs that allow the programmer to alter the behavior of the parser make syntax analysis an undecidable problem, and generally blur the distinction between parsing and execution.[26] In contrast to Lisp's macro system and Perl's BEGIN blocks, which may contain general computations, C macros are merely string replacements, and do not require code execution.[27] - [edit] Static semantics The static semantics defines restrictions on the structure of valid texts that are hard or impossible to express in standard syntactic formalisms.[1] For compiled languages, static semantics essentially include those semantic rules that can be checked at compile time. Examples include checking that every identifier is declared before it is used (in languages that require such declarations) or that the labels on the arms of a case statement are distinct.[28] Many important restrictions of this type, like checking that identifiers are used in the appropriate context (e.g. not adding a integer to a function name), or that subroutine calls have the appropriate number and type of arguments can be enforced by defining them as rules in a logic called a type system. Other forms of static analyses like data flow analysis may also be part of static semantics. Newer programming languages like Java and C# have definite assignment analysis, a form of data flow analysis, as part of their static semantics. - [edit] Type system Main articles: Type system and Type safety A type system defines how a programming language classifies values and expressions into types, how it can manipulate those types and how they interact. The goal of a type system is to verify and usually enforce a certain level of correctness in programs written in that language by detecting certain incorrect operations. Any decidable type system involves a trade-off: while it rejects many incorrect programs, it can also prohibit some correct, albeit unusual programs. In order to bypass this downside, a number of languages have type loopholes, usually unchecked casts that may be used by the programmer to explicitly allow a normally disallowed operation between different types. In most typed languages, the type system is used only to type check programs, but a number of languages, usually functional ones, perform type inference, which relieves the programmer from writing type annotations. The formal design and study of type systems is known as type theory. - [edit] Typed versus untyped languages A language is typed if the specification of every operation defines types of data to which the operation is applicable, with the implication that it is not applicable to other types.[29] For example, "this text between the quotes" is a string. In most programming languages, dividing a number by a string has no meaning. Most modern programming languages will therefore reject any program attempting to perform such an operation. In some languages, the meaningless operation will be detected when the program is compiled ("static" type checking), and rejected by the compiler, while in others, it will be detected when the program is run ("dynamic" type checking), resulting in a runtime exception. A special case of typed languages are the single-type languages. These are often scripting or markup languages, such as REXX or SGML, and have only one data type—most commonly character strings which are used for both symbolic and numeric data. In contrast, an untyped language, such as most assembly languages, allows any operation to be performed on any data, which are generally considered to be sequences of bits of various lengths.[29] High-level languages which are untyped include BCPL and some varieties of Forth. In practice, while few languages are considered typed from the point of view of type theory (verifying or rejecting all operations), most modern languages offer a degree of typing.[29] Many production languages provide means to bypass or subvert the type system. - [edit] Static versus dynamic typing In static typing all expressions have their types determined prior to the program being run (typically at compile-time). For example, 1 and (2+2) are integer expressions; they cannot be passed to a function that expects a string, or stored in a variable that is defined to hold dates.[29] Statically typed languages can be either manifestly typed or type-inferred. In the first case, the programmer must explicitly write types at certain textual positions (for example, at variable declarations). In the second case, the compiler infers the types of expressions and declarations based on context. Most mainstream statically typed languages, such as C++, C# and Java, are manifestly typed. Complete type inference has traditionally been associated with less mainstream languages, such as Haskell and ML. However, many manifestly typed languages support partial type inference; for example, Java and C# both infer types in certain limited cases.[30] Dynamic typing, also called latent typing, determines the type-safety of operations at runtime; in other words, types are associated with runtime values rather than textual expressions.[29] As with type-inferred languages, dynamically typed languages do not require the programmer to write explicit type annotations on expressions. Among other things, this may permit a single variable to refer to values of different types at different points in the program execution. However, type errors cannot be automatically detected until a piece of code is actually executed, making debugging more difficult. Ruby, Lisp, JavaScript, and Python are dynamically typed. - [edit] Weak and strong typing Weak typing allows a value of one type to be treated as another, for example treating a string as a number.[29] This can occasionally be useful, but it can also allow some kinds of program faults to go undetected at compile time and even at runtime. Strong typing prevents the above. An attempt to perform an operation on the wrong type of value raises an error.[29] Strongly typed languages are often termed type-safe or safe. An alternative definition for "weakly typed" refers to languages, such as Perl and JavaScript, which permit a large number of implicit type conversions. In JavaScript, for example, the expression 2 * x implicitly converts x to a number, and this conversion succeeds even if x is null, undefined, an Array, or a string of letters. Such implicit conversions are often useful, but they can mask programming errors. Strong and static are now generally considered orthogonal concepts, but usage in the literature differs. Some use the term strongly typed to mean strongly, statically typed, or, even more confusingly, to mean simply statically typed. Thus C has been called both strongly typed and weakly, statically typed.[31][32] - [edit] Execution semantics Further information: Formal semantics of programming languages Once data has been specified, the machine must be instructed to perform operations on the data. For example, the semantics may define the strategy by which expressions are evaluated to values, or the manner in which control structures conditionally execute statements. The execution semantics (also known as dynamic semantics) of a language defines how and when the various constructs of a language should produce a program behavior. There are many ways of defining execution semantics. Natural language is often used to specify the execution semantics of languages commonly used in practice. A significant amount of academic research went into formal semantics of programming languages, which allow execution semantics to be specified in a formal manner. Results from this field of research have seen limited application to programming language design and implementation outside academia. - [edit] Core library Main article: Standard library Most programming languages have an associated core library (sometimes known as the 'standard library', especially if it is included as part of the published language standard), which is conventionally made available by all implementations of the language. Core libraries typically include definitions for commonly used algorithms, data structures, and mechanisms for input and output. A language's core library is often treated as part of the language by its users, although the designers may have treated it as a separate entity. Many language specifications define a core that must be made available in all implementations, and in the case of standardized languages this core library may be required. The line between a language and its core library therefore differs from language to language. Indeed, some languages are designed so that the meanings of certain syntactic constructs cannot even be described without referring to the core library. For example, in Java, a string literal is defined as an instance of the java.lang.String class; similarly, in Smalltalk, an anonymous function expression (a "block") constructs an instance of the library's BlockContext class. Conversely, Scheme contains multiple coherent subsets that suffice to construct the rest of the language as library macros, and so the language designers do not even bother to say which portions of the language must be implemented as language constructs, and which must be implemented as parts of a library. - [edit] Design and implementation Programming languages share properties with natural languages related to their purpose as vehicles for communication, having a syntactic form separate from its semantics, and showing language families of related languages branching one from another.[3] But as artificial constructs, they also differ in fundamental ways from languages that have evolved through usage. A significant difference is that a programming language can be fully described and studied in its entirety, since it has a precise and finite definition.[33] By contrast, natural languages have changing meanings given by their users in different communities. While constructed languages are also artificial languages designed from the ground up with a specific purpose, they lack the precise and complete semantic definition that a programming language has. Many languages have been designed from scratch, altered to meet new needs, combined with other languages, and eventually fallen into disuse. Although there have been attempts to design one "universal" programming language that serves all purposes, all of them have failed to be generally accepted as filling this role.[34] The need for diverse programming languages arises from the diversity of contexts in which languages are used: - Programs range from tiny scripts written by individual hobbyists to huge systems written by hundreds of programmers. - Programmers range in expertise from novices who need simplicity above all else, to experts who may be comfortable with considerable complexity. - Programs must balance speed, size, and simplicity on systems ranging from microcontrollers to supercomputers. - Programs may be written once and not change for generations, or they may undergo continual modification. - Finally, programmers may simply differ in their tastes: they may be accustomed to discussing problems and expressing them in a particular language. One common trend in the development of programming languages has been to add more ability to solve problems using a higher level of abstraction. The earliest programming languages were tied very closely to the underlying hardware of the computer. As new programming languages have developed, features have been added that let programmers express ideas that are more remote from simple translation into underlying hardware instructions. Because programmers are less tied to the complexity of the computer, their programs can do more computing with less effort from the programmer. This lets them write more functionality per time unit.[35] Natural language processors have been proposed as a way to eliminate the need for a specialized language for programming. However, this goal remains distant and its benefits are open to debate. Edsger W. Dijkstra took the position that the use of a formal language is essential to prevent the introduction of meaningless constructs, and dismissed natural language programming as "foolish".[36] Alan Perlis was similarly dismissive of the idea.[37] Hybrid approaches have been taken in Structured English and SQL. A language's designers and users must construct a number of artifacts that govern and enable the practice of programming. The most important of these artifacts are the language specification and implementation. - [edit] Specification Main article: Programming language specification The specification of a programming language is intended to provide a definition that the language users and the implementors can use to determine whether the behavior of a program is correct, given its source code. A programming language specification can take several forms, including the following: - An explicit definition of the syntax, static semantics, and execution semantics of the language. While syntax is commonly specified using a formal grammar, semantic definitions may be written in natural language (e.g., as in the C language), or a formal semantics (e.g., as in Standard ML[38] and Scheme[39] specifications). - A description of the behavior of a translator for the language (e.g., the C++ and Fortran specifications). The syntax and semantics of the language have to be inferred from this description, which may be written in natural or a formal language. - A reference or model implementation, sometimes written in the language being specified (e.g., Prolog or ANSI REXX[40]). The syntax and semantics of the language are explicit in the behavior of the reference implementation. - [edit] Implementation Main article: Programming language implementation An implementation of a programming language provides a way to execute that program on one or more configurations of hardware and software. There are, broadly, two approaches to programming language implementation: compilation and interpretation. It is generally possible to implement a language using either technique. The output of a compiler may be executed by hardware or a program called an interpreter. In some implementations that make use of the interpreter approach there is no distinct boundary between compiling and interpreting. For instance, some implementations of BASIC compile and then execute the source a line at a time. Programs that are executed directly on the hardware usually run several orders of magnitude faster than those that are interpreted in software.[citation needed] One technique for improving the performance of interpreted programs is just-in-time compilation. Here the virtual machine, just before execution, translates the blocks of bytecode which are going to be used to machine code, for direct execution on the hardware. - [edit] Usage Thousands of different programming languages have been created, mainly in the computing field.[41] Programming languages differ from most other forms of human expression in that they require a greater degree of precision and completeness. When using a natural language to communicate with other people, human authors and speakers can be ambiguous and make small errors, and still expect their intent to be understood. However, figuratively speaking, computers "do exactly what they are told to do", and cannot "understand" what code the programmer intended to write. The combination of the language definition, a program, and the program's inputs must fully specify the external behavior that occurs when the program is executed, within the domain of control of that program. A programming language provides a structured mechanism for defining pieces of data, and the operations or transformations that may be carried out automatically on that data. A programmer uses the abstractions present in the language to represent the concepts involved in a computation. These concepts are represented as a collection of the simplest elements available (called primitives).[42] Programming is the process by which programmers combine these primitives to compose new programs, or adapt existing ones to new uses or a changing environment. Programs for a computer might be executed in a batch process without human interaction, or a user might type commands in an interactive session of an interpreter. In this case the "commands" are simply programs, whose execution is chained together. When a language is used to give commands to a software application (such as a shell) it is called a scripting language.[citation needed] - [edit] Measuring language usage Main article: Measuring programming language popularity It is difficult to determine which programming languages are most widely used, and what usage means varies by context. One language may occupy the greater number of programmer hours, a different one have more lines of code, and a third utilize the most CPU time. Some languages are very popular for particular kinds of applications. For example, COBOL is still strong in the corporate data center, often on large mainframes; FORTRAN in engineering applications; C in embedded applications and operating systems; and other languages are regularly used to write many different kinds of applications. Various methods of measuring language popularity, each subject to a different bias over what is measured, have been proposed: - counting the number of job advertisements that mention the language[43] - the number of books sold that teach or describe the language[44] - estimates of the number of existing lines of code written in the language—which may underestimate languages not often found in public searches[45] - counts of language references (i.e., to the name of the language) found using a web search engine. Combining and averaging information from various internet sites, langpop.com claims that [46] in 2008 the 10 most cited programming languages are (in alphabetical order): C, C++, C#, Java, JavaScript, Perl, PHP, Python, Ruby, and SQL. - [edit] Taxonomies For more details on this topic, see Categorical list of programming languages. There is no overarching classification scheme for programming languages. A given programming language does not usually have a single ancestor language. Languages commonly arise by combining the elements of several predecessor languages with new ideas in circulation at the time. Ideas that originate in one language will diffuse throughout a family of related languages, and then leap suddenly across familial gaps to appear in an entirely different family. The task is further complicated by the fact that languages can be classified along multiple axes. For example, Java is both an object-oriented language (because it encourages object-oriented organization) and a concurrent language (because it contains built-in constructs for running multiple threads in parallel). Python is an object-oriented scripting language. In broad strokes, programming languages divide into programming paradigms and a classification by intended domain of use. Traditionally, programming languages have been regarded as describing computation in terms of imperative sentences, i.e. issuing commands. These are generally called imperative programming languages. A great deal of research in programming languages has been aimed at blurring the distinction between a program as a set of instructions and a program as an assertion about the desired answer, which is the main feature of declarative programming.[47] More refined paradigms include procedural programming, object-oriented programming, functional programming, and logic programming; some languages are hybrids of paradigms or multi-paradigmatic. An assembly language is not so much a paradigm as a direct model of an underlying machine architecture. By purpose, programming languages might be considered general purpose, system programming languages, scripting languages, domain-specific languages, or concurrent/distributed languages (or a combination of these).[48] Some general purpose languages were designed largely with educational goals.[49] A programming language may also be classified by factors unrelated to programming paradigm. For instance, most programming languages use English language keywords, while a minority do not. Other languages may be classified as being esoteric or not. - [edit] History A selection of textbooks that teach programming, in languages both popular and obscure. These are only a few of the thousands of programming languages and dialects that have been designed in history. Main articles: History of programming languages and Programming language generations - [edit] Early developments The first programming languages predate the modern computer. The 19th century had "programmable" looms and player piano scrolls which implemented what are today recognized as examples of domain-specific languages. By the beginning of the twentieth century, punch cards encoded data and directed mechanical processing. In the 1930s and 1940s, the formalisms of Alonzo Church's lambda calculus and Alan Turing's Turing machines provided mathematical abstractions for expressing algorithms; the lambda calculus remains influential in language design.[50] In the 1940s, the first electrically powered digital computers were created. The first high-level programming language to be designed for a computer was Plankalkül, developed for the German Z3 by Konrad Zuse between 1943 and 1945. However, it was not implemented until 1998 and 2000.[51] Programmers of early 1950s computers, notably UNIVAC I and IBM 701, used machine language programs, that is, the first generation language (1GL). 1GL programming was quickly superseded by similarly machine-specific, but mnemonic, second generation languages (2GL) known as assembly languages or "assembler". Later in the 1950s, assembly language programming, which had evolved to include the use of macro instructions, was followed by the development of "third generation" programming languages (3GL), such as FORTRAN, LISP, and COBOL.[52] 3GLs are more abstract and are "portable", or at least implemented similarly on computers that do not support the same native machine code. Updated versions of all of these 3GLs are still in general use, and each has strongly influenced the development of later languages.[53] At the end of the 1950s, the language formalized as ALGOL 60 was introduced, and most later programming languages are, in many respects, descendants of Algol.[53] The format and use of the early programming languages was heavily influenced by the constraints of the interface.[54] - [edit] Refinement The period from the 1960s to the late 1970s brought the development of the major language paradigms now in use, though many aspects were refinements of ideas in the very first Third-generation programming languages: - APL introduced array programming and influenced functional programming.[55] - PL/I (NPL) was designed in the early 1960s to incorporate the best ideas from FORTRAN and COBOL. - In the 1960s, Simula was the first language designed to support object-oriented programming; in the mid-1970s, Smalltalk followed with the first "purely" object-oriented language. - C was developed between 1969 and 1973 as a system programming language, and remains popular.[56] - Prolog, designed in 1972, was the first logic programming language. - In 1978, ML built a polymorphic type system on top of Lisp, pioneering statically typed functional programming languages. Each of these languages spawned an entire family of descendants, and most modern languages count at least one of them in their ancestry. The 1960s and 1970s also saw considerable debate over the merits of structured programming, and whether programming languages should be designed to support it.[57] Edsger Dijkstra, in a famous 1968 letter published in the Communications of the ACM, argued that GOTO statements should be eliminated from all "higher level" programming languages.[58] The 1960s and 1970s also saw expansion of techniques that reduced the footprint of a program as well as improved productivity of the programmer and user. The card deck for an early 4GL was a lot smaller for the same functionality expressed in a 3GL deck. - [edit] Consolidation and growth The 1980s were years of relative consolidation. C++ combined object-oriented and systems programming. The United States government standardized Ada, a systems programming language derived from Pascal and intended for use by defense contractors. In Japan and elsewhere, vast sums were spent investigating so-called "fifth generation" languages that incorporated logic programming constructs.[59] The functional languages community moved to standardize ML and Lisp. Rather than inventing new paradigms, all of these movements elaborated upon the ideas invented in the previous decade. One important trend in language design for programming large-scale systems during the 1980s was an increased focus on the use of modules, or large-scale organizational units of code. Modula-2, Ada, and ML all developed notable module systems in the 1980s, although other languages, such as PL/I, already had extensive support for modular programming. Module systems were often wedded to generic programming constructs.[60] The rapid growth of the Internet in the mid-1990s created opportunities for new languages. Perl, originally a Unix scripting tool first released in 1987, became common in dynamic websites. Java came to be used for server-side programming, and bytecode virtual machines became popular again in commercial settings with their promise of "Write once, run anywhere" (UCSD Pascal had been popular for a time in the early 1980s). These developments were not fundamentally novel, rather they were refinements to existing languages and paradigms, and largely based on the C family of programming languages. Programming language evolution continues, in both industry and research. Current directions include security and reliability verification, new kinds of modularity (mixins, delegates, aspects), and database integration such as Microsoft's LINQ. The 4GLs are examples of languages which are domain-specific, such as SQL, which manipulates and returns sets of data rather than the scalar values which are canonical to most programming languages. Perl, for example, with its 'here document' can hold multiple 4GL programs, as well as multiple JavaScript programs, in part of its own perl code and use variable interpolation in the 'here document' to support multi-language programming.[61] - [edit] See also Computer science portal. Wikibooks has a book on the topic of Computer programming. Wikiquote has a collection of quotations related to: Programming languages. - Comparison of programming languages (basic instructions) - Comparison of programming languages - Computer programming - Computer science and Outline of computer science - Educational programming language - Invariant based programming - Lists of programming languages - List of programming language researchers - Literate programming - Dialect (computing) - Programming language theory - Pseudocode - Scientific language - Software engineering and List of software engineering topics - [edit] References - ^ a b c Aaby, Anthony (2004). Introduction to Programming Languages. http://burks.brighton.ac.uk/burks/pcinfo/progdocs/plbook/index.htm. - ^ In mathematical terms, this means the programming language is Turing-complete MacLennan, Bruce J. (1987). Principles of Programming Languages. Oxford University Press. p. 1. ISBN 0-19-511306-3. - ^ a b Steven R. Fischer, A history of language, Reaktion Books, 2003, ISBN 186189080X, p. 205 - ^ ACM SIGPLAN (2003). "Bylaws of the Special Interest Group on Programming Languages of the Association for Computing Machinery". http://www.acm.org/sigs/sigplan/sigplan_bylaws.htm. Retrieved 2006-06-19. , The scope of SIGPLAN is the theory, design, implementation, description, and application of computer programming languages - languages that permit the specification of a variety of different computations, thereby providing the user with significant control (immediate or delayed) over the computer's operation. - ^ Dean, Tom (2002). "Programming Robots". Building Intelligent Robots. Brown University Department of Computer Science. http://www.cs.brown.edu/people/tld/courses/cs148/02/programming.html. Retrieved 2006-09-23. - ^ R. Narasimahan, Programming Languages and Computers: A Unified Metatheory, pp. 189--247 in Franz Alt, Morris Rubinoff (eds.) Advances in computers, Volume 8, Academic Press, 1994, ISBN 012012108, p.193 : "a complete specification of a programming language must, by definition, include a specification of a processor--idealized, if you will--for that language." [the source cites many references to support this statement] - ^ Ben Ari, Mordechai (1996). Understanding Programming Languages". John Wiley and Sons. "Programs and languages can be deﬁned as purely formal mathematical objects. However, more people are interested in programs than in other mathematical objects such as groups, precisely because it is possible to use the program—the sequence of symbols—to control the execution of a computer. While we highly recommend the study of the theory of programming, this text will generally limit itself to the study of programs as they are executed on a computer." - ^ David A. Schmidt, The structure of typed programming languages, MIT Press, 1994, ISBN 0262193493, p. 32 - ^ Pierce, Benjamin (2002). Types and Programming Languages. MIT Press. p. 339. ISBN 0-262-16209-1. - ^ Digital Equipment Corporation. "Information Technology - Database Language SQL (Proposed revised text of DIS 9075)". ISO/IEC 9075:1992, Database Language SQL. http://www.contrib.andrew.cmu.edu/~shadow/sql/sql1992.txt. Retrieved June 29, 2006. - ^ The Charity Development Group (December 1996). "The CHARITY Home Page". http://pll.cpsc.ucalgary.ca/charity1/www/home.html. Retrieved 2006-06-29. , Charity is a categorical programming language..., All Charity computations terminate. - ^ XML in 10 points W3C, 1999, XML is not a programming language. - ^ Powell, Thomas (2003). HTML & XHTML: the complete reference. McGraw-Hill. p. 25. ISBN 0-07-222-942-X. "HTML is not a programming language." - ^ Dykes, Lucinda; Tittel, Ed (2005). XML For Dummies, 4th Edition. Wiley. p. 20. ISBN 0-7645-8845-1. "...it's a markup language, not a programming language." - ^ http://www.ibm.com/developerworks/library/x-xslt/ - ^ http://msdn.microsoft.com/en-us/library/ms767587(VS.85).aspx - ^ Scott, Michael (2006). Programming Language Pragmatics. Morgan Kaufmann. p. 802. ISBN 0-12-633951-1. "XSLT, though highly specialized to the transformation of XML, is a Turing-complete programming language." - ^ http://tobi.oetiker.ch/lshort/lshort.pdf - ^ Syropoulos, Apostolos; Antonis Tsolomitis, Nick Sofroniou (2003). Digital typography using LaTeX. Springer-Verlag. p. 213. ISBN 0-387-95217-9. "TeX is not only an excellent typesetting engine but also a real programming language." - ^ Robert A. Edmunds, The Prentice-Hall standard glossary of computer terminology, Prentice-Hall, 1985, p. 91 - ^ Pascal Lando, Anne Lapujade, Gilles Kassel, and Frédéric Fürst, Towards a General Ontology of Computer Programs, ICSOFT 2007, pp. 163-170 - ^ S.K. Bajpai, Introduction To Computers And C Programming, New Age International, 2007, ISBN 812241379X, p. 346 - ^ R. Narasimahan, Programming Languages and Computers: A Unified Metatheory, pp. 189--247 in Franz Alt, Morris Rubinoff (eds.) Advances in computers, Volume 8, Academic Press, 1994, ISBN 012012108, p.215: "[...] the model [...] for computer languages differs from that [...] for programming languages in only two respects. In a computer language, there are only finitely many names--or registers--which can assume only finitely many values--or states--and these states are not further distinguished in terms of any other attributes. [author's footnote:] This may sound like a truism but its implications are far reaching. For example, it would imply that any model for programming languages, by fixing certain of its parameters or features, should be reducible in a natural way to a model for computer languages." - ^ John C. Reynolds, Some thoughts on teaching programming and programming languages, SIGPLAN Notices, Volume 43, Issue 11, November 2008, p.109 - ^ Michael Sipser (1997). Introduction to the Theory of Computation. PWS Publishing. ISBN 0-534-94728-X. Section 2.2: Pushdown Automata, pp.101–114. - ^ Jeffrey Kegler, "Perl and Undecidability", The Perl Review. Papers 2 and 3 prove, using respectively Rice's theorem and direct reduction to the halting problem, that the parsing of Perl programs is in general undecidable. - ^ Marty Hall, 1995, Lecture Notes: Macros, PostScript version - ^ Michael Lee Scott, Programming language pragmatics, Edition 2, Morgan Kaufmann, 2006, ISBN 0126339511, p. 18-19 - ^ a b c d e f g Andrew Cooke. "An Introduction to Programming Languages". http://www.acooke.org/andrew/writing/lang.html#sec-types. Retrieved June 30, 2006. [dead link] - ^ Specifically, instantiations of generic types are inferred for certain expression forms. Type inference in Generic Java—the research language that provided the basis for Java 1.5's bounded parametric polymorphism extensions—is discussed in two informal manuscripts from the Types mailing list: Generic Java type inference is unsound (Alan Jeffrey, 17 December 2001) and Sound Generic Java type inference (Martin Odersky, 15 January 2002). C#'s type system is similar to Java's, and uses a similar partial type inference scheme. - ^ "Revised Report on the Algorithmic Language Scheme (February 20, 1998)". http://www.schemers.org/Documents/Standards/R5RS/HTML/r5rs-Z-H-4.html. Retrieved June 9, 2006. - ^ Luca Cardelli and Peter Wegner. "On Understanding Types, Data Abstraction, and Polymorphism". Manuscript (1985). http://citeseer.ist.psu.edu/cardelli85understanding.html. Retrieved June 9, 2006. - ^ Jing Huang. "Artificial Language vs. Natural Language". http://www.cs.cornell.edu/info/Projects/Nuprl/cs611/fall94notes/cn2/subsection3_1_3.html. - ^ IBM in first publishing PL/I, for example, rather ambitiously titled its manual The universal programming language PL/I (IBM Library; 1966). The title reflected IBM's goals for unlimited subsetting capability: PL/I is designed in such a way that one can isolate subsets from it satisfying the requirements of particular applications. ("Encyclopaedia of Mathematics » P » PL/I". SpringerLink. http://eom.springer.de/P/p072885.htm. Retrieved June 29, 2006. ). Ada and UNCOL had similar early goals. - ^ Frederick P. Brooks, Jr.: The Mythical Man-Month, Addison-Wesley, 1982, pp. 93-94 - ^ Dijkstra, Edsger W. On the foolishness of "natural language programming." EWD667. - ^ Perlis, Alan, Epigrams on Programming. SIGPLAN Notices Vol. 17, No. 9, September 1982, pp. 7-13 - ^ Milner, R.; M. Tofte, R. Harper and D. MacQueen. (1997). The Definition of Standard ML (Revised). MIT Press. ISBN 0-262-63181-4. - ^ Kelsey, Richard; William Clinger and Jonathan Rees (February 1998). "Section 7.2 Formal semantics". Revised5 Report on the Algorithmic Language Scheme. http://www.schemers.org/Documents/Standards/R5RS/HTML/r5rs-Z-H-10.html#%_sec_7.2. Retrieved 2006-06-09. - ^ ANSI — Programming Language Rexx, X3-274.1996 - ^ "HOPL: an interactive Roster of Programming Languages". Australia: Murdoch University. http://hopl.murdoch.edu.au/. Retrieved 2009-06-01. "This site lists 8512 languages." - ^ Abelson, Sussman, and Sussman. "Structure and Interpretation of Computer Programs". http://mitpress.mit.edu/sicp/full-text/book/book-Z-H-10.html. Retrieved 2009-03-03. - ^ Survey of Job advertisements mentioning a given language - ^ Counting programming languages by book sales - ^ Bieman, J.M.; Murdock, V., Finding code on the World Wide Web: a preliminary investigation, Proceedings First IEEE International Workshop on Source Code Analysis and Manipulation, 2001 - ^ Programming Language Popularity - ^ Carl A. Gunter, Semantics of Programming Languages: Structures and Techniques, MIT Press, 1992, ISBN 0262570955, p. 1 - ^ "TUNES: Programming Languages". http://tunes.org/wiki/programming_20languages.html. - ^ Wirth, Niklaus (1993). "Recollections about the development of Pascal". Proc. 2nd ACM SIGPLAN conference on history of programming languages: 333–342. doi:10.1145/154766.155378. http://portal.acm.org/citation.cfm?id=155378. Retrieved 2006-06-30. - ^ Benjamin C. Pierce writes: "... the lambda calculus has seen widespread use in the specification of programming language features, in language design and implementation, and in the study of type systems." Pierce, Benjamin C. (2002). Types and Programming Languages. MIT Press. p. 52. ISBN 0-262-16209-1. - ^ Rojas, Raúl, et al. (2000). "Plankalkül: The First High-Level Programming Language and its Implementation". Institut für Informatik, Freie Universität Berlin, Technical Report B-3/2000. (full text) - ^ Linda Null, Julia Lobur, The essentials of computer organization and architecture, Edition 2, Jones & Bartlett Publishers, 2006, ISBN 0763737690, p. 435 - ^ a b O'Reilly Media. "History of programming languages" (PDF). http://www.oreilly.com/news/graphics/prog_lang_poster.pdf. Retrieved October 5, 2006. - ^ Frank da Cruz. IBM Punch Cards Columbia University Computing History. - ^ Richard L. Wexelblat: History of Programming Languages, Academic Press, 1981, chapter XIV. - ^ François Labelle. "Programming Language Usage Graph". SourceForge. http://www.cs.berkeley.edu/~flab/languages.html. Retrieved June 21, 2006. . This comparison analyzes trends in number of projects hosted by a popular community programming repository. During most years of the comparison, C leads by a considerable margin; in 2006, Java overtakes C, but the combination of C/C++ still leads considerably. - ^ Hayes, Brian (2006). "The Semicolon Wars". American Scientist 94 (4): 299–303. - ^ Dijkstra, Edsger W. (March 1968). "Go To Statement Considered Harmful". Communications of the ACM 11 (3): 147–148. doi:10.1145/362929.362947. http://www.acm.org/classics/oct95/. Retrieved 2006-06-29. - ^ Tetsuro Fujise, Takashi Chikayama Kazuaki Rokusawa, Akihiko Nakase (December 1994). "KLIC: A Portable Implementation of KL1" Proc. of FGCS '94, ICOT Tokyo, December 1994. KLIC is a portable implementation of a concurrent logic programming language KL1. - ^ Jim Bender (March 15, 2004). "Mini-Bibliography on Modules for Functional Programming Languages". ReadScheme.org. http://readscheme.org/modules/. Retrieved 2006-09-27. - ^ Wall, Programming Perl ISBN 0-596-00027-8 p.66 - [edit] Further reading - Daniel P. Friedman, Mitchell Wand, Christopher Thomas Haynes: Essentials of Programming Languages, The MIT Press 2001. - David Gelernter, Suresh Jagannathan: Programming Linguistics, The MIT Press 1990. - Shriram Krishnamurthi: Programming Languages: Application and Interpretation, online publication. - Bruce J. MacLennan: Principles of Programming Languages: Design, Evaluation, and Implementation, Oxford University Press 1999. - John C. Mitchell: Concepts in Programming Languages, Cambridge University Press 2002. - Benjamin C. Pierce: Types and Programming Languages, The MIT Press 2002. - Ravi Sethi: Programming Languages: Concepts and Constructs, 2nd ed., Addison-Wesley 1996. - Michael L. Scott: Programming Language Pragmatics, Morgan Kaufmann Publishers 2005. - Richard L. Wexelblat (ed.): History of Programming Languages, Academic Press 1981. - [edit] External links Look up programming language in Wiktionary, the free dictionary.. - 99 Bottles of Beer A collection of implementations in many languages. - Computer Programming Languages at the Open Directory Project - Syntax Patterns for Various Languages [show] v • d • e Types of programming languages. Array · Aspect-oriented · Assembly · Class-based · Compiled · Concatenative · Concurrent · Data-structured · Dataflow · Declarative · Domain-specific · Dynamic · Esoteric · Event-driven · Extensible · Functional · High-level · Imperative · Interpreted · Logic · Low-level · Machine · Macro · Metaprogramming · Multi-paradigm · Non-English-based · Object-based · Object-oriented · Off-side rule · Pipeline · Procedural · Prototype-based · Reflective · Rule-based · Scripting · Synchronous · Very high-level · Visual.. [show] v • d • e Types of Computer languages. Architecture description · Hardware description · Markup · Modeling · Programming · Query · Specification · Stylesheet · Template processing · Transformation.. - Real-time operating system From Wikipedia, the free encyclopedia Jump to: navigation, search This article needs additional citations for verification. Please help improve this article by adding reliable references. Unsourced material may be challenged and removed. (August 2010). A real-time operating system (RTOS) is an operating system (OS) intended for real-time applications. Such operating systems serve application requests nearly real-time. A real-time operating system offers programmers more control over process priorities. An application's process priority level may exceed that of a system process. Real-time operating systems minimize critical sections of system code, so that the application's interruption is nearly critical. A key characteristic of a real-time OS is the level of its consistency concerning the amount of time it takes to accept and complete an application's task; the variability is jitter. A hard real-time operating system has less jitter than a soft real-time operating system. The chief design goal is not high throughput, but rather a guarantee of a soft or hard performance category. A real-time OS that can usually or generally meet a deadline is a soft real-time OS, but if it can meet a deadline deterministically it is a hard real-time OS. A real-time OS has an advanced algorithm for scheduling. Scheduler flexibility enables a wider, computer-system orchestration of process priorities, but a real-time OS is more frequently dedicated to a narrow set of applications. Key factors in a real-time OS are minimal interrupt latency and minimal thread switching latency, but a real-time OS is valued more for how quickly or how predictably it can respond than for the amount of work it can perform in a given period of time. - Contents [hide] - 1 Design philosophies - 2 Scheduling - 2.1 Algorithms - 3 Intertask communication and resource sharing - 3.1 Temporarily masking/disabling interrupts - 3.2 Binary semaphores - 3.2.1 Binary semaphores in existing real-time operating systems - 3.3 Message passing - 4 Interrupt handlers and the scheduler - 5 Memory allocation - 6 Examples - 7 See also - 8 References. - [edit] Design philosophies Two basic designs exist: - Event-driven which switches tasks only when an event of higher priority needs service, called preemptive priority, or priority scheduling. - Time-sharing designs switch tasks on a regular clock interrupt, and on events, called round robin. Time-sharing designs switch tasks more often than strictly needed, but give smoother, more deterministic multitasking, giving the illusion that a process or user has sole use of a machine. Early CPU designs needed many cycles to switch tasks, during which the CPU could do nothing else useful. For example, with a 20 MHz 68000 processor (typical of late 1980s), task switch times are roughly 20 microseconds. (In contrast, a 100 MHz ARM CPU (from 2008) switches in less than 3 microseconds.)[1][2] Because of this, early OSes tried to minimize wasting CPU time by avoiding unnecessary task switching. - [edit] Scheduling In typical designs, a task has three states: 1) running (executing on the CPU), 2) ready (ready to be executed), 3) blocked (waiting for input/output). Most tasks are blocked or ready most of the time because generally only one task can run at a time per CPU. The number of items in the ready queue can greatly vary, depending on the number of tasks the system needs to perform and the type of scheduler that the system uses. On simpler non-preemptive but still multitasking systems, a task has to give up its time on the CPU to other tasks, which can cause the ready queue to have a greater number of overall tasks in the ready to be executed state (see: [¦ Resource Starvation]) Usually the data structure of the ready list in the scheduler is designed to minimize the worst-case length of time spent in the scheduler's critical section, during which preemption is inhibited, and, in some cases, all interrupts are disabled. But the choice of data structure depends also on the maximum number of tasks that can be on the ready list. If there are never more than a few tasks on the ready list, then a doubly linked list of ready tasks is likely optimal. If the ready list usually contains only a few tasks but occasionally contains more, then the list should be sorted by priority. That way, finding the highest priority task to run does not require iterating through the entire list. Inserting a task then requires walking the ready list until reaching either the end of the list, or a task of lower priority than that of the task being inserted. Care must be taken not to inhibit preemption during this search. Longer critical sections should be divided into small pieces. If an interrupt occurs that makes a high priority task ready during the insertion of a low priority task, that high priority task can be inserted and run immediately before the low priority task is inserted. The critical response time, sometimes called the flyback time, is the time it takes to queue a new ready task and restore the state of the highest priority task to running. In a well-designed RTOS, readying a new task will take 3 to 20 instructions per ready-queue entry, and restoration of the highest-priority ready task will take 5 to 30 instructions. In more advanced systems, real-time tasks share computing resources with many non-real-time tasks, and the ready list can be arbitrarily long. In such systems, a scheduler ready list implemented as a linked list would be inadequate. - [edit] Algorithms Some commonly used RTOS scheduling algorithms are: - Cooperative scheduling - Preemptive scheduling - Round-robin scheduling - Fixed priority pre-emptive scheduling, an implementation of preemptive time slicing - Fixed-Priority Scheduling with Deferred Preemption - Fixed-Priority Non-preemptive Scheduling - Critical section preemptive scheduling - Static time scheduling - Earliest Deadline First approach - Advanced scheduling using the stochastic and MTG - [edit] Intertask communication and resource sharing Multitasking systems must manage sharing data and hardware resources among multiple tasks. It is usually "unsafe" for two tasks to access the same specific data or hardware resource simultaneously. "Unsafe" means the results are inconsistent or unpredictable. There are three common approaches to resolve this problem: - [edit] Temporarily masking/disabling interrupts General-purpose operating systems usually do not allow user programs to mask (disable) interrupts, because the user program could control the CPU for as long as it wishes. Modern CPUs don't allow user mode code to disable interrupts as such control is considered a key operating system resource. Many embedded systems and RTOSs, however, allow the application itself to run in kernel mode for greater system call efficiency and also to permit the application to have greater control of the operating environment without requiring OS intervention. On single-processor systems, if the application runs in kernel mode and can mask interrupts, often interrupt disablement is the best (lowest overhead) solution to prevent simultaneous access to a shared resource. While interrupts are masked, the current task has exclusive use of the CPU since no other task or interrupt can take control, so the critical section is protected. When the task exits its critical section, it must unmask interrupts; pending interrupts, if any, will then execute. Temporarily masking interrupts should only be done when the longest path through the critical section is shorter than the desired maximum interrupt latency, or else this method increases the system's maximum interrupt latency. Typically this method of protection is used only when the critical section is just a few instructions and contains no loops. This method is ideal for protecting hardware bit-mapped registers when the bits are controlled by different tasks. - [edit] Binary semaphores When the critical section is longer than a few source code lines or involves lengthy looping, an embedded/real-time algorithm must resort to using mechanisms identical or similar to those available on general-purpose operating systems, such as semaphores and OS-supervised interprocess messaging. Such mechanisms involve system calls, and usually invoke the OS's dispatcher code on exit, so they typically take hundreds of CPU instructions to execute, while masking interrupts may take as few as one instruction on some processors. But for longer critical sections, there may be no choice; interrupts cannot be masked for long periods without increasing the system's interrupt latency. A binary semaphore is either locked or unlocked. When it is locked, tasks must wait for the semaphore to unlock. A binary semaphore is therefore equivalent to a mutex. Typically a task will set a timeout on its wait for a semaphore. There are several well-known problems with semaphore based designs such as priority inversion and deadlocks. In priority inversion a high priority task waits because a low priority task has a semaphore. A typical solution is to have the task that owns a semaphore run at (inherit) the priority of the highest waiting task. But this simplistic approach fails when there are multiple levels of waiting: task A waits for a binary semaphore locked by task B, which waits for a binary semaphore locked by task C. Handling multiple levels of inheritance without introducing instability in cycles is complex and problematic. In a deadlock, two or more tasks lock semaphores without timeouts and then wait forever for the other task's semaphore, creating a cyclic dependency. The simplest deadlock scenario occurs when two tasks alternately lock two semaphores, but in the opposite order. Deadlock is prevented by careful design or by having floored semaphores, which pass control of a semaphore to the higher priority task on defined conditions. - [edit] Binary semaphores in existing real-time operating systems Operating system; Protocol; Details. uC/OS-II; Priority Ceiling; Addresses priority inversion, but suffers from chained blocking and deadlock.. - [edit] Message passing The other approach to resource sharing is for tasks to send messages in an organized message passing scheme. In this paradigm, the resource is managed directly by only one task. When another task wants to interrogate or manipulate the resource, it sends a message to the managing task. Although their real-time behavior is less crisp than semaphore systems, simple message-based systems avoid most protocol deadlock hazards, and are generally better-behaved than semaphore systems. However, problems like those of semaphores are possible. Priority inversion can occur when a task is working on a low-priority message and ignores a higher-priority message (or a message originating indirectly from a high priority task) in its incoming message queue. Protocol deadlocks can occur when two or more tasks wait for each other to send response messages. - [edit] Interrupt handlers and the scheduler Since an interrupt handler blocks the highest priority task from running, and since real time operating systems are designed to keep thread latency to a minimum, interrupt handlers are typically kept as short as possible. The interrupt handler defers all interaction with the hardware as long as possible; typically all that is necessary is to acknowledge or disable the interrupt (so that it won't occur again when the interrupt handler returns). The interrupt handler then queues work to be done at a lower priority level, such as unblocking a driver task through releasing a semaphore or sending a message. A scheduler often provides the ability to unblock a task from interrupt handler context. An OS maintains catalogs of objects it manages such as threads, mutexes, memory, and so on. Updates to this catalog must be strictly controlled. For this reason it can be problematic when an interrupt handler calls an OS function while the application is in the act of also doing so. The OS function called from an interrupt handler could find the object database to be in an inconsistent state because of the application's update. There are two major approaches to deal with this problem: the unified architecture and the segmented architecture. RTOSs implementing the unified architecture solve the problem by simply disabling interrupts while the internal catalog is updated. The downside of this is that interrupt latency increases, potentially losing interrupts. The segmented architecture does not make direct OS calls but delegates the OS related work to a separate handler. This handler runs at a higher priority than any thread but lower than the interrupt handlers. The advantage of this architecture is that it adds very few cycles to interrupt latency. As a result, OSes which implement the segmented architecture are more predictable and can deal with higher interrupt rates compared to the unified architecture. - [edit] Memory allocation Memory allocation is more critical in an RTOS than in other operating systems. First, speed of allocation is important. A standard memory allocation scheme scans a linked list of indeterminate length to find a suitable free memory block. This is unacceptable in an RTOS since memory allocation has to occur within a certain amount of time. The simple fixed-size-blocks algorithm works quite well for simple embedded systems because of its low overhead. See memory allocation for more details. - [edit] Examples An early example of a large-scale real-time operating system was the Transaction Processing Facility developed by American Airlines and IBM for the Sabre Airline Reservations System. Currently the best known, most widely deployed, real-time operating systems are[citation needed] - QNX - RTLinux - VxWorks - Windows CE - OSE See the list of real-time operating systems for a comprehensive list. Also, see the list of operating systems for all types of operating systems. - [edit] See also - Software engineering From Wikipedia, the free encyclopedia Jump to: navigation, search The Airbus A380 uses a substantial amount of software to create a "paperless" cockpit. Software engineering (SE) is a profession dedicated to designing, implementing, and modifying software so that it is of higher quality, more affordable, maintainable, and faster to build. The term software engineering first appeared in the 1968 NATO Software Engineering Conference, and was meant to provoke thought regarding the perceived "software crisis" at the time.[1][2] Since the field is still relatively young compared to its sister fields of engineering, there is still much debate around what software engineering actually is, and if it conforms to the classical definition of engineering[3]. Some people argue that development of computer software is more art than science [4], and that attempting to impose engineering disciplines over a type of art is an exercise in futility because what represents good practice in the creation of software is not even defined.[5] Others, such as Steve McConnell, argue that engineering's blend of art and science to achieve practical ends provides a useful model for software development.[6] The IEEE Computer Society's Software Engineering Body of Knowledge defines "software engineering" as the application of a systematic, disciplined, quantifiable approach to the development, operation, and maintenance of software, and the study of these approaches; that is, the application of engineering to software.[7] Software development, a much used and more generic term, does not necessarily subsume the engineering paradigm. Although it is questionable what impact it has had on actual software development over the last more than 40 years,[8][9] the field's future looks bright according to Money Magazine and Salary.com, who rated "software engineering" as the best job in the United States in 2006.[10] - Contents [hide] - 1 History - 2 Profession - 2.1 Employment - 2.2 Certification - 2.3 Impact of globalization - 3 Education - 4 Sub-disciplines - 5 Related disciplines - 5.1 Computer science - 5.2 Project management - 5.3 Systems engineering - 6 See also - 7 References - 8 Further reading - 9 External links. - [edit] History Main article: History of software engineering When the first modern digital computers appeared in the early 1940s,[11] the instructions to make them operate were wired into the machine. Practitioners quickly realized that this design was not flexible and came up with the "stored program architecture" or von Neumann architecture. Thus the first division between "hardware" and "software" began with abstraction being used to deal with the complexity of computing. Programming languages started to appear in the 1950s and this was also another major step in abstraction. Major languages such as Fortran, ALGOL, and COBOL were released in the late 1950s to deal with scientific, algorithmic, and business problems respectively. E.W. Dijkstra wrote his seminal paper, "Go To Statement Considered Harmful",[12] in 1968 and David Parnas introduced the key concept of modularity and information hiding in 1972[13] to help programmers deal with the ever increasing complexity of software systems. A software system for managing the hardware called an operating system was also introduced, most notably by Unix in 1969. In 1967, the Simula language introduced the object-oriented programming paradigm. These advances in software were met with more advances in computer hardware. In the mid 1970s, the microcomputer was introduced, making it economical for hobbyists to obtain a computer and write software for it. This in turn led to the now famous Personal Computer (PC) and Microsoft Windows. The Software Development Life Cycle or SDLC was also starting to appear as a consensus for centralized construction of software in the mid 1980s. The late 1970s and early 1980s saw the introduction of several new Simula-inspired object-oriented programming languages, including Smalltalk, Objective-C, and C++. Open-source software started to appear in the early 90s in the form of Linux and other software introducing the "bazaar" or decentralized style of constructing software.[14] Then the World Wide Web and the popularization of the Internet hit in the mid 90s, changing the engineering of software once again. Distributed systems gained sway as a way to design systems, and the Java programming language was introduced with its own virtual machine as another step in abstraction. Programmers collaborated and wrote the Agile Manifesto, which favored more lightweight processes to create cheaper and more timely software. The current definition of software engineering is still being debated by practitioners today as they struggle to come up with ways to produce software that is "cheaper, better, faster". Cost reduction has been a primary focus of the IT industry since the 1990s. Total cost of ownership represents the costs of more than just acquisition. It includes things like productivity impediments, upkeep efforts, and resources needed to support infrastructure. Service Oriented Architecture brings a unique tie of managing IT expenses in relation to value provided. Its main thesis is orienting the operation of IT infrastructure in accordance with how the business wants to operate. This is different than in the past, when many businesses were driven by what IT could provide rather than what the business needed. - [edit] Profession Main article: Software engineer Legal requirements for the licensing or certification of professional software engineers vary around the world. In the UK, the British Computer Society licenses software engineers and members of the society can also become Chartered Engineers (CEng), while in some areas of Canada, such as Alberta, Ontario,[15] and Quebec, software engineers can hold the Professional Engineer (P.Eng)designation and/or the Information Systems Professional (I.S.P.) designation; however, there is no legal requirement to have these qualifications. In Israel a person with an appropriate engineering degree has the right to be listed in Israel's Registry of Engineers and Architects[16], and Israeli engineering law[17] says that a person calling themselves an engineer without the proper license / registration could be sentenced to up to 6 months in jail. The IEEE Computer Society and the ACM, the two main professional organizations of software engineering, publish guides to the profession of software engineering. The IEEE's Guide to the Software Engineering Body of Knowledge - 2004 Version, or SWEBOK, defines the field and describes the knowledge the IEEE expects a practicing software engineer to have. The IEEE also promulgates a "Software Engineering Code of Ethics".[18] - [edit] Employment In 2004, the U. S. Bureau of Labor Statistics counted 760,840 software engineers holding jobs in the U.S.; in the same time period there were some 1.4 million practitioners employed in the U.S. in all other engineering disciplines combined.[19] Due to its relative newness as a field of study, formal education in software engineering is often taught as part of a computer science curriculum, and many software engineers hold computer science degrees.[20] Many software engineers work as employees or contractors. Software engineers work with businesses, government agencies (civilian or military), and non-profit organizations. Some software engineers work for themselves as freelancers. Some organizations have specialists to perform each of the tasks in the software development process. Other organizations require software engineers to do many or all of them. In large projects, people may specialize in only one role. In small projects, people may fill several or all roles at the same time. Specializations include: in industry (analysts, architects, developers, testers, technical support, middleware analysts, managers) and in academia (educators, researchers). Bill rates for US engagements from top-tier consulting firms can be as high as US$325 per hour.[21][Need quotation to verify] The individual consultant would see direct compensation of about 15-20% of this amount. A fresher entering into consulting as a profession may achieve 2010 compensation of US$50,000 for the year. Well-connected veterans in this field can monetize their experience and earn upwards of US$300,000 by exploiting multiple consulting engagements simultaneously and working remotely.[citation needed] - [edit] Certification The Software Engineering Institute offers certification on specific topics like Security, Process improvement and Software architecture[22]. Apple, IBM, Microsoft and other companies also sponsor their own certification examinations. Many IT certification programs are oriented toward specific technologies, and managed by the vendors of these technologies.[23] These certification programs are tailored to the institutions that would employ people who use these technologies. Broader certification of general software engineering skills is available through various professional societies. As of 2006[update], the IEEE had certified over 575 software professionals as a Certified Software Development Professional (CSDP).[24] In 2008 they added an entry-level certification known as the Certified Software Development Associate (CSDA).[25] In the U.K. the British Computer Society has developed a legally recognized professional certification called Chartered IT Professional (CITP), available to fully qualified Members (MBCS). In Canada the Canadian Information Processing Society has developed a legally recognized professional certification called Information Systems Professional (ISP)[26]. The ACM had a professional certification program in the early 1980s,[citation needed] which was discontinued due to lack of interest. The ACM examined the possibility of professional certification of software engineers in the late 1990s, but eventually decided that such certification was inappropriate for the professional industrial practice of software engineering.[27] - [edit] Impact of globalization The initial impact of outsourcing, and the relatively lower cost of international human resources in developing third world countries led to the dot com bubble burst of the 1990s. This had a negative impact on many aspects of the software engineering profession. For example, some students in the developed world avoid education related to software engineering because of the fear of offshore outsourcing (importing software products or services from other countries) and of being displaced by foreign visa workers.[28] Although statistics do not currently show a threat to software engineering itself; a related career, computer programming does appear to have been affected.[29][30] Nevertheless, the ability to smartly leverage offshore and near-shore resources in an efficient fashion has improved the overall operational capability of many organizations[31]. When Europeans are leaving work, Asians are just arriving to work. When Asians are leaving work, Europeans are arriving to work. This provides a continuous ability to have human oversight on business-critical processes 24 hours per day, without paying overtime compensation or disrupting key human resource sleep patterns. - [edit] Education A knowledge of programming is a pre-requisite to becoming a software engineer. In 2004 the IEEE Computer Society produced the SWEBOK, which has been published as ISO/IEC Technical Report 19759:2004, describing the body of knowledge that they believe should be mastered by a graduate software engineer with four years of experience.[32] Many software engineers enter the profession by obtaining a university degree or training at a vocational school. One standard international curriculum for undergraduate software engineering degrees was defined by the CCSE, and updated in 2004.[33] A number of universities have Software Engineering degree programs; as of 2010[update], there were 244 Campus programs, 70 Online programs, 230 Masters-level programs, 41 Doctorate-level programs, and 69 Certificate-level programs[34]. In addition to university education, many companies sponsor internships for students wishing to pursue careers in Information Technology. These internships can introduce the student to interesting real-world tasks that typical Software Engineers encounter every day. Similar experience can be gained through military service in software engineering. - [edit] Sub-disciplines Software engineering can be divided into ten subdisciplines. They are:[7] - Software requirements: The elicitation, analysis, specification, and validation of requirements for software. - Software design: The design of software is usually done with Computer-Aided Software Engineering (CASE) tools and use standards for the format, such as the Unified Modeling Language (UML). - Software development: The construction of software through the use of programming languages. - Software testing - Software maintenance: Software systems often have problems and need enhancements for a long time after they are first completed. This subfield deals with those problems. - Software configuration management: Since software systems are very complex, their configuration (such as versioning and source control) have to be managed in a standardized and structured method. - Software engineering management: The management of software systems borrows heavily from project management, but there are nuances encountered in software not seen in other management disciplines. - Software development process: The process of building software is hotly debated among practitioners; some of the better-known processes are the Waterfall Model, the Spiral Model, Iterative and Incremental Development, and Agile Development. - Software engineering tools, see Computer Aided Software Engineering - Software quality - [edit] Related disciplines Software engineering is related to the disciplines of computer science, management science, and systems engineering.[35][36] - [edit] Computer science Software engineering is considered an area of computer science by some academics.[citation needed] Many of the foundations of software engineering come from computer science. - [edit] Project management The building of a software system is usually considered a project and the management of it borrows many principles from the field of Project management. - [edit] Systems engineering Systems engineers have been dealing with the complexity of large systems for many decades and their knowledge is applied to many software engineering problems. - [edit] See also Main article: Outline of software engineering - Software Craftsmanship - List of basic software engineering topics - List of software engineering conferences - List of software engineering publications - List of software engineering topics - Bachelor of Software Engineering - Bachelor of Science in Information Technology; Software portal. Software Testing portal.. - [edit] References - ^ Peter, Naur; Brian Randell (7–11 October 1968). "Software engineering: Report of a conference sponsored by the NATO Science Committee" (PDF). Garmisch, Germany: Scientific Affairs Division, NATO. http://homepages.cs.ncl.ac.uk/brian.randell/NATO/nato1968.PDF. Retrieved 2008-12-26. - ^ Randell, Brian (10 August 2001). "The 1968/69 NATO Software Engineering Reports". Brian Randell's University Homepage. The School of the Computer Sciences, Newcastle University. http://homepages.cs.ncl.ac.uk/brian.randell/NATO/NATOReports/index.html. Retrieved 2008-10-11. "The idea for the first NATO Software Engineering Conference, and in particular that of adopting the then practically unknown term "software engineering" as its (deliberately provocative) title, I believe came originally from Professor Fritz Bauer." - ^ Tedre, Matti (2009). "Computing as Engineering". Journal of Universal Computer Science 15 (8). http://www.jucs.org/jucs_15_8/computing_as_engineering/jucs_15_08_1642_1658_tedre.pdf. - ^ Hey, Programmers, We Got No Theory! Dr. Dobbs Journal, March 22, 2010 (Retrieved March 26, 2010) - ^ Why We Need a Theory for Software Engineering, Ivar Jacobson and Ian Spence, Dr. Dobbs Journal, October 02, 2009 (Retrieved March 26, 2010) - ^ McConnell, Steve (January/February 1998). "The Art, Science, and Engineering of Software Development". IEEE Software 15 (1). http://www.stevemcconnell.com/ieeesoftware/bp13.htm. - ^ a b SWEBOK executive editors, Alain Abran, James W. Moore ; editors, Pierre Bourque, Robert Dupuis. (2004). Pierre Bourque and Robert Dupuis. ed. Guide to the Software Engineering Body of Knowledge - 2004 Version. IEEE Computer Society. pp. 1–1. ISBN 0-7695-2330-7. http://www.swebok.org. - ^ The end of software engineering and the start of economic-cooperative gaming - ^ 35 years on: to what extent has software engineering design achieved its goals? - ^ Kalwarski, Tara; Daphne Mosher, Janet Paskin and Donna Rosato (2006). "Best Jobs in America". MONEY Magazine. CNN. http://money.cnn.com/magazines/moneymag/bestjobs/2006/. Retrieved 2006-04-20. - ^ Leondes (2002). intelligent systems: technology and applications. CRC Press. ISBN 9780849311215. - ^ Dijkstra, E. W. (March 1968). "Go To Statement Considered Harmful". Communications of the ACM 11 (3): 147–148. doi:10.1145/362929.362947. http://www.cs.utexas.edu/users/EWD/ewd02xx/EWD215.PDF. Retrieved 2009-08-10. - ^ Parnas, David (December 1972). "On the Criteria To Be Used in Decomposing Systems into Modules". Communications of the ACM 15 (12): 1053–1058. doi:10.1145/361598.361623. http://www.acm.org/classics/may96/. Retrieved 2008-12-26. - ^ Raymond, Eric S. The Cathedral and the Bazaar. ed 3.0. 2000. - ^ Williams, N.S.W. (19–21 February 2001). "Professional Engineers Ontario's approach to licensing software engineering practitioners". Software Engineering Education and Training, 2001 Proceedings. 14th Conference on. Charlotte, NC: IEEE. pp. 77–78. - ^ [1] - ^ [2] - ^ Software Engineering Code of Ethics - ^ Bureau of Labor Statistics, U.S. Department of Labor, USDL 05-2145: Occupational Employment and Wages, November 2004, Table 1. - ^ "Software Engineering". http://computingcareers.acm.org/?page_id=12. Retrieved 2008-02-01. - ^ "Software Engineering Bill Rate". http://www.ibmemployee.com/Highlights100220.shtml. Retrieved 2010-08-30. - ^ SEI certification page - ^ Wyrostek, Warren (March 14, 2008). "The Top 10 Problems with IT Certification in 2008". InformIT. http://www.informit.com/articles/article.aspx?p=1180991. Retrieved 2009-03-03. - ^ IEEE Computer Society. "2006 IEEE computer society report to the IFIP General Assembly" (PDF). http://www.ifip.org/minutes/GA2006/Tab18b-US-IEEE.pdf. Retrieved 2007-04-10. - ^ IEEE. "CSDA". http://www.computer.org/portal/web/certification/csda. Retrieved 2010-04-20. - ^ Canadian Information Processing Society. "I.S.P. Designation". http://www.cips.ca/standards/isp. Retrieved 2007-03-15. - ^ ACM (July 17, 2000). "A Summary of the ACM Position on Software Engineering as a Licensed Engineering Profession". Association for Computing Machinery (ACM). http://www.cs.wm.edu/~coppit/csci690-spring2004/papers/selep_main.pdf. Retrieved 2009-03-03. "At its meeting in May 2000, the Council further concluded that the framework of a licensed professional engineer, originally developed for civil engineers, does not match the professional industrial practice of software engineering. Such licensing practices would give false assurances of competence even if the body of knowledge were mature; and would preclude many of the most qualified software engineers from becoming licensed." - ^ As outsourcing gathers steam, computer science interest wanes - ^ Computer Programmers - ^ Software developer growth slows in North America ¦ InfoWorld ¦ News ¦ 2007-03-13 ¦ By Robert Mullins, IDG News Service - ^ Gartner Magic Quadrant - ^ Abran, Alain, ed (2005) [2004]. "Chapter 1: Introduction to the Guide". Guide to the Software Engineering Body of Knowledge. Los Alamitos: IEEE Computer Society. ISBN 0769523307. http://www.computer.org/portal/web/swebok/html/ch1. Retrieved 2010-09-13. "The total volume of cited literature is intended to be suitable for mastery through the completion of an undergraduate education plus four years of experience." - ^ SE2004 Software Engineering Curriculum - ^ [3] Degree programs in Software Engineering - ^ Ian Sommerville (2004). Software Engineering. 7th edition. Chapter 1. Bezien 20 Okt 2008. - ^ Table 2 in Chapter 1,"Guide to the Software Engineering Body of Knowledge". February 6, 2004. http://www.swebok.org/swebokcontents-ch1.html#ch1. Retrieved 2008-02-21. - [edit] Further reading - Pressman, Roger S (2005). Software Engineering: A Practitioner's Approach (6th ed.). Boston, Mass: McGraw-Hill. ISBN 0072853182. - Sommerville, Ian (2007) [1982]. Software Engineering (8th ed.). Harlow, England: Pearson Education. ISBN 0-321-31379-8. http://www.pearsoned.co.uk/HigherEducation/Booksby/Sommerville/. - Jalote, Pankaj (2005) [1991]. An Integrated Approach to Software Engineering (3rd ed.). Springer. ISBN 0-387-20881-X. http://www.springer.com/east/home?SGWisbn=5-102-22-52090005-0&changeHeader=true. - Ghezzi, Carlo; Mehdi Jazayeri, Dino Mandrioli (2003) [1991]. Fundamentals of Software Engineering (2nd (International) ed.). Pearson Education @ Prentice-Hall. - [edit] External links Wikimedia Commons has media related to: Software engineering. - Computing Curricula 2005: The Overview Report by The Joint Task Force for Computing Curricula ACM/AIS/IEEE-CS - Curriculum Guidelines for Undergraduate Degree Programs in Software Engineering by The Joint Task Force on Computing Curricula ACM/IEEE-CS - Guidelines for Associate-Degree Transfer Curriculum in Software Engineering by The ACM Two-Year College Education Committee and The IEEE Computer Society/ACM Joint Task Force on Software Engineering - Guide to the Software Engineering Body of Knowledge - Computer Software Engineers - Definition and statistics from the U.S. Bureau of Labor Statistics - A Student's Guide to Software Engineering Projects - a free online guide for students taking SE project courses - System-on-a-chip From Wikipedia, the free encyclopedia (Redirected from System on a chip) Jump to: navigation, search This article needs additional citations for verification. Please help improve this article by adding reliable references. Unsourced material may be challenged and removed. (October 2009). AMD Geode is an x86 compatible system-on-a-chip System-on-a-chip or system on chip (SoC or SOC) refers to integrating all components of a computer or other electronic system into a single integrated circuit (chip). It may contain digital, analog, mixed-signal, and often radio-frequency functions – all on a single chip substrate. A typical application is in the area of embedded systems. The contrast with a microcontroller is one of degree. Microcontrollers typically have under 100K of RAM (often just a few KBytes) and often really are single-chip-systems; whereas the term SoC is typically used with more powerful processors, capable of running software such as Windows or Linux, which need external memory chips (flash, RAM) to be useful, and which are used with various external peripherals. In short, for larger systems System-on-a-chip is hyperbole, indicating technical direction more than reality: increasing chip integration to reduce manufacturing costs and to enable smaller systems. Many interesting systems are too complex to fit on just one chip built with a process optimized for just one of the system's tasks. When it is not feasible to construct an SoC for a particular application, an alternative is a system in package (SiP) comprising a number of chips in a single package. In large volumes, SoC is believed to be more cost effective than SiP since it increases the yield of the fabrication and because its packaging is simpler.[1] Another option, as seen for example in higher end cell phones and on the Beagle Board, is package on package stacking during board assembly. The SoC chip includes processors and numerous digital peripherals, and comes in a ball grid package with lower and upper connections. The lower balls connect to the board and various peripherals, with the upper balls in a ring holding the memory busses used to access NAND flash and DDR2 RAM. Memory packages could come from multiple vendors. - Contents [hide] - 1 Structure - 2 Design flow - 3 Fabrication - 4 See also - 5 Notes - 6 External links. - [edit] Structure A typical SoC consists of: - One microcontroller, microprocessor or DSP core(s). Some SoCs – called multiprocessor System-on-Chip (MPSoC) – include more than one processor core. - Memory blocks including a selection of ROM, RAM, EEPROM and flash. - Timing sources including oscillators and phase-locked loops. - Peripherals including counter-timers, real-time timers and power-on reset generators. - External interfaces including industry standards such as USB, FireWire, Ethernet, USART, SPI. - Analog interfaces including ADCs and DACs. - Voltage regulators and power management circuits. These blocks are connected by either a proprietary or industry-standard bus such as the AMBA bus from ARM. DMA controllers route data directly between external interfaces and memory, by-passing the processor core and thereby increasing the data throughput of the SoC. Microcontroller-based System-on-a-Chip - [edit] Design flow An SoC consists of both the hardware described above, and the software that controls the microcontroller, microprocessor or DSP cores, peripherals and interfaces. The design flow for an SoC aims to develop this hardware and software in parallel. Most SoCs are developed from pre-qualified hardware blocks for the hardware elements described above, together with the software drivers that control their operation. Of particular importance are the protocol stacks that drive industry-standard interfaces like USB. The hardware blocks are put together using CAD tools; the software modules are integrated using a software development environment. A key step in the design flow is emulation: the hardware is mapped onto an emulation platform based on a field programmable gate array (FPGA) that mimics the behavior of the SoC, and the software modules are loaded into the memory of the emulation platform. Once programmed, the emulation platform enables the hardware and software of the SoC to be tested and debugged at close to its full operational speed. (Emulation is generally preceded by extensive software simulation.[vague] In fact, sometimes the FPGAs are used primarily to speed up some parts of the simulation work.) After emulation the hardware of the SoC follows the place and route phase of the design of an integrated circuit before it is fabricated. Chips are verified for logical correctness before being sent to foundry. This process is called functional verification, and it accounts for a significant portion of the time and energy expended in the chip design life cycle (although the often quoted figure of 70% is probably an exaggeration).[2] Verilog and VHDL are typical hardware description languages used for verification. With the growing complexity of chips, hardware verification languages like SystemVerilog, SystemC, e, and OpenVera are also being used. Bugs found in the verification stage are reported to the designer. System-on-a-Chip Design Flow ... - [edit] Fabrication SoCs can be fabricated by several technologies, including: - Full custom - Standard cell - FPGA SoC designs usually consume less power and have a lower cost and higher reliability than the multi-chip systems that they replace. And with fewer packages in the system, assembly costs are reduced as well. However, like most VLSI designs, the total cost is higher for one large chip than for the same functionality distributed over several smaller chips, because of lower yields and higher NRE costs. - [edit] See also - List of system-on-a-chip suppliers - PSoC - ASIC - Microcontroller - Electronic design automation - Post silicon validation - System in package - Network On Chip - [edit] Notes - ^ "The Great Debate: SOC vs. SIP". Eetimes.com. http://www.eetimes.com/showArticle.jhtml;?articleID=159901628. Retrieved 2009-08-12. - ^ "Is verification really 70 percent?". Eetimes.com. http://www.eetimes.com/showArticle.jhtml?articleID=21700028. Retrieved 2009-08-12. - [edit] External links - SOCC Annual IEEE International SOC Conference [hide] v • d • e CPU technologies.. - Computer-on-module From Wikipedia, the free encyclopedia (Redirected from System on module) Jump to: navigation, search This article does not cite any references or sources. Please help improve this article by adding citations to reliable sources. Unsourced material may be challenged and removed. (July 2010) Find sources: "Computer-on-module" – news · books · scholar · images. A computer-on-module (COM) is a type of single-board computer (SBC), a subtype of an embedded computer system. It is also called System on Module (SOM) as an extension of the concept of System on Chip (SoC) and lying between a full-up computer and a microcontroller in nature. Today's COM/SOM modules are complete computers built on a single circuit board. The design is centered on a single microprocessor with RAM, input/output controllers and all other features needed to be a functional computer on the one board. However, unlike a single-board computer, the COM module will usually lack the standard connectors for any input/output peripherals to be attached directly to the board. Instead, the wiring for these peripherals are bussed out to connectors on the board. The module will usually need to be mounted on a carrier board (or "baseboard") which breaks the bus out to standard peripheral connectors. Some COMs also include peripheral connectors and/or can be used without a carrier. A COM/SOM solution offers a dense package computer system for use in small or specialized applications requiring low power consumption or small physical size as is needed in embedded systems. Some devices also incorporate Field Programmable Gate Arrays. The terms "Computer-on-Module" and "COM" were coined by VDC Research Group, Inc. (formerly Venture Development Corporation) (Natick, MA, USA) to describe this class of embedded computer boards. The term became more notable upon industry standardization of the COM Express format. Gumstix Overo COM, a tiny, OMAP-based embedded computer-on-module with Wifi and Bluetooth. A number of manufacturers offer products which are defined as system-on-module or computer-on-module. Among those are - Advanced Knowledge Associates (www.a-k-a.net) in Santa Clara, CA - BCM Advanced Research (www.bcmcom.com) in Irvine, CA - Embedded Systems (www.embedded-systems.net) - Gumstix, Inc. (www.gumstix.com) in San Jose, CA - KaRo Electronics GmbH (www.karo-electronics.de) in Germany - Logic Product Development (www.logicpd.com) in Minneapolis, MN - MEN Micro (www.menmicro.com) in Ambler, PA - DAVE Embedded Systems (www.dave.eu) in Italy - Kontron which is located in Eching, Germany - [edit] See also - Computer-on-module types - ETX - XTX - CoreExpress - COM Express - ESM - ESMexpress - ESMini - Qseven - SOM - Computer - Embedded system - Microcontroller - Embedded microprocessor - Accelerated System Design using FPGA Based System- - Ubiquitous computing From Wikipedia, the free encyclopedia Jump to: navigation, search Ubiquitous computing (ubicomp) is a post-desktop model of human-computer interaction in which information processing has been thoroughly integrated into everyday objects and activities. In the course of ordinary activities, someone "using" ubiquitous computing engages many computational devices and systems simultaneously, and may not necessarily even be aware that they are doing so. This model is usually considered an advancement from the desktop paradigm. This paradigm is also described as pervasive computing, ambient intelligence.[1], where each term emphasizes slightly different aspects. When primarily concerning the objects involved, it is also physical computing, the Internet of Things, haptic computing,[2] and things that think. Rather than propose a single definition for ubiquitous computing and for these related terms, a taxonomy of properties for ubiquitous computing has been proposed, from which different kinds or flavors of ubiquitous systems and applications can be described.[3] - Contents [hide] - 1 Core concept - 2 History - 3 Examples - 4 Current research - 5 See also - 6 References - 7 Resources and other external links. - [edit] Core concept At their core, all models of ubiquitous computing (also called pervasive computing) share a vision of small, inexpensive, robust networked processing devices, distributed at all scales throughout everyday life and generally turned to distinctly common-place ends. For example, a domestic ubiquitous computing environment might interconnect lighting and environmental controls with personal biometric monitors woven into clothing so that illumination and heating conditions in a room might be modulated, continuously and imperceptibly. Another common scenario posits refrigerators "aware" of their suitably-tagged contents, able to both plan a variety of menus from the food actually on hand, and warn users of stale or spoiled food. Ubiquitous computing presents challenges across computer science: in systems design and engineering, in systems modelling, and in user interface design. Contemporary human-computer interaction models, whether command-line, menu-driven, or GUI-based, are inappropriate and inadequate to the ubiquitous case. This suggests that the "natural" interaction paradigm appropriate to a fully robust ubiquitous computing has yet to emerge - although there is also recognition in the field that in many ways we are already living in an ubicomp world. Contemporary devices that lend some support to this latter idea include mobile phones, digital audio players, radio-frequency identification tags, GPS, and interactive whiteboards. Mark Weiser proposed three basic forms for ubiquitous system devices, see also Smart device: tabs, pads and boards. - Tabs: wearable centimetre sized devices - Pads: hand-held decimetre-sized devices - Boards: metre sized interactive display devices. These three forms proposed by Weiser are characterized by being macro-sized, having a planar form and on incorporating visual output displays. If we relax each of these three characteristics we can expand this range into a much more diverse and potentially more useful range of Ubiquitous Computing devices. Hence, three additional forms for ubiquitous systems have been proposed: [3] - Dust: miniaturized devices can be without visual output displays, e.g., Micro Electro-Mechanical Systems (MEMS), ranging from nanometres through micrometers to millimetres. See also Smart dust. - Skin: fabrics based upon light emitting and conductive polymers, organic computer devices, can be formed into more flexible non-planar display surfaces and products such as clothes and curtains, see OLED display. MEMS device can also be painted onto various surfaces so that a variety of physical world structures can act as networked surfaces of MEMS. - Clay: ensembles of MEMS can be formed into arbitrary three dimensional shapes as artefacts resembling many different kinds of physical object (see also Tangible interface). In his book The Rise of the Network Society, Manuel Castells suggests that there is an ongoing shift from already-decentralised, stand-alone microcomputers and mainframes towards entirely pervasive computing. In his model of a pervasive computing system, Castells uses the example of the Internet as the start of a pervasive computing system. The logical progression from that paradigm is a system where that networking logic becomes applicable in every realm of daily activity, in every location and every context. Castells envisages a system where billions of miniature, ubiquitous inter-communication devices will be spread worldwide, "like pigment in the wall paint". - [edit] History Mark Weiser coined the phrase "ubiquitous computing" around 1988, during his tenure as Chief Technologist of the Xerox Palo Alto Research Center (PARC). Both alone and with PARC Director and Chief Scientist John Seely Brown, Weiser wrote some of the earliest papers on the subject, largely defining it and sketching out its major concerns.[4][5][6] Recognizing that the extension of processing power into everyday scenarios would necessitate understandings of social, cultural and psychological phenomena beyond its proper ambit, Weiser was influenced by many fields outside computer science, including "philosophy, phenomenology, anthropology, psychology, post-Modernism, sociology of science and feminist criticism." He was explicit about "the humanistic origins of the ‘invisible ideal in post-modernist thought'",[6] referencing as well the ironically dystopian Philip K. Dick novel Ubik. MIT has also contributed significant research in this field, notably Hiroshi Ishii's Things That Think consortium at the Media Lab[7] and the CSAIL effort known as Project Oxygen.[8] Other major contributors include Georgia Tech's College of Computing, NYU's Interactive Telecommunications Program, UC Irvine's Department of Informatics, Microsoft Research, Intel Research and Equator,[9] Ajou University UCRi & CUS.[10] - [edit] Examples One of the earliest ubiquitous systems was artist Natalie Jeremijenko's "Live Wire", also known as "Dangling String," installed at Xerox PARC during Mark Weiser's time there. This was a piece of string attached to a stepper motor and controlled by a LAN connection; network activity caused the string to twitch, yielding a peripherally noticeable indication of traffic. Weiser called this an example of calm technology.[11] Ambient Devices has produced an "orb", a "dashboard", and a "weather beacon": these decorative devices receive data from a wireless network and report current events, such as stock prices and the weather, like the Nabaztag produced by Violet. - [edit] Current research Main article: List of ubiquitous computing research centers Ubiquitous computing touches on a wide range of research topics, including distributed computing, mobile computing, sensor networks, human-computer interaction, and artificial intelligence. - [edit] See also - Ambient intelligence - Context-aware pervasive systems - Human-centered computing - Human-computer interaction - List of Ubicomp Researchers - Pervasive adaptation - Sentient computing - Smart device - Ubiquitous learning - Virtual reality - Wearable computer - Task computing - [edit] References - ^ Hansmann, Uwe (2003). Pervasive Computing: The Mobile World. Springer. ISBN 3540002189. - ^ "World Haptics Conferences". Haptics Technical Committee. http://www.worldhaptics.org/hapticConferences.htm. Retrieved 2007-10-13. - ^ a b Poslad, Stefan (2009). Ubiquitous Computing Smart Devices, Smart Environments and Smart Interaction. Wiley. ISBN 978-0-470-03560-3. http://www.elec.qmul.ac.uk/people/stefan/ubicom/index.html. - ^ Weiser, Mark (1991). "The Computer for the 21st Century". http://www.ubiq.com/hypertext/weiser/SciAmDraft3.html. Retrieved 2008-05-07. - ^ Weiser; Gold; Brown (1999-05-11). "Ubiquitous computing". http://www.research.ibm.com/journal/sj/384/weiser.html. Retrieved 2008-05-07. - ^ a b Weiser, Mark (1996-03-17). "Ubiquitous computing". http://www.ubiq.com/hypertext/weiser/UbiHome.html. Retrieved 2007-11-03. - ^ "MIT Media Lab - Things That Think Consortium". MIT. http://ttt.media.mit.edu. Retrieved 2007-11-03. - ^ "MIT Project Oxygen: Overview". MIT. http://oxygen.csail.mit.edu/Overview.html. Retrieved 2007-11-03. - ^ "Equator". UCL. http://www.cs.ucl.ac.uk/research/equator/. Retrieved 2009-11-19. - ^ "Center_of_excellence_for_Ubiquitous_System". CUS. http://www.cuslab.com. Retrieved 2008-05-04. - ^ Weiser, Mark; Rich Gold and John Seely Brown (1999). ""The origins of ubiquitous computing research at PARC in the late 1980s"". IBM systems journal 38 (4): 693. doi:10.1147/sj.384.0693. - [edit] Resources and other external links An introduction to the field appropriate for general audiences is Adam Greenfield's book Everyware: The Dawning Age of Ubiquitous Computing (ISBN 0-321-38401-6). Greenfield describes the interaction paradigm of ubiquitous computing as "information processing dissolving in behavior." Notable conferences in the field include: - International Conference on Ubiquitous Computing (Ubicomp) - IEEE International Conference on Pervasive Computing and Communications (Percom) - IEEE International Conference on Pervasive Services (ICPS) - IEEE GlobeCom Workshop on Service Discovery and Composition in Ubiquitous and Pervasive Environments (SUPE) - The Second International Conference on Mobile Ubiquitous Computing, Systems, Services and Technologies (UBICOMM 2008) Academic journals and magazines devoted primarily to pervasive computing: - Pervasive Computing (IEEE) - Personal and Ubiquitous Computing (Springer) - Pervasive and Mobile Computing journal, PMC (Elsevier) - Ubiquitous Computing and Communication Journal - UbiCC Journal Mark Weiser's original material dating from his tenure at Xerox PARC: - Ubiquitous Computing Other links: - Context and Adaptivity in Pervasive Computing Environments: Links with Software Engineering and Ontological Engineering, article in Journal of Software, Vol 4, No 9 (2009), 992-1013, Nov 2009 by Ahmet Soylu, Patrick De Causmaecker and Piet Desmet - Yesterday’s tomorrows: notes on ubiquitous computing’s dominant vision, by Genevieve Bell & Paul Dourish. - Towards pervasive computing in health care – A literature review, article in BMC Medical Informatics and Decision Making (Open Access journal) by Carsten Orwat, Andreas Graefe and Timm Faulwasser. - PerAda is the Pervasive Adaptation Research Network PerAda is the Coordination Action for the Pervasive Adaptation community, supporting the funded projects under FP7: FET Proactive Initiative on Pervasive Adaptation.. Others; Single-board computer · Wireless sensor network · Microcontroller · Smartdust · Nanocomputer · Pizza Box Case.

Retrieved from "http://en.wikipedia.org/wiki/Microcontroller"