From Wikipedia, the free encyclopedia

Jump to: navigation, search

|  | This article does not cite any references or sources. Please help improve this article by adding citations to reliable sources. Unsourced material may be challenged and removed. (October 2009) |
| --- | --- |

Cisco Systems Gigabit Switch Router.

A multilayer switch (MLS) is a computer networking device that switches on OSI layer 2 like an ordinary network switch and provides extra functions on higher OSI layers.

- Contents [hide] - 1 Layer 3 Switching - 2 MultiLayer Switch (MLS) - OSI layer 3 and/or 4 - 3 Layer 4-7 switch, web-switch, content-switch - 4 Layer 4 Load Balancer - 5 See also - 6 External links

- [edit] Layer 3 Switching

The major difference between the packet switching operation of a router and that of a Layer 3 switch is the physical implementation. In general-purpose routers, packet switching takes place using software which runs on a microprocessor, whereas a Layer 3 switch performs this using dedicated application-specific integrated circuit (ASIC) hardware.[citation needed]

- [edit] MultiLayer Switch (MLS) - OSI layer 3 and/or 4

A Multilayer Switch (MLS) can prioritize packets by the 8 bits in IP DSCP (differentiated services CodePoint). These 8 bits were used in the "old days" for Type of Service (ToS).

The following 4 mappings are normally available in an MLS:

- From OSI layer 2, 3 or 4 to IP DSCP (if IP packet) and/or VLAN IEEE 802.1p.

- From VLAN IEEE 802.1p to IP DSCP.

- From IP DSCP to VLAN IEEE 802.1p.

- From VLAN IEEE 802.1p to port queue.

Many MLSs implement QoS differentiated services and/or integrated services in hardware.

Some MLS's are also able to route between VLANs and/or ports like a common router. The routing is normally as quick as switching (at wirespeed). According to Cisco, Level 3 switches are basically routers that switch based on Layer 3 information, the basic difference being processing speed and/or the way they do the switching; Level 3 switches use ASICs/hardware instead of the CPU/software that a router would.[citation needed]

- [edit] Layer 4-7 switch, web-switch, content-switch

Some switches can use up to OSI layer 7 packet information; these may be called layer 4-7 switches, content-switches, content services switches, web-switches or application-switches.

Content switches are typically used for load balancing among groups of servers. Load balancing can be performed on HTTP, HTTPS, VPN, or any TCP/IP traffic using a specific port. Load balancing often involves NAT so that the client of the load balanced service is not fully aware of precisely which server is handling its requests. Some of the layer 4-7 switches can perform NAT at wirespeed. Also, content switches can often be used to perform standard operations such as SSL encryption/decryption to reduce the load on the servers receiving the traffic, or to centralise the management of digital certificates.

Some types of application require that repeated requests from a client are directed at the same application server. Since the client isn't generally aware of which server it spoke to earlier, content switches define a notion of stickiness. For example, requests from the same source IP address are directed to the same application server each time. Stickiness can also be based on SSL IDs, and some content switches can even use cookies to provide this functionality.

- [edit] Layer 4 Load Balancer

A typical network router simply sends incoming packets onto the appropriate IP address on its network. A layer 4 router, more correctly a NAT with port and transaction awareness, uses a little trickery and sends incoming packets to one or more machines which are hidden behind a single IP address.

The layer 4 refers to the 7 layer OSI model. The router is on the Transport Layer and makes decisions on where to send the packets. Modern load balancing routers can use different rules to make decisions on where to route traffic. This can be based on least load, or fastest response times, or simply balancing requests out. This is also a redundancy method, so if one machine is not up, the router will not send traffic to it.

- [edit] See also

- Multiprotocol Label Switching (MPLS)

- LAN switching

- Network switch

- Application delivery controller

- Residential gateway

# Building Cisco Multilayer Switched Networks

From Wikipedia, the free encyclopedia

Jump to: navigation, search

|  | This article is an orphan, as few or no other articles link to it. Please introduce links to this page from related articles; suggestions may be available. (February 2009) |
| --- | --- |

BCMSN stands for Building Cisco Multilayer Switched Networks. The latest version is BCMSN v3.0. CCNP training focuses on building Enterprise level switched networks as well as VoIP and Wireless networks and applications. This is not a certification but a course taken in conjunction with the CCDP certification or CCNP certification. The course consists of 40 hours or 5 days of web based training plus five days of instructor led training and five days of follow up remote labs. Prerequisites include CCNA and ICND2. Candidates who take this course must pass the Exam 642-812 BCMSN to validate their BCMSN training.

Retrieved from "http://en.wikipedia.org/wiki/Building_Cisco_Multilayer_Switched_Networks"