- BACKGROUND:

Our company was hired to figure out if this client’s existing cooling tower was big enough to support their planned expansion. When we design a new plant that requires the use of cooling towers, we normally have access to the information of the process equipment that the cooling tower water system will be servicing, so a quick energy balance is developed to predict the size of the cooling tower.

The plant’s location, elevation, and seasonal demand are critical elements to consider when selecting the size and type of cooling tower to specify and quote. It is customary to slightly overestimate the size of cooling tower needed, to allow for future equipment additions that may have been overlooked during the original evaluation.

- THE PROBLEM:

On the other hand, when an established facility decides it’s time to expand, it’s much more difficult to define whether their current cooling tower will support the expansion. This is a challenging effort because our first task is to investigate how the system was modified over the years, and then measure how the cooling tower is performing under the current conditions. This helps us decide if we can add heat load, or if an additional cooling tower will be needed.

- THE SOLUTION:

We began with a quick survey of the cooling tower P&IDs to document all the existing pieces of process equipment that were being serviced. Needless to say, many pieces of equipment were added over the years, but never recorded on the P&IDs.

None of the main headers or branches had flow meters, so predicting the flows was difficult, and we had to find a way to measure and calculate the flows for each branch. Some of the existing temperature gauges were providing inaccurate temperatures, so we had to establish a protocol to take temperature readings for the overall system.

The cooling tower water pump curves, plus amperage readings gave us an idea of the overall “supply” flow rate. However, to figure out the current heat load, we needed to know flows and temperatures for all the “return” branches.

Once the current heat load was defined, we proceeded to add the loads for the new equipment. We performed analysis for summer and winter conditions, to make sure that the existing cooling tower was big enough or if additional capacity was needed.

Our conclusion was that the immediate additional heat load resulted in a 25% increment, so the existing cooling tower, pumps, and piping were unable to handle the incremental load, and a new cooling tower had to be added. When we sized the new cooling tower, pumps, and cooling tower water headers, we had to include the heat load for the potential expansion in the next five years.

## Background:

We received a call from a client who wanted to make energy and efficiency improvements to an existing ethanol facility. The existing facility was designed using a technology package that was no longer available. As with all facilities, this plant was built on a budget. In order to save on capital expenses, a minimal energy recovery plan was adopted to reduce equipment in the plant.

## The Problem:

As the economics of operating an ethanol facility are in flux with the price of corn, ethanol, and natural gas, it became evident that the plant was going to struggle to maintainin the coming years.

The first challenge for the facility was to lower its total energy costs. This was not as simple as adding more surface area to heat exchangers, as there were several systems that were already interconnected. This meant changing one system would affect other systems as well.

The second challenge was to improve operational reliability. Several of the decisions made to reduce capital had an adverse effect on operations. There was a single mash train that had to be shut down for cleaning, interrupting the fermenter fill sequence and beer feed heat recovery. The evaporator operated on whole stillage, causing increased fouling. The beer column feed heater frequently fouled, and the beer column was pressurized when it should have been operating under vacuum.

## The Solution:

Ethanol facilities are a classic example of pinch analysis, and there are several operating facilities using successful pinch models. The challenge for this particular facility was integrating a successful pinch model where the equipment was not originally designed for it. Our initial approach utilized plant operating data to develop a detailed mass and energy balance.

Working with plant personnel, we developed a concept energy recovery model that also addressed many of the fouling and operational issues the plant was having. The mass balance was modified to reflect these changes, determine energy savings, and a return on investment for the project.

### Mash Trains Solution

We replaced the single mash train with narrow gap plate heat exchangers with a double mash train. One train operated, while the other train was cleaned, or ready in “stand-by” mode. The new exchangers included a wide-gap plate design with more surface area that accommodated a 100F increase in beer column feed, while reducing tower water usage on fermenter fill. Finally, we added a booster pump to control the system pressure.

### Distillation Solution

With the higher temperature beer feed coming out of the mash train, CO2 degassing became more important. We installed a new flash tank, vent cooler, and blower to assist in the degassing process.

The old system did not recover any heat from the beer column bottoms, so we added new heat exchangers to fix this. The new heat exchangers made a significant reduction in steam usage at the beer column, while reducing vapor flashing in the decanter centrifuges.

In order to reduce fouling, we added a recycle pump was added at the beer feed S-T heat exchanger.

### Molecular Sieves and Evaporation Solution

The rectifier overhead vapors were split between the molecular sieves and the evaporator, where the ethanol vapor was used as the heat source to drive the evaporator. The evaporator was fed whole stillage instead of thin stillage, which increased fouling, limited the amount of evaporation capacity, and increased the drying load in the DDGS dryer.

The obvious answer was to convert the evaporator feed from whole stillage to thin stillage, but we weren’t sure how it could be done without increasing the total plant steam consumption. On the surface, it seemed counterintuitive to condense the ethanol vapor in the evaporator, and then reheat it on the way to the molecular sieves. This is another area that would not be obvious in a pinch analysis.

With some detailed mathematical modelling around the evaporator, it made sense to convert the evaporator to thin stillage, and condense all the ethanol vapor on the evaporator’s first effect to provide more evaporation. We then installed a stream-driven vaporizer, which “de-coupled” the molecular sieves. This works because the evaporator has three effects, so there is about 2.5 times more evaporation for every pound of ethanol vapor that is condensed.

In order to increase evaporation even more, a thin stillage re-boiler was later added to the system. This increased the syrup solids and decreased the dryer load.

## Results:

The end result of these modifications was a 12.5% reduction in steam and a 20% reduction in natural gas usage. The plant was able to increase annual production by reducing downtime for cleaning and running for longer periods at steady-state. This facility is profitable today because of the energy improvements we helped implement.

- Background:

We received a phone call from a customer that was planning to start up a Solvent Extraction facility designed to process wheat germ.

The facility was supposed to extract wheat germ oil and wheat germ meal from wheat germ, both very valuable products—if the user’s specifications are met. The original start up team spent several weeks trying to make the equipment work, but after running out of money, they simply walked out.

That’s when the customer called us. We knew we’d have to determine what went wrong with the original team to figure out why they quit, what we could learn from their mistakes, and how we could solve the customer’s needs.

- The Problem:

While the facility was a new installation, the equipment had previously been used to extract coffee, so switching to extracting wheat germ oil had its challenges, and major consequences.

- The first challenge was to define the capacity that the Extractor could deliver for wheat germ

- The second challenge was to figure out how to remove the fines that were liberated once the oil was removed, as they were plugging the Extractor screens, plugging the Evaporator, and plugging the pipes

- The third challenge was to figure out how to operate the Desolventizer Toaster, as it was producing finished meal that turned out very dark in color

- Last, but not least, the fourth challenge was to fix the Mineral Oil system to keep the solvent losses under control

- The Solution:

Upon arrival on the site, we performed a detailed survey of the facility to try to understand the best approach to address all the challenges.

The Solvent Extractor was a shallow bed type, so we ran calculations of the volume of the baskets in order to estimate the tonnage of material that the Extractor could handle.

Most extractors are equipped with hydroclones to remove the fines. However, the amount of fines generated by the wheat germ were extremely high, so we had to add a filtration system capable to handle the fines removal.

In a properly designed facility, the Desolventizer/Toaster is a system that is relatively easy to operate.  However, this Desolventizer/Toaster was too big for the capacity that the facility could handle. We spent a lot of time calculating heating surfaces and testing various temperatures on the steam decks, until we found the magic recipe to desolventize, dry, and toast the meal just enough to produce a “very light” color. This was crucial, as the end user needed to use it as an ingredient in a cookie dough, and bake it to produce an acceptable and attractive cookie.

Finally, we focused our attention on the mineral oil system, which is the last barrier to recover hexane that ends at the evaporator condenser vents. The Mineral Oil system is easy to operate, but we had to leave it for the end, until the rest of the systems were working at a stable state.

The key was to establish the proper operating parameters, such as oil flow, temperature of the stripper, temperature of the absorber, and the amount of sparge steam to be added. A few hours of tweaking and adjusting the parameters, and the job was completed.

Topics: Case Studies