- RESIDENTIAL STRUCTURAL DESIGN

- Part 1—Vertical Structural Forces

Before moving on to designing floor plans, we'll need to learn some residential structural design basics. The indoor space bubble diagrams you have already created will eventually turn into floor plans but first you need to consider what structural elements will need to be created within those spaces in order to support the roof, floors and the forces acting on and within the house.

This online tutorial is best followed in order. If you are just starting out with your house design, see our tutorial site map to go through the tutorial in order, or to navigate it as it suits you.

- Residential Structural Framing Primer

First let's talk about the types of loads on a house and how those loads can be supported.

The picture below shows vertical forces acting on a house roof and those forces being transferred down the roof and then to the ground by the structural walls.

The vertical forces acting on a residential structure include both live loads and dead loads. Live loads are considered anything that can move or shift, such as people, furniture and snow loading. Dead loads are forces that do not move. Dead loads include the weight of the roof, floors and built-in furniture.

Let's back up a bit and discuss how loads are transferred along a simple wood beam or joist. A wood member will always be strongest when oriented so that its widest dimension runs parallel to the majority of its loading forces. Look at the images below.

In the first image, the wood beam is loaded so that the thinnest dimension is parallel to the loading force. Note that this beam deflects noticeably.

In the second image the wood beam is loaded so that the widest dimension is parallel to the loading force. Note that this beam does not deflect as much.

Let's go back to the house example to see how the vertical forces are transferred through the structure to the ground. In the most simple case, a house could consist of four walls and a flat roof. The image below shows a cross-section of such a house. (A cross-section view shows what you would see if you took a vertical slice of the house as if you had sliced it like a loaf of bread.) Here we are looking at two of the outside walls and the flat roof.

In this image, vertical forces applied to the flat roof are applied to the flat roof which are in turn supported by the vertical support (load bearing) walls at which they can travel downwards. At these walls the forces travel down to the ground.

A house is supported entirely by vertical and horizontal systems (and sloped systems such as sloped roofs). Loading forces are supported by either columns (posts) or load bearing walls. Floors take loading forces (such as the weight of the building, people and furniture) and transfer them to these columns or walls. Likewise the forces on the roof are transferred to columns and/or walls.

Now let's take a look at the different ways to support a house like the one shown in the example above.

As mentioned earlier, the house can be supported by load bearing walls, columns (posts) and beams or by light structural elements (such a wood or steel) which is really a combination of the two other support mechanisms. Let's illustrate this with pictures. In the first image below, the vertical forces of the house are supported entirely by load-bearing structural walls. In the type of residential structural design below, we have used concrete walls. There are openings in the concrete for doorways and windows but these openings have been calculated so that the walls will be strong enough to support the floor above and the roof.

In our next example, the house is supported by columns and beams. There are many ways for these columns and beams to attach or join to one another. For now, we are simply looking at how the loading forces are transferred. The picture below shows how the loading forces are transferred in a house supported by posts and beams. Keep in mind these posts and beams can be made out of many materials including wood, concrete and steel.

Finally, let's consider structural framing with light structural elements. By this we could mean a traditionally wood-framed house or a light steel framed house. This style of residential structural design can be considered a combination of both load bearing walls and post and beam construction since, as a whole, the framed exterior wall is a load bearing wall but all the elements within in are acting as posts and beams. Look at the picture below.

Let's take a look at just a single wall of this style of residential framing and look at the force transfer.

- Designing with Structure in Mind

So you know now that your house can be supported in a few different ways—with structural walls, posts and beams or light structural elements such as wood or steel framing. At this point, it is important for you to consider the type of house you are planning and the type of construction most suited to that style.

The most common house construction method in North America is wood framing. Many types of homes can be built in this way. If your home will be a timber frame, adobe or straw bale home, you will most likely support it with posts and beams.

A home can also be made entirely of structural walls such as poured concrete, concrete block or concrete in Insulated Concrete Forms (ICFs).

- Next Tutorial Section—Designing with Wood Joist Span Tables

# DESIGNING WITH FLOOR JOIST SPAN TABLES

## Part 2 of Residential Structural Design

On this page we will explain how to design with floor joist span tables. At the bottom of this page you'll find a joist span calculator. The content on this page will explain how to interpret the results from the calculator.

If you are just starting out, you might want to start at our previous page, residential structural design page which explains basic house structure.

See our Design Your Own House tutorial site map to go through the tutorial in order, or to jump around it as it suits you.

## Using Floor Joist Span Tables

Continuing on from Part 1: Residential Structural Design Vertical Structural Forces, we were about to start learning about floor joist span tables.

Don't worry, you won't need to do a lot of calculations in determining the size and placement of the structural framing within your house design. You will however, need to become familiar with reading your local floor joist span tables for floor beams, floor joists, window and door lintels, ceiling joists, roof joists, roof rafters, and roof ridge beams, as well as the sizing and spacing of wood studs tables.

Before jumping into the span tables, let's first consider wood species and wood grades. In general, all lumber is stamped near the end of the lumber piece by the manufacturer.

The stamp displays several types of information. The information displayed varies between wood associations. But most will display:

- The wood species - often abbreviated

- The wood grade - could be a number or description

- The wood association of which the lumber manufacturer is a member

- How the wood was seasoned - this will give an indication of the moisture content

- The mill identification—indicated by name or mill number

### Wood Species

The stamp above indicates in the triangle that the wood is Douglas Fir (D FIR). Other common designations are:

- S-P-F (or Spruce – Pine – Fir)

- Hem-Fir(N) (or Hemlock – Fir)

- D.Fir-L(N) (or Douglas Fir – Larch)

- N. Species (red cedar, certain pines, balsam and poplars)

### Lumber Grade

The stamp shown above indicates the lumber grade is standard or better (STAND & BTR) this is the same as number 3 or stud. Common lumber grade designations include:

- SEL STR (Select Structural)

- No. 1, No. 2

- No. 3 / Stud / Stand & Btr. (or Standard & Better)

These designations and numbers will become important when you are reading your structural tables since the strength of a wood depends on its designations.

Now we'll look at how to read these tables. Let's start with a simple, very small house 12 feet wide by 13 feet long. The house will have a gable roof and will be framed with wood studs.

The table below shows the maximum span for a floor joist with the following designations:

- Douglas Fir-Larch (lumber species)

- No. 1 and No. 2 (lumber grade)

- All joists are bridged

### Sample Floor Joist Span Table

The table excerpt above is simply a sample and may not be valid for your region.

Looking at this table you will see there is a choice in the size of floor joist (2 X 6, 2 X 8, 2 X 10 or 2 X 12) and there is a choice in the joist spacing (12", 16" or 14"). The floor joist spacing is the distance between the centers of any two installed joists. See the image below for an example of joists spaced 16" on center (16" o.c.).

Since the example house we are designing for is 12 feet wide, we need to find, in the floor joist span table, a joist size and centering that can span 12' or wider. Reading the table, you'll see that 2 X 8s installed 16" apart on center can span 12'4".

So our house would look like the picture below (with all floor joists 2" X 8"). The picture below shows a plan view, that means looking straight down from above the house. The outer gray walls are the concrete foundation walls which support the floor joists. The narrow double lines show the floor joist themselves.

From a three-dimensional perspective, the floor joist framing would look like this:

Keep in mind the span table above shows just the allowable spans for Douglas fir or larch, there are different tables for all the construction lumber species.

## Next Tutorial Section

Now, let's make the house wider so that it is necessary to have a center support for the floor joists. To learn about designing with beam span tables, continue on with the next part of Residential Structural Design:

Part 3: Designing with Wood Beam Span Tables.

## Floor Joist Span Calculator

To use the joist span calculator below, first select the species of lumber you will use for your construction project from the drop-down list.

Next use the buttons in the table to select the maximum length in feet (or metres in brackets) that your floor joists must span.

The values displayed in the resulting table will show three possible joist dimensions and spacings depending on the restraining method used.

Top of Form

Bottom of Form

| Residential Floor Joists Span Calculator |  | Spruce-Pine-Fir |  |
| --- | --- | --- | --- |
| Table valid for No. 1 and No.2 Grade of Spruce-Pine-Fir Includes all spruce species except coast sitka spruce, jack pine, lodgepole pine, balsam fir and alpine fir |  |  |  |
| Max. Span, ft.-in. (metres) Use Buttons Below to Select Span | Joist Dimensions, inches x inches (mm x mm) | Joist Spacing, in. (mm) | Restraining Method |

Data taken from CMHC Wood-Frame House Construction Tables
Notes:
1. This table is for residential construction only and is meant only for the purposes of initial estimation. You must consult your local floor joist span tables to ensure that the joist sizing is correct for your area.
2. Maximum spans are shown in 6 inch (150mm) increments. For more precise spans you will need to consult span tables.

Return to Top of Page and Navigation Bar